***Sub Engineer***

1. In case of Raymond pile

a. diameter of top of piles varies from 40 cm to 60 cm

b. diameter of pile at bottom varies from 20 cm to 28 cm

c. thickness of outer shell depends upon pile diameter

d. all the above.

Explanation: Option D is correct.

Raymond Pile:
A pile made by driving a thin steel shell into the ground with a tapered mandrel and filling it with concrete.

2. Queen closer may be placed

a. in stretcher course

b. in header course next to first brick

c. in stretcher course next to first brick

d. in any position.

Explanation: Option B is correct.

In order to brake the aligment of vertical points in the same line queen closer is placed next to quion header in each header course. A heading course should not start with queen closer.

3. Dado is usually provided in

a. bath rooms

b. living rooms

c. verandah

d. roofs.

Explanation: Option A is correct.

Dados are rails that are mostly put in the sitting room to prevent direct contact of the couches with the wall. In other words, they prevent the couch/chair from damaging the wall.

4. The foundation in which a cantilever beam is provided to join two footings, is known as

a. strap footing

b. combined footing

c. raft footing

d. none of these.

Explanation: Option A is correct.

The beam which is connected to the two isolated footing in between for sharing the maximum load coming from one column and distributing that load between two isolated footing is known as strap footing.

5. The foundations are placed below ground level, to increase

a. strength

b. workability

c. stability of structure

d. all the above.

Explanation: Option C is correct.

Foundation increase the stability of a structure. It is substructure part and must be designed carefully as it can affect whole structure.

6. Stud(s) of a common wooden partition

a. are vertical wooden members

b. is the upper horizontal wooden member

c. is the lower horizontal wooden member

d. are the intermediate horizontal wooden members.

Explanation: Option A is correct.

Vertical wooden members - Studs.
Upper horizontal wooden member - Top plate.
Lower horizontal wooden member - Base plate.
Intermediate horizontal wooden members - Noggins.

7. Pick up the correct statement from the following :

a. inclined borings are made for taking samples under existing structures

b. inclined borings are occasionally used instead of vertical holes.

c. the spacing of inclined borings is kept such that one bore hole is vertically above the bottom of an adjacent bore hole.

d. all the above.

Explanation: Option D is correct.

8. Pick up the commonly adopted geophysical method in civil engineering from the following :

a. electrical resistivity method

b. gravitational method

c. magnetic method

d. both (a) and (b) of the above.

Explanation: Option D is correct.

Electric resistivity methods are a form of geophysical surveying that aids in imaging the subsurface. These methods utilize differences in electric potential to identify subsurface material.

9. To ensure that supporting area of an offset footing of a boundary wall is fully compressive, the C.G. of load must act

a. at the centre of the base

b. within the middle third of the base

c. within the middle fifth of the base

d. neither (a), (b) nor (c).

Explanation: Option B is correct.

C.G = Centre of Gravity (Offset Footing of a boundary wall).
Water cement ratio is the amount of water added in cement as compared to cement's quantity.
Let 200 g Cement + percentage of water added(50 ml water).

10. The 9 cm x 9 cm side of a brick as seen in the wall face, is generally known as

a. face

b. front

c. header

d. side.

Explanation: Option C is correct.

9*9 size brick uses header wall because its size standard size brick we provide easily cleaning windows and given space according to construction  building.

11. The taper of precast concrete pile should not be more than

a. 1 cm per metre length

b. 2 cm per metre length

c. 4 cm per metre length

d. 5 cm per metre length.

Explanation: Option B is correct.

It's like when you measure the pile waist at one point and then one meter below so the difference should not be more than 2cm.
So, the taper is the decreasing size of the pile towards the base and it should not be more than 2 cm per meter.

12. The under surface of an arch, is called

a. soffit

b. intrados

c. haunch

d. back.

Explanation: Option A is correct.

1. Inner surface of an arch called the soffit.
2. Inner curve of an arch called intrados.
Ref : R.S Khurmi.

13. The raft slab is projected beyond the outer walls of the structure by

a. 15 to 20 cm

b. 25 to 30 cm

c. 30 to 45 cm

d. 60 cm.

Explanation: Option C is correct.

14. The process of making the back ground rough, before plastering, is

a. dubbing

b. hacking

c. blistering

d. peeling.

Explanation: Option B is correct.

Blockwork wall joining side/ plastering min 75-80 Hacking Impressions per sqft to be present and to be made with Hacking Machine only and the Impressions to be made in cross X shape only.

15. Black cotton soil is unsuitable for foundations because its

a. bearing capacity is low

b. permeability is uncertain

c. particles are cohesive

d. property to undergo a volumetric change due to variation of moisture content.

Explanation: Option D is correct.

Because black cotton soil is expensive and gives high potential for shrinking or swelling as result of changing moisture content.

16. The loose pockets in soil mass can be bridged safely by providing a raft foundation provided the soft area is smaller than

a. one-third the column spacing

b. half the column spacing

c. three-fourth the column spacing

d. none of these.

Explanation: Option A is correct.

If a small portion of soil having area less than 1/3 of column spacing is loose/soft, then it can be covered by providing Raft Foundation. However if the loosen part is more than 1/3 of Column Spacing then you should go for deep Foundation/Piling.

17. The portion of a brick cut across the width, is called

a. closer

b. half brick

c. bed

d. bat.

Explanation: Option D is correct.

Closer Brick: The portion of a brick cut across the length.
Half Brick: The portion of brick cut to half its length.
Bed: The bed is the mortar upon which a brick is laid.
Bat: The portion of a brick cut across the width.

18. The concrete slump recommended for beams and slabs ; is

a. 25 to 75 mm

b. 30 to 125 mm

c. 50 to 100 mm

d. none of these.

Explanation: Option B is correct.

As per IS 456-2000 Recommend value of slump for beams slabs columns are 50-100 degree of workability Medium.

19. The member which is placed horizontally to support common rafter of a sloping roof, is

a. purlin

b. cleat

c. batten

d. strut.

Explanation: Option A is correct.

Purlins are horizontal wooden or steel members used to support common rafters of a sloping roof when the span is large.
Reference: Khurmi & Gupta.

20. Pick up the correct statement from the following:

a. Flush  door  is  generally  provided   in dinning room

b. Revolving door is generally provided in cinema halls

c. Sliding door is generally provided in show rooms

d. All the above.

Explanation: Option B is correct.

21. Grillage foundation

a. is light and economical

b. does not require deep cutting as the required base area with required pressure intensity is obtained at a shallow depth

c. is constructed by rolled steel joists (R.S.J.) placed in single or double tier

d. all the above.

Explanation: Option D is correct.

22. The exterior angle between outer faces of a wall, is known as

a. turn

b. junction

c. quion

d. all the above.

Explanation: Option C is correct.

23. In jack arch floor, the rise is kept

a. 1/8th of the span

b. 1/10th of the span

c. 1/12th of the span

d. 1/15th of the span.

Explanation: Option C is correct.

According to IS 2118 the rise shall kept 1/6 to 1/8 of span and deflection shall not more than L/480.

24. While designing a stair, the product of rise and going is approximately kept equal to

a. 420

b. 450

c. 500

d. 600.

Explanation: Option A is correct.

Riser = 15 cm & Tread = 30 cm.
Example- In 1st Step Finishing Material 3 cm also this condition in 2nd step then overall height not change.
30 x 15 = 450.

25. For constructing a terrazo floor. Pick up the incorrect statement from the following :

a. a 32 mm thick layer of cement concrete (1 : 2 : 4) is laid on the base course and the surface is made smooth by trowelling

b. glass strips are driven into the layer according to the pattern required

c. after final grinding is over, oxalic acid mixed with water is spread over and rubbed hard with soft material

d. none of these.

Explanation: Option A is correct.

26. The entrained concrete is used in lining walls and roofs for making

a. heat insulated

b. sound insulated

c. neither (a) nor (b)

d. both (a) and (b).

Explanation: Option D is correct.

Entrain air concrete means light weight or cellular concrete which will use for soundproof as well as insulation property.

27. The pile which is provided with a bulb filled with concrete at its lower end, is known as

a. Mac-Arthur pile

b. Raymond pile

c. Franki pile

d. none of these.

Explanation: Option A is correct.

Mac-Arthur pile is also called as Under reamed pile.. There may be single reamed pile or double reamed pile foundation. This type of pile foundation is best suitable in the Area having problems of black cotton soil.

28. In case of multi-storeyed buildings, the forms to be removed first are

a. sides of beams and girders

b. column forms

c. bottom of beams and girders

d. all the above at the same time.

Explanation: Option A is correct.

First of all, we have to remove beams because before columns we have to remove plinth beam side forms And inner side girders.

29. For providing a raft foundation, the following activities are involved

a. 1, 2, 3, 4, 5

b. 5, 4, 3, 2, 1

c. 2, 1, 3, 5, 4

d. 3, 2, 5, 1, 4.

Explanation: Option C is correct.

30. The maximum permissible deflection of a timber beam supporting a roof, is

a. L/150

b. L/260

c. L/360

d. none of these.

Explanation: Option C is correct.

31. The angular steps used for changing direction of the stairs, are called

a. angular steps

b. winders

c. radial steps

d. circular steps.

Explanation: Option B is correct.

32. During percussion drilling

a. ground water observations are hindered due to entry of the slurry in the soil below the bottom of the hole

b. caving or mixing of strata are caused in soft soils or cohesionless soils

c. the soil to a considerable depth below the bottom of the hole gets disturbed

d. all the above.

Explanation: Option D is correct.

33. In English garden wall bond

a. one course of headers to three or five course of stretchers

b. queen closer in provided in each heading course

c. the middle course of stretchers is started with a header to give proper vertical joints

d. all the above.

Explanation: Option D is correct.

34. A projecting piece usually provided to support a truss, is

a. cornice

b. coping

c. frieze

d. lintal.

Explanation: Option C is correct.

Cornice : the ornamental course near the top of a wall or at the junction of wall and ceiling is called cornice.
Corbel: a projecting stone provided to serve as support for joist, truss etc is called corbel.

35. The line of intersection of the surfaces of a sloping roof forming an external angle exceeding 180°, is

a. ridge

b. hip

c. valley

d. none of these.

Explanation: Option B is correct.

36. A solid core of rock is formed in side the cylinder in the case of

a. auger boring

b. percussion drilling

c. diamond drilling

d. wash boring.

Explanation: Option C is correct.

37. The single stage well point system of dewatering an excavation can be used if the depth of excavation does not exceed

a. 5 m

b. 10 m

c. 15 m

d. 20 m

Explanation: Option A is correct.

The single stage well point system of dewatering and excavation can be used if the depth of excavation does not exceed.

38. The piece of a brick cut with its one corner equivalent to half the length and half the width of a full brick, is known as

a. queen closer

b. bevelled closer

c. king closer

d. half king closer.

Explanation: Option C is correct.

39. The brick laid with its breadth parallel to the face of a wall, is known as

a. header

b. stretcher

c. closer

d. none of these.

Explanation: Option A is correct.

40. A wooden block hinged on post outside a door, is known

a. cleat

b. stop

c. horn

d. none of these.

Explanation: Option A is correct.

Cleat is used in place of door stopper in wooden door to keep the door in open position whenever required.

41. Pick up the incorrect statement from the following :

a. No  timbering is  required  for  shallow trenches

b. Shallow foundations can be constructed on made-up soil

c. Grillage   foundation   is   classified as a shallow foundation

d. Black cotton soil is very good for foundation bed.

Explanation: Option D is correct.

Black cotton soil has a nature of absorbing more moisture content so it is not suitable for foundation bed.

42. In horizontal D.P.C, thickness of cement concrete (1 : 2 : 4) is

a. 4 cm

b. 6 cm

c. 8 cm

d. 10 cm.

Explanation: Option A is correct.

43. Which one of the following factors is considered for the orientation of buildings :

a. the direction of the prevailing winds in the area

b. the exposure of the walls and roof of the buildings to the rays of sun

c. the extent up to which the sunrays penetrate with the verandah.

d. all the above.

Explanation: Option D is correct.

44. Pick up the incorrect statement from the following :

a. Cement is added to lime mortar to increase its hydraulic properties only

b. Lime surkhi mortar is used for pointing the walls

c. Lime should be slaked before preparing lime mortar

d. High early strength concrete is generally used in cold weather.

Explanation: Option A is correct.

45. Which one of the following rocks is used for monumental buildings :

a. granite

b. marble

c. sand stone

d. slate.

Explanation: Option B is correct.

46. The Auger borings are not common

a. in soils that require lateral support

b. in cohesive soils

c. in soft soils

d. none of the above.

Explanation: Option A is correct.

47. The form work from the underside of slabs, can be removed only after

a. 1 day

b. 4 days

c. 7 days

d. 14 days.

Explanation: Option C is correct.

48. Arches in the form of masonry arcs struck from more than four centres, are called

a. two curved arches

b. gothic arches

c. ogee arches

d. drop gothic arches.

Explanation: Option C is correct.

49. If (φ) is the angle of repose of soil of weight w kg/m3, the horizontal pressure p at a depth of h metres per metre length of wall, is

a.

b.

c.

d.

Explanation: Option A is correct.

None of the above is corrent because the pressure exerting by the wall is passive pressure which is equal to wh^2/2 (1+sinφ) (1-sin φ).

50. According to Rankine's formula, minimum depth of foundations, is

a.

b.

c.

d.

Explanation: Option B is correct.

51. Cast iron piles

a. are suitable for works under sea water

b. resist shocks or vibrations

c. are suitable for use as batter piles

d. are useful for heavy vertical loads.

Explanation: Option D is correct.

Cast iron is good in compression that's why it's used in columns and struts.
Good in compression, bad in tension and brittle property.

52. The depth of an arch is the distance between

a. ground level and springing line

b. crown and springing line

c. crown and ground level

d. intrados and extrados.

Explanation: Option D is correct.

The depth or height of an arch is The perpendicular distance between the intrados of the arch to the extrados of the arch. While Rise of an arch is The clear vertical distance between the highest point on the intrados and the springing line.

53. Expansion joints in masonry walls are provided if length exceeds

a. 20 m

b. 30 m

c. 40 m

d. 50 m.

Explanation: Option C is correct.

In brick masonry or stone masonry, it is provided at the expansion joint of 25M to 30M. It is the correct expansion space at a longer wall.

54. The platform at the end of a series of steps, is known as

a. relief

b. rest

c. landing

d. stop.

Explanation: Option C is correct.

55. Raft foundation are generally preferred to when the area required for individual footing, is more than

a. 25% to total area

b. 30% of total area

c. 40% to total area

d. 50% of total area.

Explanation: Option D is correct.

56. You are asked to design and supervise a truss for a factory to have spans 6 m to 9 m. The type of the truss you will use, is

a. queen post truss

b. king post truss

c. collar truss

d. none of these.

Explanation: Option B is correct.

57. Pick up the incorrect statement from the following:

a. The section of the retaining wall should be so proportioned that it may not overturn by the lateral pressure

b. The retaining wall should be safe against sliding

c. The foundation of the retaining wall should not be stressed beyond safe bearing capacity ; due to its weight and the force resulting from the earth pressure

d. to drain off water from the earth retained, weep holes are provided near the top of the retaining wall.

Explanation: Option D is correct.

To drain off water from the earth retained, weep holes are provided near the "bottom" of the retaining wall.

58. The arrangement of supporting an existing structure by providing supports underneath, is known as

a. shoring

b. underpinning

c. jacking

d. piling

Explanation: Option B is correct.

59. The process of keeping concrete moist for a certain period after its finishing, is known as

a. curing of concrete

b. placing of concrete

c. compaction of concrete

d. none of these.

Explanation: Option A is correct.

60. The inclined surface of an abutment to receive the arch, is known as

a. skew back

b. soffit

c. spandril

d. haunch.

Explanation: Option A is correct.

61. The strength of brick masonry in 1:6 cement mortar, is

a. 40 tonnes/m2

b. 50 tonnes/m2

c. 60 tonnes/m2

d. 75 tonnes/m2.

Explanation: Option B is correct.

Cement-sand ratio of 1:6 has a mortar strength of 30 kg/cm2 and a brickwork strength of 5.5 kg/cm2, while cement-lime and the sand-mix ratio of 1:1:6 has mortar strength of 30 kg/cm2 has higher brickwork strength of 7 kg/cm2.
1 tonnes = 100kg.

62. The dimensions of a half queen closer, are

a. 9 cm x 9 cm x 9 cm

b. 9 cm x 9 cm x 4.5 cm

c. 9 cm x 4.5 cm x 9 cm

d. 1.8 cm x 4.5 cm x 9 cm.

Explanation: Option C is correct.

Queen closer has the dimensions 9cm x 4. 5cm x 19cm.
So, half queen closer will be having the dimensions 9cm x 4.5cm x 9.5cm.
So, option C is correct.

63. In case of foundations on sandy soil, maximum permissible differential settlement, is usually limited to

a. 25 mm

b. 35 mm

c. 45 mm

d. 55 mm.

Explanation: Option A is correct.

64. For heavy embankments and dams, of height h, the depth of exploration of soil should not be less than

a. h/4

b. 1/2 h

c. h

d. 2 h.

Explanation: Option D is correct.

Here, They asked for heavy embankment and dams. Generally, for strong and durable structure 2H height will require.

65. For different layers of cement concrete floor. Pick up the incorrect statement from the following :

a. The lowest layer consists of consolidated ground

b. A 10 cm thick clean sand is laid on consolidated ground

c. A 10 cm lime concrete (1 : 4 : 8) is laid on clean sand

d. A 10 cm thick cement concrete (1 : 2 : 4) is laid on top layer.

Explanation: Option D is correct.

66. Rotary drilling

a. is not suitable for deposits containing very coarse gravel

b. hinders the ground water observations and permeability test

c. is not economical for holes of less than 10 cm.

d. all the above.

Explanation: Option D is correct.

67. In places where the soil is soft and has small resistance to the flow of concrete, which one of the following types of piles, is used

a. vibro pile

b. pressure pile

c. Franki pile

d. pedestal pile.

Explanation: Option A is correct.

68. In grillage foundations a minimum 15 cm cover is provided on

a. upper flange of top tier

b. lower beam of lower tier

c. ends of external beams

d. none to these.

Explanation: Option B is correct.

Grillage foundation is used when heavy structural loads from columns, piers or stanchions are required to be transferred to a soil of low bearing capacity.

69. The skirting/dado in a bath roof should be upto

a. ceiling

b. 15 cm above floor level

c. 200 cm

d. level of the tap.

Explanation: Option C is correct.

Mostly we do not provide skirting in the bathroom, dado provides at the height of 6-7 feet (near above 200 cm).

70. The maximum bearing capacity of soil is that of

a. loose fine sandy soil

b. dry coarse sandy soil

c. hard rocks

d. soft clay soil.

Explanation: Option C is correct.

71. The columns of multi-storeyed buildings are designed to withstand the forces due to

a. live loads

b. wind loads

c. earthquakes

d. all of these.

Explanation: Option D is correct.

72. The black cotton soil

a. swells excessively when wet

b. shrinks excessively when dry

c. has a tendency of swelling and shrinking due to clay particles

d. all the above.

Explanation: Option D is correct.

73. Pick up the incorrect statement from the following:

a. In dog-legged stairs, no space between its flights is provided

b. In open newel stair, a rectangular well is provided

c. In geometric stair, a curved shaped well between forward and backward flights, is provided

d. In geometrical stair, two quarter space landing is provided.

Explanation: Option D is correct.

74. Exposed portions of vertical surface at right angles to the door or window frame, are known as

a. jambs

b. lintels

c. reveals

d. soffits.

Explanation: Option C is correct.

75. To obtain good bonding in brick masonry

a. first class bricks are used

b. vertical joints in alternate courses are kept in plumb line

c. bats are used where necessary

d. all the above.

Explanation: Option D is correct.

76. Raft foundations are used for :

a. providing increased area of foundation over poor bearing capacity of soil

b. spanning over small soft or loose pockets

c. counter acting the hydrostatic effect

d. all the above.

Explanation: Option D is correct.

77. For brick construction, the lime-sand mortar, is

a. 1:2

b. 1:3

c. 1:4

d. 1:5.

Explanation: Option A is correct.

78. An arch constructed with finely dressed stones, is known

a. ashlar arch

b. rubble arch

c. gauged arch

d. axed arch.

Explanation: Option A is correct.

79. Cavity wall is generally provided for

a. heat insulation

b. sound insulation

c. prevention of dampness

d. all the above.

Explanation: Option D is correct.

The Cavity wall usually used for saving heat into home, also if the home has any dampness we can not use cavity wall until solve the dampness problem.

80. Dampness causes

a. bleaching of paints

b. crumbling of plaster

c. growth of termites

d. none of these.

Explanation: Option C is correct.

81. The foundation which consists of a thick reinforced cement slab covering whole area to support heavy concentrated structural loads, is known as

a. combined footing

b. strap footing

c. raft footing

d. none of these.

Explanation: Option C is correct.

Raft footing is a thick reinforced concrete slab which spreads over a large area of soil and provides support for several columns and load bearing walls.

82. Bearing capacity of soils cannot be improved by

a. ramming crushed stone in soil

b. driving sand piles

c. watering surface of soil

d. none of these.

Explanation: Option C is correct.

83. To support a heavy structure in sandy soil, the type of foundation generally used, is

a. raft footing

b. pier footing

c. strap footing

d. none of these.

Explanation: Option B is correct.

84. A floor constructed with 3 mm marble chips, is known

a. mosaic floor

b. terrazo floor

c. chips floor

d. marble floor.

Explanation: Option B is correct.

Terrazo floor is made of chips of marble, quartz, glass etc while the mosaic floor is made with arrangement of tiles and stones to get a picture, deisgn or pattern.

85. The position of a brick when laid on its side 9 cm x 9 cm with its frog in the vertical plane, is called

a. brick on edge

b. brick on end

c. brick on bed

d. brick held vertically.

Explanation: Option B is correct.

86. Dutch bond is a modification of

a. English bond

b. stretcher bond

c. header bond

d. single Flemish bond.

Explanation: Option A is correct.

87. The 19 cm x 9 cm side of a brick as seen in the wall face, is generally known as

a. stretcher

b. face

c. front

d. header

Explanation: Option A is correct.

88. The depth of the ground water table may be ascertained by

a. looking through the well in the vicinity

b. standing on the well in the vicinity

c. measuring the depth of water in the well

d. none of the above.

Explanation: Option C is correct.

89. Pick up the correct statement from the following :

a. the pile driven in sand is called sand pile

b. the drilled hole filled with sand is called sand pile

c. the sand piles are used for bearing purposes

d. None of these.

Explanation: Option A is correct.

90. Pick up the correct statement from the following :

a. the cost of square rooms is less

b. the expenditure on the foundation and roof for the double storeyed building is nearly half of that for the ground storeyed building.

c. the cost of construction of a house may be minimised by restricting the height floors

d. all the above.

Explanation: Option D is correct.

91. The maximum permissible differential settlement, in case of foundations in clayey soil, is usually limited to

a. 20 mm

b. 30 mm

c. 40 mm

d. 50 mm.

Explanation: Option C is correct.

92. The compaction of concrete in the drilled pile hole is done by compressed air in the case of

a. simplex pile

b. Franki pile

c. pressure pile

d. vibro pile.

Explanation: Option C is correct.

93. The opening provided in sloping roof with its top parallel to the roof surface, is called

a. dormer window

b. sky light window

c. lantern window

d. louvered window.

Explanation: Option B is correct.

Hello, guys, dormer window is provided to the perpendicular to the sloping roof. And skylight window provided in the horizontal plane.

94. The type of ashlar masonry in which stones are finely chisel dressed and thickness of joints does not exceed 3 mm, is

a. chamfered ashlar masonry

b. ashlar facing masonry

c. random coursed ashlar masonry

d. coursed ashlar masonry.

Explanation: Option D is correct.

95. The lower half portion between crown and skew back of the arch, is called

a. spandril

b. haunch

c. springing

d. soffit.

Explanation: Option B is correct.

96. The angle between skew back of a flat arch and the horizontal, is kept approximately equal to

a. 30°

b. 60°

c. 90°

d. 120°.

Explanation: Option B is correct.

Skew Back is an inclined surface where abutment of arch is present & makes an angle of 60 degree with the horizontal.

97. In clay soil

a. swelling and shrinkage characteristics prevail

b. consolidation continues even after several years of construction.

c. differential settlement is generally prevalent

d. all the above.

Explanation: Option D is correct.

98. Pick up the incorrect statement from the following :

a. a long vertical load transferring concrete structure is called a concrete pile

b. in pile which transfers the load to the soil by the friction between the pile and the surrounding soil is called friction pile.

c. the pile which transfers the load to a hard rock bed at certain depth is called load bearing

d. none of the these.

Explanation: Option C is correct.

99. Depth of lean concrete bed placedat the bottom of a wall footing, is kept

a. 10 cm

b. 15 cm

c. equal to its projection beyond wall base

d. less than its projection beyond wall base.

Explanation: Option C is correct.

Kindly explain which one projection, because the wall is going to be straight so which projection you are talking about.

100. For a wall carrying heavy load on low bearing capacity soil,

a. thick concrete bed is provided

b. reinforced concrete bed is provided

c. (a) and (c) of the above

d. (b) and (c) of the above.

Explanation: Option C is correct.

Lean Concrete is always provided may it be low or high bearing capacity, it used for leveling your foundation and no relation to bearing capacity.

101. In grillage foundations, distance between flanges of grillage beams, is kept

a. 40 cm

b. equal to flange width

c. twice the flange width

d. maximum of (a), (b) and (c).

Explanation: Option C is correct.

In the Grillage foundation, the distance between flanges is kept 1.5 to 2 times the width of the flange or 30 cm whichever is lesser.

102. Herringbone bond is used for

a. walls having thickness more than 4 bricks

b. architectural finish to the face work

c. ornamental panels in brick flooring

d. all the above.

Explanation: Option D is correct.

103. A cut in frame of a door to receive the shutter, is called

a. louver

b. stop

c. horn

d. rebate.

Explanation: Option D is correct.

104. The alignment of a cross joint along the plumb line is

a. bed block

b. perpend

c. lintel

d. vertical line.

Explanation: Option B is correct.

An imaginary vertical line which includes the vertical joint separating two adjoining bricks is called perpend.

105. A temporary rigid structure having platforms to enable masons to work at different stages of a building, is known as

a. scaffolding

b. dead shore

c. raking shore

d. under pinning.

Explanation: Option A is correct.

106. The additional piles which are driven to increase the capacity of supporting loads on vertical piles, are known

a. raking piles

b. eccentric piles

c. sinking piles

d. none of these.

Explanation: Option A is correct.

If a pile group is subjected to vertical force, then the calculation of load distribution on single pile that is member of the group is assumed to be the total load divided by the number of piles in the group.

107. The bearing capacity of piles is determined by

a. dynamic formula

b. static formula

c. pile load tests

d. all the above.

Explanation: Option D is correct.

108. A covering of concrete placed on the exposed top of an external wall, is known as

a. cornice

b. coping

c. frieze

d. lintal.

Explanation: Option B is correct.

109. Auger boring

a. is generally employed in cohesive and other self soils above water table

b. is most economical upto a depth of 5 metres

c. is done by portable power driven helical augers those diameters range from 7.5 to 30 cm

d. all the above.

Explanation: Option D is correct.

110. A roof which slopes in four directions, is called

a. shed roof

b. gable end roof

c. hipped roof

d. gambrel roof.

Explanation: Option C is correct.

Hip roofs on houses could have two triangular sides and two trapezoidal ones.
And Mansard roof which has four sloping sides, each of which becomes steeper halfway down.

111. The stone whose crushing strength is least, is

a. chalk

b. marble

c. slate

d. sand stone.

Explanation: Option A is correct.

Chalk is made out of pure lime and no additives are provided hence only lime is very weak in tension and compression.

112. Best type of piles for soft soil having little resistance to the flow of concrete, is

a. Simplex pile

b. Vibro pile

c. Raymond pile

d. Franki pile.

Explanation: Option B is correct.

113. A wall constructed to resist the pressure of an earth filling, is called

a. retaining wall

b. breast wall

c. buttress

d. parapet wall.

Explanation: Option A is correct.

114. Engineering news formula for obtaining safe bearing capacity of pile for drop hammer, is,

a.

b.

c.

d.

Explanation: Option A is correct.

115. The bearing capacity of a water logged soil, may be improved by

a. grouting

b. chemical action

c. drainage

d. compaction.

Explanation: Option C is correct.

Compaction cannot achieved.
Compaction achieved = removal of air voids and water voids from the soil mass.

116. The inclined support at the ends of treads and rises of a stair, is known as

a. baluster

b. header

c. string

d. beam.

Explanation: Option C is correct.

A Baluster is a vertical moulded shaft in stairways, parapets, and other architectural features While the String is inclined support at the ends of threads and rises in a Stair.

117. The depth of concrete bed of the foundation depends upon

a. the projection of the concrete block beyond the footing over it

b. the upward soil pressure

c. the mix of the concrete

d. all the above.

Explanation: Option D is correct.

118. The vertical side member of a shutter frame, is known

a. style

b. reveal

c. mullion

d. post.

Explanation: Option A is correct.

119. The brick laid with its length parallel to the face of a wall, is a known as

a. header

b. stretcher

c. closer

d. none of these.

Explanation: Option B is correct.

120. Couple roof is used for spans

a. 3.5 m or less

b. 3.5 m but less than 5 m

c. 5 m but less than 6.5 m

d. 6.5 m but less than 8 m.

Explanation: Option A is correct.

121. Two columns 50 cm x 50 cm and 60 cm x 60 cm carry 80 tonnes and 120 tonnes of loads respectively. The centre to centre distance between columns is 5.00 metres. The permissible bearing capacity of the soil is 20 t/m2. If the footing is not to project more than 25 cm beyond the outside of the smaller column, pick up the correct design parameters of the footing from the following:

a. the   length   of the foundation  slab = 7.00 m

b. area of footing slab = 11.00 m2

c. width of the footing = 1.57 m.

d. all the above.

Explanation: Option D is correct.

Total load = (120+80)*1.1=220t.
Area of footing = 220/20 = 11 sq.m.
Dist. Of CG from the center of the small column = 120(5)/(120+80).
= 3m.
Length of the footing = (3+0.25+0.25) * 2 = 7m.
Width of the footing = 11/7 = 1.57m.

122. Pick up the correct specification of one-room quarters generally adopted from the following :

a. six quarters in a row

b. the size of room is either 3.5 m x 3 m or 4.2 m x 2.5 m

c. the front verandah is kept 2 m wide.

d. all the above.

Explanation: Option D is correct.

123. Negative skin friction

a. is a downward drag acting on a pile due to downward movement of the surrounding compressible soil relative to the pile

b. develops due to lowering of ground water

c. both (a) and (b)

d. neither (a) not (b).

Explanation: Option C is correct.

124. The type of stone masonry in which stones of same height are laid in layers, is called

a. random rubble masonry

b. course rubble masonry

c. uncoursed rubble masonry

d. ashlar masonry.

Explanation: Option B is correct.

125. The range of spread from the wall base to outer edge of a brick work foundation does not exceed

a. 1/2 horizontal to 1 vertical

b. 2/3 horizontal to 1 vertical

c. 1 horizontal to 1 vertical

d. 2 horizontals to 1 vertical.

Explanation: Option C is correct.

126. Which one of the following piles has a cast iron shoe even after removal of the hollow cylindrical steel casing

a. pedastal pile

b. Franki pile

c. vibro pile

d. both (a) and (d) of the above.

Explanation: Option D is correct.

127. If height of the first storey of a building is 3.2 m and riser is 13 cm, the number of treads required, is

a. 18

b. 24

c. 25

d. 30.

Explanation: Option B is correct.

3.20/0.13 = 25  (Height of a building/ riser).
For each storey, No of threads = 25 - No of storeys (Here it is 1).
i.e 25-1 = 24.
If it is a 2-storey building, then No of threads equal to 25-2 = 23 and Vice versa.

128. Pick up the correct statement from the following :

a. the sand is kept moist at the time of placing and tamping

b. the top one metre of the pile is filled up with cement concrete to provide a cap for the filled up sand

c. sand piles are generally used under column loads

d. all of the above.

Explanation: Option D is correct.

129. Pick up the incorrect statement from the following :

a. In king post truss, one vertical post is used

b. In a queen post truss, one vertical post is used

c. In a queen post truss, two vertical posts are used

d. None of these.

Explanation: Option B is correct.

130. For effective drainage, the finished surface of flat roof should have a minimum slope of

a. 1 in 20

b. 1 to 50

c. 1 in 10

d. 1 in 5.

Explanation: Option A is correct.

131. If the depth of an excavation is 20 metres, number of single stage well points to be installed at various levels, is

a. 4

b. 3

c. 2

d. 6.

Explanation: Option B is correct.

132. The minimum distance between the centres of bulb of diameter du, of a multi under reamed piles, is

a. 1.25 du

b. 1.5 du

c. 1.75 du

d. 2 du.

Explanation: Option B is correct.

133. The method of moving each brick through a small horizontal distance before it is finally laid in any brick wall and pressing it by means of brick hammer, is known as

a. trowelling

b. laying

c. grouting

d. placing.

Explanation: Option B is correct.

134. The stone whose crushing strength is maximum, is

a. granite

b. chalk

c. slate

d. sand stone

Explanation: Option A is correct.

135. The vertical member running through middle of a shutter frame, is

a. style

b. reveal

c. mullion

d. post.

Explanation: Option C is correct.

136. The formWork including the props can be removed from beams, only after

a. 3 day

b. 7 days

c. 14 days

d. 21 days.

Explanation: Option C is correct.

Props are the compression members used as temporary supports for building and civil engineering works incorporating a means for adjusting and fixing their length.

137. The type of bond in a brick masonry containing alternate courses of stretchers and headers, is called

a. Flemish bond

b. English bond

c. Stretcher bond

d. Header bond.

Explanation: Option B is correct.

English bond - alternative header and stretchers course.
Flemish bond - each course contain header and stretchers.

138. While investigating the site, a thick layer of fairly firm clay over a deep layer of soft clay is encountered. In such a situation, the following type of foundation is useful :

a. pile formation

b. raft foundation

c. grillage foundation

d. none of these.

Explanation: Option B is correct.

139. The concrete slump recommended for columns, is

a. 25 to 50 mm

b. 25 to 75 mm

c. 75 to 125 mm

d. 50 to 100 mm.

Explanation: Option C is correct.

According to R.Agore - Concrete Slumps For;
Slabs and Beams = 30 to 125 mm.
Columns = 75 to125 mm.
Foundations = 50 to 100 mm.

140. The inner section of a cavity wall, is generally known as

a. butress

b. leaf wall

c. pillaster

d. pillar.

Explanation: Option B is correct.

The inner and outer wall of the cavity wall is used for the protection of the cavity wall. Seems like a leaf Wich is protected the leaf's stem.

141. Nogging of a common wooden partition is

a. upper horizontal wooden member

b. lower horizontal wooden member

c. intermediate horizontal wooden member

d. vertical wooden member.

Explanation: Option C is correct.

142. Ornamental moulded course placed on the top of a wall, is

a. cornice

b. coping

c. frieze

d. lintal.

Explanation: Option A is correct.

Option A is right.
The cornice is moulding at corner b/w celling and top of the wall.
Coping is covering of stone or concrete on the exposed top of the external wall.

143. The size of a floor tile commonly used, is

a. 15 cm x 15 cm x 1.8 cm

b. 20 cm x 20 cm x 2 cm

c. 22.5 x 22.5 cm x 2.2 cm

d. all the above.

Explanation: Option D is correct.

144. The triangular portion between any two adjacent arches and the tangent to their crowns, is

a. haunch

b. spandril

c. soffit

d. rise.

Explanation: Option B is correct.

145. The depth of excavation of foundations, is generally measured with a

a. ranging rod

b. steel tape

c. levelling staff

d. bonning rod.

Explanation: Option D is correct.

Bonning rods are used to set out horizontal lines or lines with a constant slope. In particular they are used for setting out canal excavation works, but also for roads and dyke construction.

146. The floor is rubbed with oxalic acid, for making its surface

a. free from voids

b. glossy

c. durable

d. uniform.

Explanation: Option B is correct.

147. For the construction of flyovers in sandy soils, the type of foundation provided, is

a. raft footing

b. combined footing

c. pier footing

d. none of these.

Explanation: Option C is correct.

148. If a is the offset of concrete bed in cms, and d is the depth of concrete bed in cms, then

a. d = 0.445 a

b. 0.557 a

c. d = 0.775 a

d. none of these.

Explanation: Option C is correct.

149. The type of bond in which every course contains both headers and stretchers, is called

a. English bond

b. Flemish bond

c. Russian band

d. Mixed bond.

Explanation: Option B is correct.

It's a Flemish bond. In English bond: alternate course of headers and stretcher. In Flemish bond: every course contains header and stretcher.

150. Pile foundation is generally provided if soil is

a. compressible

b. water logged

c. made up

d. all the above.

Explanation: Option D is correct.

151. To stagger vertical joints in successive courses of a wall, a piece of brick is generally used at the end of the course, which is known as

a. bat

b. header

c. stretcher

d. closer.

Explanation: Option D is correct.

152. The concrete slump recommended for foundations, is

a. 30 to 125 mm

b. 50 to 100 mm

c. 75 to 125 mm

d. none of these.

Explanation: Option B is correct.

For slabs and beams, it should be 30 to 125.
For columns 75 to 125.
For foundation, it should be 50 to 100.

153. The minimum thickness of walls built in cement mortar (1 : 6) for a single storey building, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm.

Explanation: Option B is correct.

154. The form work from the slabs excluding props, can be removed only after

a. 1 day

b. 4 days

c. 7 days

d. 14 days

Explanation: Option B is correct.

Deshuttering,( formwork, stripping) time.
Vertical side of beams and column 24- 48 hours.
Slab span upto 4.5 m  7 days.
Slab span 4.5 to 6 m   14 days.
Slab span over 6 m     21 days.

155. The pile provided with one or more bulles in its vertical shaft, is generally known as

a. under-ream pile

b. friction pile

c. bearing pile

d. sheet pile.

Explanation: Option A is correct.

156. To construct a 10 cm thick partition wall, you will prefer

a. English bond

b. Flemish bond

c. Header bond

d. Stretcher bond.

Explanation: Option D is correct.

10 cm thick means brick laid with along the length of wall. which Means by viewing the front side of the wall,  You see the stretcher side of the brick. Hence thickness has to 10 cm.

157. The pile which supports the load due to friction between pile face and surrounding soil, is generally known as

a. bearing pile

b. friction pile

c. sheet pile

d. battered pile.

Explanation: Option B is correct.

158. The voussoir placed at crown of an arch, is known as a

a. key

b. soffit

c. springer

d. haunch.

Explanation: Option A is correct.

159. The vertical members fixed between steps and hand rail, are known

a. balusters

b. strings

c. newel posts

d. soffits.

Explanation: Option A is correct.

160. Pick up the consideration to the taken while designing a hospital from the following :

a. the operation theatre unit to be detached as it requires sterlized zone but near the ward for the patients and doctor

b. the mortuary should be detached from the main circulation with a postmortem room

c. causalty unit should be provided a separate entrance

d. all the above.

Explanation: Option D is correct.

161. Pick up the incorrect statement from the following :

a. Horizontal D.P.C. is provided at plinth level in internal walls

b. D.P.C. is provided under door and verandah openings

c. Vertical D.P.C. is not provided in internal walls

d. Cement concrete is a rigid damp-proofing material.

Explanation: Option B is correct.

162. For plastering the exposed brick walls, the cement sand mortar should be

a. 1:3

b. 1:4

c. 1:6

d. 1:8.

Explanation: Option B is correct.

163. The construction joints in buildings are provided after

a. 15 m

b. 20 m

c. 40 m

d. 60 m.

Explanation: Option C is correct.

164. The arrangement made to support an unsafe structure temporarily, is known as

a. shoring

b. scaffolding

c. underpinning

d. jacking

Explanation: Option A is correct.

165. Pick up the correct statement from the following :

a. D.P.C. should be continuous

b. D.P.C.   should  be  of good  impervious material

c. D.P.C. may be horizontal or vertical

d. All the above.

Explanation: Option D is correct.

166. An arch may fail due to

a. uneven settlement of abutments

b. sliding of voussoirs

c. crushing of the material

d. all the above.

Explanation: Option D is correct.

167. Pick up the correct statement from the following :

a. Cavity of a cavity wall should terminate near eaves level of sloping roof

b. Cavity of a cavity wall should terminate near coping of flat roof with parapet wall

c. Damp proof course for two leaves of a cavity wall, is laid separately but at the same level

d. All the above.

Explanation: Option D is correct.

168. In soft clay of low bearing capacity, the type of steel pile generally used, is

a. screw pile

b. disc pile

c. pipe pile

d. raking pile.

Explanation: Option A is correct.

169. Stability of an existing structure may be disturbed by

a. vibrations caused by traffic movements

b. mining in the neighbourhood

c. excavation in the neighbourhood

d. all the above.

Explanation: Option D is correct.

170. According to National Building Code, the hydrants in water mains is provided at minimum interval of

a. 50 m

b. 60 m

c. 75 m

d. 90 m.

Explanation: Option C is correct.

171. Open test pit is only suitable upto a depth of

a. 2 metres

b. 2.5 metres

c. 3 metres

d. none of the above.

Explanation: Option C is correct.

The commonly used method to find the nature of the subsoil strata is to dig a hole and see. The holes which are large enough to permit the entry of persons for inspection are called 'Test Pits'.

172. Pick up the correct statement from the following :

a. Double-flemish bond is suitable for brick masonry to give uniform face appearance

b. The double-flemish bond is used for the construction of single brick residential building

c. The stretcher bond is used for the construction of half brick masonry brick

d. all the above.

Explanation: Option D is correct.

173. The projections of head or sill of a door or window frame, are

a. transoms

b. horns

c. stops

d. chocks.

Explanation: Option B is correct.

174. Pick up the correct statement from the following :

a. the first coat of stucco plaster is called scratch coat

b. the second coat of stucco plaster is called brown coat

c. the third coat of stucco plaster is called white coat

d. all the above.

Explanation: Option D is correct.

175. The local swelling of a finished plaster, is termed

a. cracking

b. dubbing

c. blistering

d. hacking.

Explanation: Option C is correct.

176. The form work from the sides of beams can be removed only after

a. 1 day

b. 4 days

c. 7 days

d. 14 days.

Explanation: Option A is correct.

As per IS 456 2000.
Beam more than 6m is 21 days.
Soffit beams after 7 days.
For column it is 1 day.

177. In soils possessing low bearing capacity, the type of foundation generally provided, is

a. grillage footing

b. raft footing

c. mat footing

d. all the above.

Explanation: Option D is correct.

178. Which one of the following activities is not correct as applicable to brick corbels

a. the maximum projection of the corbel should not be more than the thickness of the wall

b. the maximum projection of each corbel course should be limited to a quarter brick at a time

c. the discontinuous corbels are used to carry heavy concentrated loads

d. stretcher bond is generally used for the construction of brick corbel.

Explanation: Option D is correct.

179. Pick up the correct statements from the following :

a. cracks appear on the plastered surface in the form of hair cracks

b. in brick work, the efflorescence is removed by applying a solution of zinc sulphate and water

c. excessive thermal variations in the backing or plaster causes the plaster to fall

d. all the above.

Explanation: Option D is correct.

180. Rotary drilling is the fastest method in case of

a. rocky soils

b. clay soils

c. sandy soil

d. all of these.

Explanation: Option D is correct.

181. The type of pointing in which upper side of mortar joints is kept about 12 mm inside the face of the masonry and bottom is kept flushed with face of wall, is

a. truck pointing

b. recessed pointing

c. struck pointing

d. grooved pointing.

Explanation: Option C is correct.

182. The steel pile which is generally sunk in soft clay or loose sand of low bearing capacity, is

a. pipe pile

b. screw pile

c. disc pile

d. none of these.

Explanation: Option B is correct.

183. The stepped structure provided for lateral support of a structure, is

a. retaining wall

b. breast wall

c. buttress

d. parapet wall.

Explanation: Option C is correct.

184. In the method of tube boring of soil investigation, the following is essential :

a. a flap valve at the bottom of tube is provided to extract the soil sample

b. the tube is raised and lowered by 4 thick rope moving over a pulley suspended on a tripod stand

c. the tube is dropped to fall under gravity in side a metalic casing pipe which is driven as the depth of excavation proceeds.

d. all the above.

Explanation: Option D is correct.

185. For each storey of a building, the depth of exploration should be

a. 1 metre

b. 2 metres

c. 3 metres

d. 4 metres.

Explanation: Option C is correct.

Here, depth for exploration of each story means floor to floor height of a building therefore as we know floor to floor height Of building is 3m.

186. Pick up the correct statement from the following :

a. the lime with surkhi used as mortor for construction reduces the cost and provides equal strength to wall

b. for very cold or very hot climate, a compact and closed plan should be provided

c. on the sea coast, an exposed and open house is generally preferred

d. all the above.

Explanation: Option D is correct.

187. Pile foundations are suitable for

a. water logged soils

b. soft rocks

c. compact soils

d. multistoreyed buildings

Explanation: Option A is correct.

188. In verandah floors outward slope is

a. 1 in 50

b. 1 in 60

c. 1 in 70

d. 1 in 100.

Explanation: Option B is correct.

189. A floor constructed with the 4 to 6 mm marble chips, is known

a. terrazo floor

b. marble floor

c. chip floor

d. mosaic floor.

Explanation: Option A is correct.

190. In flat roof of reinforced cement concrete, the recommended angle of slope, is

a. zero

b. a few degrees

c. 10°

d. 200°.

Explanation: Option B is correct.

191. Safe bearing capacity of black cotton soil varies from

a. 2 to 3 t/m2

b. 5 to 7.5 t/m2

c. 8 to 10 t/m2

d. 10 to 12 t/m2.

Explanation: Option B is correct.

192. Crown is located at

a. highest point on the extrados of the arch

b. highest point on the intrados of the arch

c. skew-back of the arch

d. none of these.

Explanation: Option A is correct.

193. The nominal thickness of an expansion joint in brick walls, is kept more than

a. 10 mm

b. 15 mm

c. 20 mm

d. 40 mm.

Explanation: Option C is correct.

194. The nominal thickness of one brick wall in mm, is

a. 90 mm

b. 150 mm

c. 190 mm

d. 200 mm.

Explanation: Option D is correct.

195. The pile which supports the load partly by friction and partly by resting on hard stratum, is called

a. friction pile

b. bearing pile

c. friction bearing pile

d. rough pile.

Explanation: Option C is correct.

196. The sound which continues even after its source is cut off, is called

a. reverberation

b. echo

c. intensity of sound

d. interference.

Explanation: Option A is correct.

Reverberation time is measured by recording a sound level as it decreases after the source of the sound has been cut off.

197. Pick up the incorrect statement from the following :

a. In Flemish bond every header in each course lies centrally over every stretcher of the underlying course

b. In English bond, stretchers are laid in every course

c. In English bond, headers and stretchers are laid in alternate courses

d. None of these.

Explanation: Option B is correct.

198. Which of the following metal sheets is most effective in preventing dampness ?

a. Copper sheets

b. lead sheets

c. aluminium sheets

d. all the above.

Explanation: Option D is correct.

199. The least bearing capacity of soil is that of

a. moist clay

b. soft rock

c. laminated

d. coarse sandy soil.

Explanation: Option A is correct.

200. Pick up the correct statement from the following :

a. the roof slabs of multi-storeyed buildings are constructed monolithically to carry the various floor loads

b. the beams of multi-storeyed buildings rest on girders and are the main load transferring members to the columns

c. the slab is spanned across the secondary beams provided between the main beams

d. All of these.

Explanation: Option D is correct.

201. The sill of a common wooden partition is

a. vertical wooden member on either end

b. lower horizontal wooden member

c. upper horizontal wooden member

d. intermediate horizontal wooden member.

Explanation: Option B is correct.

202. Suitable spacing of timber piles, is

a. 60 cm

b. 70 cm

c. 80 cm

d. 90 cm.

Explanation: Option D is correct.

It is because timber piles have a diameter of approx 30cm to 45 cm and mostly used for soil compaction. The min spacing of piles used for this purpose is 3 dia.

203. The members which support covering material of a sloping roof, are

a. rafters

b. purlins

c. battens

d. struts.

Explanation: Option A is correct.

Sloping roof have no purloin it only comes in truss it has batten for support of rafter and the covering material support by rafter.

204. An ordinary concrete may be made water proof by adding

a. impermo

b. snowcem

c. cico

d. all of these.

Explanation: Option D is correct.

205. The Auger boring method is  not suitable for

a. cemented soil

b. vary soft soil

c. fully saturated cohesionless soils

d. all the above.

Explanation: Option D is correct.

206. The mortar in which both cement and lime are used as binding materials, is called

a. lime mortar

b. fire resistant mortar

c. gauged mortar

d. light weight mortar.

Explanation: Option C is correct.

Gauged mortar consists cement, lime, sand and water & the amount of cement added to lime to equalise its quality is known as gauged mortar.

207. The rock formed from the solidification of molten matter (magma) is called :

a. sedimentary rock

b. metamorphic rock

c. igneous rock

d. none of the above.

Explanation: Option C is correct.

208. The minimum width of a stair in residential buildings, is

a. 70 cm

b. 85 cm

c. 100 cm

d. 120 cm.

Explanation: Option B is correct.

209. The bond in which headers and stretchers are laid in alternate courses and every stretcher course is started with a three fourth brick bat, is known as

a. English cross bond

b. Dutch bond

c. Monk bond

d. Rat-trap bond.

Explanation: Option B is correct.

When in this same arrangement of alternate header and stretcher courses, the header starts with a queen closer then it is called English Bond.

210. The vertical faces of a door opening which support frame of the door, are

a. jambs

b. posts

c. reveals

d. styles.

Explanation: Option A is correct.

211. The type of pointing in which a V-shaped projection outside the wall surface, is provided, is called

a. recessed pointing

b. weather pointing

c. V-pointing

d. tuck pointing.

Explanation: Option B is correct.

212. Pick up the correct statement from the following:

a. In a king post truss, principal rafter and tie beams are jointed together with a bridle joint.

b. Joint between the principal rafter and the king post is made by making tenon and mortice respectively

c. Joint between strut and king post, is generally of mortice and tenon type

d. All the above.

Explanation: Option D is correct.

213. Pick up the correct statement about silt soil from the following :

a. the silt soil has particle size from 0.02 mm to 0.06 mm.

b. in organic fine grained silt soil possesses no plasticity.

c. the least plastic type normally consists of more or less equi-dimensional grains of quartz

d. all the above.

Explanation: Option D is correct.

Coarse silt soil has particle size is  = 0.02 - 0.06.
Fine silt =  0.002 - 0.006.
Medium silt = 0.006 - 0.02.

214. Pick up the correct statement from the following :

a. The ultimate bearing capacity of a pile is defined as the maximum load which the pile carries and continues to sink without any further increases of load

b. The safe bearing capacity of a pile is obtained by dividing the ultimate bearing capacity with a suitable factor of safety

c. The factor of safety for piles is taken as 6

d. All the above.

Explanation: Option D is correct.

215. Pick up the correct statement from the following:

a. Slump test is performed to check concrete strength

b. Curing of concrete is done for proper compaction of cement

c. Fineness modulus is the index number expressing the relative sizes of both coarse and fine aggregates

d. Concrete is a mixture of binding material, coarse aggregate and water.

Explanation: Option C is correct.

Concrete is the mixture of fine aggregate, coarse aggregate, binding material and water.
So, D is correct.

216. A pre-stressed concrete pile is

a. lighter in weight

b. extremely durable

c. suitable for heavy load

d. all the above.

Explanation: Option D is correct.

217. The highest line of sloping roof, where two oposite slopes meet, is known as

a. rafter

b. ridge

c. crown

d. eave.

Explanation: Option B is correct.

218. The vertical sides of a door and window openings provided in a wall, are known as

a. verticals

b. reveals

c. jambs

d. none of these.

Explanation: Option C is correct.

Jambs are generally the outer part of the doors and windows which act as a vertical side and it is main source of joining the door and wall by a hinge.

219. The piece of a brick cut along the centre of width in such a way that its length is equal to that of full brick, is called

a. half brick

b. queen closer

c. king closer

d. bevelled closer.

Explanation: Option B is correct.

220. A stair should not have pitch more than

a. 30°

b. 40°

c. 50°

d. 60°.

Explanation: Option B is correct.

1 rise & 1 tread make an angle of 45°.
1 rise & 2 tread make an angle of 26°.
So the maximum is 45°.
The correct option is C because 40 degree is near to 45°.
While 50 degree is greater than 45 degree so we choose the safe side.

221. The window which is provided in flat roof of a room, is known

a. dormer window

b. lantern window

c. louvered window

d. sky window.

Explanation: Option B is correct.

222. Gravels

a. vary in size between 2 to 20 mm

b. never swell when they come into contact with water

c. seldom shrink when dried

d. all the above.

Explanation: Option D is correct.

223. A wooden block fixed on back side of a door frame on its post, is known as

a. cleat

b. stop

c. horn

d. none of these.

Explanation: Option B is correct.

224. Brick nogging type of partition wall, is constructed by

a. laying bricks as stretchers in cement mortar

b. laying bricks as headers in cement mortar

c. reinforcing brick wall with iron straps

d. constructing brick work within a wooden framework.

Explanation: Option D is correct.

225. In high mountaneous region, the type of roof generally recommended for buildings, is

a. shed type

b. gable type

c. gambrel type

d. mansord type.

Explanation: Option C is correct.

226. The stone masonry of finely dressed stones laid in cement or lime, is

a. random rubble masonry

b. coursed rubble masonry

c. dry rubble masonry

d. ashlar masonry.

Explanation: Option D is correct.

227. The process of filling hollow spaces of walls before plastering, is known

a. dubbing out

b. blishering

c. peeling

d. all the above.

Explanation: Option A is correct.

228. The stone blocks approximately triangular in shape, used as steps, are known

a. stone steps

b. built up steps

c. spandril steps

d. none of these.

Explanation: Option C is correct.

229. A concrete structure is set on fire and the temperature raises to 1000°C. The strength of concrete as compared to original strength reduces to

a. 10%

b. 15%

c. 20%

d. 25%.

Explanation: Option C is correct.

You cannot predict only with temperature. Time is also a very important factor that governs the concrete strength in a fire.

230. Pick up the correct statement from the following :

a. A mortar joint projecting beyond the face of a masonry wall, is called tucked joint

b. A mortar joint having an inward-upward slope in brick masonry, is called weathered joint

c. A mortar joint having a recess in it, is called ruled joint

d. All the above.

Explanation: Option D is correct.

231. The window which projects outside a room of a building for admitting more light and air, is known

a. bay window

b. casement window

c. lantern window

d. dormer window.

Explanation: Option A is correct.

232. Weep holes are provided in retaining and breast walls

a. to drain off the water from the filling

b. to ventilate the stone masonry

c. to add architectural beauty

d. to reduce the weight of the earth retained

Explanation: Option A is correct.

233. The window which is provided on a sloping roof of a building, is called

a. dormer window

b. louvered window

c. rash window

d. air window.

Explanation: Option A is correct.

234. Vertical construction joints are provided where the shearing forces are minimum in the case of

a. slabs

b. beams

c. girders

d. all of these.

Explanation: Option D is correct.

235. Pick up the correct statement from the following :

a. column footings may have steps or projections in the concrete base

b. heavily loaded column base must be provided steel reinforcement in both directions

c. the concrete offset should be at least 15 cm on all sides

d. all the above.

Explanation: Option D is correct.

236. The  bearing  capacity  of granite  is generally

a. 5 to 10 kg/cm2

b. 15 to 20 kg/cm2

c. 30 to 35 kg/cm2

d. 40 to 45 kg/cm2.

Explanation: Option C is correct.

237. A wall constructed with stones to protect slopes of cuttings in natural ground from the action of weathering agents, is called

a. retaining wall

b. breast wall

c. buttress

d. parapet wall.

Explanation: Option B is correct.

Retaining walls are provided to resisit earth fillings.
Whereas breast wall are provided to on naturally sloping grounds in cutting to resist the action of weathering.

238. Pick up the correct statement from the following :

a. sand consists of coarse particles of silica formed due to the disintegration of rocks.

b. the grains of sand are not affected by frost

c. sand beds are permeable and do not allow water to rise up between pores due to capillary action

d. all the above.

Explanation: Option D is correct.

239. The ceiling height of a building is

a. between ceiling and ground level

b. between ceiling and floor level

c. upto roof above ground level

d. upto ceiling from the ground level.

Explanation: Option B is correct.

240. Pick up the correct statement from the following :

a. A combined footing may be either rectangular or trapezoidal in shape

b. Rectangular footings are provided if two column loads are equal or interior column carries relatively greater load

c. Trapezoidal shaped footings may be provided under any loading

d. All the above.

Explanation: Option D is correct.

241. For a rectangular foundation of width b, eccentricity of load should not exceed

a. b/3

b. b/4

c. b/5

d. b/6.

Explanation: Option D is correct.

242. The width of the hollow space between two walls of a cavity wall should not exceed

a. 5 cm

b. 7.5 cm

c. 10 cm

d. 15 cm.

Explanation: Option C is correct.

243. The service area in a building means the area occupied by

a. stairs

b. toilets

c. light and shafts

d. all the above.

Explanation: Option D is correct.

244. The X-ray rooms are plastered with

a. Plaster of Paris

b. Barium plaster

c. Martin's cement

d. Keen's cement.

Explanation: Option B is correct.

245. Slate

a. is a metamorphic rock

b. splits into thin sheets along its bedding planes

c. has a smooth surface and contains alumina and silica

d. possesses good water absorption capacity

Explanation: Option A is correct.

Slate water absorption is 1% and less than 5% water absorption, consider as Good building stone.
So, Option D is absolutely Wrong.

246. The line of intersection of two surfaces of a sloping roof forming an internal angle less than 180°, is known as

a. ridge

b. hip

c. valley

d. none of these.

Explanation: Option C is correct.

247. The process of working a flat for the finishing coat, is known

a. dubbing out

b. floating

c. knetting

d. blistering.

Explanation: Option B is correct.

248. The type of arch used for high class buildings where appearance is of prime importance, is known as

a. ashlar arch

b. rubble arch

c. gauged brick arch

d. axed brick arch.

Explanation: Option A is correct.

249. The minimum strength of the mortar used in load bearing brick masonry, is

a. 50 N/cm2

b. 100 N/cm2

c. 150 N/cm2

d. 200 N/cm2.

Explanation: Option B is correct.

250. In ordinary residential and public buildings, the damp proof course is generally provided at

a. ground level

b. plinth level

c. water table level

d. midway ground level and watertable level.

Explanation: Option B is correct.

251. Under reamed piles are generally used for

a. factory buildinp

b. transmission linetowers

c. tall structures.

d. All the above.

Explanation: Option D is correct.

252. The thickness of a reinforced brick partition wall, is generally kept

a. 10 cm

b. 15 cm

c. 20 cm

d. 25 cm.

Explanation: Option A is correct.

253. The wedge shaped bricks forming an arch ring, are called

a. Soffits

b. voussoirs

c. haunchs

d. spandrils.

Explanation: Option B is correct.

254. A pointed arch which forms isosceles or equilateral triangle, is generally known as

a. three centred arch

b. two centred arch

c. Lancet arch

d. Bull's eye arch.

Explanation: Option C is correct.

255. The art of bringing the floor to a true level surface by means of screads, is called

a. topping

b. bedding

c. screading

d. none of these.

Explanation: Option C is correct.