***Sub Engineer***

1. Surge tanks are used

a. for storage water

b. to increase the velocity in a pipeline

c. as overflow valves

d. to guard against water hammer.

Explanation: Option D is correct.

A Surge tank is a water storage device used as a pressure neutralizer in hydropower water conveyance systems in order to dampen excess pressure variance.

2. As per IS : 1172-1963, water required per head per day for average domestic purposes, is

a. 65 litres

b. 85 litres

c. 105 litres

d. 135 litres.

Explanation: Option D is correct.

3. In slow sland filters, the turbidity of raw water can be removed only up to

a. 60 mg/litre

b. 75 mg/litre

c. 100 gm/litre

d. 150 mg/litre.

Explanation: Option A is correct.

4. Biochemical Oxygen Demand (B.O.D.) of safe drinking water must be

a. nil

b. 5

c. 10

d. 15

Explanation: Option A is correct.

BOD5 in India is also because of 5 days working schedule and two days weekend off.
So, the reading is measured only for 5 days.

5. Acidity in water is caused due to

a. Free CO2

b. Iron sulphate

c. Aluminium sulphate

d. All the above.

Explanation: Option D is correct.

6. The transitional middle portion of a logistic curve follows

a. a geometric growth

b. a logarithmic growth

c. a first over curve

d. a constant rate.

Explanation: Option D is correct.

Logistic curve.
ABCDE.
AB - early growth.
BC - exponential or logarithmic growth.
CD - constant growth.
DE-  late growth.

7. Pick up the correct statement from the following :

a. Lead and barium salts have toxic effect

b. Arsenic and selenium are poisonous to human health

c. Higher copper content affects the lungs

d. All the above.

Explanation: Option D is correct.

8. Pick up the incorrect statement from the following regarding fire hydrants

a. Fire hydrants are fitted in water mains at 100 m to 150 m apart at fire

b. The minimum water pressure hydrants, is kept 1.5 kg/cm2

c. The water at pressure 1 to 1.5 kg/cm2 is made available for 4 to 5 hours for constant use

d. None of these.

Explanation: Option D is correct.

9. The specific retention is least in case of

a. Clay

b. Sand

c. Silt

d. Coarse gravel.

Explanation: Option D is correct.

Porosity is a measure of volume of voids. But not big voids (as in case of gravel).
Whereas permeability is the ability of any material to pass fluid from its voids.

10. B.O.D. of treated water should be

a. 25 ppm

b. 20 ppm

c. 30 ppm

d. Nil.

Explanation: Option D is correct.

11. Most commonly used pump for lifting water in water supply mains, is

a. axialflow pump

b. reciprocating pump

c. rotary type pump

d. centrifugal pumps

Explanation: Option A is correct.

The most commonly adopted pumps in water supplies are --> centrifugal pumps.
Pumps commonly used for lifting of water supply main is --> Axial flow pump.

12. Economic height of a dam is the height corresponding to which

a. cost of the dam per unit of storage is minimum

b. amount of silting is less

c. cost of dam per unit storage is maximum

d. free board provided is least

Explanation: Option A is correct.

Generally for the construction of big projects like dams, bridges etc.
The cost plays a very crucial role. Thus lesser the cost more is the economy.

13. Distribution mains of any water supply, is normally designed for its average daily requirement

a. 100%

b. 150%

c. 200%

d. 225%.

Explanation: Option D is correct.

Design criteria - coincidental draft or max hourly demand.
Max hourly demand = 1.5 x max daily demand.
                                      = 2.7 x avg daily demand.
                                      = 270%.

14. If pH value of water is

a. 7 water it is said to be neutral

b. less than 7 it is said to be acidic

c. more than 7 it is said to be alkaline

d. all the above.

Explanation: Option D is correct.

pH value of pure water is 7, it is neither acidic nor alkaline.When then pH value of water is below 7, it is said to be soft water and when the pH value is higher than 7 it is said to be Hard water.

15. In pressure supply mains, water hammer pressure is reduced by providing

a. sluice valves

b. air valves

c. pressure relief valves

d. none of the these.

Explanation: Option C is correct.

16. P0, P1, P2 be the populations of a city at times t0, t1 and t2 = 2t1, the saturation value of the population Ps of the city, is

a.

b.

c.

d.

Explanation: Option A is correct.

17. Water supply system includes

a. digging a well for water

b. construction of dams

c. construction of canals

d. entire arrangement from source to distribution.

Explanation: Option D is correct.

18. Corrosion of a pipe

a. reduces its carrying capacity

b. adds colour to water

c. adds odour to water

d. all the above.

Explanation: Option D is correct.

19. Most satisfactory formula for an estimate of fire demand Q for a city of population P in thousands for Indian conditions, is

a.

b. Q = 1640 √P (1 - 0.01 √P)

c. Q = 3180 √P

d. none of these.

Explanation: Option C is correct.

20. Pick up the correct statement from the following :

a. Detention period for plain sedimentation tanks ranges between 4 to 8 hours

b. Detention period for sedimentation tanks, using coagulants usually ranges between 2 to 4 hours

c. The horizontally flow velocity in sedimentation tanks, is generally limited to 0.3 m/minute

d. All the above.

Explanation: Option D is correct.

Detention period for plain sedimentation is 3 to 4 hr and for coagulated sedimentation is 2 to 2.5 hr.
The horizontal flow velocity in the sedimentation tank is limited to 0.3m/min.

21. According to Kuichling's formula, fire demand in litres per minute for a population of P thousands, is

a. 3182 √P

b.

c. 4637 √P [1 - 0.01 √P]

d. 5663 √P.

Explanation: Option A is correct.

22. The maximum permissible nitrites in public water supplies, is

a. Nil

b. 0.5 P.P.M.

c. 1.0. P.P.M.

d. 1.5 P.P.M.

Explanation: Option A is correct.

Nitrites are highly dangerous for human health.
It represents partial decomposition of organic matter and it should not be present in water at all.

23. A high velocity of wash water is required for

a. rapid gravity filter without strainers

b. slow sand filter with strainers

c. slow sand filter without strainers

d. none of these.

Explanation: Option A is correct.

24. In distribution pipes, drain valves are provided at

a. lower point

b. higher point

c. junction points

d. any where.

Explanation: Option A is correct.

In distribution pipes, drain valves are provided at Lower point.
In distribution pipes, Air valves are provided at higher point.

25. Corrosion of well pipes may not be reduced by

a. reducing the flow velocity

b. using thicker pipes

c. using screens having larger area of openings

d. none of these.

Explanation: Option C is correct.

If velocity will high Re will be more losses will be more so surface of pipe will get eroded if large size mesh used then head loss will be very less and erosion can't be controlled in this way.

26. Manholes along the mains from the source to a city are provided at 500 m intervals in

a. steel pipes

b. R.C.C. pipes

c. hume steel pipes

d. all the above.

Explanation: Option D is correct.

27. The population growth curve is

a. S-shaped curve

b. parabolic curve

c. circular curve

d. straight line

Explanation: Option A is correct.

The growth of population is represented by Logistic curve i.e. this curve seems like S shape. Tha's why it is S curve.

28. Turbidity of raw water is a measure of

a. suspended solids

b. acidity of water

c. B.O.D.

d. none of these.

Explanation: Option A is correct.

Turbidity is the measure of extent to which light is either absorbed or scattered by suspended material in water but it is not quantitative measure of suspended solid.

29. If V is total consumption of water in litres for a population of N individuals, per capita consumption or water allowance for the water supply Q, is given by

a.

b.

c.

d. none of these.

Explanation: Option C is correct.

30. Water supply includes

a. distribution of water to consumers

b. provision of hydrants for fire fighting

c. mains, sub-mains and branch lines of water supply

d. all the above.

Explanation: Option D is correct.

31. Asbestos pipes are

a. highly resistant to corrosion

b. high flexible to accommodate deflection upto 12°

c. very much smooth and hydraulically efficient

d. all the above.

Explanation: Option D is correct.

32. The maximum depth of sedimentation tanks is limited to

a. 3 m

b. 4 m

c. 5 m

d. 6 m.

Explanation: Option D is correct.

For continuous flow type tank -- Depth 3 to 4.5 m,
For intermittent settling tank -- Depth 2.5 to 6 m,
Therefore, max = 6m.

33. By boiling water, hardness can be removed if it is due to

a. magnesium sulphate

b. calcium nitrate

c. calcium bicarbonate

d. none of these.

Explanation: Option C is correct.

Lime treatment can remove CH caused by Ca and Mg and NCH caused by Mg.
Lime soda treatment can remove CH caused by Ca and Mg and NCH caused by Ca and Mg.
Ion exchange process can produce zero hardness water.

34. Q is the discharge from an unconfined tube well with depression head s through its pipe of radius rw. If the radius of influence is R, the length of the required strainer, is

a.

b.

c.

d.

Explanation: Option C is correct.

35. For determining the velocity of flow of underground water, the most commonly used non-empirical formula is

a. Darcy's formula

b. Slichter's formula

c. Hazen's formula

d. Lacy's formula.

Explanation: Option A is correct.

36. The least thickness of class B cast iron (spun) pipe, is

a. 7.2 mm

b. 7.9 mm

c. 8.6 mm

d. 10 mm.

Explanation: Option C is correct.

37. According to IS : 1172-1963, a minimum of 135 litres of water capita per day, is required for

a. Boarding schools

b. Nurses home and medical quarters

c. hostels

d. all the above.

Explanation: Option D is correct.

38. The R.L. of ground water table on the sides of a valley is 1505 m whereas R.L. of the stream water is 1475 m. If 60° slope consists of pervious soil between R.L. 1485 m to 1500 m, the gravity spring may be expected at the point of reduced level

a. 1500 m

b. 1505 m

c. 1475 m

d. 1485 m.

Explanation: Option D is correct.

39. The factor affecting per capita demand, is

a. climatic conditions

b. pressure in water mains

c. cost of water

d. all the above.

Explanation: Option D is correct.

40. Pick up the incorrect statement from the following. The underground sources of water, is from

a. springs

b. infiltration wells

c. storage reservoirs

d. none of these.

Explanation: Option C is correct.

41. Disinfection of drinking water, is done to remove

a. odour

b. bacterias

c. turbidity

d. colour.

Explanation: Option B is correct.

42. Sunlight

a. helps growth of bacterias

b. impedes growth of algae

c. increases dissolved oxygen content

d. reduces turbidity.

Explanation: Option B is correct.

Sunlight depletes nutrients from the surface of the water in summer. So the growth of algae reduces so b correct. But the same time due to sunlight due to photosynthesis DO increase. But due to high temperature DO decrease. So option B is correct.

43. The expected discharge to be obtained from an open well sunk in coarse sand is 0.0059 cumec. If the working depression head of the well is 3 m, the minimum diameter of the well, is

a. 2.25 m

b. 2.50 m

c. 2.75 m

d. 3.00 m.

Explanation: Option D is correct.

Q=KiA ; i=3m; A = 0.785d^2; K=1 Q=0.0059m^3/sec=21.24m^3/hr.     
21.24=1*3*0.785*d^2,
d^2=(21.24)/(3*0.785)=9.0191,
d=3m.

44. Average annual rainfall at any station is the average of annual rainfall over a period of

a. 14 years

b. 21 years

c. 28 years

d. 35 years.

Explanation: Option D is correct.

45. Time of concentration

a. is the time taken, for precipitation

b. duration of rainfall.

c. time taken for all the ran off to reach the drain

d. time taken for the storm water to travel from the most remote point to the drain.

Explanation: Option D is correct.

It is defined as the time needed for water to flow from the most remote point in a watershed to the watershed outlet.

46. If four fires break out in a city of population 40 lakhs and if each hydrant has three streams and duration of each fire is four hours, the total quantity of water required, is

a. 1880 kilo litres

b. 2880 kilo litres

c. 3880 kilo litres

d. 4880 kilo litres.

Explanation: Option B is correct.

Total quantity of water required = No of fires x Discharge x Time of each fire.
= 4 x (3 x 1000) x (4 x 60) = 2880000 liters = 2880 kilo liters. (where, 1000 liters/minute is discharge of each stream).

47. The fire demand of a city may be worked out by

a. Freeman formula

b. Under Writers formula

c. Bustan's formula

d. All the above.

Explanation: Option D is correct.

Kuchling's = 3182√p.
Freeman's = 1136((p/5)+10).
Bustan's = 5663√p.
National board of fire = 4367√p (1 - 0.01√p).

48. Aeration of water is done to remove

a. odour

b. colour

c. bacterias

d. hardness

Explanation: Option A is correct.

Aeration is done to remove unpleasant tastes and odour. So, according to option only (A) is right.
Also, activated carbon, ozone, etc can be used for aeration.

49. The yield of a rapid gravity filter as compared to that of slow sand filter, is

a. 15 times

b. 20 times

c. 30 times

d. 35 times.

Explanation: Option C is correct.

Slow sand filter -> 100 - 200 litre per hr per m2.
Rapid sand filter -> 3000 - 6000 litre per hr per m2.
Pressure filter -> 10000 - 15000 litre per hr per m2.

50. While determining the yield of open wells by the pumping test

a. velocity of recharging water, increases with depression head

b. depression head resulting at critical velocity, is called critical depression head

c. working head is generally limited to  rd of the critical depression head

d. maximum safe yield of an open well, is expected at critical depression head.

Explanation: Option D is correct.

51. Increase in population of a rapidly growing city, may be estimated by

a. arithmetical mean method

b. geometrical method

c. incremental increase method

d. graphical comparison method.

Explanation: Option B is correct.

Young city &new City is adopted Geometrical method note- Geometric method is more precise than arithmetic mean method.

52. Pick up the correct statement from the following regarding the pressure conduits :

a. Pressure conduits are permitted to run  th full

b. Pressure conduits are always laid along down grades

c. The hydraulic gradient line always coincides the invert of the conduit

d. None of these.

Explanation: Option D is correct.

53. The standard B.O.D. at 20&Deg;C, is taken for the consumption in

a. 2 days

b. 3 days

c. 4 days

d. 5 days.

Explanation: Option D is correct.

The BOD value is most commonly expressed in milligrams of oxygen consumed per litre of the sample during 5 days of incubation at 20°C.

54. The superimposed load transmitted to the pipe, is generally evaluated by Bousiness formula

a.

b.

c.

d.

Explanation: Option B is correct.

55. Percussion drilling is unsuitable in

a. unconsolidated sand

b. unconsolidated gravel

c. quick sand

d. consolidated rocks.

Explanation: Option D is correct.

PERCUSSION.
Although the technique is slower than other drilling techniques, it is the only manual drilling technique that is able to drill through consolidated rock layers.

56. Disinfection of water with ozone is not good because

a. it removes the colour, taste and odour from water as bacterias

b. it adds taste to the water

c. it is more efficient than chlorine in killing bacterias

d. none of these.

Explanation: Option C is correct.

I also marked the option A, as in chlorination residual chlorine is left in water so as to avoid any contamination during distribution.

57. An earth formation which, although porous and capable of absoring water does not provide an appreciable supply to wells, is known as

a. acquifer

b. aquiclude

c. aquifuge

d. none of these.

Explanation: Option B is correct.

Acquifer - Porous and Permeable.
Aouitard - Porous and Semi impermeable.
Aquiclude - Porous and Impermeable.
Aquifuge - Non porous and impermeable.

58. Mathamoglobinemia, or blue baby disease is caused due to

a. chlorides

b. nitrites

c. nitrates

d. sulphides.

Explanation: Option C is correct.

59. The main process to purify water by filtration, is

a. flocculation and sedimentation

b. biological metabolism

c. electronic change

d. all the above.

Explanation: Option D is correct.

60. Pick up the correct statement from the following :

a. The internal pressure in a pipe running full is equal to vertical ordinate between hydraulic gradient line and centre of the pipe

b. In pressure pipes with water at rest, the pressure is equal to water head

c. Sudden closure of a valve, causes the water hammer pressure

d. All the above.

Explanation: Option D is correct.

61. Surface water is obtained from

a. well

b. springs

c. artesian well

d. rain.

Explanation: Option D is correct.

62. Hardness of water is caused due to

a. magnesium sulphate

b. calcium nitrates

c. calcium bicarbonates

d. all the above.

Explanation: Option D is correct.

Hardness of water is mainly due to the presence of multivalent metallic cations like mg+2, ca+2, al+3, etc.

63. Flow through period, in sedimentation tanks, is

a. more than detention period

b. less than detention period

c. detention period divided by displacement efficiency

d. none of these.

Explanation: Option B is correct.

Flowing through period is the time taken by a batch of water to travel from the entry to exit of settling tank. Because of short circuits produced in water current, flowing through period is less than the detention period.

64. Normal values of overflow rate for plain sedimentation tank, is

a. 250 to 500 litres/hr/m2

b. 500 to 750 litres/hr/m2

c. 750 to 1000 litres/hr/m2

d. 1000 to 1250 litres/hr/m2.

Explanation: Option B is correct.

Surface overflow rates:
Plain sedimentation :
V = 12 to 18 m3/day/m2.
V = 500 to 750 Liters/hr/m2.
Coagulation sediment:
V = 24 to 30 m3/day/m2.
V = 1000 to 1250 Liters/hr/m2.

65. If p is total internal pressure, d is diameter of pressure conduit and t is thickness of conduit, the Hoop's stress is

a.

b.

c.

d.

Explanation: Option C is correct.

66. The force which develops in a pressure conduit supported on trestles, is

a. tension

b. compression

c. temperature stress

d. flexural stress.

Explanation: Option D is correct.

Trestles are supports provided for pipe. This makes a pipe line act like a beam. So when water flows, flexural stress is induced.

67. After cleaning a slow sand filter, the filtered water is not used for

a. 12 hours to 18 hours

b. 18 hours to 24 hours

c. 24 hours to 36 hours

d. 48 hours.

Explanation: Option C is correct.

It is 24 to 48 hours since we need to wait for the formation of schmudzdecke layer, which plays a crucial role in the working of slow sand filter.

68. A pressure conduit laid under ground, may not be subjected to

a. pressure due to external load

b. longitudinal temperature stress

c. longitudinal stresses due to unbalanced pressure to bends

d. none of these.

Explanation: Option B is correct.

69. The period of cleaning of a slow sand filter, is usually

a. two weeks to three weeks

b. one month to three months

c. three months to six months

d. one year.

Explanation: Option B is correct.

70. At the socket and spigot joint,

a. normal end of the pipe is called spigot

b. spigot is fitted into the socket

c. molten lead is used for water proofing

d. all the above.

Explanation: Option D is correct.

71. Gravity conduits for carrying water from the source are

a. canals

b. flumes

c. acqueducts

d. all the above.

Explanation: Option D is correct.

72. Continuous flow of water can be expected from

a. gravity springs

b. surface springs

c. artesian springs

d. none of these.

Explanation: Option C is correct.

73. The maximum hourly consumption, is generally taken as

a. 120%

b. 130%

c. 140%

d. 150%.

Explanation: Option D is correct.

Minimum daily flow = 2/3 * average daily flow.
Minimum hourly flow = 1/3 * average daily flow.
Maximum daily flow = 2 * average daily flow.
Maximum hourly flow = 3 * average daily flow.

74. A city supply includes

a. industrial and commercial water demands

b. demand for public uses and fire

c. water losses

d. all the above.

Explanation: Option D is correct.

75. During treatment of water, sedimentation is done

a. before filtration

b. after filtration

c. simultaneously with filtration

d. along with chlorination.

Explanation: Option A is correct.

The sequence of treating water is screening-sedimentation-filtration-disinfection. Hence this option A is correct.

76. E. Coli bacterias die in water having pH greater than

a. 6.5

b. 7.5

c. 8.5

d. 9.5

Explanation: Option D is correct.

If ph greater than 9 the bacteria will going to die so we will add lime to increase alkalinity in excess lime treatment of disinfection process.

77. Check valves are installed

a. on the delivery side of the pumping set

b. at the interconnections between polluted water system and a potable water system

c. both (a) and (b)

d. neither (a) nor (b).

Explanation: Option C is correct.

Both are incorrect because the check valve is installed suction side and b check valve could not stop bacterias from polluted water to potable water.

78. The equation  in which Q is yearly run off in cm, P is yearly total rainfall in cm, H is difference of R.L.s. of lowest and highest points and A is area of catchment in square metres, is known as

a. English formula

b. Khosla's formula

c. Justin's formula

d. Vermule's formula.

Explanation: Option C is correct.

79. If intensity of rainfall in cm per hour is I, percentage coefficient of run-off is P, area of catchment in square kilometres is A, the total run-off Q, is given by

a. Q = 2.758 x 102 x API

b. Q = 2.758 x 103 x API

c.

d.

Explanation: Option B is correct.

80. Hard water contains

a. calcium

b. magnesium bicarbonates

c. magnesium sulphate

d. all the above.

Explanation: Option D is correct.

81. The main disadvantage of hard water, is

a. scaling of boilers

b. corrosion and incrustation of pipes

c. making food tasteless

d. all the above.

Explanation: Option D is correct.

Option A is correct.
Explanation:
Bathing: Hard water does not lather freely with soap solution, but produces sticky scum on the bath-tub and body. Thus, the cleansing quality of soap is depressed and a lot of it is wasted.

82. An area is declared drought affected if its mean rainfall is less than

a. 60%

b. 75%

c. 80%

d. 85%.

Explanation: Option D is correct.

The Indian Meteorological Department (IMD) has defined drought as a situation occurring in any area when the mean annual rainfall is less than 75% of the normal rainfall.

83. The best quality of filter material is obtained from quartzite if it does not loose weight when placed in hydro-chloric acid for 24 hours, more than

a. 5%

b. 8%

c. 10%

d. 12%

Explanation: Option A is correct.

84. If discharge of a pump is 0.16 cumecs, the economic diameter of pipe, is

a. 0.488 m

b. 4.88 cm

c. 0.488 cm

d. 4.88 m

Explanation: Option A is correct.

85. Hard water for public water supply is discarded because

a. it consumes more soap

b. it contains lot of turbidity

c. it contains pathogenic bacterias

d. it possesses bad taste and odour

Explanation: Option A is correct.

86. Pick up the correct statement from the following regarding radial flow centrifugal pumps :

a. In involute type of radial flow centrifugal pump, the impeller discharges into a gradually expanding spiral casing

b. In turbine type of radial flow centrifugal pumps, the impeller is surrounded by stationary guide vanes to reduce the velocity of water

c. The efficiency of turbine type of radial flow centrifugal pump, is always higher than that of volute type

d. All the above.

Explanation: Option D is correct.

87. Quality of water is said to be good if it is

a. colourless

b. free from pathogenic organism

c. tasteless

d. all the above.

Explanation: Option D is correct.

88. Pick up the incorrect statement from the following :

a. An impervious layer a few metres below the water table in the sub-soil, is generally called Mota layer (6) Mota layer is very useful to give structural support to open deep wells

b. A bore hole is generally provided in the mota layer for deep wells

c. Shallow wells are always of less depth as compared to deep wells

d. None of these.

Explanation: Option D is correct.

Option C is incorrect as this terminology has nothing to do with depth. Sometimes shallow wells have more depth than deep wells.

89. The main draw-back of centrifugal pump, is

a. discharge from pump varies with the load of water

b. for high heads, efficiency is low up to 50%

c. the suction lift is limited to 6 m

d. all the above.

Explanation: Option D is correct.

90. The water level in an open well was depressed by pumping 2.5 m and recuperated 2.87 m in 3 hours and 50 minutes. The yield of the well per minute is

a. 0.0033

b. 0.0044

c. 0.0055

d. 0.0066

Explanation: Option C is correct.

Yield=(2.3/T)*log(s1/s2),
T= time after the pumping is stopped=3.83hrs,
s1= depression head=2.5m,
s2= recuperation head=2.5-2.87=0.37m,
Y=(2.3/3.83)*log(2.5/0.37),
Y=0.49828 m2/hr,
Y=0.0083 m2/min.

91. Rapid gravity filter can only remove turbidity of water upto

a. 25 to 30 gm/litre

b. 30 to 35 cm/litre

c. 35 to 40 gm/litre

d. 50 gm/litre.

Explanation: Option C is correct.

92. Pick up the correct statement from the following :

a. The amount of rainfall in one year, is known as annual rainfall

b. The rain cycle period in India is taken as 35 years

c. The ratio of the actual rainfall in the year to the normal rainfall at a place, is called index of the wetness

d. All the above.

Explanation: Option D is correct.

93. The best process of disinfection of public water supply, is by

a. boiling

b. chlorination

c. adding lime

d. adding zone.

Explanation: Option B is correct.

Because chlorination is cheaper and also future contamination can be prevented due to residual chlorine present in water.

94. Pick up the correct statement from the following :

a. A mass curve is a plot of accumulated flow versus time

b. The mass curve continuously rises

c. The slope of the mass curve at any time is a measure of the inflow rate at that time

d. All the above.

Explanation: Option D is correct.

95. A water channel supported above the ground over trestles, is generally called

a. flume

b. canal

c. adueduct

d. tunnel

Explanation: Option A is correct.

A flume is a human-made channel for water in the form of an open declined gravity chute whose walls are raised above the surrounding terrain.

96. If d is the diameter of the pipe, p is the total internal pressure, f is the permissible tensile stress and n is the effective of the joint, the thickness t of metal pipe, is

a.

b.

c.

d. none of these.

Explanation: Option A is correct.

97. One degree of hardness of water means a content of salts of

a. 12.25 mg/litre

b. 14.25 mg/litre

c. 16.25 mg/litre.

d. 20 mg/litre.

Explanation: Option B is correct.

98. Mostly used coagulant, is

a. chlorine

b. alum

c. lime

d. bleaching powder.

Explanation: Option B is correct.

99. If G is the specific gravity of particles of diameter d, the velocity of settlement V in still water at T°C, according to Stoke's law, is

a.

b.

c.

d.

Explanation: Option A is correct.

When d<0.1mm
V=418(G-1)d^2{(3T+70)/100}.
When 0.1<d<1mm
V= 418(G-1)d{(3T+70)/100}.
When d>1mm
V = 1.8*(gd(G-1)).

100. The most commonly used chemical for dechlorination of water, is

a. sodium bisulphate

b. sodium sulphite

c. sulphur-dioxide

d. all the above.

Explanation: Option B is correct.

When residual chlorine is removed from water which is disinfected then the process is known as dechlorination. The most commonly used chemical for dechlorination of water is sulfur dioxide.

101. The prescribed hardness limit of potable water ranges between

a. 75 to 115 P.P.M.

b. 100 to 150 P.P.M.

c. 150 to 200 P.P.M.

d. none of these.

Explanation: Option A is correct.

As per WHO  hardness.
Less than 60ppm is soft,
60 to 120 ppm moderately soft,
120 to 180 ppm hard,
Greater than 180 very hard,
As per IS 10500,
Limited up to 300ppm,
Maximum limit of 600ppm.

102. For centrifugal pumps

a. initial cost is low

b. limited space is required

c. the discharge obtained is steady and non-pulsating

d. all the above.

Explanation: Option D is correct.

103. The cast iron pipes for water supply system are used for

a. strength

b. easy connection

c. low maintenance cost

d. all the above.

Explanation: Option D is correct.

104. For plain chlorination of water, the quantity of chlorine used, is

a. 0.2 mg/litre

b. 0.3 mg/litre

c. 0.4 mg/litre

d. 0.5 mg/litre.

Explanation: Option D is correct.

Super chlorination is the application of chlorine beyond the stage of the break point. It is followed by a contact period of 30-60 minutes.

105. For estimating the run off of catchments, the mean value of the constant 'K' in the Khosla's formula Qy = Py - K (1.8 Ty + 32) is

a. 1.23

b. 1.25

c. 1.27

d. 1.31

Explanation: Option C is correct.

106. Service connections to consumers houses, are generally provided with

a. hume pipes

b. galvanised iron pipes

c. P.V.C. pipes

d. none of these.

Explanation: Option B is correct.

It's not PVC. Other than G.I pipes, the department does not allow pipeline connections (in the states).

107. Sluice valves are fitted in a distribution system

a. along straight length of pipes at suitable intervals

b. at the junctions of the pipes

c. at the branching off points of sub-mains

d. all the above.

Explanation: Option D is correct.

108. The detention period for plain sedimentation water tanks, is usually

a. 4 to 8 hours

b. 8 to 16 hours

c. 16 to 24 hours

d. 24 to 36 hours.

Explanation: Option D is correct.

Fill and draw-type of sedimentation tank 24 to 36 hrs.
Continuous flow type 2 to 4 hr for CAS.
And 4 to 8 hrs for plain sedimentaton.

109. The maximum permitted loss of head in a rapid sand filter, is

a. 2 m

b. 3 m

c. 4 m

d. 5 m.

Explanation: Option B is correct.

Slow sand filter:
Loss of head: 0.1 m ( initial limit).
: 0.8 to 1.2 m ( final limit).
Rapid sand filter :
Loss of head: 0.3 m ( initial limit).
: 2.5 to 3.0 m (final limit).

110. Mud balls may be removed by

a. breaking and washing

b. washing the filter with a solution of caustic soda

c. removing, cleaning and replacing the damaged sand

d. all the above.

Explanation: Option D is correct.

Mud balls are formed in filters due to insufficient washing of sand grains. Mud accumulates on the sand surface and forms a dense mat.

111. For maximum alkalinity of water, pH value should be

a. less than 7

b. more than 7

c. 14

d. 21

Explanation: Option C is correct.

0 to 6.5 acidic.
7 neutral.                                                 
7.5 to 14 alkaline.

112. The fire demand for a city of 50, 000 population, according to Godrich formula, is

a. 40 mld

b. 42 mld

c. 44 mld

d. 48 mld.

Explanation: Option C is correct.

Goodrich's formula.
P = 180*t^(-0.1),
P = Population in thousand.
t = time in days.
Goodrich's formula is to find the peak demand for water. Not for fire demand.

113. The efficiency of sedimentation tank does not depend upon

a. depth of tank

b. length of tank

c. detention period

d. velocity of water.

Explanation: Option A is correct.

114. When the reduced level of the water source is higher than the reduced level of the consumer's place, water is generally supplied

a. by pumping system

b. by gravitational system

c. both (a) and (b)

d. all the above.

Explanation: Option B is correct.

115. Most commonly used section in grade aqueducts, is

a. circular

b. rectangular

c. parabolic

d. horse shoe section.

Explanation: Option B is correct.

116. Pressure exerted at 90° bend in a pipe of cross-sectional area A and carrying water with a velocity V under a pressure p, is

a.

b.

c.

d.

Explanation: Option C is correct.

117. The lowest outlet sluice in a dam is provided

a. below the dead storage

b. on the top level of dead storage

c. on the top level of useful storage

d. at the centre of the dam.

Explanation: Option B is correct.

118. The four major water supply distribution systems, are

a. dead end, tree, grid iron and reticulation

b. dead end, tree, grid iron and circular

c. tree, grid iron, ring and radial

d. tree, reticulation, circular and ring.

Explanation: Option C is correct.

It's
1) Dead end method/ Tree method.
2) Grid iron method.
3) Circular method/Ring Method.
4) Radial method.

119. Most important source of water for public water supply, is from

a. ponds

b. streams

c. rivers

d. sea.

Explanation: Option C is correct.

Sea water is saltish and requires enormous treatment to make it adequate to Drink River water is good source for public Water supply as it is economical to treat.

120. In pumping stations, the type of joint generally used, is

a. flanged joint

b. expansion joint

c. dresser coupling joint

d. flexible joint.

Explanation: Option A is correct.

Flexible joint : This joint is used at such places where settlement is likely to occur after laying of the pipe.

121. The permissible pH value for public water supplies may range between

a. 5.5 to 6.5

b. 6.5 to 8.5

c. 8.5 to 10.5

d. none of these.

Explanation: Option B is correct.

122. The strainer type tube well, is unsuitable for

a. coarse gravels

b. fine sandy strata

c. clean gravels

d. none of these.

Explanation: Option B is correct.

123. Pick up the correct statement from the following :

a. Excessive use of lime may kill the bacterias

b. Excessive lime when added to water, raises its pH value

c. When lime raises pH value of water to about 9.5, 100% bacterias are removed

d. All the above.

Explanation: Option D is correct.

124. Specific capacity or yield of wells, is generally expressed, as

a. m3/hour

b. m3/hour/m2

c. m3/hour/m2/m

d. m3/hour/m2/sec.

Explanation: Option C is correct.

Specific Yield of Wells: Rate of water percolation in the well or yield of a well in m3/hr under a head of one metre is called the specific yield of the well.

125. The depth of the water table at a place is 45 m below the general ground level. To lift water from a deep tube well in such a locality, the type of pump to be installed is

a. Centrifugal pump

b. Reciprocating pump

c. Deep well turbine pump

d. None of these.

Explanation: Option C is correct.

126. Critical time for developing a water hammer, is the time required for

a. closing the valve

b. the wave to travel from valve to the reservoir

c. the wave to travel from the valve to the reservoir and back

d. none of these.

Explanation: Option C is correct.

127. According to Buston's formula, fire demand in litres per minute for a population of P thousands, is

a.

b. 4637 √P (1 - 0.01√ P)

c. 5663 √P

d. none of these.

Explanation: Option C is correct.

128. The ratio of maximum hourly consumption and average hourly consumption of the maximum day, is

a. 1.5

b. 1.8

c. 2.4

d. 2.7

Explanation: Option D is correct.

The ratio of max. hourly consumption =( 2.7 LPCD /24).
The ratio of avg. hourly consumption = (1.8 LPCD /24).
So, Ans is  B

129. Cast iron pipes are generally preferred to, because

a. of ease to join

b. of durability

c. of longer life

d. all the above.

Explanation: Option D is correct.

130. The fire demand for ascertaining the empirical formula  known as

a. Buston's formula

b. Freeman formula

c. Under Writers formula

d. None of these.

Explanation: Option B is correct.

131. To detect the turbidity of the order of 0 to 1000 P.P.M. the instrument used is

a. turbidimeter

b. Jackson turbidimeter

c. Baylis turbidimeter

d. Hallige turbidimeter.

Explanation: Option C is correct.

Practically it is not possible to use Jackson's turbidimeter for the water of 1000 PPM and is possible with Baylis's hence Option-C is correct.

132. Standard process of chlorination of water, is done by

a. plain chlorination

b. pre-chlorination

c. post-chlorination

d. double chlorination.

Explanation: Option C is correct.

133. If the chosen diameter of a pipe, is less than the economical diameter

a. cost of pipe will be less

b. head loss will be high

c. cost of pumping will be more than saving

d. all the above.

Explanation: Option D is correct.

134. Water gets evaporated from water surfaces and land surfaces, get converted into water drops at lower temperatures, flows over ground surface and finally meets its source, i.e. lake, sea, etc. This entire process is generally known as

a. hydrological cycle

b. water cycle

c. evaporation and precipitation cycle

d. all the above.

Explanation: Option D is correct.

135. In the equation  of a logistic curve of population growth, the constant m is

a. Ps x P

b.

c.

d. KPs .

Explanation: Option C is correct.

136. High pH value of water does not produce

a. sediment deposits

b. turberculation

c. corrosion

d. both (c) and (d).

Explanation: Option D is correct.

137. Distribution of wash water is provided in

a. sedimentation tank

b. slow sand filter

c. rapid gravity filter

d. all the above.

Explanation: Option C is correct.

138. The valves provided at low points of pipes to drain off water quickly under gravity, are called

a. blow off valves

b. drain valves

c. sewer valves

d. all the above.

Explanation: Option D is correct.

Blow off valve.
Drain valve.
Sewer valve.
Shut off valve.
Flow off valve.
Washout valve.
Scour valve.
All are  same.

139. Pick up the incorrect statement from the following :

a. Water table surface rises and falls with seasons

b. Depth of water table is directly proportional to the rate of drawal of water

c. Level of water table remains stationary

d. None of these.

Explanation: Option C is correct.

As the rate of withdrawal increases the depth of the water table should decrease, therefore statement C is also incorrect.

140. The bacterias which require free oxygen for their survival, are called

a. Aerobic bacterias

b. Anaerobic bacterias

c. Faculative bacteria

d. None of these.

Explanation: Option A is correct.

Facultative bacteria : Bacteria that can use dissolved oxygen or oxygen obtained from food materials such as sulfate or nitrate ions, or some can respire through glycolysis. The bacteria can live under aerobic, anoxic, or anaerobic conditions.

141. Maximum threshold number permitted for indicating the odour of public water supplies, is

a. 2

b. 3

c. 4

d. 5

Explanation: Option B is correct.

The drinking water should not have any objectionable taste and odour and for this the flavour threshold number (FTN) and the threshold odour number (TON) should not exceed 3 and should preferably be 1.

142. 45 litres of water per person per day, is provided in

a. office buildings

b. hotels

c. hostels

d. nurse's homes.

Explanation: Option A is correct.

IS code : 1172-1963 Indian standard code of basic requirements for water supply, drainage and sanitation.

143. Turbidity of water may be caused due to

a. Suspended clay

b. Suspended silt

c. finely divided organic material

d. all the above.

Explanation: Option D is correct.

144. Pick up the correct statement from the following :

a. Water with hardness more than 200 ppm is considered hard

b. Water needed for laundaries should contain hardness less than 75 ppm

c. To provide taste during drinking, water should possess hardness between 75 to 120

d. All the above.

Explanation: Option D is correct.

145. The maximum pressure to which cast iron pipes may be subjected to, is

a. 3 kg/cm2

b. 5 kg/cm2

c. 7 kg/cm2

d. 10 kg/cm2.

Explanation: Option C is correct.

146. The type of pipe commonly used in water supply distribution schemes, is

a. R.C.C. pipes

b. Hume pipes

c. Cast iron pipes

d. G.I. pipes.

Explanation: Option C is correct.

Options A, B, C are not pressurized pipes generally used to carry sewage. Actually steel pipes are pressurized pipes used in distribution system. So Opt. D is correct.

147. The valve shown in the below figure, is generally known as

a. air valve

b. pressure relief valve

c. sluice valve

d. none of these.

Explanation: Option B is correct.

148. The population of a city in 2000 is 50, 000. The average per decade of the previous records of population is 5000 and average percentage per decade is 20%. The population of the city based on geometrical increase method, in the year 2020 will be

a. 60, 000

b. 64, 000

c. 70, 000

d. 72, 000

Explanation: Option D is correct.

pn = p(1+%/100)^n.
n = no of decade.
so p2 = 50000(1+20/100)^2.
= 50000*36/25 = 72000.
p2 means after 2 decade the population is 72000.

149. Pick up the correct statement from the following :

a. The odour of water sample is generally measured by a term called odour intensity

b. The colour of water sample may be detected by Nessler tube

c. Specific conductivity of water measures the amount of dissolved salt

d. All the above.

Explanation: Option D is correct.

150. Before constructing a dam, the factor to be considered for controlling sedimentation, is

a. construction of check dams

b. providing vegetation screens

c. providing under-sluices in the dam

d. All the above.

Explanation: Option D is correct.

151. The process of passing water through beds of granular materials, is called

a. screening

b. sedimentation

c. filtration

d. none of these.

Explanation: Option C is correct.

152. If Q is discharge is cubicmetres per sec and D is the economical diameter of the pipe. According to Lea

a. D = 0.67 to 0.87 √Q

b. D = 0.77 to 0.97 √Q

c. D = 0.97 to 1.22 √Q

d. D = 1.22 to 1.33 √Q.

Explanation: Option C is correct.

153. Water to the consumers may be supplied from

a. infiltration galleries connected to sump well

b. infiltration well dug out on the banks of rivers

c. ranney wells sunk to the water level

d. all the above.

Explanation: Option D is correct.

154. If the average daily demand of a city of 50, 000 population, is 20 m.l.d., the maximum daily demand is

a. 24 mld

b. 30 mld

c. 36 mld

d. 54 mld.

Explanation: Option C is correct.

Max daily demand = 1.8 * avg daily demand.
Max hourly demand= 1.5* max daily demand.
Max hourly demand= 2.7 *avg daily demand.

155. Recuperation test is carried out to determine

a. turbidity of water

b. pH value of water

c. yield of well

d. discharge from a well.

Explanation: Option C is correct.

156. The discharge  for the confined tube well is obtained from

a. Thiem's formula

b. Darcy's formula

c. Tolman's formula

d. Dupuits formula.

Explanation: Option D is correct.

This equation for confined well obtained from steady flow using darcy equation and the equation achieved is called Thieme's equation.

157. Pick up the correct statement from the following :

a. Jet pumps are generally used for pumping water from small wells

b. The hydraulic ram works on the principle of water hammer

c. Rotary pumps are provided cams which rotate in opposite direction

d. all the above.

Explanation: Option D is correct.

158. Turbidity of water is expressed

a. in ppm

b. in numbers in an arbitrary scale

c. by pH value

d. by colour code.

Explanation: Option A is correct.

159. The ratio of the maximum daily consumption to the average daily demand, is

a. 1.2

b. 1.4

c. 1.6

d. 1.8

Explanation: Option D is correct.

1. Maxi daily demand to average daily demand = 1.8,
2. Hourly water demand to average hourly demand = 2.7,
3. Maximum monthly water demand to average monthly demand = 1.28.

160. The ratio of discharge and plan area of a continuous flow type settling tank, is known

a. surface loading

b. overflow

c. overflow rate

d. all the above.

Explanation: Option D is correct.

The volume of water applied per unit time per unit horizontal surface area is called over flow velocity. It is also called as surface loading rate or surface over flow rate.

161. Pick up the correct statement from the following :

a. At zero discharge, discharge valve remaining closed, the head developed is maximum which is known as shut-off head

b. At a particular speed the discharge at which efficiency of a pump is maximum, is known as normal discharge

c. The brake horse power required from the primemover to drive the pump, increases with the increase of discharge

d. all the above.

Explanation: Option D is correct.

162. The maximum pressure to which a pipe is subjected to during its operation, is known

a. working pressure

b. design pressure

c. test pressure

d. pipe pressure

Explanation: Option A is correct.

163. To control the growth of algae in reservoirs, the compound which is used, is

a. copper sulphate

b. lime solution

c. alum solution

d. all the above.

Explanation: Option A is correct.

164. Pick up the wrong nominal internal diameter of cast iron (spun) pipes in mm from the following :

a. 400

b. 500

c. 550

d. 600

Explanation: Option C is correct.

165. Hardness of water can be removed by boiling if it is due to

a. calcium bicarbonates

b. calcium sulphates

c. calcium chloride

d. calcium nitrates.

Explanation: Option A is correct.

166. A well is considered to be good if it is sunk into

a. Clay

b. Sand

c. Coarse gravel

d. Silt.

Explanation: Option C is correct.

167. The nitrate concentration in domestic water supplies, is generally limited to

a. 15 ppm

b. 25 ppm

c. 30 ppm

d. 45 ppm.

Explanation: Option D is correct.

168. Sluice valves in main water supplies

a. are used to regulate the flow of water in pipes

b. are spaced about 5 km apart

c. are usually placed at the summits

d. all the above.

Explanation: Option D is correct.

169. Dead storage of a reservoir of Q capacity generally provided for silt deposition during its life time, is generally kept

a. 1/6th Q

b. 1/5th Q

c. 1/4th Q

d. 1/3rd Q.

Explanation: Option C is correct.

170. If the specific capacity of a well is 0.3183 x 10-3 per sec, the discharge from a well of 4 m diameter under a depression head of 4 m, is

a. 10 litres/sec

b. 12 litres/sec

c. 14 litres/sec

d. 16 litres/sec.

Explanation: Option D is correct.

Discharge Q = C*A*H.
C = Specific capacity of well.
A = Area of the well.
H = Depression head.
Q = 16 lit/sec.

171. The factor to be considered for the source of city water supply, is

a. quantity and quality of the available water

b. elevation of the source of water

c. general terrain intervening the area

d. all the above.

Explanation: Option D is correct.

172. Pick up the correct statement from the following :

a. Average consumption of commercial use of water is 25% of total consumption

b. Consumption of water on public use is 10% of total consumption

c. Waste water and leakage is 15% of total consumption

d. all the above.

Explanation: Option D is correct.

173. Yield of a drainage basin is :

a. run off of the area expressed as instantaneous rate

b. average run off over a short period

c. total volume of water flowing annually

d. all the above.

Explanation: Option C is correct.

174. Normal values of overflow rate for plain sedimentation tank using coagulants, is

a. 750 to 1000 litres/hr/m2

b. 1000 to 1250 litres/hr/m2

c. 1250 to 1500 litres/hr/m2

d. 1500 litres/hr/m2.

Explanation: Option B is correct.

175. The formula Q = P - K[1.8T + 32] in which Q is runoff, P is annual rain fall in cm, T is mean annual temperature in centigrades and K is a constant, is known

a. Justin's formula

b. Khosla's formula

c. English formula

d. Vermule's formula.

Explanation: Option B is correct.

176. The formula for pipe flows, suggested by, Hazen-William is

a.

b. V = 0.85 CH R0.63 . S0.54

c.

d. None of these.

Explanation: Option B is correct.

177. While selecting the location of an intake for collecting surface water, the factor considered, is

a. The intake point should be in purer zone of the water source

b. The intake point should be upstream of the point of disposal of waste water

c. The intake point in meandering rivers should be on concave banks

d. All the above;

Explanation: Option D is correct.

178. Aqueducts are generally designed

a. circular

b. rectangular

c. horse shoe section

d. all the above.

Explanation: Option D is correct.

179. Gravity conduits

a. follow the hydraulic gradient line

b. are carried on trestles in valleys and depressions

c. are carried through tunnels in deep cuttings

d. All the above.

Explanation: Option D is correct.

180. Carbonates in water produce

a. temporary hardness

b. permanent hardness

c. acidity

d. alkanity.

Explanation: Option A is correct.

Temporary hardness : due to bicarbonates of calcium and it is removed by boiling, freezing and by addition of lime.
Permanent hardness : due to chlorides and sulphates of calcium and magnesium. And it is removed by zeolite process.

181. Hardness of water is caused by

a. presence of soap lather

b. presence of chlorides and sulphates of sodium and potassium

c. presence of bicarbonates, sulphates or chlorides of calcium and magnesium

d. turbidity.

Explanation: Option C is correct.

182. A slow sand filter is cleaned if its filter head is higher than

a. 20 cm to 40 cm

b. 40 cm to 70 cm

c. 70 cm to 120 cm

d. 120 cm to 150 cm.

Explanation: Option C is correct.

183. A strainer type well sunk through three pervious layers intervened by three impervious aquicludes, draws water from

a. top most pervious layer

b. central pervious layer

c. lowest pervious layer

d. all the pervious layers.

Explanation: Option D is correct.

184. The permissible amount of nitrites present in potable water, is

a. 15 ppm

b. 20 ppm

c. 45 ppm

d. Nil.

Explanation: Option D is correct.

185. Water having pH value as 6, is

a. alkaline

b. acidic

c. neutral

d. none of these.

Explanation: Option B is correct.

Permissible limit of pH in water is 7 to 8.5 and cause for rejection is pH <6.5.
So, pH 6 is acidic.

186. Alum is chemically

a. Copper sulphate

b. Aluminium sulphate

c. Ferrous sulphate

d. Ferric sulphate.

Explanation: Option B is correct.

187. If w is the weight of water per cubic metre, Q is the discharge in cubic metres per sec and H is the total head, the required water horse power of the pump, is

a.

b.

c.

d.

Explanation: Option A is correct.

If
w = unit wt of water in KN/cum.
Q = Discharge in cum/sec.
H = total head.
Then, required H.P. = wQH/0.735.

188. Perched acquifers are generally found

a. on the surface of the ground

b. below the surface of the ground but above water table

c. below the water table

d. all the above.

Explanation: Option B is correct.

189. Pickup the incorrect statement from the following :

a. The invert of pressure conduit is independent of the grade of the hydraulic gradient line

b. The pressure conduits may be taken uphill upto a maximum height of 8.3 m

c. Aqueducts and tunnels sections are generally kept circular

d. None of these.

Explanation: Option D is correct.

190. Detention period of a settling tank is

a. time required for flow of water to fill the tank fully

b. average time for which water is retained in tank

c. ratio of volume of basin of a sedimentation tank to the rate of flow

d. all the above.

Explanation: Option D is correct.

My opinion is option A is correct because Detention period is time required to empty the tank and detention time is time required to fill the tank.

191. Pick up the incorrect statement from the following :

a. Water disinfected with ozone is free from any odour

b. Ozone removes bacterias as well as colour and odour

c. ozonised water becomes tasteless

d. None of these.

Explanation: Option C is correct.

192. The order of existence of three portions of water in a reservoir from the bottom to top is

a. useful storage + dead storage + surcharge storage

b. dead storage + useful storage + surcharge storage

c. surcharge storage + useful storage + dead storage

d. none of these.

Explanation: Option B is correct.

193. The approximate diameter of a water mains for supplying 7.2 mld with a velocity 1.2 m/sec, is

a. 26 cm

b. 28 cm

c. 30 cm

d. 32 cm.

Explanation: Option C is correct.

7.2 mld = 7.2 x 1000/1 x 24 x 60 x 60 = 0.08334 m^3/sec.
Q = AV, 0.08334 = 3.14d^2/4 x 1.2.
So, 0.08334 x 4/ 1.2 x 3.14 = d = 0.2966 m say 30 cm.

194. The chlorine supply cylinders are generally kept at 38°C to 40°C to prevent

a. conversion into crystals

b. it from burning

c. it from explosion

d. none of these.

Explanation: Option A is correct.

195. The capability of a soil mass of full width and depth to transmit water, is known

a. porosity

b. permeability

c. transmissibility

d. none of these.

Explanation: Option B is correct.

196. Well treated water is generally supplied for

a. commercial use

b. public use

c. waste water

d. all the above.

Explanation: Option D is correct.

197. Non-pathogenic bacterias cause the following water borne disease,

a. cholera

b. typhoid

c. infections hepatitis

d. none of these.

Explanation: Option D is correct.

198. Steel pipe are

a. connected by riveted or welded joints

b. generally laid under ground and no expansion joint is required

c. likely to last 100 years under ordinary conditions

d. None of these.

Explanation: Option C is correct.

Because steel pipes do not resistant outer pressure like landfill, other loads. Steel pipe carries large water supply which is suitable on adequate.

199. The most ideal disinfectant used for drinking water throughout the world, is

a. lime

b. chlorine

c. nitrogen

d. ammonia.

Explanation: Option B is correct.

200. A centrifugal pump is required to be primed before starting if it is located

a. at higher level than water level of reservoir

b. at lower level than water level of reservoir

c. both (a) and (b)

d. neither (a) nor (b).

Explanation: Option A is correct.

201. The maximum pressure which the pipe can withstand without any leakage during hydrostatic pressure test, is called

a. Working pressure

b. Design pressure

c. Test pressure

d. Hydrostatic pressure.

Explanation: Option C is correct.

202. Filtration of water is done to remove

a. odour

b. turbidity

c. pathogenic bacteria

d. all the above.

Explanation: Option B is correct.

Yes, definitely filtration removes bacteria. But specifically, to remove bacteria there is a disinfection process separately like chlorination. The purpose of filtration is to remove turbidity.

203. Generally, first portion of a logistic curve for the population growth of a developing city, represents the growth of

a. increasing

b. decreasing

c. constant

d. all the above.

Explanation: Option A is correct.

204. Ground water from artesian wells

a. contains dissolved salts

b. may be saltish and hard

c. generally require lesser treatment

d. all the above.

Explanation: Option D is correct.

205. Property of earth to allow water to pass through it, is known as

a. perviousness

b. porosity

c. permeability

d. transmissibility.

Explanation: Option A is correct.

The capacity to transmit water is called "permeability". The ideal rock material for the accumulation of groundwater is both porous and permeable. This kind of material is known as an "aquifer" from the Latin for "Waterbearing".

206. Derivation of Thiem's formula  is based on the assumption

a. the well is sunk through the full depth of the aquifer

b. the flow lines are radial and horizontal, and the flow is laminar

c. for hydraulic gradient in Darcy's equation, tangent value is used for sine value

d. all the above.

Explanation: Option D is correct.

207. Pick up the incorrect statement from the following :

a. Porosity of pure sand is 35%

b. Porosity of sand stone is up to 15%

c. Least porosity is that of granite and quartz

d. None of these.

Explanation: Option D is correct.

208. Water is supplied from the source to the treatment plant under gravity in

a. Calcutta

b. Delhi

c. Madras

d. Bombay.

Explanation: Option D is correct.

209. Normal values of overflow rate for sedimentation tanks using coagulants in litres/hr/m2 , generally range between

a. 250 to 500

b. 500 to 750

c. 750 to 1000

d. 1000 to 1250

Explanation: Option D is correct.

210. In a rapid sand filter, air binding is caused due to excessive

a. negative pressure

b. pressure

c. turbidity

d. water pressure

Explanation: Option A is correct.

211. If average volume of sediment deposits is one-tenth million cubic metres per year in a reservoir of total capacity 10 million cubic metres, the dead storage will be filled up, in

a. 15 years

b. 20 years

c. 25 years

d. 50 years.

Explanation: Option C is correct.

Dead storage=1/4 of total capacity 10.
= 2.5 million cubic meters.
Deposition rate =1/10 per year.
Then silting year= 2.5/1/10= 2.5 * 10,
= 25 year.

212. A circular gravity aqueduct is not generally preferred to because of

a. its maximum hydraulic mean depth

b. maximum area per unit of wetted perimeter

c. minimum cost of construction

d. its proper support on the ground.

Explanation: Option D is correct.

As the main design concepts, to maximize hydraulic mean depth to have max discharge and in a circular section, we get maximum hydraulic mean depth for the same area of c/s than another type of section.

213. Standard unit of turbidity of water is in one litre of distilled water, one milligram of finely divided

a. silica

b. mud

c. clay

d. organic matter.

Explanation: Option A is correct.

214. Water losses in water supply, is assumed as

a. 7.5%

b. 10%

c. 22.5%

d. 15%.

Explanation: Option D is correct.

215. The pH value of water is kept slightly less than 7 so that hydrochloride ions may combine with ammonia ions to form

a. Mono-chloramine (NHCl)

b. Di-chloramine (NH2Cl)

c. Nitrogen trichloramine (NCl3)

d. All the above.

Explanation: Option D is correct.

216. 'Shrouding' is essentially provided in

a. strainer type wells

b. cavity type wells

c. slotted type well

d. all the above.

Explanation: Option C is correct.

The pipe will be clogged by the fine particles. To avoid it, we provide shrouding in slotted type tubewell.

217. The ensure proper growth of children's teeth, the quantity of flouride used in water mains, is

a. 1 mg/litre

b. 2 mg/litre

c. 3 mg/litre

d. 4 mg/litre

Explanation: Option A is correct.

Fluorides with more than 1.5 mg/l would lead to mottling of teeth and less than 1 mg/l will cause tooth decay due to cavity formation. Ideal concentration is 1 mg/l to 1.2 mg/l. Hence option A is correct.

218. Chemical coagulation of drinking water, is done

a. to settle suspended materials

b. to increase rate of settlement of suspended materials

c. to remove the bacterias

d. none of these.

Explanation: Option B is correct.

219. Permanent hardness of water can be removed by

a. adding lime

b. adding chlorine

c. boiling

d. zeolite process.

Explanation: Option D is correct.

220. Demand for public uses in a city, does not include water required for

a. watering of public parks

b. watering of public gardens

c. sprickling on roads

d. drinking purposes.

Explanation: Option D is correct.

221. The chloride content of treated water for public supplies should not exceed

a. 150 ppm

b. 200 ppm

c. 250 ppm

d. 300 ppm.

Explanation: Option C is correct.

222. An aquiclude is

a. a non artesian aquifer

b. an artesian acquifer

c. a confined bed of impervious material between acquifers

d. a large water body under ground.

Explanation: Option C is correct.

223. Pick up the incorrect statement from the following :

a. The amount of water which joins the stream from the underground water, is called base flow

b. The yearly run off in cm depth over the catchment, is termed as the yield of the drainage basin

c. The yield of drainage in cm depth multiplied by the area of the catchment gives the annual volume of water

d. None of these.

Explanation: Option D is correct.

224. In an artesian acquifer, the draw downs in two observation wells at distances 100 m, and 200 m were found same after one hour and x hours respectively. The value of x, is

a. 2 hours

b. 4 hours

c. 9 hours

d. 16 hours.

Explanation: Option B is correct.

225. Dissolved carbon dioxide, can be removed from the supply main by

a. aeration

b. chlorination

c. coagulation

d. none of these.

Explanation: Option A is correct.

226. Slow sand filter is used if maximum turbidity of raw water is less than

a. 20 gm/litre

b. 30 gm/litre

c. 40 gm/litre

d. 50 gm/litre.

Explanation: Option D is correct.

227. The external load per unit length of

a. a pipe laid on, or projecting above the undisturbed ground and covered with fills, is proportional to the square of the external diameter of the pipe

b. a flexible pipe burried in narrow trenches and thoroughly compacted side fills, is proportional to the product of the width of the trench and diameter of the pipe

c. a rigid pipe burried in a narrow trenches and thoroughly compacted side fills, is proportional to the square of the width of the trench

d. all the above.

Explanation: Option D is correct.

228. The total domestic consumption in a city water supply, is assumed

a. 20%

b. 30%

c. 40%

d. 60%.

Explanation: Option D is correct.

Domestic = 50 to 60%.
Industrial purposes = 20 to 25%,
Public use = 5%,
Water losses = 15% of allowance for water wastages should be made.

229. Alum is a

a. coagulant

b. flocculant

c. catalyst

d. disinfectant.

Explanation: Option A is correct.

230. If L, B and D length, breadth and depth of water in a rectangular sedimentation tank of total discharge Q, the settling velocity, is

a.

b.

c.

d.

Explanation: Option C is correct.

231. In a reservoir the average volume of sediment deposition is 0.15 million cubic per year. If the dead storage and total capacity of the reservoir are 8 million cubic metres and 36 million cubic metres respectively,

a. the reservoir will theoretically be silted in 240 years

b. the reservoir will start reducing after 60 years

c. both (a) and (b)

d. neither (a) nor (b).

Explanation: Option C is correct.

Total storage of 36 million cm.
So, 36/0.15 = 240 yr.
Hence 240/4 = 60 yr for dead storage.
Bcause dead storage is 1/4th of total.

232. Q is the discharge from an unconfined tube well with depression head S through a pipe of radius rw. If the radius of influence is R, the minimum required length of the strainer, is

a.

b.

c.

d.

Explanation: Option A is correct.

233. The requirement of water per capita per day, is

a. 90 litres

b. 150 litres

c. 250 litres

d. 400 litres.

Explanation: Option A is correct.

Domestic use =135 liters/capita/day.
Industrial use = 40 litres/capita/day.
Public use = 25 litres/capita/day.
Fire demand= 15 litres/capita /day.
Losses = 55  litres/capita/day.
Total= 270 litres/capita/day.

234. Pick up the incorrect statement from the following. In take of water supply should

a. be nearer to the treatment plant

b. receive water from purer zone of the source

c. be located downstream of waste water disposal point

d. remain easily accessible during floods.

Explanation: Option C is correct.

235. Reciprocating pumps

a. are not suitable for variable heads

b. are four times costlier than centrifugal pumps

c. are not suitable for pumping water containing sediments

d. single stroke produce pulsating flow

Explanation: Option A is correct.

236. The temporary hardness of water can be removed by

a. boiling

b. adding lime

c. adding alum

d. filtration

Explanation: Option A is correct.

237. Rapid gravity filters

a. make use of coarser sand with effective size as 0.5 mm

b. yield as high as 30 times the yield of slow sand filters

c. are fed with coagulation treated water

d. all the above.

Explanation: Option D is correct.

238. If n is porosity, y is specific yield and r is specific retention of any soil, the relationship which holds good, is

a. n + y = r

b. n + r = y

c. y + r = n

d. n + r + y = 0

Explanation: Option C is correct.

239. The coefficient of permeability of soils, is generally expressed in

a. cm/sec

b. cm/minute

c. cm2/sec

d. cm2/minute.

Explanation: Option A is correct.

Unit of Coefficient of permeability is same as that of velocity such as m/s.
Unit of coefficient of compressibility is inverse of pressure.
Unit of coefficient of consolidation is cm2/S.

240. Pick up the correct statement from the following :

a. Head loss in smaller size pipes at equal velocities, is less

b. Cost of pumping is less in smaller size pipes

c. Cost of smaller pipes is comparatively more

d. None of these.

Explanation: Option D is correct.

241. Total flow in stream is known

a. run off

b. stream flow

c. discharge

d. all the above.

Explanation: Option D is correct.

242. Disinfection of drinking water is done to remove

a. odour

b. colour

c. bacterias

d. all the above.

Explanation: Option C is correct.

243. The ratio of total capacity and dead storage is kept

a. 6

b. 4

c. 3

d. 2

Explanation: Option B is correct.

244. The U.C. (uniformity coefficient)  for the best filter media sand should be

a. 2

b. 3

c. 4

d. 5

Explanation: Option A is correct.

245. Pick up the correct statement from the following :

a. Incrustation of the pipe reduces the discharge

b. Acidic waters cause corrosion of the pipes

c. Corrosion results due to excessive withdrawal of sand along with water

d. All the above.

Explanation: Option D is correct.

246. Specific yield of a well is

a. quantity of water than can be drawn from the well

b. flow of water per unit time

c. total quantity of water available in the well

d. quantity of water per unit time per unit draw-down.

Explanation: Option C is correct.

247. Normal values of overflow rate for plain sedimentation tanks in litres/hr/m2, generally range between

a. 100 to 250

b. 250 to 500

c. 500 to 750

d. 750 to 1000

Explanation: Option C is correct.

248. The formula  for the head loss in conduits is generally known as

a. Hazen-William's formula

b. Manning's formula

c. Darcy-Weisbach formula

d. Nikuradse formula.

Explanation: Option B is correct.

249. Manholes are less common in

a. cast iron pipes

b. steel pipes

c. hume steel pipes

d. R.C.C. pipes.

Explanation: Option A is correct.

This is because cast iron pipes are used for treated water supply. In water supply pipes there aren't any manholes.

250. Pick up the correct statement from the following :

a. Even if ports are closed, the wet intake tower will have water filled up to reservoir level

b. The dry intake towers are generally constructed with heavier construction to resist buoyant forces

c. No buoyant force acts on wet intake towers

d. All the above.

Explanation: Option D is correct.

251. While designing a water supply of an industrial township, industrial and commercial water demand of total supply, is assumed

a. 10%

b. 10 to 15%

c. 15 to 20%

d. 20 to 25%.

Explanation: Option D is correct.

Domestic = 50 to 60%.
Industrial = 20 to 25%.
Public use = 5%.
Compensate allowance demand = An allowance of 15% for losses & wastages should be made.

252. The storage capacity of a reservoir may be divided into three zones. The lowest zone is

a. Dead storage

b. Useful storage

c. Surcharge storage

d. None of these.

Explanation: Option A is correct.

253. In a rapid gravity filter

a. raw water from the source is supplied

b. disinfected raw water is supplied

c. raw water passed through coagulation tank is supplied

d. none of these.

Explanation: Option C is correct.

254. In distribution pipes, air valves are provided at

a. lower points

b. junction points

c. higher points

d. any where.

Explanation: Option C is correct.

255. The flow of water gets retarded, in

a. sedimentation tank

b. clarifer

c. sedimentation basin

d. all the above.

Explanation: Option D is correct.

Flow of water is modulated by a site weir at the end of a grit chamber. While, the flow is retarded by a sedimentation tank.

256. Surcharge storage zone of a reservoir, is

a. below dead storage

b. between dead storage and useful storage

c. above useful storage

d. also known as valley storage.

Explanation: Option C is correct.

257. Surface water may

a. contain large amount of impurities

b. be contaminated by impurities

c. contain disease producing bacterias

d. all the above.

Explanation: Option D is correct.

258. For a city developed haphazardly, the layout of distribution pipes preferred to, is

a. ring system

b. radial system

c. grid iron system

d. dead end system.

Explanation: Option D is correct.

259. Asbestos pipes are joined by means of

a. flexible joint

b. dresser coupling joint

c. simplex joint

d. socket and spigot joint.

Explanation: Option C is correct.

260. Abyssinian tube well is a special type of

a. slotted type wells

b. cavity type wells

c. strainer type well

d. none of these.

Explanation: Option C is correct.

261. Chlorination of water does not remove

a. ammonia content

b. B.O.D.

c. organic matter content

d. dissolved oxygen.

Explanation: Option D is correct.

262. The efficiency of a pumping set, is generally assumed

a. 55%

b. 60%

c. 65%

d. 70%.

Explanation: Option C is correct.

263. To remove very fine suspended particles from water, the method adopted is

a. screening

b. sedimentation

c. boiling

d. filtration.

Explanation: Option D is correct.

264. Velocity of flow of water in plain sedimentation water tank, is normally kept

a. 10 cm/minute

b. 20 cm/minute

c. 30 cm/minute

d. 40 cm/minute.

Explanation: Option C is correct.

For grit chamber= 30 cm/s and detention= 1min.
For detritus tank= 0. 09 m/s and detention= 3-4min.

265. The most important and widely used tube well in India, is

a. strainer well

b. cavity well

c. slotted well

d. perforated pipe well.

Explanation: Option A is correct.

266. Pick up the correct statement from the following :

a. Rain water which percolates through the ground, is free from suspended materials

b. Under ground water may dissolve minerals and salts present in the earth's layers

c. Small quantities of iron, calcium and magnesium dissolved in water, may be useful for human health

d. All the above.

Explanation: Option D is correct.

267. The maximum permissible chloride content in treated water of public water supplies should not exceed

a. 100 P.P.M.

b. 150 P.P.M.

c. 200 P.P.M.

d. 250 P.P.M.

Explanation: Option D is correct.

According to Indian Standard, 250mg/l is the maximum amount of chloride that should be present in water.

268. To pump water from a water reservoir 3 m deep and maximum water level at 135 m, a pump is installed to lift water up to R.L. 175 m at a constant rate of 36, 00, 000 litres per hour. If the length of the pipe is 1506 m and f = 0.01, ignoring other minor losses and assuming the economical diameter from Lea's formula D = 1.2 √Q, the water horse power of the pump is

a. 450

b. 500

c. 580

d. 600

Explanation: Option D is correct.

269. Disappearance of pink colour of water of a well due of KMnO4 indicates that water contains

a. alkalinity

b. turbidity

c. organic matter

d. all the above.

Explanation: Option C is correct.

270. Shales are

a. permeable

b. porous and permeable

c. porous but not permeable

d. neither porous nor permeable.

Explanation: Option C is correct.

271. Water is distributed to consumers by gravitational system, in

a. Bombay

b. Delhi

c. Calcutta

d. Both (a) and (b).

Explanation: Option D is correct.

272. Grade aqueducts are not allowed to run

a. full

b.  th full

c.  full

d.  th full.

Explanation: Option A is correct.

273. The type of joint generally used in cast iron pipes, is

a. flanged joint

b. dresser coupling joint

c. flexible joint

d. all the above.

Explanation: Option D is correct.

274. Growth of population can be conveniently represented by

a. a semi-logarithmic curve

b. a logistic curve

c. a straight line curve

d. all the above.

Explanation: Option B is correct.

275. In a well planned city, the layout of distribution pipes generally adopted, is

a. grid-iron system

b. interlaced system

c. reticulation system

d. all the above.

Explanation: Option D is correct.

276. Air valves are generally provided in pressure pipes of water supply

a. at pipe junctions

b. at summits

c. at low points

d. near service pipes.

Explanation: Option B is correct.

277. Pick up the correct statement from the following :

a. The difference between water table level and the water level in a well after pumping, is called depression head

b. The surface of water table surrounding a well during pumping, forms a cone of depression

c. The distance from the centre of the well up to the place where level of water table is not affected, is called radius of influence

d. All the above.

Explanation: Option D is correct.

278. Bacterias which can survive with or without free oxygen, are known

a. aerobic bacterias

b. anerobic bacterias

c. facultative bacterias

d. none of these.

Explanation: Option C is correct.

279. The maximum permissible colour for domestic supplies based on cobalt scale, is

a. 10 ppm

b. 15 ppm

c. 20 ppm

d. 25 ppm.

Explanation: Option C is correct.

As per IS 10500:
As per Cobalt scale= 10 (preferable) & 20 (permissible).
As per Haven scale= 5 (preferable) & 15 (permissible).

280. The dilution ratio at which the odour is hardly detectable is generally called threshold odour number and for public supplies it should not exceed

a. 3

b. 5

c. 7

d. 9

Explanation: Option A is correct.

281. The joint used for joining the plain ends of cast iron pipes, is

a. socket and spigot joint .

b. dresser coupling joint

c. flexible joint

d. expansion joint.

Explanation: Option B is correct.

282. The bacterias which may survive with or without free oxygen, are called

a. Aerobic bacterias

b. Anaerobic bacterias

c. Faculative bacteria's

d. None of these.

Explanation: Option C is correct.

283. Detention time for plain sedimentation tank usually ranges from

a. 4 to 8 hours

b. 6 to 10 hours

c. 8 to 12 hours

d. 12 to 16 hours.

Explanation: Option A is correct.

284. The rate of silting in a reservoir

a. remains constant throughout

b. is more in the beginning

c. is more in the beginning and reduces in the end

d. none of these.

Explanation: Option C is correct.

Siltation: It's a process of accumulation of sediments (dust, soil, sand, gravels and other particles) in the river and dam it is called as siltation.

285. Estimates of a water supply project depends upon the rate of water supply per capita consumption and probable population estimated at the end of the design period of

a. 10 to 15 years

b. 15 to 20 years

c. 20 to 30 years

d. 25 years.

Explanation: Option C is correct.

Ssewers: design period: 40 to 50 years.
Water supply system : design period: 20 to 30 years.
Sewage treatment plant: design period: 15 to 20 years.

286. In centrifugal pump installation,

a. the diameter of the pipe at inlet as well as at outlet, is kept smaller than the delivery pipe

b. the foot valve fitted at the bottom end of the suction pipe, prevents the movement of water from the pump when it is stopped

c. the motor may burn out if it is started with empty delivery pipe and with open gate valve

d. all the above.

Explanation: Option D is correct.

287. Pipes are laid parallel

a. to increase the capacity of the water supply

b. to provide a means of repairing without closing water supply

c. to meet the requirement of excessive supply

d. all the above.

Explanation: Option D is correct.

288. Maximum permissible colour for domestic water supplies, based on Cobalt scale, is

a. 10 P.P.M.

b. 15 P.P.M.

c. 20 P.P.M.

d. 25 P.P.M.

Explanation: Option C is correct.

289. 135 litres of water per person per day, is provided in

a. nurses homes

b. hostels

c. residential schools

d. all the above.

Explanation: Option D is correct.

290. For a network of distribution pipes of a proposed township shown in the below figure, the layout of distribution system, is

a. dead end system

b. grid-iron system

c. ring system

d. radial system.

Explanation: Option D is correct.

291. If P is population of a city in thousands and Q is fire demand in litres per minute, for proper estimate of water, the Empirical formula  is suggested by

a. National Board of fire under-writers

b. Freeman

c. Kuichling

d. None of these.

Explanation: Option B is correct.

292. The gaseous form of chlorine gets converted into liquid form when subjected to a pressure of

a. 6 kg/cm2

b. 7 kg/cm2

c. 8 kg/cm2

d. 10 kg/cm2.

Explanation: Option B is correct.

293. The pH value of water fit for drinking, is

a. 11

b. 9

c. 7

d. 5

Explanation: Option C is correct.

The pH value range 6.5-8.5 for water to drink but the water which has pH value 7 is not also good for health.
Ref- R.S. KHURMI.

294. Pick up the incorrect statement from the following. The source of surface water is from

a. storage reservoirs

b. springs

c. ponds and lakes

d. none of these.

Explanation: Option B is correct.

295. For the prediction of future population of a city, the factor to be considered, is

a. births

b. deaths

c. migrants

d. all the above.

Explanation: Option D is correct.

296. Pick up the correct statement from the following :

a. The bigger suspended particles get deposited in the head reaches of the reservoir

b. The fine sediment deposits are generally near the face of the dam

c. Deposition of sediment in the reservoir, reduces the capacity of the reservoir

d. All the above.

Explanation: Option D is correct.

297. Pick up the incorrect statement from the following :

a. The flow in cavity type wells in spherical

b. In strainer type wells, area of flow depends upon the length of the strainer pipe

c. In cavity type wells, area of flow depends upon the size of the cavity

d. None of these.

Explanation: Option B is correct.

298. Water may not contain much impurities if its source is

a. stream flowing in plains

b. lakes in lower regions

c. spring along hill slopes

d. none of these.

Explanation: Option C is correct.

299. Raw water treated with only chlorine, is known as

a. plain chlorination

b. pre-chlorination

c. first-chlorination

d. de-chlorination

Explanation: Option A is correct.

300. The level of underground water is called

a. water level

b. water table

c. negative level

d. invert level.

Explanation: Option B is correct.

301. Per capita demand of water is calculated in litres

a. per person per day

b. per person per month

c. per person per year

d. none of these.

Explanation: Option A is correct.

302. P.V.C. pipes can withstand pressure bead of water upto

a. 50 m

b. 75 m

c. 100 m

d. 125 m.

Explanation: Option C is correct.

303. The population of a city in 2000 is 50,000. The average increase in population over last 8 decades is 7500 and average incremental increase during 8 decades is 750. The population of the city based on incremental method, in the year 2020 will be

a. 60,500

b. 66,500

c. 72,500

d. 76,500

Explanation: Option B is correct.

Pn = Po + nx + [(n(n+1))/2]*y.
n = no Of decades,
x = avg increase in population,
y = avg. incremental increase,
Po = initial population.

304. Air inlet valve in water mains, is generally provided at

a. upstream of sluice valve

b. down stream of sluice valve

c. valley of the pipe

d. both (a) and (c) of above.

Explanation: Option D is correct.

Air inlet valve - down stream of sluice valve.
Pressure relief Valve - Up stream of sluice valve.

305. The duration of contact of chlorine with water before it is served to the first consumer, should be at least

a. 15 to 20 minutes

b. 20 to 30 minutes

c. 30 to 40 minutes

d. one hour.

Explanation: Option B is correct.

306. The diameter of pipes in bath rooms and lavatories in domestic water supply, is

a. 6 mm

b. 12 mm

c. 18 mm

d. 24 mm.

Explanation: Option B is correct.

307. Pick up the incorrect statement from the following :

a. The depth of water on the filter should be twice the depth of the filter sand

b. When the filter head is 0.75 times the depth of filter sand, the water obtained is purest

c. The cleaning of slow sand filters is done by back washing

d. All the above.

Explanation: Option D is correct.

308. m and n are monsoon duration factor and catchment factor respectively. If P is yearly rainfall in cm, runoff can be calculated by

a. velocity area method

b. weir or spillway method

c. use of venturi-meter

d. using power plant consumption

Explanation: Option A is correct.

309. Most important method for calculating discharge for planning a water supply project, is

a. velocity area method

b. weir or spillway method

c. use of venturi-meter

d. using power plant consumption

Explanation: Option A is correct.

310. Alkalinity in water may be caused due to

a. sodium carbonate

b. potassium carbonate

c. calcium hydroxide

d. all the above.

Explanation: Option D is correct.

311. Open channels supported over tres-les, are generally known as

a. raised canals

b. acqueducts

c. syphons

d. flumes.

Explanation: Option D is correct.

312. For least effect on the water table, the tube wells must be dug one in every

a. 0.75 sq km

b. 1.0 sq km

c. 1.25 sq km

d. 1.5 sq km.

Explanation: Option D is correct.

313. In plain sedimentation tanks under normal conditions, impurities are removed upto

a. 60%

b. 70%

c. 80%

d. 90%.

Explanation: Option B is correct.

314. Pick up the correct statement from the following :

a. Lime may be used for softening hard water

b. Excess lime when added to water, raises the pH value of water

c. the bacterias do not survive in water having pH value exceeding 9.5

d. All the above.

Explanation: Option D is correct.

315. For the same draw down in two observations wells at distances r1 and r2, the times after start of pumping are t1 and t2 hours respectively. The relation which holds good is

a.

b.

c.

d. none of these.

Explanation: Option A is correct.

316. The valve fitted closely in a recess against an opening in a pipe, is generally

a. wedge shaped circular disc

b. spherical disc

c. parallelopiped disc

d. conical shaped circular disc.

Explanation: Option A is correct.

317. For calculation of economical diameter D of a pipe in metres for a discharge Q to be pumped in cumecs, Lea suggested the empirical formula

a. D = 1.22 √Q

b. D = 2.22 √Q

c. D = 3.22 √Q

d. D = 1.33 √Q.

Explanation: Option A is correct.

318. S1 and S2 are the draw downs in an observation well at times t1 and t2 after pumping. For discharge Q and coefficient of transmissibility T, the relationship, is

a.

b.

c.

d.

Explanation: Option B is correct.

319. An ideal sand for filters should be

a. free from dirt and other impurities

b. uniform in nature and size

c. hard and resistant

d. all the above.

Explanation: Option D is correct.

320. The maximum non-verticality of the bore of a well 200 m deep, may be permitted up to

a. 50 cm

b. 75 cm

c. 100 cm

d. 150 cm.

Explanation: Option C is correct.

321. By adding 1 ml of orthotolidine solution to 100 ml chlorinated water taken after contact period, the residual chlorine makes the colour of solution

a. yellowish

b. greenish

c. bluish

d. reddish

Explanation: Option A is correct.

The color indicates the presence of residual chlorine. The deeper the yellow colour the greater is the concentration of residual chlorine.

322. From the surface of reservoir, evaporation may be minimised by sprinkling

a. Hydrochloric acid

b. Cetyl alcohol

c. Methane

d. None of these.

Explanation: Option B is correct.

323. Gravity conduits are generally in the form of

a. flumes

b. aqueduct

c. tunnels

d. all the above.

Explanation: Option D is correct.

324. Pick up the correct statement from the following :

a. The ratio of the reservoir capacity to the total inflow of water in it, is generally called capacity inflow ratio

b. Small capacity reservoirs on large rivers generally, silt less

c. Large capacity reservoirs on small rivers generally silt more

d. All the above.

Explanation: Option D is correct.

325. Pressure relief valves are provided in water mains

a. to reduce the pressure

b. at low points

c. upstream of sluice

d. all the above.

Explanation: Option D is correct.

326. Pick up the incorrect statement from the following :

a. Iron salts produce heavy flocs and hence remove more suspended matter

b. Iron salts remove hydrogen sulphides

c. Iron salts can be used over a limited range of pH values

d. Iron salts impart corrosiveness to water.

Explanation: Option C is correct.

327. The percentage of chlorine in fresh bleaching powder is roughly

a. 50 to 60

b. 30 to 35

c. 40 to 50

d. 20 to 25

Explanation: Option B is correct.

328. Higher yield may be expected from

a. gravity springs

b. surface springs

c. artesian springs

d. all the above.

Explanation: Option C is correct.

329. The intake opening is generally covered by a screen to prevent entry of debris etc. and its level is kept

a. at the level of water of the source

b. at the bottom of water of the source

c. at about 2.5 m above the bottom

d. none of these.

Explanation: Option C is correct.

330. In rapid sand filters the ratio of length and diameter of the lateral, should not be greater than

a. 10

b. 15

c. 20

d. 25

Explanation: Option C is correct.

331. The maximum permissible hardness for public supplies is

a. 105 mg/litre

b. 115 mg/litre

c. 125 mg/litre

d. 150 mg/litre.

Explanation: Option B is correct.

332. Plain chlorination is used for water

a. obtained from clear lakes

b. consumed during emergencies

c. supplies to armies during war

d. all the above.

Explanation: Option D is correct.

333. Underground water is obtained from

a. rivers

b. lakes

c. reservoirs

d. springs.

Explanation: Option D is correct.

334. Water may contain

a. carbonate alkalinity

b. hydroxide alkalinity

c. caustic alkalinity

d. all the above.

Explanation: Option D is correct.

335. Pick up the correct statement from the following :

a. The pH value of neutral water is 7

b. The maximum acidity is obtained when pH value is zero

c. The maximum alkalinity is obtained when pH value is 14.

d. All the above.

Explanation: Option D is correct.

336. For controlling algae, the most commonly used chemical, is

a. copper sulphate

b. alum

c. lime

d. bleaching powder.

Explanation: Option A is correct.

337. According to Godrich the ratio of peak demand rate to mean demand is

a.

b.

c.

d. All the above.

Explanation: Option D is correct.

338. The head against which the motor works for lifting water, is

a. maximum depression head

b. velocity head

c. frictional losses in the delivery pipe

d. all the above.

Explanation: Option D is correct.

339. Rapid sand filter

a. should be preceeded by coagulation and sedimentation

b. uses rapid sand as filter media

c. is used after slow sand filtering has been done

d. can combine disinfection also.

Explanation: Option A is correct.

340. Pick up the correct statement from the following :

a. Wrought iron pipes are lighter than cast iron pipes

b. Wrought iron pipes corrode quickly and are less durable

c. Wrought iron pipes coated with zinc, are called galvanised iron pipes

d. All the above.

Explanation: Option D is correct.

341. Rapid gravity filters can remove bacterial impurities up to a maximum of

a. 60%

b. 70%

c. 80%

d. 90%.

Explanation: Option C is correct.

The rate of filtration - 3000-6000L/m^2/hour.
Bacteria removal - about 80 to 90%.
Turbidity removal 35-40 mg/l.

342. Cast iron pipes

a. resist corrosion satisfactorily

b. may last for 100 years

c. are normally manufactured in lengths of about 3.5 m

d. all the above.

Explanation: Option D is correct.

343. Run off is the quantity of water which flows

a. in sewer pipes

b. due to leakage in pipes

c. in rivers

d. none of these.

Explanation: Option C is correct.

344. Efficiency of removing bacterias from raw water by a slow sand filter, is

a. 80% to 81%

b. 85% to 86%

c. 90% to 97%

d. 98% to 99%.

Explanation: Option D is correct.

Rate of filtration - to 200L/m2/hour.
Bacteria Removal - 98%to 99%.
Turbidity removal 50ppm.
Colour removal- 30ppm.

345. De-chlorination is followed by

a. post-chlorination

b. pre-chlorination

c. double-chlorination

d. super-chlorination.

Explanation: Option D is correct.

346. The bed slope in slow sand filters, is generally kept

a. 1 in 50

b. 1 in 75

c. 1 in 100

d. 1 : 200

Explanation: Option C is correct.

347. When gravity and pumping systems of water distribution are adopted, the type of distribution reservoir, is

a. elevated tank

b. ground source reservoir

c. intz tank

d. stand pipe.

Explanation: Option A is correct.

348. The average domestic consumption under normal conditions in an Indian city per day per person, is

a. 115 litres

b. 125 litres

c. 135 litres

d. 150 litres.

Explanation: Option C is correct.

In India, on an average, the domestic consumption of water under normal condition is about 135 litres/day/capita.

349. Capacity of soil to absorb moisture, is generally known as

a. permeability

b. porosity

c. infiltration capacity

d. perviousness.

Explanation: Option C is correct.

350. At break point of chlorination,

a. chlorine is used to oxidise

b. residual chloride is zero

c. residual chloride is maximum

d. residual chlorine reappears.

Explanation: Option D is correct.

At break point, residual is not zero as a certain chlorine always stays and hence residual chlorine is minimum. But from this point if any chlorine is added it remains as residual chlorine so D is correct.

351. The difference in the reservoir level and the lowest point of the water mains is 180 m. The expected pressure due to water hammer is 7.5 kg/cm2 in a pressure conduit of diameter 1 m. Assuming the efficiency of the riveted joints of the pipe as 0.6 and minimum cover 3 mm for corrosion, the thickness of the pipe materials, is

a. 15 mm

b. 20 mm

c. 25 mm

d. 30 mm.

Explanation: Option B is correct.

352. The factor for the selection of pumping station site, is

a. height above the H.F.L. of the river

b. future expansion

c. proximity of the site from the railway station

d. all the above.

Explanation: Option D is correct.

353. Low turbidity of water is detected by

a. Jackson turbidity meter

b. Baylis turbidimeter

c. Hellipe turbidimeter

d. None of these.

Explanation: Option B is correct.

Jackson turbidity meter range is 0-1000ppm.
Hellige turbidity meter range is 0-50 ppm
Baylis turbidity meter range is 0-2 ppm.

354. Pick up the incorrect statement from the following :

a. Higher value of pH means lower hydrogen ion concentration

b. Lower value of pH means higher hydrogen ion contraction

c. Lower value of pH gives alkaline solution

d. None of these.

Explanation: Option C is correct.

355. Pick up the correct statement from the following :

a. The ratio of volume drained to the total volume of material drained, is known as specific yield

b. The volume of water retained by the soil against the pull of gravity, is called specific retention or field capacity

c. The sum of specific yield and specific retention, is equal to porosity

d. All the above.

Explanation: Option D is correct.

356. Perched acquifers generally occur

a. above water table

b. in acquicludes

c. in artesian acquifers

d. none of these.

Explanation: Option A is correct.

357. Silt storage is the same as

a. dead storage

b. live storage

c. effective storage

d. none of these.

Explanation: Option A is correct.

Yes. Why because, In the dead storage zone, the accumulation of silt particles will takes place. There the approximate is similar.

358. For complete stabilisation of organic matter in polluted water, it takes

a. 10 days

b. 20 days

c. 30 days

d. 40 days.

Explanation: Option C is correct.

359. A pressure conduit carrying water beneath a stream or a canal, is known as

a. sag

b. depressed pipe

c. inverted syphon

d. all the above.

Explanation: Option D is correct.

360. Suction lift of a pump depends upon

a. water temperature

b. velocity of water in suction pipe

c. frictional losses

d. all the above.

Explanation: Option D is correct.

361. For 3.25 x 10-2 cumecs discharging from a well, a pump is installed to lift water against a total head of 30 m. The minimum required horse power, is

a. 15 H.P.

b. 18 H.P.

c. 20 H.P.

d. 25 H.P.

Explanation: Option C is correct.

P = WQH/75.
W= Density of Fluid  = 1000 kg/m^3.
P=1000 * 3.25 * 10^-2 * 30/75 = 13,
Assume efficiency of 65%,
Required Power = 13/0.65 = 20 HP.