***Sub Engineer***

1. Borrow pits should preferably be located in

a. field on the left side of the canal

b. field on the right side of the canal

c. fields on both sides of the canal

d. central half width of the section of the canal.

Explanation: Option D is correct.

Borrow Pits serves the purpose of depositing silt i.e. running with water. So it is one of the purposes of borrow pit.

2. Irrigation canals are generally aligned along

a. ridge line

b. contour line

c. valley line

d. straight line.

Explanation: Option A is correct.

Is we made canal along the ridge line then we can irrigate in both side of ridge line. Ridge line is the peak of a drainage area which divide the drainage area into two half.

3. The length of a meander is the distance along the river between the tangent point of one curve to the tangent point of

a. reverse curve

b. next curve of the same order

c. reverse curve plus the width of the river

d. none of these.

Explanation: Option B is correct.

Formation of successive bends of reverse order by the river scouring leads to the formation of S shape curve called meander.

4. If B and d are the bed width and depth of a channel in metres, the combined losses due to evaporation and seepage in cumecs per kilometre length of the channel, is

a.

b.

c.

d.

Explanation: Option D is correct.

5. Pick up the incorrect statement from the following

a. Side walls of a venturi head flume are splayed out from the end of the throat at 1 : 10 for a length of 4.5 m

b. Length of side walls should be such that the width of the flume is made equal to 2/3rd the bed width of the distributary

c. Once the width of the flume becomes 2/3rd of the width of the distributary, the splayed walls are increased to 1 in 3 to get full bed width

d. None of these.

Explanation: Option D is correct.

6. The difference in level between the top of a bank and supply level in a canal, is called

a. berm

b. free board

c. height of bank

d. none of these.

Explanation: Option B is correct.

Generally, the freeboard is the distance between top-level and supply level which is used in the emergency period. The value is generally taken 0.5.

7. F.S.L. of a canal at its head with respect to parent channel is kept

a. at the same level

b. 15 cm lower

c. 15 cm higher

d. none of these.

Explanation: Option B is correct.

F.S.L= It is canal x-section which can holding (carrying) the amount of water needed for irrigation/supply.

8. The measure to remove water logging of land, is

a. to reduce percolation from canals and water courses

b. to increase outflow from the ground water reservoir

c. both (a) and (b)

d. neither (a) nor (b)

Explanation: Option C is correct.

However we do the measure to remove water logging of land to increase outflow from the ground water reservoir.

9. Attracting groynes are built

a. perpendicular to the bank

b. inclined down stream

c. inclined up stream

d. none of these.

Explanation: Option B is correct.

Groynes are generally provided for prevention of banks of rivers and 3 category form in provide to that.
Perpendicular to river, inclined in towards upstream, inclined in towards downstream.

10. Retrogression of the bed level of a river downstream a weir, occurs due to

a. heavy impact of water

b. increase of the bed level

c. less percentage of silt

d. soft soil strata.

Explanation: Option C is correct.

11. According to G.W. Pickles the effect of confining the flood water of a river between levee, is to increase

a. water surface elevation during floods

b. maximum discharge

c. velocity and scouring action

d. surface slope of streams above the leveed portion.

Explanation: Option D is correct.

Here all the given options are correct. These all effects will be observed by confining the flood water of the river between marginal embankments/Levees.

12. In a canal syphon, the flow is

a. under atmospheric pressure

b. pipe flow

c. with critical velocity

d. under negative pressure.

Explanation: Option B is correct.

When the canal passes below the trough as a pressure flow, then it is termed as a syphon or a canal syphon.

13. A water-logged land is found suitable for cultivation due to

a. ease of tillage for preparation of the field for optimum condition of germination

b. absence of aeration of soil from root zone of the plant

c. regular supply of water to plants from the water table by capillary action

d. none of these.

Explanation: Option D is correct.

In the water-logged area, the salt content is very much. To make suitable for cultivation it requires leaching process (irrigation without cultivation). It reduces the salt content to such an extent so that inferior crops can be cultivated.

14. When a canal and a drainage approach each other at the same level, the structure so provided, is

a. an aqueduct

b. a syphon

c. a level crossing

d. inlet and outlet.

Explanation: Option C is correct.

Inlet Outlest - we use in small discharges, and not using hydraulic structures.
Level crossing - we use in large discharges and also using hydraulic structures.

15. For smooth entry of water in a canal, the angle between head regulator and water is generally kept

a. 80°

b. 90°

c. 110°

d. 120°

Explanation: Option C is correct.

16. If L is total length of a canal in kilometres, P is total perimeter of its lining in metres and C is the cost of lining per square metre, the additional expenditure involved on lining, is

a. 1000 PLC

b.

c.

d.

Explanation: Option A is correct.

Length=1000L (in metre).
Perimeter= P.
So, total area= 1000PL (in square metre).
So, total cost= 1000PLC.

17. Pick up the incorrect statement from the following:

a. In free flooding irrigation, water is admitted at one corner of a field and is allowed to spread over the entire area

b. In check method of irrigation, the field is divided into smaller compartments and water is admitted to each in turn

c. In furrow irrigation water is admitted between the rows of plants in the field

d. None of these.

Explanation: Option D is correct.

When water enters through the sewerage system free flooding is provided.
When we grow plants like fruits then check method will but provide.
When we grow vegetables and food of animals we adopt rows of plants it's called furrow.

18. Fundamental momentum equation for a hydraulic jump, is

a.

b.

c.

d.

Explanation: Option B is correct.

19. In a Sarda type fall, the rectangular crest, may be used for discharge upto

a. 10 cumecs

b. 14 cumecs

c. 20 cumecs

d. 25 cumecs.

Explanation: Option B is correct.

For a discharge 4m^3/s in Sarda type fall rectangular crest is used.
The trapezoidal crest is used for discharge over 14M^3/s.

20. When a canal is carried over a natural drainage, the structure provided, is known as

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

Explanation: Option B is correct.

Super passage:
Canal bed level below the drainage & drainage bed level above the full supply levels.

21. For the stability of a structure against seepage pressure according to Khosla's creep theory, the critical gradient is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

Explanation: Option D is correct.

Fine sand - 1/6 to 1/7.
Medium sand - 1/5 to 1/6.
Coarse sand - 1/4 to 1/5.
Sand mixed with gravel 1/4 to 1/5.

22. Pick up the correct statement from the following

a. Approach of the water line in a flumed channel section should not be steeper than

b. Departure of the water line from a flumed channel section should not be steeper than 30°

c. Approach of the water line should not be steeper than 30° and departure line not steeper than  in a flumed channel section

d. Approach and departure of the water line in a flumed channel section, should not be steeper than

Explanation: Option C is correct.

23. Groynes are generally built

a. perpendicular to the bank

b. inclined up stream up to 30°

c. inclined down stream upto 30°

d. all the above.

Explanation: Option B is correct.

The groynes are generally aligned either perpendicular to bank or pointing upstream. Here written generally means normal groynes. This point to be noted. So, Only Option A is correct.

24. The top of the capillary zone

a. lies below the water table at every point

b. lies above the water table at every point

c. coincides the water table at every point

d. none of these.

Explanation: Option B is correct.

25. Pick up the incorrect statement from the following. Culturable commanded area is the gross area of an irrigation canal system less

a. populated area

b. alkaline area

c. forest area

d. fallow land.

Explanation: Option D is correct.

Option A
Culturable Command Area (CCA).
It is the area which can be physically irrigated from a scheme and is fit for cultivation.
The culturable cultivated area is a portion of CCA which is actually cultivated during a crop season.

26. Pick up the correct statement from the following

a. In a level crossing, a crest with its top at the canal F.S.L. is provided across the drainage at its up-stream junction with canal

b. In a level crossing a regulator is provided across the drainage at its down-stream

c. In a level crossing, a cross regulator is provided on the canal below the crossing

d. All the above.

Explanation: Option D is correct.

27. For the design of major hydraulic structures on the canals, the method generally preferred to, is based on

a. Bligh's theory

b. Electrical analogy method

c. The relaxation method

d. Khosla's method of independent variables.

Explanation: Option D is correct.

28. For designing the venturi head,

a. width of the throat should not be less than the width obtained from the formula B = Q / 1.2d3/2, where d is the difference in levels of F.S.L. and sill

b. minimum throat width should not be less than 60 cm

c. length of the throat should be 2.5 times the depth measured to F.S.L. in the main canal

d. all the above.

Explanation: Option D is correct.

29. The sinuosity of a meander is the ratio of

a. meander length and the width of meander

b. meander length and half width of the river

c. curved length and the straight distance

d. none of these.

Explanation: Option C is correct.

Degree of Sinuosity : the ratio of curved length and straight axial distance.
Tortuosity: the ratio of curved length along the river to the direct axial length of river.

30. The top soil of a water logged field becomes more alkaline and infertile if its pH value is

a. 8

b. 9

c. 10

d. 11

Explanation: Option D is correct.

PH 7 to 8.5 normal yields.
8.0 to 9.0 yield decreases.
If 11 soil becomes more alkaline and infertile.

31. The main function of a diversion head works of a canal from a river, is

a. to control floods

b. to store water

c. to raise water level

d. all the above.

Explanation: Option C is correct.

32. Side slopes generally kept for canals in loam soil, are :

a. 1 : 1 in cutting and  : 1 in filling

b.  : 1 in cutting and 2 : 1 in filling

c. Neither (a) nor (b)

d. Both (a) and (b)

Explanation: Option D is correct.

Loam is soil composed mostly of sand (particle size > 63 μm), silt (particle size > 2 μm), and a smaller amount of clay (particle size < 2 μm) loam is preferred for both types of slope.

33. When a canal flowing under pressure is carried below a natural drainage such that its F.S.L. does not touch the underside of the supporting structure, the structure so provided, is called

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

Explanation: Option C is correct.

34. For the conditions enumerated to provide a crossing at C1 You will probably provide

a. an aqueduct

b. a super-passage

c. a syphon aqueduct

d. none of these.

Explanation: Option B is correct.

When the canal bed is high than a river bed, the canal will go below the river. So, we have to provide Super passage.
When the canal is lower than the river bed, River will go below the canal. So, we have to provide Aqueduct.

35. If the straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the perimeter of the canal is

a. 2R(θ + tan θ)

b. R(θ + cos θ)

c. 2R(θ + cos θ)

d. none of these.

Explanation: Option C is correct.

Consider half portion.
For straight line portion cos θ = (x/R).
Therefore x = R.Cos θ.
Now Circular portion making angle of θ.
Length of arc = R.θ.
Therefore full length of perimeter = 2xR(θ + cos θ).

36. The crest level of a canal diversion head work, depends upon

a. discharge perimeters

b. pond level

c. all the above

d. none these.

Explanation: Option C is correct.

37. The depth of rice root zone, is

a. 60 cm

b. 70 cm

c. 80 cm

d. 90 cm

Explanation: Option D is correct.

Delta means the depth of water required during the growing period. Delta is 12 cm. It doesn't mean that root zone depth is 120 cm.

38. The intensity of irrigation means

a. percentage of culturable commanded area to be irrigated annually

b. percentage of gross commanded area to be irrigated annually

c. percentage of the mean of culturable commanded area and the gross commanded area to be irrigated annually

d. total depth of water supplied by the number of waterings.

Explanation: Option A is correct.

39. The saturation line is the line up to which banks get saturated after the canal runs for some time. The saturation gradient in ordinary loam soil, is generally

a. 3 : 1

b. 4 : 1

c. 5 : 1

d. 6 : 1

Explanation: Option B is correct.

The common soil types throughout Wisconsin and found the ratio ranged from 8.1:1 to 1.0:1 (Table 5).
Some of the silt loam soils like Antigo fell near 4:0.1 compared to soils with more clay, like Marathon with a ratio of 7.7:1.

40. The depth of the crest of a scouring sluice below the crest of a head regulator, is generally kept

a. 1.20 m

b. 2.20 m

c. 3.20 m

d. 4.90 m

Explanation: Option A is correct.

41. The discharge over a trapezoidal crest of a Sarda fall with free over fall conditions, is

a. 0.45 √2g . LH3/2 (H/B)1/3

b. 0.45 √2g . LH2/3 (H/B)2/3

c. 0.45 √2g . LH3/2 (H/B)1/6

d. 0.45 √2g . LH1/6 (H/B)3/2

Explanation: Option C is correct.

42. If q is the discharge per unit width of a channel and D1 D2 are the depths of water before and after hydraulic jump, the following relationship is true

a.

b.

c.

d.

Explanation: Option C is correct.

43. A river training work is generally required when the river is

a. meandering

b. aggrading

c. degrading

d. all the above.

Explanation: Option A is correct.

Alternate deposition of silt and erosion of bank in the opposite sides of river bank known as Meandering.
It makes one side concave and the other side convex in shape.

44. For loss of head in a canal inverted syphon barrel, the factor in the Unwin formula  is a coefficient for loss of head due to

a. friction

b. exit

c. entry

d. gradient

Explanation: Option C is correct.

45. Pick up the correct statement from the following:

a. If the setting of an outlet is higher than that required for proportionality, the outlet is hyper-proportional

b. If the setting of an outlet is lower than that required for proportionality, the outlet is sub-proportional

c. If the flexibility is zero, it is a rigid module

d. All the above.

Explanation: Option D is correct.

46. Pick up the correct statement from the following:

a. Duty of canal water is usually calculated at the head discharge of a channel

b. Duty of canal water at the head of a channel and at the outlet of a distributary, differ considerably due to progressive losses of water

c. Duty of canal water at the outlet of a distributary, is generally known as outlet factor or outlet discharge factor

d. All the above.

Explanation: Option D is correct.

47. Water-shed line is abondened for aligning an irrigation canal if

a. water shed forms a sharp loop

b. canal has to take off from a river

c. towns and villages are located on the water shed line

d. all the above.

Explanation: Option D is correct.

48. Pick up the incorrect statement from the following

a. A bank in the cross-section of a canal is provided on the right

b. A bank in the cross-section of a canal is provided on the left

c. A 'dowla' in the cross-section of a canal is provided on the left

d. None of these.

Explanation: Option B is correct.

49. For a given discharge in a channel, Blench curves give the relationship between the loss of head (HL) and

a. specific energy up-stream

b. specific energy down-stream

c. critical depth of water down-stream

d. depth of water down-stream

Explanation: Option B is correct.

50. A deficit of sediments in flowing water may cause a river

a. meandering type

b. aggrading type

c. degrading type

d. sub-critical type.

Explanation: Option C is correct.

In aggrading silting is occur.
Degrading Scouring.
And in menadring, both occur simultaneously.
So, the deficit of sediment causes degrading type.

51. Pick up the correct statement from the following:

a. The full supply level of a canal should be above ground level

b. According to Lacey, regime conditions require a particular slope for a given discharge and silt factor

c. In case the ground slope is less than the required bed slope, the silt factor must be reduced by permitting the entry of coarse silt

d. All the above.

Explanation: Option D is correct.

Ground level means bed level of canal. Full supply level always above the ground level. FSL must be below than the bank of canal.

52. If Δ is the depth of water in metres, B is the number of days of base period and D is the duty in hactare/cumec, the relationship which holds good, is

a.

b.

c.

d. none of these.

Explanation: Option C is correct.

53. Meandering of a river generally occurs, in

a. rocky stage

b. delta stage

c. boulder stage

d. trough stage.

Explanation: Option D is correct.

A meander, in general, is a bend in a sinuous watercourse or river. A meander forms when moving water in a stream erodes the outer banks and widens its valley, and the inner part of the river has less energy and deposits silt.

54. The consumptive use of water for a crop

a. is measured as the volume of water per unit area

b. is measured as depth of water on irrigated area

c. may be supplied partly by precipitation and partly by irrigation

d. all the above.

Explanation: Option D is correct.

55. The velocity of drainage water in the barrels of a syphon-aqueduct, is normally limited to

a. 1 to 2 m per second

b. 2 to 3 m per second

c. 3 to 4 m per second

d. 4 to 5 m per second.

Explanation: Option B is correct.

56. A fall in a canal bed is generally provided, if

a. ground slope exceeds the designed bed slope

b. designed bed slope exceeds the ground slope

c. ground slope is practically the same as the designed bed slope

d. none of these.

Explanation: Option A is correct.

When designed slope exceeds the ground slope, fall in the canal bed is provided to dissipate the energy in flow.

57. According to Khosla, the exist gradient of surface flow

a. depends upon the b/d ratio

b. is independent of the b/d ratio

c. is independent of the depths of d/s cut off walls

d. none of these.

Explanation: Option A is correct.

Exit gradient=(H/d)*(1/(&pi*√ λ)).
λ=function (α),
α=b/d.
Hence exit gradient is function of b/d ratio.

58. Pick up the correct statement from the following:

a. Gravity water is harmful to crops

b. Hygroscopic water remains attached to soil molecules by chemical bond

c. Capillary moisture held in the soil pores against gravity by surface tension, is utilised by plants

d. All the above.

Explanation: Option C is correct.

C is right because of hydroscopic water attached by soil by adhesive force not by a chemical bond. Structural water is attached to a chemical bond.

59. If the irrigation efficiency is 80%, conveyance losses are 20% and the actual depth of watering is 16 cm, the depth of water required at the canal outlet, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

Explanation: Option C is correct.

Let depth at canal be x then 20% loss = x - 20% of x = 0.8x,
From 0.8x only 80% taken by soil that equals 16cm in depth,
So,
80% of 0.8X = 16, Solve x=25.

60. The scour depth D of a river during flood, may be calculated from the Lacey's equation

a.

b.

c.

d.

Explanation: Option C is correct.

61. If V0 is the critical velocity of a channel, its silt transporting power, according to Kennedy, is proportional to

a.

b.

c.

d.

Explanation: Option B is correct.

62. The useful moisture of soil, is equal to its

a. field capacity

b. saturation capacity

c. moisture content at permanent wilting point

d. difference between filed capacity and permanent wilting point within the root zone of plants.

Explanation: Option D is correct.

63. Canals taken off from ice-fed perennial rivers, are known

a. ridge canals

b. perennial canals

c. inundation canals

d. ice canals.

Explanation: Option B is correct.

64. The down stream expansion head of a guide bank is extended to subtend an angle at the centre, equal to

a. 30°

b. 45°

c. 60°

d. 90°

Explanation: Option B is correct.

65. Trial procedure of canal design by Kennedy's theory is based upon the equation

a. Q = A . Y

b.

c.

d. all the above.

Explanation: Option D is correct.

66. The main cause of silting up a channel,

a. inadequate slope

b. defective head regulator

c. defective outlets

d. all the above.

Explanation: Option D is correct.

67. In north Indian Plains, optimum depth of kor watering for wheat, is

a. 19.0 cm

b. 17.5 cm

c. 13.5 cm

d. 12.0 cm

Explanation: Option C is correct.

The first watering is done when the crop has grown to about three centimeters. This watering is known as Kor watering.

68. Bed bars in a canal are provided

a. to watch the general behaviour of canal

b. to measure the discharge

c. to raise the supply level

d. to control the silting

Explanation: Option A is correct.

Bed bars are used for permanent reference for an unlined channel. These are provided on certain interval. Like for minor canals these are provided by a distance of 200 m.

69. The most suitable location of a canal head work, is

a. boulders stage of the river

b. delta stage of the river

c. rock stage of the river

d. trough stage of the river.

Explanation: Option D is correct.

70. Useful soil moisture for plant growth, is

a. capillary water

b. gravity water

c. hygroscopic water

d. chemical water

Explanation: Option A is correct.

71. A hydraulic structure is designed to withstand

a. seepage forces

b. hydraulic jump

c. hydraulic pressure

d. all the above.

Explanation: Option D is correct.

72. In gravity canals, F.S.L. is

a. always at the ground level

b. always below the ground level

c. generally 4 to 5 metres above the ground level

d. only a few cm above the ground level.

Explanation: Option D is correct.

73. If d1 is the depth of cutting, d2 is the height of the bank from bed level r2 : 1 and r1 : 1 are the slopes in filling and cutting respectively, the horizontal distance n between the bed and bank, is

a. x = r1 d1

b. x = r2 d2

c. x = d1 / r1

d. x = d2 / r2

Explanation: Option B is correct.

74. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

Explanation: Option D is correct.

75. The optimum depth of kor watering for a rice crop, is

a. 19.0 cm

b. 17.5 cm

c. 13.5 cm

d. 12.0 cm

Explanation: Option A is correct.

76. Bligh's theory of seepage assumes

a. equal weightage to the horizontal and vertical creep

b. more weightage to horizontal creep than vertical creep

c. less weightage to horizontal creep than vertical creep

d. loss of head follows the sine curve.

Explanation: Option A is correct.

Option (c) given by Lane's weighted creep theory with horizontal creep factor 1/3 as compared to 1 of vertical creep.

77. The Lacey's regime velocity is proportional to

a. R1/2 S3/4

b. Q3/4 S1/3

c. R3/4 S1/3

d. R2/3 S1/2

Explanation: Option C is correct.

None of the option is true because according to the formula the powers are 2/3 & 1/3 for R & S respectively.

78. A hydraulic jump is generally formed when a stream moving with

a. a hyper-critical velocity meets a stream moving with a critical velocity

b. a hyper-critical velocity meets a stream moving with a hyper-critical velocity

c. A hyper-critical velocity meets a stream moving with a sub-critical velocity

d. a sub-critical velocity meets a stream moving with a hyper-critical velocity.

Explanation: Option C is correct.

79. For a unique design of a channel by Kennedy's theory

a. its breadth must only be known

b. its depth must only be known

c. its breadth and depth ratio must only be known

d. all the above.

Explanation: Option C is correct.

In its design problem, we were assumed "D" value just and then followed the relations.
So, the right option is Depth only.

80. In a barrage, the crest level is kept

a. low with large gates

b. high with large gates

c. high with no gates

d. low with no gates.

Explanation: Option A is correct.

81. For a standing crop, the consumptive use of water is equal to the depth of water

a. transpired by the crop

b. evaporated by the crop

c. transpired and evaporated by the crop

d. used by the crop in transpiration, evaporation and also the quantity of water evaporated from adjacent soil.

Explanation: Option D is correct.

82. The state of the soil when plants fail to extract sufficient water for their requirements, is

a. maximum saturated point

b. permanent wilting point

c. ultimate utilisation point

d. none of these.

Explanation: Option B is correct.

Permanent wilting point (PWP) or wilting point (WP) is defined as the minimal amount of water in the soil that the plant requires not to wilt.

83. The field capacity of a soil is 25%, its permanent wilting point is 15% and specific dry unity weight is 1.5. If the depth of root zone of a crop, is 80 cm, the storage capacity of the soil, is

a. 10 cm

b. 12 cm

c. 14 cm

d. 16 cm

Explanation: Option B is correct.

Here, s = Specific gravity of soil,
d = Depth of root zone,
Fc = Field capacity,
PWP = Permanent wilting point.

84. Solution of Laplacian equation in three dimensions  of water in a syphon, is done by

a. analytical method

b. Khosla's method

c. method of relaxation

d. Unwin's method.

Explanation: Option C is correct.

85. The length and width of a meander and also the width of the river, vary roughly as

a. square root of the discharge

b. discharge

c. square of the discharge

d. cube of the discharge.

Explanation: Option A is correct.

86. Regime conditions in a channel may occur if

a. discharge is constant

b. channel flows uniformly in incoherent alluvium as that transported in suspension

c. silt grade and silt charge are constant

d. all the above.

Explanation: Option D is correct.

87. If the optimum depth of kor watering for a crop is 15.12 cm, the outlet factor for the crop for four week period in hectares per cumec, is

a. 1200

b. 1400

c. 1600

d. 2000

Explanation: Option C is correct.

Delta= 15.12 cm = 0.1512 m.
B = 4 week = 28 days.
Delta = (8.64 * 28)/0.1512.
         =  1600 hect/cumec.

88. If H and d are the water depth and drop in the bed level at a Sarda fall, the width B of the trapezoidal crest, is given by

a. B = 0.22 √H + d

b. B = 0.33 √H + d

c. B = 0.44 √H + d

d. B = 0.55 √H + d

Explanation: Option D is correct.

89. According to Bligh's creep theory, percolating water flows along

a. straight path under the foundation of the dam

b. circular path under the foundation of the dam

c. the outline of the base of the foundation of the dam

d. none of these.

Explanation: Option C is correct.

C is correct.
According to Bligh's Theory, the percolating water follows the outline of the base of the foundation of the hydraulic structure.

90. The width of a meander belt is the transverse distance between

a. apex point of one curve and apex point of the reserve curve

b. apex point and the crossing

c. two banks of meandering river

d. none of these.

Explanation: Option A is correct.

91. The sensitivity of a rigid module, is

a. 1.50

b. 1.00

c. 0.50

d. zero

Explanation: Option D is correct.

For rigid modules, the discharge is fixed, and hence sensitivity is zero.
The greater the variation of discharge through an outlet for a given rise or fall in water level of the distributary, the larger is the sensitivity of the outlet.

92. An outlet which maintains a constant discharge irrespective of fluctuation in the water levels of the supplying channel or water course, is known as

a. non-modular outlet

b. semi-modular outlet

c. flexible modular outlet

d. right modular outlet.

Explanation: Option D is correct.

It's rigid modular outlet.
Description: These modules allow constant discharge within reasonable working limits of head.
Irrespective of water levels in the distribution and the watercourse of the main canal.

93. Canals constructed for draining off water from water logged areas, are known

a. drains

b. inundation canals

c. valley canals

d. contour canals

Explanation: Option A is correct.

Inundation canal/Flood canal:
1. They took out excess water from the rivers without constructing dams, barrage, embankments.
2. They are full of water only in the rainy season or when the river is in flood.

94. A minimum of 90 cm free board is provided if the discharge in the canal is between

a. 30 to 33 cumecs

b. 30 to 60 cumecs

c. Over 60 cumecs

d. Over 100 cumecs

Explanation: Option C is correct.

If Free Board is 90cm than discharge will be greater than 85 cumecs (>85cumecs). i.e, F.B. =90cm than, Q is > 85cumecs.

95. If D is the depth of water upstream of the throat above its sill, B is the width of the throat, to achieve critical flow in an open venturi flume, the theoretical maximum flow Q, is

a. Q = 1.71 BD

b. Q = 1.71 BD3/2

c. Q = 1/71 BD2/3

d. none of these.

Explanation: Option B is correct.

96. If straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the area of its cross-section, is

a. R(θ + cot θ)

b. R2(θ + tan θ)

c. R2(θ + cot θ)

d. R(θ + tan θ)

Explanation: Option C is correct.

97. Lacy's regime condition is obtained if

a. silt grade in the channel is variable

b. discharge in the channel is variable

c. silt charge in the channel is variable

d. channel flows in unlimited, incoherent alluvium of the same character as that transported material.

Explanation: Option D is correct.

98. In Montague type fall

a. a straight glacis is provided

b. a circular glacis is provided

c. a parabolic glacis is provided

d. no glacis is provided.

Explanation: Option C is correct.

Montage fall is similar to straight glacis fall but in this case the glacis is not straight. It is provided in parabolic shape to introduce the vertical component of velocity which improves the energy dissipation to more extent.

99. If the straight sides of a triangular section of a lined canal with circular bottom of radius D, make an angle θ with horizontal, the hydraulic mean depth is

a. D/2

b. D/3

c. D/4

d. D/5

Explanation: Option A is correct.

100. If water table is comparatively high, the irrigation canal becomes useless, due to

a. water logging of the cultivated areas

b. uncertain water demand

c. spread of malaria

d. all the above.

Explanation: Option D is correct.

101. A river is said to be of

a. aggrading type if it builds up its bed to a certain slope

b. degrading type if it cuts its bed to a certain slope

c. meandering type if it flows in sinuous curve

d. all the above.

Explanation: Option D is correct.

102. A land is said to be water-logged if its soil pores within

a. a depth of 40 cm are saturated

b. a depth of 50 cm are saturated

c. root zone of the crops are saturated

d. all the above.

Explanation: Option C is correct.

103. In case of a trapezoidal notch fall

a. top length of the piers should not be less than their thickness

b. splay up stream from the notch section is 45°

c. splay down stream from the notch section is

d. all the above.

Explanation: Option D is correct.

104. If D1 and D2 are depths of water upstream and down stream of a hydraulic jump, the loss of head at the jump, is

a.

b.

c.

d.

Explanation: Option D is correct.

105. If Dc is the critical depth, D is the depth of water down stream and HL is the total head loss at the cistern of a vertical fall, the depth x of the cistern below the bed level down stream suggested by Blench, is

a.

b.

c.

d.

Explanation: Option B is correct.

106. The uplift pressure on the roof of an inverted syphon, is maximum when

a. drain in running dry

b. canal is running dry

c. canal is running with F.S.L.

d. drain is running with H.F.L.

Explanation: Option D is correct.

107. In rigid module, the discharge

a. is independent of water levels in the distributary and water course

b. depends upon the water level in distributary

c. depends upon the water level in the water course

d. depends upon the water levels of both, i.e. distributary and water course

Explanation: Option A is correct.

108. The ratio of the rate of change of discharge of an outlet and parent channel, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

Explanation: Option C is correct.

109. Cross regulators in main canals are provided

a. to regulate water supply in the distributaries

b. to increase water head upstream when a main canal is running with low supplies

c. to overflow excessive flow water

d. none of these.

Explanation: Option B is correct.

110. The ratio of the head recovered to the head put in, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

Explanation: Option A is correct.

111. The level of the fioor of a syphon aqueduct can be obtained

a. by subtracting the depth of the culvert from the canal bed level

b. by subtracting the thickness of culvert plus the depth of the culvert from the canal bed level

c. both (a) and (b)

d. none of these.

Explanation: Option B is correct.

112. A fall which maintains its depth, is

a. a trapezoidal notch fall

b. a low weir fall

c. a rectangular notch fall

d. all the above.

Explanation: Option D is correct.

113. The water face of the guide banks, is protected by

a. one men stone pitching

b. two man stone pitching

c. three man stone pitching

d. four man stone pitching

Explanation: Option A is correct.

One man stone pitching means stone weighing 40-60 kg that can be handled by a single person.
As equipment may be inaccessible to certain locations thus stones are lay manually at many locations.

114. Effective precipitation for a crop may be defined as

a. total precipitation minus the loss due to evaporation

b. total precipitation minus the loss due to infiltration

c. total precipitation during the crop period

d. available water stored in soil within root zone of the crop.

Explanation: Option D is correct.

115. If average particle size of the silt in millimetres is m, the Lacey's silt factor f is proportional to

a. m3

b. m1/2

c. m1/3

d. m2/3

Explanation: Option B is correct.

116. Lane's weighted creep theory assumes

a. double weightage to horizontal creep and one weightage to vertical creep

b. triple weightage to horizontal creep and one weightage to vertical creep

c. triple weightage to vertical creep and one weightage to horizontal creep

d. double weightage to vertical creep and one weightage to horizontal creep.

Explanation: Option C is correct.

We know,
Horizontal factor=1/3 and vertical factor = 1,
Horizontal factor/vertical factor = 1/3,
Horizontal factor=1 and vertical factor=3. Therefore,  D is correct.

117. Disposal of extra excavated earth of canals, is utilised to provide a spoil bank on

a. left side

b. right side

c. both sides

d. all the above.

Explanation: Option D is correct.

118. If A is the area of the surface, x is the depth of its C.G. from the surface of the water and ω is the density of water, then

a. total pressure on the surface is equal to ω x

b. depth of the point at which total pressure acts is equal to its moment of inertia divided by A x

c. depth of the centre of pressure is 2/3H vertically below the surface

d. all the above.

Explanation: Option D is correct.

119. V and R are the regime mean velocity and hydraulic mean depth respectively in metres. Lacey's silt factor f is

a.

b.

c.

d.

Explanation: Option C is correct.

120. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

Explanation: Option D is correct.

121. According to Kennedy, the critical velocity (V0) in metres in a channel is the mean velocity which keeps the channel free from silting or scouring. Its value is given by (where m is critical velocity ratio and D is the depth of the channel).

a.

b.

c.

d.

Explanation: Option B is correct.

122. According to Lacey, depth of scour in a river depends upon the straightness of the reach. If D is the depth of scour in regime flow in a right angled bend, it is

a. 1.25 D

b. 1.50 D

c. 1.75 D

d. 2.00 D

Explanation: Option D is correct.

123. The width of a dowla is generally kept between 30 to 60 cm and its height above the road level should invariably be more than

a. 10 cm

b. 20 cm

c. 30 cm

d. 40 cm

Explanation: Option C is correct.

124. The field capacity of a soil depends upon

a. capillary tension in soil

b. porosity of soil

c. both (a) and (b)

d. neither (a) nor (b).

Explanation: Option C is correct.

125. Finally formed berms in canals are provided for

a. protection of banks erosion by the waves

b. control of seepage losses

c. strengthening of banks

d. all the above.

Explanation: Option D is correct.

126. An outlet is said to be proportional if its flexibility, is

a. zero

b. less than one

c. more than one

d. one

Explanation: Option D is correct.

Proportional outlet, Flexibility, F = 1.
Hyper-proportional outlet, Flexibility, F > 1.
Sub-proportional outlet, Flexibility, F < 1.

127. The ratio of the discharge over a trapezoidal crest to a rectangular crest of Sarda falls of identical parameters, is

a. 1.084

b. 1.074

c. 1.064

d. 1.054

Explanation: Option A is correct.

128. For standing crops in undulating sandy fields, the best method of irrigation, is

a. sprinkler irrigation

b. free flooding

c. check method

d. furrow method

Explanation: Option A is correct.

Standing crops are cereal crops like wheat, maize or simply the crops that grow on by the plant of grass family.

129. In a syphon aqueduct

a. drainage passes over the canal and F.S.L. of the canal is below the bottom of the drainage trough

b. drainage passes over the canal and F.S.L. of the canal is above the bottom of the drainage trough

c. canal passes over the drainage and H.F.L. of the drainage is above the bottom of the canal trough

d. canal passes over the drainage and H.F.L. of the drainage is below the bottom of the canal trough.

Explanation: Option C is correct.

130. Which one of the following statements is correct ?

a. The canal system and the drainage system are complementary

b. The canal system runs on ridges, gets divided into smaller channels and spreads its water on the land

c. The drainage system runs along the lowest line, collects drainage water from the slopes and joins the main stream

d. All the above.

Explanation: Option D is correct.

131. Garnett's diagrams are used for graphical solution of design equations of a canal by

a. Lacey's theory

b. Kennedy's theory

c. Gibb's theory

d. Lindlay theory.

Explanation: Option B is correct.

132. A current meter measures the velocity of flow, if it is held

a. on the surface of water

b. at the bottom of channel

c. at any point within the cross-section

d. none of these.

Explanation: Option C is correct.

133. In a concrete canal the approximate permissible velocity of water should not exceed

a. 1 m/sec

b. 1.5 m/sec

c. 2 m/sec

d. 2.5 m/sec

Explanation: Option C is correct.

For precast cement, concrete velocity should not exceed 2 m/sec.
For cast in situ concrete, the velocity should not exceed 2.5 m/sec.

134. Pick up the correct sequence of the part of a canal system from the following

a. Head work-distributary-branch canal-minor

b. Head works-main canal-branch canal-dis-tributary-minor

c. Head works-main canal-branch canal-minor-distributary

d. Heads works-branch canal-main canal distributary, minor.

Explanation: Option B is correct.

135. A counter berm is

a. a horizontal benching provided on the inside slope

b. a horizontal benching provided on the outside slope

c. a vertical benching provided on the outer edge of the bank

d. a vertical benching provided on the inner edge of the bank.

Explanation: Option B is correct.

136. Pick up the correct statement from the following:

a. Static level of water in an open well indicates the position of the water table

b. Saturated pores of a soil in the capillary zone above water table have negative pressure

c. At water table, hydrostatic pressure is zero

d. All the above.

Explanation: Option D is correct.

137. For diversion of flood water of rivers, the type of canal constructed, is

a. ridge canal

b. perennial canal

c. inundation canal

d. canal.

Explanation: Option C is correct.

138. To control the silt entry into a distributary at head regulator, King's vanes are provided which are the walls

a. of height l/3rd to l/4th the depth of water in the main canal

b. spaced at interval of  times their heights

c. extending about one metre beyond a line drawn at an inclination of 2 : 1 to the axis of main canal

d. all the above.

Explanation: Option D is correct.

139. According to Lacey, in regime conditions

a. silt is kept in suspension by vertical components of eddies

b. entire cross-section of the channel is generated at all points by the forces normal to the wetted perimeter

c. both (a) and (b)

d. neither (a) nor (b).

Explanation: Option C is correct.

140. Process of meandering is due to

a. sediment load of streams

b. discharge and hydraulic properties of streams

c. relative erodibility of the bed and banks

d. all the above.

Explanation: Option D is correct.

141. If the height of the hydraulic gradient line above the floor of thickness t is h and the specific gravity of the material of the floor is G, the minimum thickness t of the floor down stream of the crestwall, is given by the equation

a.

b.

c.

d.

Explanation: Option C is correct.

142. For cereal crops the most commonly adopted method of irrigation, is

a. free flowing method

b. check method

c. furrow method

d. sprinkling method.

Explanation: Option B is correct.

Standing crop -- sprinkler(high efficiency).
Orchard tree -- basin.
Rolling land. -- free(wild flooding).
Cereal -- check flooding.
Low water -- drip.
Low water with high pr -- sprinkler.
Sewage water -- effluent irrigation.

143. While deciding the alignment of a proposed canal taking off from a river at B, three alignments of approximately equal lengths are available. These cross a drainages at C1, C2 and C3 where drainage bed levels C1 > C2 > C3, you will provide at site C3

a. an aqueduct

b. a syphon aqueduct

c. a super passage

d. a syphon.

Explanation: Option A is correct.

144. The ratio of the rate of change of discharge of an outlet to the rate of change in level of water surface in a distributary at its normal depth, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

Explanation: Option B is correct.

It's not inverse.
Sensitivity is with respect to Distributory normal depth.
And flexibility is with respect to Parent channel.

145. Lacey's equation for calculating flood discharge in rivers, is

a. V = 10.8 R1/2 S1/2

b. V = 10.8 R2/3 S1/2

c. V = 10.8 R2/3 S1/3

d. V = 10.8 R1/3 S2/3

Explanation: Option C is correct.

The three fundamental equations are:
V = 0.639 √fR
Where V is regime velocity in m/sec.
Af2 = 141.2 V5(2)
V = 10.8 R2/3 S1/3 ..(3)
Where S is the slope of water surface.

146. The most suitable section of a lined canal, is

a. trapezoidal section with rounded corners for large canals

b. rectangular section with rounded corners for large canals

c. both (a) and (b)

d. both (a) and (c)

Explanation: Option C is correct.

147. To hold hydraulic jumps, baffle walls are provided in

a. Sarda type falls

b. English type falls

c. Montague type falls

d. Vertical type falls.

Explanation: Option B is correct.