***Sub Engineer***

1. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

Explanation: Option C is correct.

Excavation up to 30 cm is measured in sq m and if it is greater than 30 cm then it is measured in the cubic meter.

2. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

Explanation: Option C is correct.

It is an approximate method. It varies from city to city. A mason is completing 7.5sq m for cement concrete floor per day.

3. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

Explanation: Option D is correct.

No, it is wrong because this formula is used to calculate the volume cut cone where r = upper portion radius and are is the bottom portion radius.

4. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

Explanation: Option B is correct.

Both options B and C are right.
If we take A1, A2, A3.......,An-1, An then option B is right.
If we take A0, A1, A2, A3.....An-1, An then option C is right.

5. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

Explanation: Option A is correct.

Length is more than Width and height. But some times in case of height walls/Excavations/Cuts etc. Length and height are more than Width, hence we cannot define mandatory as Length, Width and height.

6. As per Indian Standard Specifications, the peak discharge for domestic purposes per capita per minute, is taken

a. 1.80 litres for 5 to 10 users

b. 1.20 litres for 15 users

c. 1.35 for 20 users

d. All the above.

Explanation: Option D is correct.

Is code is 135 litre/capita/day.
Now, 135/(24*60)= 0.09375 litre/capita/minute.
capita is multiplying per minute.

7. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

Explanation: Option D is correct.

8. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

Explanation: Option D is correct.

To resist hogging moment at supports in continuous beam reinforcement provided in the form of bent up bars may be in 45 degrees.

9. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

Explanation: Option D is correct.

10. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

Explanation: Option A is correct.

10cm, Actually in India half brick thick wall is 115 mm thick. Half brick Modular brick wall is 10cm. Thick including thickness of plaster. However plaster is paid separately in both the cases. In India half brick thick wall is paid in sq.meter.

11. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

Explanation: Option D is correct.

12. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

Explanation: Option D is correct.

13. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

Explanation: Option B is correct.

14. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

Explanation: Option C is correct.

15. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

Explanation: Option D is correct.

Length is measured 0.01.
Width is measured 0.01.
Area 0.01 Volume 0.01.
For woodwork 0.001.
For Steel work 0.001

16. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

Explanation: Option D is correct.

17. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

Explanation: Option D is correct.

18. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

Explanation: Option D is correct.

19. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

Explanation: Option D is correct.

For 10 m3 of cement concrete work.
Masons needed are 2.
Hence there is one mason, then the quantity in m3 is 5m3.

20. The area of the cross-section of a road fully in banking shown in the given figure, is

a.

b.

c.

d. None of these

Explanation: Option A is correct.

21. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

Explanation: Option D is correct.

22. For 12 mm thick cement plastering 1 : 6 on 100 sq.m new brick work, the quantity of cement required, is

a. 0.200 m3

b. 0.247 m3

c. 0.274 m3

d. 0.295 m3

Explanation: Option C is correct.

dry vol=area*thckness*1.3*1.25.
dry vol=100*0.012*1.3*1.25.
          =1.95cum,
Cement=1.95/1+6,
Cement=0.278cum,

23. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

Explanation: Option B is correct.

As per is 2470(part 1):1985--minimum width 750 mm,
Min capacity 1000 litre,
Min freeboard 300 mm,
Min grade of concrete for septic tank m15.

24. The item of the brick structure measured in sq.m, is

a. Reinforced brick work

b. Broken glass coping

c. Brick edging

d. Brick work in arches.

Explanation: Option B is correct.

25. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

Explanation: Option D is correct.

26. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

Explanation: Option D is correct.

27. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

Explanation: Option D is correct.

28. The value of 'C' of Indian type W.C. shown in the given figure is :

a. 400 mm

b. 450 mm

c. 500 mm

d. 550 mm

Explanation: Option C is correct.

29. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. √d2 x (ds)2

c. L.D √1 + s2

d. 2 LD √1 + s2

Explanation: Option C is correct.

30. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

Explanation: Option D is correct.

31. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

Explanation: Option A is correct.

32. The expected out turn of 12 mm plastering with cement mortar is

a. 4.0 sq m

b. 6.0 sq m

c. 8.0 sq m

d. 10 sq m

Explanation: Option C is correct.

33. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

Explanation: Option D is correct.

34. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

Explanation: Option B is correct.

35. The reduced levels of points, 30 metres apart along the longitudinal section of a road portion between chainages 5 and 9 are shown in the given figure. If there is a uniform up-gradient of the road 120 in 1, the chainage of the point with no filling or cutting is

a. (6 + 15) chains

b. (6 + 12) chains

c. (6 + 18) chains

d. None of these.

Explanation: Option B is correct.

36. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

Explanation: Option A is correct.

37. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

Explanation: Option D is correct.

38. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

Explanation: Option A is correct.

39. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

Explanation: Option D is correct.

40. Pick up the item whose weight is added to the weight of respective item, is

a. Brackets

b. Bolts

c. Distance separators

d. All the above.

Explanation: Option D is correct.

41. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

Explanation: Option B is correct.

42. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

Explanation: Option D is correct.

43. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

Explanation: Option D is correct.

44. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

Explanation: Option C is correct.

45. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

Explanation: Option D is correct.

The increase of length due to bending stress is called bond length.
Bond length = (diameter/4)X stress/strength.
= 1400x D/ 4x6.
= 59 D.

46. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

Explanation: Option D is correct.

A Chhajja is the projecting or overhanging eaves or cover of a roof, usually supported on large carved brackets.

47. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

Explanation: Option A is correct.

48. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

Explanation: Option D is correct.

49. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

Explanation: Option D is correct.

50. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

Explanation: Option C is correct.

51. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

Explanation: Option B is correct.

For 4cm thick flooring constant of rate analysis is 6.6.
6.6/(1+2+4) = 0.94285,
for 2.5cm thick flooring 4.125.

52. For the construction of buildings, the subheads of the estimate are

a. Brickwork, Stone work, Roofing

b. Brickwork Flooring, Wood work, Steel work

c. Plastering or pointing, finishing, water supply and sanitary work

d. All the above.

Explanation: Option D is correct.

53. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

Explanation: Option D is correct.

54. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

Explanation: Option D is correct.

55. The value of 'A' of Indian type W.C. shown in the given figure is :

a. 25 cm

b. 30 cm

c. 40 cm

d. 45 cm

Explanation: Option B is correct.

56. Referring of given figure, pick up the correct statement from the following:

a. The total length of centre line of four walls is 20 m

b. Length of long wall out-to-out is 6.80 m

c. Length of short walls in-to-in is 3.20 m

d. All the above.

Explanation: Option D is correct.

As per the center line method;
Centerline length is;
Long side= 6.80-.8= 6.0
Short side= 4.80-.8 = 4.0
So total length of c/c by center line method is = 2*(6+4.0) = 20.

57. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

Explanation: Option D is correct.

58. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

Explanation: Option C is correct.

Original volume= 1000*8*(15+10) = 2000m3.
Cut off volume= 1/4 * 2000 = 500m3.
Box cutting=2000-500= 1500m3.

59. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

Explanation: Option C is correct.

60. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet.

Explanation: Option C is correct.

It is used at top of p trap W.C. As more pressure at inlet puts water downwards and air comes outside from there.

61. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

Explanation: Option D is correct.

62. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

Explanation: Option D is correct.

63. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

Explanation: Option D is correct.

64. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

Explanation: Option D is correct.

Dist less than 500m-- lead 50m.
Distance 500m-5km--lead 500m.
Distance more than 5km--lead 1km.
So, Option D incorrect.

65. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

Explanation: Option D is correct.

66. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

Explanation: Option D is correct.

Glazing means glasses ( used in commercial buildings).
Skirting means tiles between floors and walls (used in the home).

67. The expected out turn of half brick partition wall per mason per day is

a. 1.5 m3

b. 2.0 m3

c. 4.0 m2

d. 5.0 m2

Explanation: Option B is correct.

It's 5 for half-brick.
Rcc work - 3.25.
Distemper - 35.
Excavation of soil - 3.
Concrete - 5.
12 mm plaster - 8.

68. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

Explanation: Option B is correct.

Top width is B.
Bottom width is b + sd + sd = b + 2sd.
Depth of embankment is d.
Area of trapezoidal = (d/2) * (b + b + 2sd) .
= (d/2) * (2b + 2sd).
= (d/2) * 2 * (b + sd).
= bd + sd * d.
= bd + s(d^2).

69. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

Explanation: Option C is correct.

70. The cross-section of a road partly in banking and partly in cutting is shown in the given figure. The area of the shaded portion is

a.

b.

c.

d.

Explanation: Option A is correct.

Option C is correct for non shaded portion.
And in option A 1/2 instead of 1/3 and this is for non shaded portion.

71. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

Explanation: Option C is correct.

Actual Size of Modular brick = 19cm * 9cm  * 9cm.
Nominal Size of Modular brick = 20cm * 10cm * 10cm.

72. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

Explanation: Option D is correct.

73. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

Explanation: Option D is correct.

74. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

Explanation: Option B is correct.

75. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

Explanation: Option D is correct.

Because shops and factories that wire netting seal in quintal. So, wire and fabric reinforcement consider in quintal.

76. Due to change in price level, a revised estimate is prepared if the sanctioned estimate exceeds

a. 2.0%

b. 2.5%

c. 4.0%

d. 5.0%

Explanation: Option D is correct.

When sanctioned estimate is likely to exceed by more than 5% either from the rates being found insufficient or from any cause whatsoever except important structural alterations.

77. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

Explanation: Option D is correct.

78. The expected out turn of brick work in cement mortar in foundation and plinth per mason per day, is

a. 1.00 m3

b. 1.25 m3

c. 1.50 m3

d. 1.75 m3

Explanation: Option B is correct.

79. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

Explanation: Option D is correct.

80. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

Explanation: Option D is correct.

81. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

Explanation: Option D is correct.

82. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

Explanation: Option B is correct.

83. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

Explanation: Option B is correct.

84. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

Explanation: Option D is correct.

85. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

Explanation: Option C is correct.

86. The rate of payment is made for 100 cu m (per % cu m) in case of

a. Rock cutting

b. Excavation in trenches for foundation

c. Earth work in filling the plinth

d. all the above.

Explanation: Option D is correct.

87. The cost of the earthwork in excavation for the surface drain of cross-section shown in the given figure for a total length of 5 metres @ Rs. 450% cum, is

a. Rs. 400

b. Rs. 425

c. Rs. 450

d. Rs. 500

Explanation: Option C is correct.

88. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

Explanation: Option D is correct.

89. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

Explanation: Option D is correct.

Half brick wall is measured in square meter (m^2).
The rest brick wall measurement are done in cubic meter (m^3).

90. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

Explanation: Option B is correct.

91. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

Explanation: Option D is correct.

92. The cross-sectional area of the embankment of a canal fully in embankment in the given figure is

a.

b.

c.

d.

Explanation: Option C is correct.

93. The value of 'B' of Indian type W.C. shown in the given figure is :

a. 45 cm

b. 50 cm

c. 30 cm

d. 25 cm

Explanation: Option A is correct.

94. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

Explanation: Option D is correct.

95. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

Explanation: Option D is correct.