***Sub Engineer***

1. In a mortar, the binding material is

a. cement

b. sand

c. surkhi

d. cinder.

Explanation: Option A is correct.

By human test, cement is cool & soft felling.
When Cement & water mixing then water density less than cement. That's my point.

2. Lacquer paints

a. are less durable as compared to enamel paints

b. consist of resin and nitro-cellulose

c. contain alcohol as thinner

d. all the above.

Explanation: Option D is correct.

It's a liquid made of shellac dissolved in alcohol, or of synthetic substances, that dries to form a hard protective coating for wood, metal, etc.

3. Wrought iron contains carbon upto

a. 0.25%

b. 1.0%

c. 1.5%

d. 2%.

Explanation: Option A is correct.

C.I and Pig iron - 2 to 5%.
Steel - 0.5 to 1.5%.
W.I - 0.1 to 1.25%.
Mild steel - less than 0.25.

4. Pick up the polymineralic rock from the following:

a. Pure gypsum

b. Magnesite

c. Granite

d. None of these.

Explanation: Option C is correct.

Granite = mica, felspar, quartz, amphibole.
Pure gypsum = only mineral of sulphates.
Magnesite = mineral which has nickel, iron, cobalt, manganese as an admixture.

5. Pick up the correct statement from the following:

a. For mass concrete structures subjected to wetting and drying, the water ratio should be 0.55

b. For thin structures which remain continuously under water, the water-cement ratio by weight should be 0.55

c. For massive concrete structures which remain continuously under water, the water cement ratio by weight should be 0.65

d. All the above.

Explanation: Option D is correct.

In case of under water mass concrete structure, w/c ratio should be high as the concrete needs high degree of work ability. Similarly for thin concrete structures also.

6. Ultimate strength to cement is provided by

a. Tricalcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

Explanation: Option B is correct.

Di-calcium Silicate (C2S): It is responsible for the ultimate strength of concrete. This is also called as Belite.

7. Elastomers can extend upto

a. five times their original dimensions

b. seven times their original dimensions

c. ten times their original dimensions

d. three times their original dimensions.

Explanation: Option C is correct.

Elastomer is high elastic material used in PIER on Bearing plate (Elastomeric). It can be stretched to 10 time of it's original dimension.

8. Bitumen felt

a. is used as water proofing material

b. is used as damp proofing material

c. is made from bitumen and hessian fibres

d. all the above.

Explanation: Option D is correct.

Destructive distillation is separation of solid components from the mixture by heat. But fractional distillation is separation of liquid components from the mixture.

9. In   the   method   of   condensation polymerization,

a. low-molecular substances are removed from the high molecular substance

b. the reaction proceeds with an evolution of ammonia

c. the reaction proceeds with an evolution of hydrogen chloride

d. all the above.

Explanation: Option D is correct.

Condensation polymerization is formed by condensation reaction which is joining of molecules losing small molecules such as water or methanol.

10. In the cement the compound quickest to react with water, is

a. Tricalcium aluminate

b. Tetra-calcium alumino-ferrite

c. Tricalcium silicate

d. Dicalcium silicate.

Explanation: Option A is correct.

Option B is correct.
The Rate of hydration C4AF>C3A>C3S>C2S.
The Heat of hydration C3A>C3S>C4AF>C2S.

11. The initial setting time of lime-pozzolana, is

a. 30 minutes

b. 60 minutes

c. 90 minutes

d. 120 minutes.

Explanation: Option D is correct.

Aluminum has a quick setting quality when mixed as raw material then how high alumina cement has more setting time as compared to opc, Even the aluminum amount is high.

12. The clay to be used for manufacturing bricks for a large project, is dugout and allowed to weather throughout

a. the monsoon

b. the winter

c. the summer

d. none of these.

Explanation: Option A is correct.

Why the weathering is preferred in monsoon because of the condition of atmosphere is some amount of humidity so while Weathering the clay there is no loss of water.

13. The rocks which are formed due to cooling of magma at a considerable depth from earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Igneous rocks.

Explanation: Option A is correct.

Plutonic rocks are coarse-grained crystalline igneous rocks formed by consolidation of molten rock material or magma below the Earth's surface (Latin Pluto, god of the Underworld).

14. Quartzite is a

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

Explanation: Option D is correct.

A and D both are correct.
It is metamorphic by physical properties, it is siliceous by chemical properties.

15. The variety of pig iron used for manufacture of wrought iron, is

a. Bessemer pig

b. Grey or foundry pig

c. White forge pig

d. Mottled pig.

Explanation: Option C is correct.

Bessemer pig iron- cast iron(1.5 to 4%).
Gray pig iron/foundary pig iron- Mild steel 0.25%,
White pig iron /forge pig iron- wrought iron >0.25%,
Mottoled pig iron- heavy foundry work.

16. Sand stone is

a. sedimentary rock

b. metamorphic rock

c. igneous rock

d. volcanic rock.

Explanation: Option A is correct.

Yes, the sandstone ia silicious rock for chemical classification It is sedimentary and Fragmental rock for geological classification.

17. If the furnace is provided with insufficient fuel at low temperatures, the type of pig iron produced, is called

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

Explanation: Option C is correct.

Bessemer pig iron - Mild steel.
Grey pig iron - Cast iron.
White Forge pig - Wrought Iron.
Mottled Pig Iron - Heavy work.

18. Stainless steel contains

a. 18% of chromuim and 8% nickel

b. 8% of chromium and 18% of nickel

c. 12% of chromium and 36% of nickel

d. 36% of chromium and 12% of nickel.

Explanation: Option A is correct.

Chromium is used in stainless steel to make it more corrosion resistant and durable.
Nickel to make it strong and versatile.
18% Chromium.
8%nickel.

19. Pick up the hypabyssal rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

Explanation: Option B is correct.

Examples of hypabyssal rocks include dolerite, microgranite and microdiorite.
Examples of plutonic rocks are granite, gabbro, and granodiorite.

20. Depending on the chemical composition and mechanical properties, iron may be classified as

a. cast iron

b. wrought iron

c. steel

d. all the above.

Explanation: Option D is correct.

Carbon in cast iron= 2- 4%.
Steel = less then 2, more then 0.15% (depending on steel type).
Wrought iron = 0.15.

21. Wrought iron contains carbon about

a. 1.5% to 5.5%

b. 0.5% to 1.75%

c. 0.1% to 0.25%

d. none to these.

Explanation: Option C is correct.

Wronght iron <0.25%(carbon content).
Low carbon content steel or mild steel 0.25%.
Medium  0.25 to 0.70%.
High 0.7 to 1.5%.

22. The main constituent of fly-ash, is

a. aluminium oxide

b. silica

c. ferrous oxide

d. All of these.

Explanation: Option D is correct.

Fly ash is volcanic ash and coal combustion process ash in powdered form,
It doesn't contain cementious property in itself but when react with Caoh2 in cement it form cementious compounds CSH gel! Increase its strength!

23. Bitumen in

a. solid state, is called asphalt

b. semi fluid state, is called mineral tar

c. fluid state, is called petroleum

d. all the above.

Explanation: Option D is correct.

Bitumen is obtained from the fractional distillation of petroleum. While tar is obtained from the district distillation of coal. So, Options D is wrong.

24. The plastics made from cellulose resin

a. are as clear as glass

b. are tough and strong

c. possess excellent electrical properties

d. All the above.

Explanation: Option D is correct.

25. Kaolin is chemically classified as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

Explanation: Option B is correct.

Kaolin is a type of clay found in nature. It can also be made in a laboratory.
ie main constituent is clay so it is argillaceous rock.

26. Which one of the following is acid resistant asbestos:

a. amosite asbestos

b. anthophylite asbestos

c. crocidolite asbestos

d. All the above.

Explanation: Option D is correct.

Asbestos is a generic name given to six fibrous minerals that have been used in commercial products. The six types of asbestos are chrysotile, crocidolite, amosite, anthophyllite asbestos, tremolite asbestos, and actinolite asbestos.

27. Due to attack of dry rot, the timber

a. cracks

b. shrinks

c. reduces to powder

d. none of these.

Explanation: Option C is correct.

Dry rot is wood decay caused by certain species of fungi that digest parts of the wood which give the wood strength and stiffness.

28. Brittleness of cold is due to an excess of

a. sulphur

b. carbon

c. phosphorus

d. silicon.

Explanation: Option C is correct.

When the steel contains an excess of phosphorus content that brittle at low heat (is called COLD short steel).
When the steel contains an excess of sulphur content that brittle at high heat (is called HOT short steel).

29. For the manufacture of Portland cement, the proportions of raw materials used, are

a. lime 63% ; silica 22% ; other ingredients 15%

b. lime 22% ; silica 63% ; other ingredients 15%

c. silica 40% ; lime 40% ; other ingredients 20%

d. silica 70% ; lime 20% ; other ingredients 10%.

Explanation: Option A is correct.

Cement manufacturing needs two types of materials:
1. Calcarius material.
2. Argilacious material.

30. Asbestos cement

a. is brittle

b. warps due to changes in humidity

c. strength is lowered when saturated by water

d. all the above.

Explanation: Option D is correct.

31. Gniess is obtained from

a. igneous rocks

b. metamorphic rocks

c. sedimentary rocks

d. sedimentary metamorphic rocks.

Explanation: Option D is correct.

Gneiss is obtained from metamorphic rock and it is formed by metamorphism of sedimentary and igneous rock.

32. Galvanising means covering iron with a thin coat of

a. tin

b. zinc

c. glaze

d. coal tar.

Explanation: Option B is correct.

Galvanizing is a process in which iron and steel is coated with zinc in order to protect its from rusting.

33. For preparing porcelains, the clay should be

a. sufficiently pure

b. of high degree of tanacity

c. of good plasticity

d. All the above.

Explanation: Option D is correct.

Ceramic materials are inorganic, non-metallic materials made from compounds of a metal and a non-metal. Ceramic materials may be crystalline or partly crystalline. They are formed by the action of heat and subsequent cooling.

34. Polymerization helps to improve the property of

a. strength

b. rigidity

c. elasticity

d. all of these.

Explanation: Option D is correct.

Polymerization is done on concrete during construction it is sprayed on the concrete and increase its physical property like elasticity, tensile strength etc.

35. Good quality stones must

a. be durable

b. be free from clay

c. resist action of acids

d. all the above.

Explanation: Option D is correct.

36. Sewer pipes are made of

a. stone ware

b. refractory clay

c. terracota

d. all the above.

Explanation: Option A is correct.

Sewage (not fresh) is acidic in nature and is composed of nearly 99% water.
Therefore, Stoneware is best suitable to avoid the acid attack and water percolation.

37. Fibre glass

a. has a higher strength to weight ratio

b. is shock proof and fire retardent

c. does not decay

d. all the above.

Explanation: Option D is correct.

38. Pick up the correct statement from the following:

a. The bulking of sand is taken into account while volumetric proportioning of the aggregates

b. The dry sand and the sand completely flooded with water, have practically the same volume

c. The expansion and contraction joints are provided if concrete structures exceed 12 m in length

d. All the above.

Explanation: Option D is correct.

When moisture content is increased by adding more water, the sand particles pack near each other and the amount of bulking of sand is decreased. Thus the dry sand and the sand completely flooded with water have practically the same volume.

39. Pick up the correct statement from the following:

a. In stone arches, the stones are placed with their natural beds radial

b. In cornices, the stones are placed with their natural beds as vertical

c. In stone walls, the stones are placed with their natural beds as horizontal

d. All the above.

Explanation: Option D is correct.

A cornice is decorative trim located at the meeting point between walls and a roof or ceiling. Cornices are used on building exteriors and interiors. On the outside of structures, a cornice is located where the wall meets the roof.

40. The commonly used colour pigment in paints, is

a. carbon black

b. iron oxide

c. lamp black

d. all the above.

Explanation: Option D is correct.

Zinc oxide - white color.
Copper sulphate - green color.
Indigo - blue color.
Burnt umber - brown color.

41. Varnish is a transparent or semi-transparent solution of resinuous substances in

a. alcohol

b. linseed

c. turpentine

d. all the above.

Explanation: Option D is correct.

42. Initial setting time of cement for asbestos cement products should be not less than

a. 30 minutes

b. 50 minutes

c. 75 minutes

d. 90 minutes.

Explanation: Option D is correct.

Portland Pozzolana cement and Asbestos cement have less C3A content as compare to Portland cement. So its setting time between 90 min to 120 min.

43. The variety of pig iron used for the manufacture of steel by Bessemer process, is

a. Bessemer pig

b. Grey pig

c. White forge pig

d. Mottled pig.

Explanation: Option A is correct.

The Bessemer process was the first inexpensive industrial process for the mass-production of steel from molten pig iron. The key principle is removal of impurities from the iron by oxidation with air being blown through the molten iron.

44. For melting one tonne of cast iron

a. 700 m3 air is required

b. 20 kg limestone is required

c. one quintal coke is required

d. all the above.

Explanation: Option D is correct.

45. For filling cracks in masonry structures, the type of bitumen used, is

a. cut-back bitumen

b. bitumen-emulsion

c. blown bitumen

d. plastic bitumen.

Explanation: Option D is correct.

46. Plastic

a. consists of natural or synthetic binders

b. finished products are rigid and stable at normal temperature

c. is capable of flow when necessary heat and pressure are applied

d. All the above.

Explanation: Option D is correct.

In chemistry terminology, plastic is an organic substance because plastics are synthetic materials derived from organic (carbon-containing) compounds.

47. Vanadium steel is generally used for

a. railway switches and crossing

b. bearing balls

c. magnets

d. axles and springs.

Explanation: Option D is correct.

48. Pick up the correct statement from the following:

a. In basic Bessemer process, the steel heats the converter

b. In open-hearth process, the furnace heats the steel

c. In Siemens process, the impurities of pig iron are oxidised by the oxygen of the ore

d. all the above.

Explanation: Option D is correct.

49. The process of manufacturing steel by heating short lengths of wrought iron bars mixed with charcoal in fire clay crucibles and collecting the molten iron into moulds, is known as

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

Explanation: Option B is correct.

Crucible process.
Crucible process, a technique for producing fine or tool steel. The steel was produced by heating wrought iron with materials rich in carbon, such as charcoal in closed vessels. It was known as wootz and later as Damascus steel.

50. The rocks in which argil (or clay) predominates, are called

a. sillicious rocks

b. argillaceous rocks

c. calcareous rocks

d. igneous rocks.

Explanation: Option B is correct.

The rocks having alumina or clay as their major components are known as argillaceous rocks.
Like kaolin chemically classified as argillaceous rock.

51. A badly mixed cement concrete results in

a. segregation

b. bleeding

c. honey combing

d. none to these.

Explanation: Option C is correct.

Segregation is due to the improper placing and occurs due to over-vibration or compaction of concrete.

52. Pick up the correct statement regarding low heat cement from the following:

a. Its initial setting time is about one hour

b. Its final setting time is about 10 hours

c. Its mainly used for mass concrete work

d. All the above.

Explanation: Option D is correct.

In low heat cement c2s containt is high. It means least Heat of hydration and least rate of Hydration.

53. Chemically, marble is known as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

Explanation: Option C is correct.

Marble firstly calcareous rock then metamorphic rock. Limestone with metamorphic action makes marble.

54. Inner part of a timber log surrounding the pith, is called

a. sapwood

b. cambium layer

c. heart wood

d. none to these.

Explanation: Option C is correct.

PHSCB from innermost to outermost,
1. Pitch
2. Heartwood
3. Sap wood
4. cambium
5. Bark

55. The filler used in plastic bitumen, is

a. talc powder

b. asbestos powder

c. plastic powder

d. none of these.

Explanation: Option B is correct.

The quality and durability of bituminous road is influenced by the type and amount of filler material is used.

56. Resins are

a. soluble in spirit

b. used in varnishes

c. left behind on evaporation of oil

d. all the above.

Explanation: Option D is correct.

Option E is correct. Resins are insoluble in water and soluble in other solvents like alcohol and also as the oil or solvent evaporates, resin remains.

57. Refractory bricks are used for

a. retaining walls

b. columns

c. piers

d. combustion chambers.

Explanation: Option D is correct.

58. Expanded metal is

a. manufactured from steel sheets

b. used for reinforced concrete in road pavements

c. measured in term of SWM (shortway mesh) and LWM (long way mesh)

d. all the above.

Explanation: Option D is correct.

59. The rock generally used for roofing, is

a. granite

b. basalt

c. slate

d. pumice.

Explanation: Option C is correct.

60. Apiece of sawn timber whose cross-sectional dimensions exceed 5 cm, in one direction and 20 cm in the other direction, is called a

a. cant

b. deal

c. baulk

d. strip.

Explanation: Option C is correct.

Baulk is a roughly squared timber piece which is obtained by removing the bark and sap-wood one of c/s dimensions exceeds 50mm and other 2200mm.

61. Quick lime (or caustic lime)

a. is obtained by the calcination of pure lime stone

b. has great affinity to moisture

c. is amorphous

d. All the above.

Explanation: Option D is correct.

62. Pick up the correct statement from the following:

a. soft stones are required for carving

b. light stones are required for arches

c. hard stones are required to stand high pressure

d. All the above.

Explanation: Option D is correct.

63. Name the type of cement from the following for canal linings :

a. sulphate resisting cement

b. rapid hardening cement

c. quick setting cement

d. pozzuolana cement.

Explanation: Option A is correct.

Sulfate resisting cement is used in construction exposed to severe sulfate action by water and soil in places like canals linings, culverts, retaining walls, siphons, etc.

64. Mastic asphalt is generally used for

a. damp proof course

b. water proof layer

c. partition walls

d. both (a) and (b).

Explanation: Option D is correct.

65. Black marble is generally found in the district of

a. Jaipur

b. Jabalpur

c. Jaisalmer

d. Pune.

Explanation: Option A is correct.

Marble that contains impurities such as clay minerals, iron oxides, or bituminous material can be bluish, gray, pink, yellow, or black in color.

66. The most fire resistant paints are :

a. enamel paints

b. aluminium paints

c. asbestos paints

d. cement paints.

Explanation: Option C is correct.

67. If P is the percentage of water required for normal consistency, water to be added for determination of initial setting time, is

a. 0.75 P

b. 0.80 P

c. 0.85 P

d. 0.90 P.

Explanation: Option C is correct.

A neat cement paste is prepared by adding 0.85 times the water required to give a paste of standard consistency.
The Quantity of water added to cement = 0.85 X Quantity of water required for standard consistency.

68. A pug mill is used for

a. moulding brick earth

b. tempering brick earth

c. providing brick earth

d. all the above.

Explanation: Option B is correct.

69. A good brick earth should contain :

a. about 50% to 60% of silica

b. not more than 5% of lime

c. about 5 to 6% of oxide of lime

d. All the above.

Explanation: Option D is correct.

Normally, bricks contain the following ingredients:
Silica (sand) : 50% to 60% by weight.
Alumina (clay) : 20% to 30% by weight.
Lime : 2 to 5% by weight.
Iron oxide : 7% by weight.
Magnesia : Less than 1% by weight.

70. The commonly used lime in white washing, is

a. fat lime

b. hydraulic lime

c. lime

d. quick lime.

Explanation: Option A is correct.

71. Shingle is

a. decomposed laterite

b. crushed granite

c. water bound pebbles

d. air weathered rock.

Explanation: Option C is correct.

72. Pick up the plutonic rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

Explanation: Option A is correct.

73. The lime which contains mainly calcium oxide and slacks with water, is

a. quick lime

b. hydraulic lime

c. poor lime

d. none of these.

Explanation: Option A is correct.

Quick lime is correct!  Major part is calcium oxide which is quick to react with H2O and cause slacking.

74. Seasoning is

a. a process of removing sap

b. creosoting

c. painting with sodium silicate

d. coating with tar.

Explanation: Option A is correct.

Sap Means the fluid, chiefly water with dissolved sugars and mineral salts, that circulates in the vascular system of a plant.

75. Inhaling of fly-ash over a long period causes

a. fibrosis of lungs

b. bronchitis

c. pneumonitis

d. All of these.

Explanation: Option D is correct.

Inhaling means tha air is does not remove from their body of the material in flow inside the body of the materials.

76. Minimum required water cement ratio for a workable concrete, is

a. 0.40

b. 0.50

c. 0.60

d. 1.0.

Explanation: Option A is correct.

In concrete, for chemical reaction 0.23% water content is require. and 0.15% water content require for evaportion loss. 0.23 + 0.15 = 0.38.
But on-site for convenience, we take 0.38 as 0.40

77. The compound of Portland cement which contributes to the strength after two to three years is

a. Tricalcium silicate

b. Di-calcium silicate

c. Tricalcium aluminate

d. Tetracalcium alumino ferrite.

Explanation: Option B is correct.

78. For slaking of 10 kg of CaO, the theoretical amount of water is

a. 2.2 kg

b. 1.5 kg

c. 3.2 kg

d. None of these.

Explanation: Option C is correct.

Slaking is of the process by adding water of turning the traditional building material lime into burned lime.

79. Pick up the correct statement from the following: Method of sawing timber

a. tangentially to annual rings, is known as tangential method.

b. in four quarters such that each board cuts annual rings at angles not less than 45°, is known as quarter sawing method.

c. cut out of quarter logs, parallel to the medullary rays and perpendicular to annual rings, is known as radial sawing.

d. all the above.

Explanation: Option D is correct.

Medullary rings are the rings which ties all annular rings while annular rings are the rings which are formed every year spreading outside or inside depending on the type of trees.

80. Clay and silt content in a good brick earth must be at least

a. 50%

b. 40%

c. 30%

d. 25%

Explanation: Option A is correct.

Silica (sand) " 50% to 60% by weight.
Alumina (clay) " 20% to 30% by weight.
Lime " 2 to 5% by weight.
Iron oxide "7% by weight.
Magnesia " less than 1% by weight.

81. Bessemer process is used for the manufacture of

a. Pig iron

b. cast iron

c. Wrought iron

d. Steel.

Explanation: Option D is correct.

Before the open heart process, the impurities are been removed by the Bessemer process from iron to make steel.

82. The portion of the brick without a triangular corner equal to half the width and half the length, is called

a. closer

b. queen closer

c. king closer

d. squint brick.

Explanation: Option C is correct.

Option 'C' is correct.
Queen closer: it is a portion of brick obtains by cutting length-wise into two equal parts.

83. The standard size of masonry bricks, is

a. 19 cm x 9 cm x 9 cm

b. 20 cm x 10 cm x 10 cm

c. 21 cm x 11 cm x 11 cm

d. none of these.

Explanation: Option A is correct.

IS 456 recommended as the standard size of brick is 19 * 9 * 9cm and nominal size of brick is with mortar is 20 * 10 * 10 cm 1 cm thickness of the mortar.

84. Which one of the following is an air binding material ?

a. Gypsum

b. Acid-resistant cement

c. Quick lime

d. All of these.

Explanation: Option D is correct.

Air biding materials react in presence of air and form a hard mass. All these material show such property hence these are air binding material.

85. A good quality stone absorbs water less than

a. 5%

b. 10%

c. 15%

d. 20%

Explanation: Option A is correct.

Not more than 10% for stone.
For bricks ; 1st class - 20%.
                   2nd class - 22.5%.
                   3rd class - 25%.

86. Pick up the correct statement from the following:

a. Alexander Parkes, a Scottish chemist prepared a hard material by mixing camphor and alcohol with nitro cellulose and called it, as Parkesite

b. Dr. L. Bakeland, a Belgian scientist prepared a product known as Bakelite

c. Pollark, an Austrian scientist prepared a substance from urea and formaldehyde and called it Plastic

d. All the above.

Explanation: Option D is correct.

87. Plywood is made from

a. common timber

b. bamboo fibre

c. teak wood only

d. asbestos sheets.

Explanation: Option C is correct.

Plywood may be made from hardwoods, softwoods, or a combination of the two. Some common hardwoods include ash, maple, mahogany, oak, and teak.

88. Soundness test of cement determines

a. quality of free lime

b. ultimate strength

c. durability

d. initial setting

Explanation: Option A is correct.

A soundness test is used to know the resistance agency weathering action. In open weather conditions how much amount cemented material expands or shrinks.

89. The slag which floats on the surface of the molten iron generally contains

a. Silica (SiO2) 35%

b. Alumina (Al2O3) 12%

c. MgO, CaSO4, KMnO2 and FeO 8%

d. all the above.

Explanation: Option D is correct.

90. Minimum of 40% of iron, is available in

a. Red haemetite

b. Limonite

c. Siderite

d. Black band.

Explanation: Option D is correct.

91. If the iron ore contains clay as an impurity, the flux added during calcination, is

a. clay

b. lime stone

c. argillaceous iron ore

d. all the above.

Explanation: Option B is correct.

92. Cement is said to be of good quality if

a. one feels cool by thrusting one's hand in the cement bag

b. it is not smooth when rubbed in between fingers

c. a handful of cement thrown into a bucket of water does not float

d. none of these.

Explanation: Option D is correct.

Both (B and D) option are correct. But D option is more accurate as compare to option B. According to R. Agor.

93. Cast steel is manufactured by

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

Explanation: Option B is correct.

Cast steel and Cast iron.
Bessemer process for Cast iron  (carbon content more than 2% and up to 4%).
Crucible Process for Cast Steel  ( carbon content 0.1-0.5%).

94. For the manufacture of plywood, veneers are placed so that grains of adjacent veneers

a. run at right angles

b. parallel

c. inclined at 45°

d. inclined at 60°.

Explanation: Option A is correct.

Grains of adjacent veneers should be parallel to each other. Grains of veneers placed one below the other should be perpendicular.

95. A 1st class brick immersed in water for 24 hours, should not absorb water (by weight) more than

a. 15%

b. 20%

c. 25%

d. 5%.

Explanation: Option B is correct.

1Class brick water absorption 15 to 20% and 2nd class brick water absorption 20 to 22% &3rd class brick water absorption 25%.

96. The proportions of charcoal, saltpetre and sulphur in gun powder by weight, are respectively:

a. 15, 75, 10

b. 75, 10, 15

c. 10, 15, 75

d. 10, 75, 15.

Explanation: Option A is correct.

Gunpowder is an explosive material which is used to Rock blasting in a source of rack area. Gun cotton is a same material to a gun powder.

97. Geologically, marble is known as

a. sedimentary rock

b. igneous rock

c. metamorphic rock

d. stratified rock.

Explanation: Option C is correct.

98. The commonly used drying oil for oil paints, is

a. olive oil

b. linseed oil

c. kerosine oil

d. accetate of lead.

Explanation: Option D is correct.

Drying oil is a kind of vegetable oil which drys in normal temperature. Linseed oil, Poppy oil, Walnut oil, Sunflower oil and Safflower oil are known as typical drying oil. Acetate of lead is a drying agent.

99. Initial setting of cement is caused due to

a. Tri-calcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

Explanation: Option C is correct.

When water is added in cement, first tricalcium aluminate is reacted with water and it is responsible for initial setting of cement.

100. The foliated structure is very common in

a. sedimentary rocks

b. igneous rocks

c. metamorphic rocks

d. none of these.

Explanation: Option C is correct.

Because metamorphic rocks formed due to high pressure and temp. So foliated str is generally formed in metamorphic rocks.

101. Turpentine oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

Explanation: Option A is correct.

As per book of civil engineering by R Agor publisher khanna publisher, turpentine oil is used as thinner.

102. The hardest rock is

a. marble

b. diamond

c. talc

d. quartz.

Explanation: Option B is correct.

A rock formed by only one mineral is called Monomineralic rock and several minerals are  polymineralic rock.

103. The silica is used for preparing

a. silica bricks

b. coke oven

c. lining for glass furnaces

d. all of these.

Explanation: Option D is correct.

104. Wrought iron is manufactured from pig iron by

a. puddling

b. shingling

c. rolling

d. all the above.

Explanation: Option D is correct.

105. Pick up the correct statement from the following:

a. The phenol is either extracted from coal-tar or prepared from benzene

b. Phenol reacts with formaldehyde, to form phenol formaldehyde resin

c. The plastics prepared from phenol-formaldehyde are used for paints, varnishes, w.c. seats

d. All the above.

Explanation: Option D is correct.

106. Fibre boards can be

a. distempered

b. painted

c. painted and distempered

d. used for furniture.

Explanation: Option D is correct.

Fiber boards are used for preparing flush doors, top of tables, insulating materials of heat and sound.

107. Pick up the correct statement from the following:

a. Catalysts are added to assist and accelerate the hardening of resin,

b. The fillers are inert materials and they impart strength and hardness

c. Fibrous fillers increase thermal resistance

d. All the above.

Explanation: Option D is correct.

108. Portland pozzolana cement possesses

a. lower heat of hydration

b. lower shrinkage on drying

c. water tightness

d. all the above.

Explanation: Option D is correct.

109. Iron ore may contain

a. silicon

b. sulphur

c. phosphorus and maganese

d. all the above.

Explanation: Option D is correct.

110. Wrought iron is used for

a. structural works in beams

b. small sized water pipes

c. columns and struts

d. none to these.

Explanation: Option B is correct.

Wrought iron is a low contact of Corbin so use for small size water pipe and it's natural brittle material.

111. The timber having maximum resistance against white ants, is obtained from

a. chir

b. shisham

c. sal

d. teak.

Explanation: Option C is correct.

Teak is the most appropriate among all other woods against termites because it is not only termite resistance but also kills it.

112. Knots in timber are

a. defects caused by crushing fibres

b. splits radiating from the centre

c. speckled strains

d. signs of branches cut off.

Explanation: Option D is correct.

113. Pick up the correct statement from the following:

a. Solder material is an alloy which melts at a temperature above 400°C

b. Brazing is done at temperature above 600°C to 1100°C

c. Brazing joint is stronger than the solder joint

d. all the above.

Explanation: Option D is correct.

Brazing is a metal-joining process in which two or more metal items are joined together by melting and flowing a filler metal into the joint.

114. Lime concrete is generally used for

a. wall foundations

b. flooring at ground level

c. both (a) and (b)

d. neither (a) nor (b).

Explanation: Option B is correct.

Lime concrete is very widely used for foundation bases of load bearing walls, columns, and under layers of floors. So, C is correct.

115. Bituminous fells are used for

a. covering A.C. sheets

b. covering sloping roofs

c. D.P.C.

d. none to these.

Explanation: Option A is correct.

116. The minimum compressive strength of Ilnd class bricks should be

a. 75 kg/cm2

b. 90 kg/cm2

c. 100 kg/cm2

d. 120 kg/cm2

Explanation: Option A is correct.

The minimum crushing / compressive strengths of burnt bricks tested flat-wise prescribed are:
(i) Common building bricks"35 kg/sq. cm.
(ii) Second class bricks"70 kg/sq. cm.
(iii) First class bricks" 105 kg/sq. cm.

117. Stuco paints are suitable for

a. stone masonry

b. brick walls

c. both (a) and (b)

d. neither (a) nor (b).

Explanation: Option C is correct.

118. Pick up the correct statement from the following:

a. Strength and stiffness of corrugated sheets are considerably increased

b. Corrugated sheets are generally gal-venised to protect iron from corrosion by rust

c. Corrugated sheets are generally used on slanting roofs

d. all the above.

Explanation: Option D is correct.

119. Quick setting cement is produced by adding

a. more amount of gypsum in very fine powdered form

b. aluminium sulphate in very fine powdered form

c. pozzolana in very fine powdered form

d. none of these.

Explanation: Option B is correct.

C -, cz gypsum is retardar, which mean increase setting time,
But here asked about the quick setting, i.e accelerator, mean decrease setting time which is aluminium sapphire (Al2So4).

120. Duco is one of the patent forms of

a. plastic paints

b. bituminous paints

c. aluminium paints

d. cellulose paints.

Explanation: Option D is correct.

121. Pick up the correct statement from the following:

a. The polymer consists of thousands of monomers joined together

b. The polymer molecule is called macro-molecule

c. A polymetric material consists of a large number of long-chain molecules

d. All the above.

Explanation: Option D is correct.

122. Bulking of sand is caused due to

a. surface moisture

b. air voids

c. viscosity

d. clay contents

Explanation: Option A is correct.

123. The normal curing period for lime mortar, is:

a. 3 days

b. 7 days

c. 10 days

d. 14 days.

Explanation: Option B is correct.

124. The type of steel used for precision levelling staff, is

a. Titanium steel

b. Carbon steel

c. Invar

d. Stainless steel.

Explanation: Option C is correct.

125. During pudding

a. carbon is converted into carbonic acid gas

b. silicon forms a slag

c. metal is heated by the burning of gases

d. all the above.

Explanation: Option D is correct.

126. German silver is an alloy of

a. silver, gold and lead

b. copper, nickel and zinc

c. copper, brass and zinc

d. brass, silver and zinc.

Explanation: Option B is correct.

127. Pick up the incorrect statement from the following:

a. Plastics are chemical resistant

b. Plastics are durable

c. Plastics are ductile

d. Plastics are excellent electric insulators.

Explanation: Option C is correct.

Plastics are Ductile, Durable and also Good Insulators but less resistant to chemicals, so option A is correct.

128. Pick up correct statement from the following:

a. Fibre boards are used for thermal and acoustic control

b. Fibre boards are used for light weight standing members

c. Fibre boards are obtained by impregnating a resin product on fibres

d. all the above.

Explanation: Option D is correct.

129. Quartzite, a metamorphic stone is

a. brittle

b. crystalline

c. compact

d. All the above.

Explanation: Option D is correct.

130. Pick up the correct statement from the following:

a. Plastics have generally low melting point

b. The coefficient of thermal expansion of plastics is about three times than that of steel

c. The acoustical boards prepared by impregnating fibre-glass with phenolic resins has absorption coefficient of about 0.67

d. All the above.

Explanation: Option D is correct.

131. Dextrin is

a. starch glue

b. albumin glue

c. rubber based adhesive

d. none to these.

Explanation: Option C is correct.

Dextrin are a group of low-molecular-weight carbohydrates produced by the hydrolysis of starch or glycogen.
So, B is correct.

132. Formula for quick lime, is

a. CaCO3

b. CaO

c. CO3CO2

d. none to these.

Explanation: Option B is correct.

133. For the manufacture of stainless steel, steel is mixed with

a. chromium

b. nickel

c. tungsten

d. none of these.

Explanation: Option A is correct.

134. Stones used for ornamental work must be

a. soft

b. hard

c. light

d. heavy.

Explanation: Option A is correct.

Diamond is the example for ornament, and it is the hardest material. So option B, hard might be right.

135. Pick up the correct statement from the following:

a. Quick lime is obtained by burning pure lime stone:

b. Hydraulic lime is obtained by burning lime stone containing clay 5% to 30%

c. Poor lime is obtained by burning lime stone containing impurities more than 5%

d. All the above.

Explanation: Option D is correct.

136. The presence of original rounded surface on the manufactured piece of timber, is called

a. Wane

b. Torn grain

c. Diagonal grain

d. Chipmark.

Explanation: Option A is correct.

A defect in a plank or board characterised by bark or insufficient wood at a corner or along an edge, due to the curvature of the log.

137. Smith's test of stones is performed to find out

a. the presence of soluble matter of stone

b. the compressive strength of the stone

c. the hardness of the stone

d. the toughness of the stone.

Explanation: Option A is correct.

Smith test used to find mud substance in stone.
Acid test if sandstone consis caco3, to find how much it in sandstone.

138. The cracks caused by shrinkage of the exterior surface of the wood exposed to atmosphere, are called:

a. radial shakes

b. heart shakes

c. wind cracks

d. twisted fibres.

Explanation: Option C is correct.

If the wood is exposed continuously to the high-speed winds, the outer surface shrinks and forms crack externally which are called wind cracks.

139. The process of decarbonising the pig iron completely and then adding proper percentage of carbon for manufacturing steel, is called

a. Cementation process

b. crucible process

c. Bessemer process

d. Open hearth process.

Explanation: Option A is correct.

In cementation process, the pig iron is converted to almost pure wrought iron and then preparing steel by adjusting carbon content.

140. In paints, the pigment is responsible for

a. colour

b. smoothness

c. glassy face

d. none of these.

Explanation: Option A is correct.

141. The operation of removal of impurities or clay adhering to iron ores, is known as

a. dressing

b. calcination

c. roasting

d. smelting.

Explanation: Option A is correct.

142. If water required for 1 bag of cement is 30 litres, the water cement ratio is :

a. 0.40

b. 0.50

c. 0.60

d. None of these.

Explanation: Option C is correct.

Wt of  1bag cement = 50 kg.
Wt of 30litre water = 30 kg.
Wc ratio = wt of water /wt of cement = 30/50.
Wc ratio = 0.6.

143. Portland cement manufactured from pure white chalk and clay but free from iron-oxide, is known as

a. quick setting cement

b. rapid hardening cement

c. white cement

d. low heat Portland cement.

Explanation: Option C is correct.

Iron oxide is basically used to impart colour but if there is no iron oxide cement will be white cement.

144. Dry rot

a. reduces the strength of timber

b. reduces the timber to powder

c. shrinks the timber

d. spoils the appearance of timber.

Explanation: Option B is correct.

145. Which one of the following polymers is obtained from condensation polymerization?

a. phenol formaldehyde

b. carbamide

c. melamine-formaldehyde

d. all of these.

Explanation: Option D is correct.

146. The weight of 1 m3 of brick earth, is about

a. 1200 kg

b. 1500 kg

c. 1800 kg

d. 2000 kg.

Explanation: Option C is correct.

The weight of a brick = 3.5 kg.
No. Of bricks in 1m^3 = 500 bricks.
Therefore;
The weight of 1m^3of brick earth  = 500 * 3.5.
                                        =  1750 kg.

147. Pozzolana (or surkhi) is used in lime

a. to impart hydraulicity

b. to prevent shrinkage

c. to decrease the cost of construction

d. to decrease the setting time.

Explanation: Option A is correct.

148. Mastic asphalt is normally used for

a. sound insulation

b. water proofing

c. fire proofing

d. none to these.

Explanation: Option B is correct.

Mastic asphalt is used for covering terasses, carparks and pavements, urban roads, public facilities, waterproofing engineering structures and sound insulation.

149. The size of modular bricks, is

a. 10 x 10 x 9 cm

b. 19 x 9 x 9 cm

c. 22.5 x 10 x 8.5 cm

d. 22.5 x 8.0 x 9 cm.

Explanation: Option B is correct.

Bricks Sizes as per Indian Standard (IS 1077):
Brick sizes based on IS 1077 specification (Common burnt clay building bricks), the standard modular size of common building brick is 190 x 90 x 90 mm or 190 x 90 x 40 mm (length x depth x height).

150. A piece of timber whose thickness and width are respectively 5 cm and 10 cm is called

a. slate

b. plank

c. board

d. strip.

Explanation: Option D is correct.

A piece of timber having-
Width and thickness both <5cm- Batten.
Thickness <5cm & width <=10cm- Strip.
Thickness <5cm & width >10cm- Board.

151. Pig iron is manufactured from the ores by

a. dressing

b. calcination and roasting

c. smelting

d. all the above.

Explanation: Option D is correct.

152. Quick lime

a. reacts with carbon dioxide

b. may be used for white-washing

c. when mixed with water forms slaked lime

d. all the above.

Explanation: Option D is correct.

153. Age of a tree may be ascertained by

a. radius of its stem

b. circumference of its stem

c. number of branches

d. number of annual rings.

Explanation: Option D is correct.

154. The portion of a brick cut to form angles other than right angles in plan, is known as

a. queen closer

b. king closer

c. closer

d. squint brick.

Explanation: Option D is correct.

King closer is a triangular cut not passing through center of width or length.
Squint Brick is any cut (may or may not be triangular) and doesn't form a right angle.

155. Pick up the correct statement from the following:

a. Quick lime contains calcium oxide

b. Slaked lime may be obtained from quick lime

c. Slaked lime is obtained by adding water to quick lime

d. All the above.

Explanation: Option D is correct.

156. Plywood is obtained by gluing wooden sheets at

a. 100 to 150 N/cm2

b. 100 to 130°C

c. both (a) and (b)

d. Neither (a) nor (b).

Explanation: Option C is correct.

Plywood is obtained by gluing wooden sheets at a pressure of 100 to 150 N/cm2 and a temperature of 100 to 130 degrees Celsius. Plywood is made up of three or more thin layers of wood. These layers are bonded together with an adhesive.

157. The low voltage porcelain is mainly used for

a. switch block

b. insulating tubes

c. lamp sockets

d. All of these.

Explanation: Option D is correct.

Porcelain is a ceramic material made by heating clay-type materials to high temperatures. It includes clay in the form of kaolinite.

158. Putty is

a. made with finely powdered chalk and linseed oil

b. used for fixing glass panes

c. softened by a solution of pearl ash and quick-lime soaked in water

d. all the above.

Explanation: Option D is correct.

Painter's Putty is typically a linseed oil based product used for filling holes, minor cracks and defacements in wood only.

159. The initial setting time of hydraulic lime, is

a. 60 minutes

b. 90 minutes

c. 120 minutes

d. 150 minutes.

Explanation: Option C is correct.

There are 3 types of hydraulic lime.
But, acc to IS code class A lime's (eminent hydraulic lime) initial setting time is 2 hrs.

160. PVC stands for

a. plastic very compact

b. polythene vinyl chloride

c. polythene vinyl carbon

d. polythene vanadium carbide.

Explanation: Option B is correct.

161. Generally wooden moulds are made from

a. shisham wood

b. deodar wood

c. teak wood

d. hard wood.

Explanation: Option A is correct.

162. Cement paints usually

a. contain 5% to 10% colour pigments

b. are prepared with white cement

c. contain 5% sodium chloride

d. all the above.

Explanation: Option D is correct.

163. The cement becomes unsound by the presence of excess

a. sulphur

b. magnesia

c. lime

d. All of these.

Explanation: Option D is correct.

Soundness due to the lime is measured in Le-Chaltier's apparatus.
Soundness due to magnesia is measured in auto clave test.
Soundness due to sulphur can't be measure.

164. The kiln which may work throughout the year, is

a. Clamp

b. Bull's kiln

c. Hoffman's kiln

d. none of these.

Explanation: Option C is correct.

It is a continuous kiln and has a permanent roof as a result of which it can function even during rains.

165. The minimum compressive strength of 1st class bricks should be

a. 90 kg/cm2

b. 100 kg/cm2

c. 120 kg/cm2

d. 130 kg/cm2.

Explanation: Option B is correct.

Minimum compressive strength of 1st class brick is 105 kg/sq. Cm, 2nd class brick is 70 kg/sq. Cm and 3rd class brick is 35 kg/sq. Cm.

166. A volatile substance added to a paint to make its application easy and smooth, is known as

a. base

b. solvent

c. vehicle

d. none to these.

Explanation: Option B is correct.

Actually vehicle= solvent+binder.
Binder is like linseed oil, tung oil, etc. But solvent also called as thinner is a volatile liquid which evaporates, eg. turpentine.

167. The percentage of water for normal consistency, is

a. 10% to 25%

b. 15% to 25%

c. 20% to 30%

d. 25% to 35%.

Explanation: Option D is correct.

168. Pick up the correct statement from the following:

a. The commercial plywoods are available upto 150 cm wide and upto 300 cm long

b. The plywoods possess uniform tensile strength in all directions

c. The plywoods are light in weight

d. All the above.

Explanation: Option D is correct.

169. The main ingredient of a good quality brick earth, is

a. magnesia

b. lime

c. silica

d. alumina.

Explanation: Option D is correct.

The most important ingredient in the brick earth is SILICA. The amount of silica present in brick earth is 50-60% and it prevents cracking and warping of raw bricks.

170. Bitumen is generally obtained from

a. organic material

b. synthetic material

c. petroleum product

d. coal.

Explanation: Option C is correct.

171. For one cubic metre of brick masonry, number of bricks required, is

a. 425

b. 450

c. 500

d. 550.

Explanation: Option C is correct.

THE STANDARD SIZE OF BRICK IS 190 X 90 X 90, BUT IF WE CONSIDER MORTAR ALSO, THEN THIS SIZE BECOMES 200X10X10.
SO THE NUMBER OF BRICK =SPACE VOLUME /(VOLUME OF ONE BRICK) ,1/(.20X.10X.10) = 500.

172. The steel which contains fissures and cavities, is manufactured by

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

Explanation: Option A is correct.

173. Pick up the volcanic rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

Explanation: Option C is correct.

Granite: Plutonic igneous rock.
Dolerite: hypabyssal igneous rock.
Basalt: volcanic igneous rock.
So, option C is correct.

174. In stone masonry, stones (stratified rocks) are so placed that the direction of pressure to the plane of bedding is

a. right angles

b. 45°

c. 60°

d. parallel

Explanation: Option A is correct.

175. The most durable varnish is

a. spirit varnish

b. turpentine varnish

c. oil varnish

d. none of these.

Explanation: Option C is correct.

176. Softer variety of steel may be obtained by

a. Cementation process

b. crucible process

c. Bessemer process

d. Open hearth process.

Explanation: Option C is correct.

177. Pick up the correct statement from the following:

a. The articles prepared from clay which is burnt at low temperature and cooled down slowly, are called earthen-ware

b. The articles prepared from refractory clays which as mixed with stone and crushed pottery, are called stone ware

c. The articles prepared from fine earthen ware which is white, thin and semi-transparent, are called procelain

d. All the above.

Explanation: Option D is correct.

178. Calcination of iron ores is done

a. to remove carbonic acid

b. by roasting in heaps

c. after dressing

d. all the above.

Explanation: Option D is correct.

Calcination --> heated strongly in the absence of air in order to drive out the impurities such as sulphur, arsenic, carbon dioxide and moisture.

179. Strength of cement concrete primarily depends upon

a. quality of water

b. quantity of aggregate

c. quantity of cement

d. water-cement ratio.

Explanation: Option D is correct.

180. Rapid hardening cement contains

a. Tri-calcium silicate

b. Tri-calcium aluminate

c. Tetra-calcium alumino-ferrite

d. Dicalcium silicate.

Explanation: Option A is correct.

Rapid Hardening cement has a maximum percentage of C3S.
Rapid Hardening Cement is also called high early strength cement.

181. Bitumen emulsion is

a. a liquid containing bitumen in suspension

b. a paint

c. used as anti-corrosive paint

d. all the above.

Explanation: Option D is correct.

182. The cast iron when heated to red heat with powdered red haemetite in an oven for increasing its toughness, is converted to

a. grey cast iron

b. white cast iron

c. mottled cast iron

d. toughed cast iron.

Explanation: Option C is correct.

183. Jhumb bricks are

a. under burnt

b. over burnt

c. kutcha

d. none of these.

Explanation: Option B is correct.

Over Burnet vitrified brick, having an irregular shape. Falls as in the qualification of bricks classification.

184. Plastic bitumen is generally used for

a. road pavements

b. expansion joints

c. crack fillings

d. none to these.

Explanation: Option C is correct.

Plastic bitumen is used for making Road Pavements.
It makes it water-resistant thereby increasing its life.

185. Pick up the correct statement from the following:

a. The heating of a material to redness in contact with air, is known as calcination

b. The property of lime by which it sets or hardens in damp places having no free circulation of air is called setting

c. The product that remains after calcination of limestone, is called lime

d. All the above.

Explanation: Option D is correct.

Property of lime by which it sets or hardens in damp places having no free circulation of air is called hydraulicity of lime.

186. The commonly used thinner in oil paints, is

a. turpentine

b. both (a) and (b)

c. neither (a) nor (b)

d. none the these.

Explanation: Option B is correct.

187. Cast iron

a. is manufactured in required shapes

b. may contain 2 to 5 per cent of carbon with other impurities

c. is remelted in a cupola furnace

d. all the above.

Explanation: Option D is correct.

188. Asbestos is

a. an incombustible fire proof material

b. an organic substance

c. a bad insulator for sound and heat

d. all the above.

Explanation: Option D is correct.

189. In a rock calcium carbonate predominates. State whether it is :

a. Sillicious rock

b. Argillaceous rock

c. Calcareous rock

d. None of these.

Explanation: Option C is correct.

190. In order of increasing percentage of silica, the correct sequence is

a. sandy clay, calcareous clay, pure clay

b. calcareous clay, pure clay, sandy clay

c. pure clay, sandy clay, calcareous clay

d. None of these.

Explanation: Option B is correct.

191. The rocks which are formed due to pouring of magma at the earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Igneous rocks

Explanation: Option C is correct.

All are types of Igneous Rocks
Place of solidification varies:
Very Deep under the earth --> Plutonic.
Shallow depth under earth --> Hypabyssal.
On earth Surface --> Volcanic.

192. Pick up the correct statement from the following:

a. Steel produced by open hearth process is milder than that obtained by the bessemer process

b. Engineers prefer open hearth steel for structural purpose as it is more homogenous

c. Basic Bessemer process is suitable for converting poor ore containing a large proportion of sulphur and phosphorus into steel

d. all the above.

Explanation: Option D is correct.

193. Pick up the correct statement from the following:

a. The distinct plane of division along which a stone can easily be split, is called natural bed of stone

b. The natural bed of sedimentary rocks is along the planes of stratification

c. The natural bed of igneous rocks is not defined

d. All the above.

Explanation: Option D is correct.

194. Invar contains

a. 12% of nickel

b. 24% of nickel

c. 30% to nickel

d. 36% of nickel.

Explanation: Option D is correct.

195. Based on flow quality, the sequence of pipes is

a. A.C. pipes, G.I. pipes, C.I. pipes, PVC pipes

b. C.I. pipes, G.I. pipes, A.C. pipes, PVC pipes

c. C.I. pipes, G.I. pipes, PVC pipes, A.C. pipes

d. None of these.

Explanation: Option B is correct.

196. The pigment used in paints for corrosive resistance, is

a. ferrous oxide

b. zinc white

c. red lead

d. gypsum.

Explanation: Option C is correct.

197. A well seasoned timber may contain moisture up to

a. 4 to 6%

b. 6 to 8%

c. 8 to 10%

d. 10 to 12%.

Explanation: Option D is correct.

198. The stones obtained by blasting are used as

a. ballast in railways

b. aggregates for concrete

c. roadmetal

d. All the above.

Explanation: Option D is correct.

199. Distemper is

a. a paint consisting of powdered chalk, pigments, and water

b. a water proofing agent

c. a paint consisting of coloured cement and water

d. a drying agent.

Explanation: Option A is correct.

200. Oil varnish generally consists of

a. synthetic resin and spirit

b. oil, wax and resin

c. resin, oil and turpentine

d. spirit, oil and wax.

Explanation: Option C is correct.

Hard resins such as amber and copal are dissolved in linseed oil. If the varnish is found unworkable, a small amount of turpentine oil may be added.
Ref : S. K. Duggal.

201. Pick up the correct statement from the following:

a. The average crushing strength of hand moulded bricks is 6000 t/m2

b. The average tensile strength of hand moulded brick is 200 t/m2

c. The average shearing strength of hand moulded brick is 600 t/m2

d. All the above.

Explanation: Option D is correct.

202. The cracks which extend from bark towards the sap wood in the cross section of a tree, are called

a. radial shakes

b. star shakes

c. heart shakes

d. cup shakes.

Explanation: Option B is correct.

It should be heart shake because it starts at the bark and towards the pith. In the other hand, in star shakes starts at pith and crack gets towards the bark.

203. Sea sand used in structures causes

a. dampness

b. efflorescence

c. disintegration

d. All of these.

Explanation: Option D is correct.

Efflorescence is the white powdery substance on the surfaces of unsealed concrete and the white blush seen with sealed floors. Efflorescence is caused by vapor migrating through the slab bringing soluble salts to the surface of the concrete.

204. The compound of Portland cement which reacts immediately with water and also sets first is

a. Tri-calcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

Explanation: Option C is correct.

205. Refractory bricks resist

a. high temperature

b. chemical action

c. dampness

d. all the above.

Explanation: Option A is correct.

206. When a brick is immersed in water for 24 hours and then dried, if

a. no grey or white deposits appear on the surface, the brick is free from soluble salts

b. 10 per cent surface is covered with grey or white deposits, the brick has slight efflorescence

c. 50 per cent surface is covered with grey or white deposits, the brick has serious efflorescence

d. All the above.

Explanation: Option D is correct.

207. In arches, stratified stones are placed so that their planes are

a. parallel

b. perpendicular

c. radial

d. none of these.

Explanation: Option C is correct.

In arches! Stratified rockz are arranged in vertical. Stratified stones are obtained from sedimentary classification.

208. Pick up the correct statement from the following:

a. Attrition test is done to find out the rate of wear of stones which are used in road construction

b. Crushing test is done to find out the compressive strength of the stone

c. Impact test is done to determine toughness of a stone

d. All the above.

Explanation: Option D is correct.

209. The fire clay contains pure

a. lime

b. oxide of iron

c. hydrated aluminium silicate

d. magnesium.

Explanation: Option C is correct.

210. Pick up the correct characteristic of Pyroxene from the following:

a. It converts to chlorine by hydration

b. Its density is 2.3 to 3.6 g/cm2

c. Its hardness is 5 to 6

d. All the above.

Explanation: Option D is correct.

The pyroxenes (commonly abbreviated to Px) are a group of important rock-forming inosilicate minerals found in many igneous and metamorphic rocks. Pyroxenes are silicon-aluminum oxides with Ca, Na, Fe, Mg, Zn, Mn, Li substituting for Si and Al.

211. For sanitary pipes and chemical stonewares,

a. salt glazing is used

b. lead glazing is used

c. opaque glazing is used

d. None of these.

Explanation: Option A is correct.

212. The steel used for rails under heavy traffic and on sharp curves, is

a. Nickel steel

b. Chrome steel

c. Magnese steel

d. Vanadium steel.

Explanation: Option C is correct.

213. The weight of a good quality brick when immersed in water for a period of 16 hours should not exceed the weight of dry brick

a. 20%

b. 15%

c. 10%

d. None of these.

Explanation: Option A is correct.

1st class brick in immersed 24 hours, water absorb is 20% but good quality brick is 15% absorbed in dry weight of a brick.

214. Snowcrete is one of the patent forms of

a. distempers

b. water proof cement paints

c. enamel paints

d. cellulose paints.

Explanation: Option B is correct.

215. The usual percentages of clay and metal in cermet are :

a. 50%, 50%

b. 60%, 40%

c. 70%, 30%

d. 80%, 20%.

Explanation: Option D is correct.

A Cermet is a composite material composed of Ceramic (Cer) and Metal (met) materials. It has both the properties of ceramics and metal.

216. Pick up the correct statement from the following:

a. Horn blende mineral is brittle

b. Muscovite is also known as white mica and potashmica

c. Biotite is also known as blackmica

d. All the above.

Explanation: Option D is correct.

217. The rocks which are formed due to cooling of magma at a relatively shallow depth from the earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Ignoeous rocks.

Explanation: Option B is correct.

TYPE OF IGNEOUS ROCK.
On the surface -- VOLCANIC.
At a Shallow depth -- HYPABYSSAL.
At a GREAT DEPTH -- PLUTONIC.

218. The base material for distemper, is

a. chalk

b. lime

c. lime putty

d. cement wash.

Explanation: Option A is correct.

219. The thermosetting plastic

a. becomes rigid when moulded at suitable pressure and temperature

b. at 127°C to 177°C permanently set and further application of heat does not soften it

c. charrs at 343°C

d. All the above.

Explanation: Option D is correct.

220. Pick up the synthetic resin from the following:

a. Phenolic resin

b. Resorcinol resin

c. Urea resin

d. All of these.

Explanation: Option D is correct.

221. Slacking of lime is affected by

a. keeping it exposed to air

b. immersing the lime in water

c. crushing the lime lumps

d. none of these.

Explanation: Option B is correct.

The process of adding water to calcium oxide to produce calcium hydroxide is referred to as hydration process or lime slaking. This hydration process, when done with just the right amount of water for chemical reaction, is called "Dry Hydration".

222. The steel used in R.C.C. work is

a. stainless steel

b. mild steel

c. high carbon steel

d. wrought iron.

Explanation: Option B is correct.

223. Mastic asphalt is

a. water proof

b. fire proof

c. elastic

d. all the above.

Explanation: Option D is correct.

There are two options match 1 waterproof 2 elastic. So may be used for damp proofing. But not 100% sure that mastic asphalt can be used in dmp.

224. Non acid-resistant asbestos is :

a. tremolite asbestos

b. chrysolite abestos

c. amosite absestos

d. none of these.

Explanation: Option B is correct.

Chrysotile or white asbestos is the most commonly encountered form of asbestos, accounting for approximately 95% of the asbestos in the United States and a similar proportion in other countries.

225. Stones used for the construction of retaining walls must be

a. soft

b. hard

c. light

d. heavy.

Explanation: Option D is correct.

226. Permanent magnets are made of high carbon steel and

a. 15% of cobalt

b. 20% of cobalt

c. 35% of cobalt

d. 45% of cobalt.

Explanation: Option D is correct.

Permanent magnets are materials where the magnetic field is generated by the internal structure of the material itself. Inside atoms and crystals you have both electrons and the nucleus of the atom.

227. Plastics are compounds of carbon with element

a. hydrogen

b. nitrogen

c. oxygen

d. All of these.

Explanation: Option D is correct.

228. Pick up the correct statement from the following:

a. The plastic sheets are made by the calendering process in which the plastic material is allowed to pass between cylindrical rollers

b. The application of thermo-setting resins on sheets of paper, is called laminating process

c. The plastic articles made by placing raw material in the desired moulds, is known as moulding process

d. All the above.

Explanation: Option D is correct.

229. Pick up the constituent of good brick earth whose excess causes the raw bricks shrink and warp during drying and burning, from the following:

a. Alumina

b. Lime

c. Ironoxide

d. Magnesia.

Explanation: Option A is correct.

230. Cast iron is used for

a. structural works in beams

b. small sized water pipes

c. columns and struts

d. none to these.

Explanation: Option C is correct.

Wrought iron is used for small sized water pipes.
And cast iron is used for columns and struts because the column is designed for comp. Stresses and cast iron is good in compression.

231. Stainless steel resists corrosion due to

a. sulphur

b. vanadium

c. chromium

d. maganese.

Explanation: Option C is correct.

232. Commonly used thinner in

a. cellulose paints is ethyle acetate

b. oil paints, is naptha

c. distemper, is water

d. all the above.

Explanation: Option D is correct.

233. Good quality sand is never obtained from

a. river

b. nala

c. sea

d. gravel powder.

Explanation: Option C is correct.

234. Pick up the rock which is not  a sedimentary rock from the following:

a. sand stone

b. gypsum

c. dolerite

d. lignite.

Explanation: Option C is correct.

235. For a good building stone, its specific gravity should the greater than

a. 1.5

b. 1.7

c. 2.2

d. 2.7.

Explanation: Option D is correct.

236. Blister steel

a. is full of fissures and cavities

b. can not be forged

c. can be easily welded

d. all the above.

Explanation: Option D is correct.

237. The most important constituent of varnish, is

a. drier

b. solvent

c. resin

d. all the above.

Explanation: Option D is correct.

238. A ferrous metal is

a. cast iron

b. wrought iron

c. steel

d. all the above.

Explanation: Option D is correct.

239. For making fly-ash building bricks, the following mix of fly-ash, sand and lime, is

a. 80 : 13 : 7

b. 70 : 20 : 10

c. 60 : 35 : 5

d. none of these.

Explanation: Option A is correct.

240. Pick up the correct statement from the following:

a. Lime is available by dissolving calcium carbonate in water

b. Lime is available by calcining calcium carbonate at 900°C

c. Lime is nothing but calcium chloride

d. None of these.

Explanation: Option B is correct.

241. Water paint is a

a. colour wash

b. whiting

c. distemper

d. all the above.

Explanation: Option D is correct.

242. Seasoning of timber is done

a. to make it water proof

b. to paint its surface

c. to increase its temperature

d. to remove water.

Explanation: Option D is correct.

243. The specific gravity of marble, is

a. 2.60

b. 2.66

c. 2.72

d. 3.00.

Explanation: Option C is correct.

Granite ----> 2.60 - 2.70.	
Dolerite ----> 3.00 - 3.05.
Sandstone ----> 2.00 - 2.60.
Shale ----> 2.00 - 2.40.
Limestone	 ----> 2.20 - 2.60.	
Dolomite ---->  2.50 - 2.60.	
Quartzite ----> 2.65.

244. The rocks having alumina or clay as their major constituents, are known as

a. argillaceous rocks

b. calcareous rocks

c. sedimentary rocks

d. igneous rocks.

Explanation: Option A is correct.

245. The curved swellings from the growth of layers or wounds left after branches are cut off in an irregular manner, are known as

a. knots

b. rindgalls

c. burls

d. none of these.

Explanation: Option B is correct.

Rindgalls is correct.
Due to improperly cut off or removed of the tree curved swellings found on body of tree.

246. Pick up the correct statement from the following:

a. The percentage of silica in silica bricks is to the extent of about 95 to 97 percent

b. Roughly 1 to 2 percent of lime in silica bricks is added to act as binding material

c. The compressive strength of silica bricks is about 150 kg/cm2

d. All the above.

Explanation: Option D is correct.

247. Spirit varnish generally consists of

a. alcohol, wax and turpentine

b. pigment and synthetic resin

c. spirit and shellac

d. none of these.

Explanation: Option C is correct.

248. Pick up the correct statement from the following:

a. Impurities float on the molten iron as slag

b. The slag contains lime about 45%

c. The molten iron is made to run, in a long channel formed in sand called 'sow'

d. All the above.

Explanation: Option D is correct.

249. Acrylic is the name of

a. cellulose resin

b. alkyd resin

c. methyl metha crylate

d. cumarone-indene.

Explanation: Option C is correct.

250. Glazing of clay products, is done

a. to improve their appearance

b. to protect them from atmospheric effect

c. to protect them from corrosive action

d. all the above.

Explanation: Option D is correct.

251. Lime mortar is generally made with

a. fat lime

b. hydraulic lime

c. plain lime

d. none of these.

Explanation: Option B is correct.

252. Duco paints are

a. cellulose paints

b. emulsion paints

c. bituminous paints

d. oil paints.

Explanation: Option D is correct.

Duco paints are oil paints. It gives a hard and rich coating with outstanding gloss and shows the natural beauty of the wood.

253. Pick up the correct statement from the following:

a. Formaldehyde is prepared synthetically from methane

b. The melamine when reacted with formaldehyde forms the melamine-formaldehyde resin

c. The plastics made from melamine formaldehyde resin, are used for electrical insulators

d. All the above.

Explanation: Option D is correct.

254. Steel contains carbon approximately

a. 1.50% to 5.6%

b. 0.05% to 1.75%

c. 0.25%

d. none to these.

Explanation: Option B is correct.

Steel Types.
Mild Steel or Low carbon steel = 0.05-0.25%.
Medium Carbon steel = 0.29-0.54%,
High carbon steel = 0.55-0.95%,
Very High carbon Steel = 0.96-2.1%.

255. Cast iron contains carbon approximately

a. 1.5% to 5.5%

b. 0.05% to 1.75%

c. 0.250%

d. none to these.

Explanation: Option A is correct.

256. Plastic asphalt is

a. used as a water proofing layer over roof

b. a mixture of cement and asphalt

c. a natural asphalt

d. a refinery product.

Explanation: Option B is correct.

Plastic asphalt = a mixture of cement and asphalt.
Mastic asphalt = used for damp proof and waterproof layer.
Plastic bitumen = used for filling cracks in masonry structure using asbestos powder as a filler.

257. For obtaining vinyl chloride acetate, the method used, is

a. addition polymerization

b. condensation polymerization

c. co-polymerization

d. none of these.

Explanation: Option C is correct.

258. The yield strength and tensile strength of low carbon steel may be improved by the addition of

a. chromium

b. nickel

c. venadium

d. tungsten.

Explanation: Option C is correct.

259. Bitumen completely dissolves in

a. chloroform

b. benzol

c. coaltar

d. All of these.

Explanation: Option D is correct.

260. Good quality cement contains higher percentage of

a. Tricalcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite

Explanation: Option A is correct.

No, it's correct.
The typical compound compositions of this type are:
55% (C3S), 19% (C2S), 10% (C3A), 7% (C4AF), 2.8% MgO, 2.9% (SO3), 1.0% ignition loss, and 1.0% free CaO.

261. Soundness of cement is tested by

a. Vicat's apparatus

b. Le-chatelier apparatus

c. compressive strength testing apparatus

d. none of the these.

Explanation: Option B is correct.

Soundness mainly are caused by three material.
1. Lime (check by blames air permeability test).
2. Alumina (check by  Le-Chatelier test).
3. Sulphur (no apparatus are available to check).

262. The PVC doors and windows are preferred as they are

a. rot proof

b. termile proof

c. water proof

d. all of these.

Explanation: Option D is correct.

263. Based on its dry weight, a freshly felled tree may contain water

a. 25%

b. 50%

c. 75%

d. 100%.

Explanation: Option D is correct.

A newly felled tree contains water and sap (plant juice) both about 50% of its own dry weight>>>>.
So, the correct option should be (B).

264. Forge pig may be converted to wrought iron by

a. rolling

b. pudding

c. shingling

d. refining.

Explanation: Option B is correct.

265. To give a brilliant finish, the type of varnish used, is

a. spirit varnish

b. turpentine varnish

c. oil varnish

d. none of these.

Explanation: Option A is correct.

266. Pick up the correct statement from the following:

a. Bull's trench kiln a trench excavated in ground

b. Hoffman's kiln is constructed overground

c. Tunnel Kiln is constructed as a tunnel

d. All the above.

Explanation: Option D is correct.

267. The softest rock is

a. marble

b. diamond

c. talc

d. quartz.

Explanation: Option C is correct.

Moh's hardness number be like;
Talc 1
Gypsum 2.
Calcite 3.
....
Topaz 8.
Diamond 10.
So, the talc is the softest and the diamond is hardest.

268. Bitumen may be dissolved in

a. water

b. sodium chloride

c. carbon disulphide

d. none of these.

Explanation: Option C is correct.

269. Pick up the correct statement from the following:

a. During rusting, first ferrous bicarbonates are formed

b. On further oxidation ferrous bicarbonates get converted to ferric bicarbonates

c. Ultimately hydrated ferric oxide is formed during rusting and carbon dioxide gets liberated

d. All the above.

Explanation: Option D is correct.

270. Whitworth compressed steel is obtained when molten steel is subjected to a pressure of

a. 9 kg/mm2

b. 13 kg/mm2

c. 15 kg/mm2

d. 10 kg/mm2.

Explanation: Option A is correct.

271. Asbestos

a. is fire-proof and acid proof

b. has sp. gravity equal to 3.10

c. is smooth like glass and silk

d. All the above.

Explanation: Option D is correct.

272. Mild steel is used for

a. structural works in beams, joints and girders

b. small sized water pipes

c. columns and struts

d. none of these.

Explanation: Option A is correct.

Cast iron is used in compression member like column and struts because cast iron is good in compression due to more carbon content.
While for fluxure member like beams we use mild steel because mild steel is good in tension.

273. The sequence of refractory materials according to increasing melting points is :

a. Dolomite, Magnesia, Bauxite, Chromite

b. Bauxite, Chromite, Dolomite, Magnesia

c. Magnesia, Bauxite, Dolomite, Chromite.

Explanation: Option B is correct.

274. Veneering means

a. carving out designs on timber planks

b. chemically treating timber planks

c. thick   layer   of superior wood glued to inferior wood

d. thin   layer   of superior   wood glued to inferior wood.

Explanation: Option C is correct.

275. Laterite is a/an

a. volcanic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

Explanation: Option B is correct.

276. A stone is rejected if it absorbs water more than

a. 10%

b. 15%

c. 20%

d. 25%.

Explanation: Option A is correct.

A good quality stone absorbs water less than 5%.
A stone is rejected if it absorbs water more than 10%.

277. The most valuable timber may be obtained from

a. chir

b. shisham

c. sal

d. teak.

Explanation: Option D is correct.

278. Pick up the correct statement from the following:

a. The edges of a finished wrought iron, are rough due to red shortage

b. Pig iron (charcoal) is manufactured from magnetic ore (Fe3O4)

c. For the manufacture of wrought iron, non-sulphurous fuel is not necessary

d. All the above.

Explanation: Option D is correct.

279. Pick up the compound responsible for early strength of cement from the following:

a. Tetra-calcium alumino-ferrite

b. Tricalcium silicate

c. Tricalcium aluminate

d. Dicalcium silicate.

Explanation: Option B is correct.

Initial setting is responsible by aluminate as it first compounds to react but early strength is provided by tricalcium silicate and ultimate strength is by dicalcium silicate.

280. Mastic asphalt is

a. acid resisting material

b. non-corrosive material

c. corrosive material

d. heating-resisting material.

Explanation: Option B is correct.

Option D is correct.
Mastic asphalt is the combination of lime and bitumen which is used in Damp and waterproofing and in the bridge deck. It is also fireproof and elastic.

281. Which one of the following is used for preparing porcelain

a. feldspar

b. quartz

c. minerals

d. All of these.

Explanation: Option D is correct.

282. The commonly used base for iron and steel work, is

a. red lead

b. zinc white

c. white lead

d. titanium white.

Explanation: Option A is correct.

283. Laterite is found in

a. U.P.

b. Punjab

c. West Bengal

d. Kerala.

Explanation: Option A is correct.

Laterite soil generally found in Kerala, Karnataka, Tamilnadu, and MP. And also found in hilly areas Asam, Meghalaya.

284. Lacquer is

a. oil paint

b. distemper

c. spirit varnish

d. none to these.

Explanation: Option C is correct.

285. Upto a maximum of 72% of iron, is available in

a. Magnetite

b. Red haemetite

c. Limonite

d. Siderite

Explanation: Option A is correct.

286. For lime concrete,

a. flexural strength at 90 days is 0.2 N/mm2

b. compressive strength at 90 days is 1.5 N/mm2

c. compressive strength at 26 days is 1.2 N/mm2

d. all the above.

Explanation: Option D is correct.

287. Pig iron made from heamatite ores free from sulphur, phosphorus and copper, is known as

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig

Explanation: Option A is correct.

288. Pick up the correct statement from the following:

a. Blistering may be cured by applying water paint finished with oil paint dried with a little copal varnish

b. Cracked paints may be cured by removing paint and giving a fresh coat of paint

c. Crawling paints may be cured by sand preparing the surface and giving a fresh coat with plenty of turps

d. All the above.

Explanation: Option D is correct.

289. The material generally not used as extender in paints, is

a. powdered silica

b. gypsum

c. talc

d. zinc white.

Explanation: Option D is correct.

290. Pick up the non-inflammable plastic from the following:

a. Cellulose acetate plastics

b. Polyvinyl chloride plastics

c. Phenol formaldehyde plastic

d. Urea formaldehyde plastic.

Explanation: Option B is correct.

291. The most commonly used synthetic abrasive is

a. aluminium carbide

b. boric acid

c. silicon

d. All of these.

Explanation: Option D is correct.

292. For light and ornamental casting, the most unsuitable pig iron, is

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

Explanation: Option D is correct.

Mottled pig- irregular patern and form in many colours, Not in proper shape. That's why not suitable for ornamental casting.

293. Seasoning of timber is essential to remove

a. knots from timber

b. sap from timber

c. twisted fibre from timber

d. roughness of timber.

Explanation: Option B is correct.

294. The melting point of silica is :

a. 1570°C

b. 1630°C

c. 1730°C

d. 1850°C.

Explanation: Option C is correct.

295. A bull nose brick is not used for

a. rounding off sharp corners

b. pillars

c. decoration purpose

d. arches.

Explanation: Option D is correct.

D is correct because arches require standard brick whose compression is 100kg/cm2 and bullnose had minimum in the area as an area of brick decrease compression decreases.

296. Red short iron cracks when bent due to the presence of

a. sulphur

b. carbon

c. phosphorus

d. silicon.

Explanation: Option A is correct.

Red hot iron is otherwise know as hot metal (product from blast furnace) impurities in hot metal are:
Silica-imparts non-homoginity.
Carbon-brittleness.
Sulphur-surface cracks.

297. Linseed oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

Explanation: Option D is correct.

Carrier or vehicle is the same.
Linseed oil - carrier or vehicle in paint.
Red lead, litharge - Drier in paint.

298. Plaster of Paris is obtained by calcining

a. gypsum

b. kankar

c. lime stone

d. none of these.

Explanation: Option A is correct.

Gypsum plaster, or plaster of paris, is produced by heating gypsum to about 300 degrees F (150 Degrees C)

299. For high grade instruments the steel preferred to, is

a. cast steel

b. bessemer steel

c. mild steel

d. whitworth compressed steel.

Explanation: Option A is correct.

Steel castings are used for a variety of reasons. They are mainly used when other types of cast irons do not have enough strength or shock resistance. Steel's physical properties allow the castings to be both strong and ductile.

300. If the ore impurities is

a. clay, lime stone is used as flux

b. lime stone, clay is used as flux

c. quartz, lime stone and argallaceous iron ores are used as flux

d. All the above.

Explanation: Option D is correct.

301. The plastics prepared from Vinyl resin are

a. non-toxic

b. transparent

c. colourless

d. all of these.

Explanation: Option D is correct.

302. Chlorite, a green colour mineral is mainly derived from the decomposition of

a. augite

b. biotite

c. horn blende

d. All of these.

Explanation: Option A is correct.

303. During smelting process, the combination of fuel in the furnace

a. forms carbon dioxide

b. carbon dioxide with carbon forms carbon mono-oxide

c. carbon mono-oxide reacts with Fe2O3 to form iron and liberates CO2

d. all the above.

Explanation: Option D is correct.

304. Lime stones are generally known as

a. aqueous rocks

b. sedimentary rocks

c. stratified rocks

d. all the above.

Explanation: Option D is correct.

Limestone is a sedimentary rock composed primarily of calcium carbonate (CaCO3) in the form of the mineral calcite. It most commonly forms in clear, warm, shallow marine waters. Hence it is aqueous, sedimentary and stratified rock.

305. The method of addition polymerization is used for obtaining:

a. polypropylene

b. polyvinylchloride

c. polystyrene

d. All of these.

Explanation: Option D is correct.

An addition polymer is a polymer that forms by simple linking of monomers without the co-generation of other products.

306. Pick up the correct statement from the following:

a. The silica in excess makes the cement stronger but its setting time also increases

b. The excess amount of alumina weakens the cement

c. The addition of gypsum increases the initial setting time of cement

d. All the above.

Explanation: Option D is correct.

307. A rock contains only one mineral. It is called

a. homogeneous

b. non-homogeneous

c. monomineralic

d. polymineralic.

Explanation: Option C is correct.

Rocks are comprised of minerals, but minerals are not comprised of rocks. Minerals are inorganic compounds with unique chemical structures and physical properties that occur naturally. Rocks are solid masses that don't naturally occur. For example, the minerals quartz and feldspar, when formed together, make granite, which is a rock.

308. Asbestos

a. is a natural fibrous mineral substance

b. is composed of hydrous silicates of calcium and magnesium (CaSiO3, 3MgSiO3)

c. contains iron oxide and alumina

d. all the above.

Explanation: Option D is correct.

309. Pegmatite is a/an

a. intrusive igneous rock

b. extrusive igneous rock

c. sedimentary rock

d. metamorphic rock.

Explanation: Option A is correct.

A pegmatite is an igneous rock, formed underground, with interlocking crystals usually larger than 2.5 cm in size (1 in). Most pegmatites are found in sheets of rock (dikes and veins) near large masses of igneous rocks called batholiths.

310. The percentage of alumina and silica in good fire clay vary respectively is

a. 25, 75

b. 30, 70

c. 35, 65

d. All of these.

Explanation: Option A is correct.

Fire clay is used to make fire bricks and is generally found under the coal seams. Fire clay contains two major constituents- silica and alumina, of which, the silica percentage varies from 60 to 70% and alumina varies from 25 to 35%.

311. The term frog means

a. an apparatus to lift the stone

b. a depression on a face of brick

c. vertical joint in a brick work

d. soaking brick in water.

Explanation: Option B is correct.

312. Asphalt is obtained from

a. petroleum distillation

b. bitumen distillation

c. plastic distillation

d. none of these.

Explanation: Option A is correct.

313. To retard the initial setting time of cement, the compound responsible, is

a. Tricalcium silicate

b. Gypsum

c. Di-calcium silicate

d. Tri calcium aluminate.

Explanation: Option B is correct.

The tricalcium aluminate and Tetracalcium aluminoferrite compounds are responsible for the initial setting of cement. So, option D  is correct.

314. Dorry's testing machine is used for

a. crushing test of stone

b. hardness test of stone

c. impact test of stone

d. water absorption test.

Explanation: Option B is correct.

315. Stones used for rubble masonry must be

a. soft

b. hard

c. light

d. heavy.

Explanation: Option B is correct.

316. Index number expressing the relative sizes of both coarse and fine aggregates, is called

a. proportioning of aggregates

b. fineness modulus

c. grading of aggregates

d. none of these.

Explanation: Option B is correct.

317. Dolomite is a lime stone which contains carbonate of magnesia upto

a. 20%

b. 25%

c. 35%

d. 45%.

Explanation: Option D is correct.

318. Priming consists of

a. one part of white lead, 8 parts of chalk and four parts of twice boiled linseed oil

b. 8 parts of white lead, one part of chalk and four parts of twice boiled linseed oil

c. one part of white lead, 8 parts of chalk and one part of linseed oil

d. none to these.

Explanation: Option A is correct.

319. The portion of the brick cut across its width and having its length equal to that of a full brick, is known as

a. queen closer

b. king closer

c. prince closer

d. none of these.

Explanation: Option A is correct.

In King Closer brick is cut across WIDTH.
In Queen Closer brick is cut across LENGTH.
So, Option C is correct.

320. Seasoning of timber is done for

a. increasing moisture content

b. decreasing moisture content

c. increasing strength of timber

d. none to these.

Explanation: Option B is correct.

321. Bitumen felt is used for

a. water proofing

b. damp proofing

c. both (a) and (b)

d. neither (a) nor (b).

Explanation: Option C is correct.

322. According to ISI, bitumen is classified into

a. 4 grades

b. 6 grades

c. 8 grades.

d. 10 grades.

Explanation: Option D is correct.

As per IS 73:1992. There are six penetration grade (S35, S45, S55, S65, S90&S200). And as per Is73:2013 there are 4 viscosity grade (VG10, VG20, VG30 & VG40).

323. Lime putty

a. is made by adding lime to water

b. can be used only upto three days

c. may be obtained from drying lime water mix passing through IS sieve No. 300

d. all of above.

Explanation: Option D is correct.

324. The minimum percentage of silica, alumina and ferric oxide in lime for white washing, is

a. 15

b. 10

c. 5

d. 0.

Explanation: Option D is correct.

If this added then colour property is change of white cement, for white purpose these should be zero.

325. For a 50 kg cement bag water required, is

a. 18.5 litres

b. 20.5 litres

c. 22.5 litres

d. 25 litres.

Explanation: Option C is correct.

Min water required for complete hydration of cement is 42% by wt of cement
So, water req is 42*50 = 21.

326. The presence of sand in brick earth prevents:

a. cracking of bricks

b. shrinkage of bricks

c. warping of bricks

d. none of these.

Explanation: Option C is correct.

Option (C) is correct.
The amount of silica (sand) present in brick earth is 50-60% and it prevents cracking and warping of raw bricks.

327. The property by virtue of which lime sets under water, is known as

a. slacking

b. setting

c. hydraulicity

d. calcining.

Explanation: Option C is correct.

Slacking is process of chemical combination of quick lime with water.
Lime sets under water because of Hydraulicity.

328. Percentage content of silica in window glass, is

a. 40 to 45

b. 50 to 55

c. 60 to 65

d. 70 to 75.

Explanation: Option D is correct.

329. The coefficient of hardness of stones used in road work should be greater than

a. 10

b. 12

c. 15

d. 17.

Explanation: Option D is correct.

Coefficient of hardness(Ch) = 20 - (loss of wt./3)
Ch<14--> poor.
14<Ch<17--> medium.
Check>17--> hard.

330. Pick up the correct statement from the following:

a. Air bubbles in casting produce a dull sound by tapping their surfaces lightly with a hammer

b. Cupola furnace is used for the manufacture of cast iron

c. Red short iron is of no value for welding purpose

d. All the above.

Explanation: Option D is correct.

331. The steel used for the manufacture of rails, is

a. Bessemer steel

b. mild steel

c. cast steel

d. stainless steel.

Explanation: Option A is correct.

332. Slump test for concrete is carried out, to determine

a. strength

b. durability

c. workability

d. water content.

Explanation: Option C is correct.

Slump test for concrete is carried out, to determine water content (consistency) not for workability, hence the correct option is D.

333. Basalt is

a. sedimentary rock

b. metamorphic rock

c. extrusive igneous rock

d. intrusive igneous rock.

Explanation: Option C is correct.

Extrusive igneous rocks form when magma reaches the Earth's surface a volcano and cools quickly.
Most extrusive (volcanic) rocks have small crystals. Examples include basalt, rhyolite, andesite, and obsidian.

334. The frog of a brick is normally made on its

a. top face

b. bottom face

c. longer face

d. shorter side.

Explanation: Option A is correct.

Frog is made on top of Brick.
The mortar inside the frog of bricks forms a keyed joint between the brick and the mortar, thus negating any relative movement between two layers of brick and providing stability to the brickwork.

335. The most important constituent of an oil paint, is

a. vehicle

b. pigment

c. base

d. all the above.

Explanation: Option D is correct.

336. Bullet proof glass is made of thick glass sheet sandwiched by a layer of

a. steel

b. stainless steel

c. high test plastic

d. chromium plate.

Explanation: Option C is correct.

337. Pig iron obtained from the furnance which is properly provided with fuel at a very high temperature, is called

a. Grey or foundry pig

b. White or forge pig

c. Mottled pig

d. None of the above.

Explanation: Option A is correct.

338. Refractory bricks are

a. neutral refractory bricks

b. acid refractory bricks

c. basic refractory bricks

d. all the above.

Explanation: Option D is correct.

339. Porcelain is used as :

a. electric insulators

b. storage vessels

c. reactor chambers

d. All of the above.

Explanation: Option D is correct.

340. Durability of building stone is affected by its

a. texture

b. resistance to atmosphere

c. location in structure

d. all the above.

Explanation: Option D is correct.

341. Magnese steels

a. possess high electrical resistance

b. possess low coefficient of expansion

c. are used for the manufacture of rails

d. all the above.

Explanation: Option D is correct.

342. Pick up the metal refractory from the following:

a. Molyblendum

b. Tungsten

c. Zicronium

d. All of these.

Explanation: Option D is correct.

343. According to IS 399-1963, the weight of the timber is specified at

a. 8% moisture content

b. 10% moisture content

c. 12% moisture content

d. 14% moisture content.

Explanation: Option C is correct.

344. The size of mould for bricks, is generally kept

a. a little large to specified size

b. a little small to specified size

c. equal to specified size

d. 10% larger than specified size

Explanation: Option A is correct.

Yes. It should be 10%.
Because the size of mould is kept to be 8-12% greater than the actual size of brick.

345. Most commonly used solvent in oil paints, is

a. petroleum

b. spirit

c. coaltar

d. turpentine.

Explanation: Option A is correct.

Of course, turpentine! Because of its high solving power, excellent flattening properties & the ideal rate of evaporation.

346. Acrylic sheets

a. are generally unaffected by most household detergents

b. possess the light transmission rate of 93%

c. are available in various shapes

d. all the above.

Explanation: Option D is correct.

347. With storage, strength of cement

a. increases

b. decreases

c. remains the same

d. none to these.

Explanation: Option B is correct.

Period of Storage -  Relative Strength (percent) at 28 days
Fresh - 100%
3 months - 80%
6 months - 70%
1 year - 60%
5 year - 50%

348. The commonly used raw material in the manufacture of cement, is

a. slate

b. sand stone

c. lime stone

d. basalt.

Explanation: Option C is correct.

349. Pick up the correct statement from the following:

a. The lime stone resists fire upto about 800°C and at higher temperature it splits into CaO and CO2

b. The sand stone with silicates resist a fire in a better way

c. The argillaceous stone though poor in strength can resist fire quite weak

d. All the above.

Explanation: Option D is correct.

According to textbook of Engineering geology by Kesavulu (para 17.3.6. Frost and fire resistance), all the statements are correct.

350. The most commonly used base for timber painting, is

a. red lead

b. zinc white

c. white lead

d. titanium white.

Explanation: Option C is correct.

351. Pick up the most favourable condition for the rapid growth of fungus for dry rot from the following:

a. dampness

b. presence of sap

c. stagnant air

d. All the above,

Explanation: Option D is correct.

352. Bitumen paints offer

a. pleasing surface

b. hard surface

c. smooth surface

d. protective surface

e. rough surface.

Explanation: Option D is correct.

353. Pick up the correct statement from the following:

a. Styrene resin is light in weight

b. Styrene resin transmits ultraviolet waves of light

c. Styrene resin is used to manufacture utensils which are unaffected by chemicals

d. All the above.

Explanation: Option D is correct.

354. The lime which contains high percentage of calcium oxide, is generally called

a. fat lime

b. rich lime

c. white lime

d. All of these.

Explanation: Option D is correct.

Fat lime, white lime and rich lime are same thing. They are used for white washing. So all of these option is right.

355. For construction of structures under water, the type of lime used, is

a. hydraulic lime

b. fat lime

c. quick lime

d. pure lime

Explanation: Option A is correct.

356. Pick up the correct statement from the following:

a. Cast iron oxidises less

b. Steel oxidises most

c. Wrought iron oxidises moderately

d. All the above.

Explanation: Option D is correct.

357. A prime coat is given to steel work with

a. an oxide of iron paint

b. a mixture of white lead and lead paint

c. a special paint

d. cement paint.

Explanation: Option B is correct.

358. Ground glass

a. is made by grinding its one side

b. is made by melting powdered glass paints surface

c. is used for getting light without transparency

d. all the above.

Explanation: Option D is correct.

359. The harmonious mixing of the clay ingredients, is known as

a. weathering

b. blending

c. tempering

d. None of these.

Explanation: Option B is correct.

Tampering is a process of manufacturing brick in which 1/4 to 1/3 water by weight of clay is added to mixture, for making paste for moulding.

360. Brass is an alloy of

a. copper and zinc

b. zinc and lead

c. tin and silver

d. zinc and nickel

Explanation: Option A is correct.

361. The wedging is adopted for quarrying costly stratified rock such as

a. marble

b. limestone

c. sandstone

d. All the above.

Explanation: Option D is correct.

Marble is a stratified rock but it has irregular patches that's why it is consider as a unstratified rock.

362. Rapid hardening cement attains early strength due to

a. larger proportion of lime grounded finer than normal cement

b. lesser proportion of lime grounded coarser than normal cement

c. lesser proportion of lime grounded finer than normal cement

d. larger proportien of lime grounded coarser than normal cement

Explanation: Option A is correct.

363. The colour of statuary marble used for sculptor's work, is

a. blue

b. white

c. green

d. yellow

Explanation: Option B is correct.

364. Second class bricks

a. are of dark brown colour

b. produce a metallic sound when struck

c. are well burnt

d. are under burnt.

Explanation: Option B is correct.

A class bricks produce metallic sound.
B class bricks are over burnt.
c class bricks are under burnt.

365. Pick up the correct statement from the following:

a. The low voltage porcelain is prepared by wet process

b. The high voltage porcelain is prepared by dry process

c. The low voltage porcelain is prepared by dry process

d. None of the above.

Explanation: Option C is correct.

366. Pick up the correct statement from the following:

a. The bulking of fine sand is more than that of coarse sand

b. If the percentage content of moisture exceeds 10%, increase in bulk of sand starts increasing

c. The volume of fully saturated sand equals that of dry sand

d. All the above.

Explanation: Option D is correct.

C option is incorrect because Bulking of sand more in 5% (4-6%) after then increases moisture decreased the volume.

367. Teak wood is suitable for

a. sports articles

b. furnitures

c. railway sleepers

d. all the above.

Explanation: Option B is correct.

Teak wood suitable for furniture.
Mulberry wood suitable for Sports Goods.
Deodar is suitable for railways sleepers.

368. Plywood is normally available

a. 1 mm thick

b. 2 mm thick

c. 2 to 3 mm thick

d. 3 mm to 4 mm thick.

Explanation: Option D is correct.

369. French polish is

a. oil paint

b. distemper

c. spirit varnish

d. none to these.

Explanation: Option C is correct.

370. In stone masonry, if stones are so placed that their layers are parallel to the direction of load, they

a. split easily

b. are affected by moisture

c. both (a) and (b)

d. none of these.

Explanation: Option C is correct.