***Engineer***

1. Keeping the depth d constant, the width of a cantilever of length l of uniform strength loaded with a uniformly distributed load w varies from zero at the free end and

a.  at the fixed end

b.  at the fixed end

c.  at the fixed end

d.  at the fixed end

2. A material may fail if

a. maximum strain exceeds

b. maximum shear stress exceeds

c. total strain energy exceeds

d. all the above.

3. At yield point of a test piece, the material

a. obeys Hooke's law

b. behaves in an elastic manner

c. regains its original shape on removal of the load

d. undergoes plastic deformation.

4. The width b and depth d of a beam cut from a wooden cylindrical log of 100 cm diameter for maximum strength are :

a. b = 57.73 cm d = 81.65 cm

b. b = 81.65 cm d = 57.73 cm

c. b = 50.00 cm d = 50.00 cm

d. b = 40.00 cm d = 80.00 cm

5. The shear force on a simply supported beam is proportional to

a. sum of the forces

b. sum of the transverse forces

c. algebraic sum of the transverse forces of the section

d. curvature of the neutral axis.

6. Planimeter is used for measuring

a. area

b. contour gradient

c. slope angle

d. none of these

7. The limiting length of an offset does not depend upon

a. accuracy of the work

b. method of setting out perpendiculars

c. scale of plotting

d. indefinite features to be surveyed.

8. The distances AC and BC are measured from two fixed points A and B whose distance AB is known. The point C is plotted by intersection. This method is generally adopted in

a. chain surveying

b. traverse method of surveys

c. triangulation

d. none of these.

9. Keeping the instrument height as 1.5 m, length of staff 4 m, the slope of the ground as 1 in 10, the sight on the down-slope, must be less than

a. 30 m

b. 25 m

c. 20 m

d. 15 m

10. Pick up the correct statement from the following:

a. the theodolite in which telescope can be rotated in vertical plane is called a transit

b. when the vertical circle is to the left of the telescope during observation, it is called to be in left face

c. when the vertical circle is to the right of the telescope during observation, it is called to be in right face

d. all the above.

11. Accurate measurement of deflection angles with a transit not properly adjusted may be made by

a. setting the vernier A at zero at back station and then plunging the telescope

b. setting the vernier A at zero at back station and turning the instrument to the forward station

c. taking two back sights one with the telescope normal and the other with telescope inverted

d. none of these.

12. In quadrantal bearing system, back bearing of a line may be obtained from its forward bearing, by

a. adding 180°, if the given bearing is less than 180°

b. subtracting 180°, if the given bearing, is more than 180°

c. changing the cardinal points, i.e. substituting N for S and E for W and vice-versa

d. none of these.

13. For light and ornamental casting, the most unsuitable pig iron, is

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

14. Brittleness of cold is due to an excess of

a. sulphur

b. carbon

c. phosphorus

d. silicon.

15. The presence of sand in brick earth prevents:

a. cracking of bricks

b. shrinkage of bricks

c. warping of bricks

d. none of these.

16. Portland pozzolana cement possesses

a. lower heat of hydration

b. lower shrinkage on drying

c. water tightness

d. all the above.

17. Pick up the correct statement from the following:

a. The lime stone resists fire upto about 800°C and at higher temperature it splits into CaO and CO2

b. The sand stone with silicates resist a fire in a better way

c. The argillaceous stone though poor in strength can resist fire quite weak

d. All the above.

18. Ultimate strength to cement is provided by

a. Tricalcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

19. Pick up the correct statement from the following:

a. Continuous grading is not necessary for obtaining a minimum of air voids

b. The omission of a certain size of aggregate is shown by a straight horizontal line on the grading curve

c. The omission of a certain size of aggregate in concrete increases the workability but also increases the liability to segregation

d. All the above.

20. If a grading curve is horizontal bet-wen the portions of 20 mm I.S. Sieve and 4.75 mm I.S. Sieve, the graded aggregates do not contain

a. 20 mm particles

b. 10 mm particles

c. 4.75 mm particles

d. all the above.

21. Percentage of pozzolanic material containing clay upto 80% used for the manufacture of pozzolana cement, is

a. 30%

b. 40%

c. 50%

d. 60%

22. The percentage of the aggregate of F.M. 2.6 to be combined with coarse aggregate of F.M. 6.8 for obtaining the aggregates of F.M. 5.4, is

a. 30%

b. 40%

c. 50%

d. 60%.

23. Pick up the correct statement from the following:

a. The quantity of water required for concreting, depends upon the grading of aggregate amd method of compaction

b. 10% excess of water reduces the strength of concrete by 15%

c. 30% excess of water reduces the strength of concrete by 50%

d. All the above.

24. Pick up the correct statement from the following:

a. When water table is above the base of a footing, the dry weight m should be used for soil below water table

b. When water table is located somewhat below the base of a footing, the elastic wedge is partly of moist soil and partly of submerged soil, and a suitable reduction factor is used

c. When water table is just at the base of the footing, no reduction factor is used

d. None of these.

25. Pick up the correct statement from the following:

a. At critical hydraulic gradient, the saturated sand becomes quick

b. The critical gradient depends on the void ratio and the specific gravity

c. The quick sand occurs more in fine sand and silt than coarse material

d. All the above.

26. The expression  ρω is used for

a. dry density

b. bulk density

c. degree of saturation

d. optimum water content.

27. Over-consolidation of soils is caused due to

a. erosion of over burden

b. melting of ice sheets after glaciation

c. permanent rise of water table

d. all the above.

28. Failure of a slope occurs only when total shear force is

a. equal to total shearing strength

b. greater than total shearing strength

c. less than total shearing strength

d. none of these.

29. A stratum of clay 2 m thick will get consolidated 80% in 10 years. For the 80% consolidation of 8 m thick stratum of the same clay, the time required is

a. 120 years

b. 140 years

c. 160 years

d. 180 years.

30. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

31. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

32. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

33. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

34. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

35. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

36. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

37. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

38. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

39. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

40. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

41. As built drawing is normally constructed................the construction.

a. before construction

b. simultaneously with construction

c. after construction

d. all of the above

42. The best drawing paper used for water colour is :

a. hand made paper (hand made drawing sheet)

b. tracing drawing sheet

c. ordinary drawing sheet

d. ammonia paper

43. Size of A4 size paper is

a. 148 x 210 mm

b. 210 x 297 mm

c. 297 x 210 mm

d. 320 X 460 mm

44. To draw or measure angles .......... is used.

a. set-square

b. T-square

c. protractor

d. all of the above

45. Pick up the correct statement from the following:

a. The ratios which show profitability in relation to sales and those which show profitability in relation to investment, are called profitability ratios.

b. The ratio of gross profit and net sales, is called profitability in relation to sales ratio.

c. The ratio of net profit after taxes to total assests is known as profitability in relation to investment ratio

d. All of these

46. Ratio analysis of a construction firm is used for analysis by :

a. firm's management

b. Banks of the firm

c. financial analysts

d. All of these.

47. Earning per share is the most important ratio for

a. share holders

b. banks

c. company's management

d. All of these

48. What is the abbreviation used for German Technical Co-operation?

a. GTZ

b. GTC

c. GTCO

d. all of the above

49. For the registiration of professional engineer amount fee required is:

a. NRS 5000/-

b. NRS 10000/-

c. NRS 15000/-

d. NRS 20000/-

50. For the formation of users committee, members are selected

a. among the people of whole district

b. among the beneficiaries of the project

c. by nomination by the chief District officer of the district

d. none of the above