***Engineer***

1. A truss containing j joints and m members, will be a simple truss if

a. m = 2j - 3

b. j = 2m - 3

c. m = 3j - 2

d. j = 3m - 2

2. A three hinged arch is generally hinged at its supports and

a. at one quarter span

b. at the crown

c. any where in the rib

d. none of these.

3. In case of principal axes of a section

a. sum of moment of inertia is zero

b. difference of moment inertia is zero

c. product of moment of inertia is zero

d. none of these.

4. For keeping the stress wholly compressive the load may be applied on a circular column anywhere within a concentric circle of diameter

a. d/3

b. d/4

c. d/8

d. d/10

5. The deflection of any rectangular beam simply supported, is

a. inversely proportional to its width

b. inversely proportional to the cube of its depth

c. directly proportional to the cube of its length

d. none of these.

6. The most reliable method of plotting a theodolite traverse, is

a. by consecutive co-ordinates of each station

b. by independent co-ordinates of each station

c. by plotting included angles and scaling off each traverse leg

d. by the tangent method of plotting.

7. Diopter is the power of a lens having a focal length of

a. 50 cm

b. 75 cm

c. 100 cm

d. 125 cm

8. If a tacheometer is fitted with an anal-latic lens

a. additive constant is 100, multiplying constant is zero

b. multiplying constant is 100, additive constant is zero

c. both multiplying and additive constants are 100

d. both multiplying and additive constants are 50.

9. The formula for the horizontal distances for inclined sights, on staff held normalis  s cos θ + (f + d) cos θ ± h sin θ

a. minus sign is used for angle of depression

b. plus sign is used for angle of depression

c. minus sign is used for angle of elevation

d. non of these.

10. The 'point of curve' of a simple circular curve, is

a. point of tangency

b. point of commencement

c. point of intersection

d. mid-point of the curve

11. A sewer is laid from a manhole A to a manhole B, 250 m away along a gradient of 1 in 125. If the reduced level of the invert at A is 205.75 m and the height of the boning rod is 3 m, the reduced level of the sight rail at B, is

a. 208.75 m

b. 202.75 m

c. 206.75 m

d. 211.75 m

12. Ramsden eye-piece consists of

a. two convex lenses short distance apart

b. two concave lenses short distance apart

c. one convex lens and one concave lens short distance apart

d. two plano-convex lenses short distance apart, with the convex surfaces facing each other.

13. The percentage of water for normal consistency, is

a. 10% to 25%

b. 15% to 25%

c. 20% to 30%

d. 25% to 35%.

14. The variety of pig iron used for manufacture of wrought iron, is

a. Bessemer pig

b. Grey or foundry pig

c. White forge pig

d. Mottled pig.

15. Wrought iron is manufactured from pig iron by

a. puddling

b. shingling

c. rolling

d. all the above.

16. Sand stone is

a. sedimentary rock

b. metamorphic rock

c. igneous rock

d. volcanic rock.

17. Bitumen emulsion is

a. a liquid containing bitumen in suspension

b. a paint

c. used as anti-corrosive paint

d. all the above.

18. Cement paints usually

a. contain 5% to 10% colour pigments

b. are prepared with white cement

c. contain 5% sodium chloride

d. all the above.

19. A concrete having a slump of 6.5 cm, is said to be

a. earth moist

b. semi-plastic

c. plastic

d. none of these.

20. Permissible compressive strength of M 150 concrete grade is

a. 150 kg/cm2

b. 200 kg/cm2

c. 250 kg/cm2

d. 300 kg/cm2

21. For preparing a test-specimen, it is necessary

a. to mix coarse aggregates

b. to mix water to the cement, fine aggregates and coarse aggregates

c. to oil inner surface of the mould and the base plate, before placing concrete

d. all the above.

22. Construction joints are provided

a. where the member is supported by other member

b. at 18 m apart in huge structures

c. in concrete wall at sill level of windows

d. all the above.

23. Pick up the incorrect statement from the following:

a. Concrete for which preliminary tests are conducted, is called controlled concrete

b. Bulking of sand depends upon the fineness of grains

c. Concrete mix 1 : 6 : 12, is used for mass concrete in piers.

d. All the above.

24. The ratio of the weight of water to the weight of solids in a given mass of soil, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

25. Pick up the correct statement from the following:

a. The percent settlement at any time is called degree of consolidation

b. Time factor is a dimensionless quantity

c. The initial curve on either side of the point of unloading and reloading is called 'virgin curve'

d. All the above.

26. Sedimentation analysis is based on the assumption:

a. soil particles are spherical

b. particles settle independent of other particles

c. walls of the jar do not affect the settlement .

d. all the above.

27. A fundamental equation of void ratio(e), specific gravity (G), water content (ω) and degree of saturation (Sr) is

a. e =

b. ω =

c. G =

d. Sr =

28. Fundamental relationship between dry density (γd), bulk density (γ) and water content (ω), is :

a. γ =

b. γd =

c. ω =

d. ω =

29. Pick up the correct statement from the following:

a. A soil classification system is meant to provide an accepted and systematic method of describing the various types of soils eliminating personal factors

b. The first category of soil classification is based on grain size of the soil

c. The second category of soil classification is based on fine as well as coarse grains

d. All the above.

30. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

31. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

32. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

33. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

34. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

35. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

36. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

37. The expected out turn of 12 mm plastering with cement mortar is

a. 4.0 sq m

b. 6.0 sq m

c. 8.0 sq m

d. 10 sq m

38. The item of the brick structure measured in sq.m, is

a. Reinforced brick work

b. Broken glass coping

c. Brick edging

d. Brick work in arches.

39. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

40. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

41. Which of the following is not a drawing hand tool?

a. alidade

b. straight edge

c. french curve

d. set squares

42. To draw very light line which pencil is used

a. B

b. HB

c. H

d. 2H

43. View of circle in orthographic is

a. ellipse

b. circle

c. ellipse and circle

d. triangle

44. The purpose of sectional view is to show the

a. surface of the object

b. internal of the object

c. shape of the object

d. none of the above

45. Both architect and engineer make use of the cost estimate of the project:

a. for site selection

b. for designing of the project

c. for choosing alternatives

d. All of these

46. Renu Bala deposits Rs 1200 now, Rs 800 two years from now and Rs 1000 five years from now. If the savings bank's rate of interest in 5%, she will receive an amount of Rs X 10 years from now, where X is

a. Rs 3415

b. Rs 4225

c. Rs 4413

d. Rs 4826

47. In a cash-flow diagram :

a. Time 1 is considered to be the end of time period 1

b. A vertical arrow pointing up indicates a positive cash flow

c. An arrow pointing downward indicates a negative cash flow

d. All of these

48. During construction price variation clause in a contract applies to

a. increase in cost of important materials only

b. variation in the cost of material labour and fuel

c. variation in total cost of the project estimated as lump sum figure

d. the rate of inflation

49. For procurement of works up to 2 million, buy which of the following method is to be considered?

a. by sealed tender

b. by direct negotiation

c. by bidding

d. by direct labour or direct negotiation

50. As per the NEC Act, 2055 the name of engineers

a. general, professional, and foreigner

b. special, local and international wngineer

c. civil, electrical and mechanical engineer.

d. computer, electronics and agriculture engineer