***Engineer***

1. Slenderness ratio of a long column, is

a. area of cross-section divided by radius of gyration

b. area of cross-section divided by least radius of gyration

c. radius of gyration divided by area of cross-section

d. length of column divided by least radius of gyration.

2. Maximum strain theory for the failure of a material at the elastic limit, is known as

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

3. If a solid shaft (diameter 20 cm, length 400 cm, N = 0.8 x 105 N/mm2) when subjected to a twisting moment, produces maximum shear stress of 50 N/mm2, the angle of twist in radians, is

a. 0.002

b. 0.0025

c. 0.003

d. 0.005

4. The equivalent length of a column fixed at one end and free at the other end, is

a. 0.7 l

b. l

c. 2 l

d. 1.5 l.

5. Stress in members of statically determinate simple frames, can be determined by

a. method of joints

b. method of sections

c. graphical solution

d. all the above.

6. An imaginary line joining the points of equal elevation on the surface of the earth, represents

a. contour gradient

b. contour line

c. level line

d. none of these.

7. An ideal vertical curve to join two gradients, is

a. parabolic

b. elliptical

c. hyperbolic

d. none of these.

8. While measuring a chain line between two stations A and B intervened by a raised ground

a. vision gets obstructed

b. chaining gets obstructed

c. both vision and chaining get obstructed

d. all the above.

9. Number of links per metre length of a chain are

a. 5

b. 8

c. 10

d. 20

10. Pick up the correct statement from the following :

a. the system of a series of lines which forms a circuit which ends at the starting point, is called a closed traverse

b. the traverse that starts from a point already fixed in some survey system and ends on another such point, is called a controlled traverse

c. the traverse that is not controlled is called a fly traverse

d. all the above.

11. In levelling operation,

a. The first sight on any change point is a back sight

b. The second sight on any change point is a fore sight

c. The line commences with a fore sight and closes with a back sight

d. The line commences with a back sight and closes with a foresight.

12. For a curve of radius 100 m and normal chord 10 m, the Rankine's deflection angle, is

a. 0°35'.95

b. 1°25'.53

c. 1°35'.95

d. 2°51'.53.

13. A 1st class brick immersed in water for 24 hours, should not absorb water (by weight) more than

a. 15%

b. 20%

c. 25%

d. 5%.

14. Quick lime (or caustic lime)

a. is obtained by the calcination of pure lime stone

b. has great affinity to moisture

c. is amorphous

d. All the above.

15. Brittleness of cold is due to an excess of

a. sulphur

b. carbon

c. phosphorus

d. silicon.

16. Pick up the correct statement from the following:

a. The edges of a finished wrought iron, are rough due to red shortage

b. Pig iron (charcoal) is manufactured from magnetic ore (Fe3O4)

c. For the manufacture of wrought iron, non-sulphurous fuel is not necessary

d. All the above.

17. Pick up the correct statement from the following:

a. Lime is available by dissolving calcium carbonate in water

b. Lime is available by calcining calcium carbonate at 900°C

c. Lime is nothing but calcium chloride

d. None of these.

18. Wrought iron contains carbon upto

a. 0.25%

b. 1.0%

c. 1.5%

d. 2%.

19. On a grading curve, the gap grading is represented by

a. a horizontal line

b. a vertical line

c. N.W. inclined line

d. N.E. inclined line

20. Water cement ratio is generally expressed in volume of water required per

a. 20 kg

b. 30 kg

c. 40 kg

d. 50 kg

21. Pick up the correct statement from the following:

a. Water lubricates the mixture of gravel, sand and cement

b. Only a small quantity of water is required for hydration of cement

c. Strength of concrete structure largely depends upon its workability

d. All the above.

22. The entrained air in concrete

a. increases workability

b. decreases workability

c. decreases resistance to weathering

d. increases strength

23. The maximum thickness of concrete floor of a cement warehouse, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

24. Hydrometer readings are corrected for:

a. meniscus correction

b. dispersing agent correction

c. meniscus and dispersing agent corrections

d. temperature, meniscus and dispersing agent corrections.

25. The expression  ρω is used for

a. dry density

b. bulk density

c. degree of saturation

d. optimum water content.

26. Pile foundations are generally preferred to for

a. bridge foundations

b. sky scrapper buildings

c. residential buildings

d. runways.

27. Negative skin friction on piles

a. is caused due to relative settlement of the soil

b. is caused in soft clays

c. decreases the pile capacity

d. all of the above.

28. The equation τ = C + σ tan φ is given by

a. Rankine

b. Coulomb

c. Culaman

d. Mohr.

29. Pick up the incorrect definition from the following:

a. The rotation of soil particles into stable state while remoulding, is known as the thiostropy of soil

b. The water content at which a soil changes from the liquid state to solid state, is known liquid limit of the soil

c. The water content at which a soil flows, is known plastic limit of the soil

d. None of these.

30. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

31. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

32. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

33. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

34. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

35. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

36. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

37. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

38. The expected out turn of 12 mm plastering with cement mortar is

a. 4.0 sq m

b. 6.0 sq m

c. 8.0 sq m

d. 10 sq m

39. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

40. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

41. The edge of the board on which T-square is made to slide is called

a. working edge

b. drawing line

c. border line

d. deader line

42. In orthographic projection, the line of sight and plane of projection are

a. perpendicular

b. parallel

c. horizontal

d. vertical

43. Presentation drawing are generally rendered due to

a. decoration

b. give real effects

c. attract the client

d. satisfaction

44. For drawing of small objects, the scale used is

a. enlarged

b. full size

c. reducing

d. all of the above

45. In a cash flow series :

a. uniform gradient signifies that an income or disbursement changes by the same amount in each interest period.

b. Either an increase or a decrease in the amount of a cash flow is called the gradient.

c. The gradient in the cash flow may be positive or negative.

d. All of these

46. Pick up the correct statement from the following:

a. The capital required to get a project started is the first cost.

b. The first cost is a single cash flow or a series of cash flows that are made in the beginning of the activity's life span

c. The first cost of purchasing a car is the sum of the down payment, taxes and dealers charges.

d. All of these

47. In a cash-flow diagram :

a. Time 1 is considered to be the end of time period 1

b. A vertical arrow pointing up indicates a positive cash flow

c. An arrow pointing downward indicates a negative cash flow

d. All of these

48. "Fraudulent practice” means

a. offering anything to influence the action of public official

b. buy more than one bid document

c. mis representation of facts in order to influence a procurement process

d. all of the above

49. According to labour act considering health condition is

a. first aid after accident

b. safety on inst. and cloth

c. education& training

d. none of the above

50. NEC does not approach for

a. engineer

b. overseer to engineer

c. minimum degree B.E.

d. all of the above