***Engineer***

1. The shape factor of standard rolled beam section varies from

a. 1.10 to 1.20

b. 1.20 to 1.30

c. 1.30 to 1.40

d. 1.40 to 1.50

2. The forces acting normally on the cross section of a bar shown in the given figure introduce

a. compressive stress

b. tensile stress

c. shear stress

d. none of these.

3. The general expression for the B.M. of a beam of length l is the beam carries

a. a uniformly distributed load w/unit length

b. a load varying linearly from zero at one end to w at the other end

c. an isolated load at mid span

d. none of these.

4. If a steel rod of 20 mm diameter and 5 metres long elongates by 2.275 mm when subjected to an axial pull of 3000 kg, the stress developed, is

a. 9.5541 kg/cm2

b. 95.541 kg/cm2

c. 955.41 kg/cm2

d. 9554.1 kg/cm2.

5. Columns of given length, cross-section and material have different values of buckling loads for different end conditions. The strongest column is one whose

a. both ends are hinged or pin jointed

b. one end is fixed and the other end entirely free

c. both the ends are fixed

d. none of the these.

6. Determining the difference in elevation between two points on the surface of the earth, is known as

a. levelling

b. simple levelling

c. differential levelling

d. longitudinal levelling.

7. Which one of the following statements is correct ?

a. when the axes of rotation of the graduated circle and the verniers are not coincident, the instrument possesses eccentricity

b. the mean of the readings of the two verniers gives correct reading free from the eccentricity

c. one vernier may be used if the readings of two verniers differ by a constant

d. all the above.

8. If the declination of the needle is 10° W

a. each of the whole circle reckoning has to be micros by 10°

b. in the quadrantal method, the correction is positive in the 1st and 3rd quadrants

c. in the quadrantal method, the corrections is negative in 2nd and 4th quadrants

d. all the above.

9. Pick up the correct statement from the following :

a. if the image of the object does not fall on the plane of the cross-lines, parallax exists

b. parallax has nothing to do with the eyepiece

c. the eyepiece is adjusted for clear vision of the cross hairs

d. all the above.

10. The horizontal angle between true meridian and magnetic meridian, is known

a. bearing

b. magnetic declination

c. dip

d. convergence.

11. Reduced bearing of a line is an angle between

a. north line and given line measured clockwise

b. north line and given line measured anticlockwise

c. east or west and the given line

d. given line and the part of the meridian whether N end or S end, lying adjacent to it.

12. The method of finding out the difference in elevation between two points for eliminating the effect of curvature and refraction, is

a. reciprocal levelling

b. precise levelling

c. differential levelling

d. flying levelling

13. Fibre glass

a. has a higher strength to weight ratio

b. is shock proof and fire retardent

c. does not decay

d. all the above.

14. The initial setting time of hydraulic lime, is

a. 60 minutes

b. 90 minutes

c. 120 minutes

d. 150 minutes.

15. Plastic asphalt is

a. used as a water proofing layer over roof

b. a mixture of cement and asphalt

c. a natural asphalt

d. a refinery product.

16. The commonly used colour pigment in paints, is

a. carbon black

b. iron oxide

c. lamp black

d. all the above.

17. Soundness of cement is tested by

a. Vicat's apparatus

b. Le-chatelier apparatus

c. compressive strength testing apparatus

d. none of the these.

18. The usual percentages of clay and metal in cermet are :

a. 50%, 50%

b. 60%, 40%

c. 70%, 30%

d. 80%, 20%.

19. Grading of sand causes great variation in

a. strength of concrete

b. durability of concrete

c. handing and placing of concrete

d. all the above.

20. The condition not applicable to water cement ratio law, is

a. concrete specimens may be tested at any temperature

b. concrete specimens need be of same age

c. concrete specimens need be of same size

d. none of these.

21. Workability of concrete mix with low water cement ratio is determined by

a. slump test

b. compaction factor test

c. flexural strength test

d. none of these.

22. The percentage of the aggregate of F.M. 2.6 to be combined with coarse aggregate of F.M. 6.8 for obtaining the aggregates of F.M. 5.4, is

a. 30%

b. 40%

c. 50%

d. 60%.

23. The risk of segregation is more for

a. wetter mix

b. larger proportion of maximum size aggregate

c. coarser grading

d. all the above.

24. Pick up the correct statement from the following:

a. A soil having pH value more than 7 is an acidic soil

b. A soil having pH value less than 7 is an acidic soil

c. A soil having pH value more than 7 is an alkaline soil

d. A soil containing chemicals for the manufacture of portland cement is preferred.

25. The ratio of settlement at any time 't' to the final settlement, is known as

a. co-efficient of consolidation

b. degree of consolidation

c. consolidation index

d. consolidation of undisturbed soil.

26. In a purely cohesive soil, the critical centre lies at the intersection of

a. perpendicular drawn at l/3rd slope from toe and the locus of the centre

b. perpendicular drawn at 2/3rd slope from toe and the locus of the centre

c. directional angles

d. none of these.

27. The soil moisture driven off by heat, is called

a. free water

b. hydroscopic water

c. gravity water

d. none of these.

28. Pick up the correct statement from the following:

a. At low value of water content most soils tend to be stiff.

b. At high water content, the dry density decreases with an increase of water content.

c. An airless soil will have maximum possible value of dry density for the given water content

d. All the above.

29. A pile is being driven with a drop hammer weighing 1800 kg and having a free fall of 1.00 m. If the penetration with last blow is 5 mm, the load carrying capacity of the pile, -according to the Engineering News formula, is

a. 100 tonnes

b. 50 tonnes

c. 20 tonnes

d. 10 tonnes.

30. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

31. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

32. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

33. Pick up the correct statement from the following with regards to C.P.M. network analysis of projects

a. Earliest occurrence time of the event from which the activity arrow originates plus the duration of the activity, is called earliest finish time of the activity

b. The latest occurrence time of the node of which the activity arrow terminates minus the duration of the activity, is called latest start time

c. The latest occurrence time for the node at which the activity arrow terminates, is called latest finish time

d. All the above.

34. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

35. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

36. As per Indian Standard Specifications, the peak discharge for domestic purposes per capita per minute, is taken

a. 1.80 litres for 5 to 10 users

b. 1.20 litres for 15 users

c. 1.35 for 20 users

d. All the above.

37. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

38. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

39. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

40. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

41. For drawing thin lines of uniform thickness the pencil should be sharpened in the form of

a. chisel edge

b. conical

c. pointed

d. circular

42. In third angle projection top view is put

a. above front view

b. below font view

c. side view

d. none of the above

43. To remove unnecessary lines ....... is used

a. duster

b. chalk

c. sand box

d. eraser

44. The main purpose of leader line is

a. show dimension

b. to connect a note with the feature

c. to show beauty

d. to show hatch

45. The owner of the construction company makes use of the estimate :

a. to assist in financial arrangements

b. to determine economic feasibility of the project.

c. to determine the tax, insurance and evaluation purpose.

d. All of these

46. The CRF (ep) is also known as: [CRF(EP) - 8% - 7], where

a. 8% is the rate of interest per year

b. money is borrowed for n = 7 years

c. both (a) and (b)

d. Neither (a) nor (b)

47. The estimate based on a detailed quantity survey and furnishes the most accurate and reliable estimate possible is known as

a. Conceptual estimate

b. Definitive estimate

c. Probabilistic estimate

d. None of these

48. The registration book for general and professional engineers is written in Nepali language while for foreginer engineer

a. Nepali

b. English

c. Nepali and English

d. French

49. Engineers shall build their professional reputation on the ..........of their services and shall not compete unfairly with others.

a. publicity

b. basis

c. merit

d. reputation

50. Who is the Chairperson of House Rent Fixation Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer