***Engineer***

1. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

2. Pick up the correct statement from the following:

a. Hoop strain of the walls of a cylinder due to liquid is

b. Longitudinal strain in the walls of a cylinder due to liquid is

c. Volumetric change in the cylinderdue to liquid is

d. All the above.

3. For determining the support reactions at A and B of a three hinged arch, points B and C are joined and produced to intersect the load line at D and a line parallel to the load line through A at D'. Distances AD, DD' and AD' when measured were 4 cm, 3 cm and 5 cm respectively. The angle between the reactions at A and B is

a. 30°

b. 45°

c. 60°

d. 90°

4. The shear force on a simply supported beam is proportional to

a. sum of the forces

b. sum of the transverse forces

c. algebraic sum of the transverse forces of the section

d. curvature of the neutral axis.

5. A cantilever carries is uniformly distributed load W over its whole length and a force W acts at its free end upward. The net deflection of the free end will be

a. zero

b.  upward

c.  downward

d. none of these.

6. In chain surveying field work is limited to

a. linear measurements only

b. angular measurements only

c. both linear and angular measurements

d. all the above.

7. For a closed traverse the omitted measurements may be calculated

a. bearing of one side only

b. both length and bearing of one side

c. length or bearing of adjacent side

d. all the above.

8. A traverse deflection angle is

a. less than 90°

b. more than 90° but less than 180°

c. the difference between the included angle and 180°

d. the difference between 360° and the included angle.

9. Pick up the correct statement from the following :

a. the eyepiece plays no part in defining the line of sight

b. the diaphragm plays no part in defining the line of sight

c. the optical centre of the objective plays no part in defining the line of sight

d. none of these.

10. ABCD is a rectangular plot of land. If the bearing of the side AB is 75°, the bearing of DC is

a. 75°

b. 255°

c. 105°

d. 285°

11. Transition curves are introduced at either end of a circular curve, to obtain

a. gradually decrease of curvature from zero at the tangent point to the specified quantity at the junction of the transition curve with main curve

b. gradual increase of super-elevation from zero at the tangent point to the specified amount at the junction of the transition curve with main curve

c. gradual change of gradient from zero at the tangent point to the specified amount at the junction of the transition curve with main curve

d. none of these.

12. The minimum range for sliding the focusing lens in the internal focusing telescope for focusing at all distances beyond 4 m is

a. 5 mm

b. 10 mm

c. 15 mm

d. 20 mm.

13. In arches, stratified stones are placed so that their planes are

a. parallel

b. perpendicular

c. radial

d. none of these.

14. Pick up the correct statement from the following:

a. The lime stone resists fire upto about 800°C and at higher temperature it splits into CaO and CO2

b. The sand stone with silicates resist a fire in a better way

c. The argillaceous stone though poor in strength can resist fire quite weak

d. All the above.

15. Veneering means

a. carving out designs on timber planks

b. chemically treating timber planks

c. thick   layer   of superior wood glued to inferior wood

d. thin   layer   of superior   wood glued to inferior wood.

16. If P is the percentage of water required for normal consistency, water to be added for determination of initial setting time, is

a. 0.75 P

b. 0.80 P

c. 0.85 P

d. 0.90 P.

17. Cast steel is manufactured by

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

18. Which one of the following polymers is obtained from condensation polymerization?

a. phenol formaldehyde

b. carbamide

c. melamine-formaldehyde

d. all of these.

19. Transport of concrete by pumps, is done for a distance of

a. 200 m

b. 300 m

c. 400 m

d. none of these.

20. The final operation of finishing floors, is known as

a. floating

b. trowelling

c. finishing

d. all the above.

21. Pick up the incorrect statement from the following:

a. Tricalcium silicate (C3S) generates more heat of hydration

b. Tricalcium silicate (C3S) develops early strength

c. Tricalcium silicate (C3S) has more resistance to sulphate attack

d. None of these.

22. Pick up the correct statement from the following:

a. Water lubricates the mixture of gravel, sand and cement

b. Only a small quantity of water is required for hydration of cement

c. Strength of concrete structure largely depends upon its workability

d. All the above.

23. The minimum percentage of chemical ingredient of cement is that of

a. magnesium oxide

b. iron oxide

c. alumina

d. lime

24. The maximum net pressure intensity causing shear failure of soil, is known

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

25. The specific gravity of quartz, is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

26. If the coefficient of the active pressure Ka is 1/3, the coefficient of passive pressure Kp, is

a. 2/3

b. 1

c. 3/2

d. 3

27. The ratio of the difference between the void ratio of the soil in its loosest state and its natural void ratio (e) to the difference between the void ratios in the loosest and fully dense state, is generally termed as

a. degree of density

b. relativity

c. density index

d. all the above.

28. Factor of safety against sliding of a slope, is the ratio of

a. actual cohesion to that required to maintain stability of slope

b. shear strength to shear stress along the surface

c. neither (a) nor (b)

d. both (a) and (b).

29. For determining the moisture content of a soil sample, the following data is available Weight of container = 260 g, Weight of soil sample and = 320 g container, Weight of soil sample (dried) and = 310 g container. The moisture content of the soil sample, is

a. 15%

b. 18%

c. 20%

d. 25%

30. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

31. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

32. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

33. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

34. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

35. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

36. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

37. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

38. Referring of given figure, pick up the correct statement from the following:

a. The total length of centre line of four walls is 20 m

b. Length of long wall out-to-out is 6.80 m

c. Length of short walls in-to-in is 3.20 m

d. All the above.

39. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

40. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

41. The section obtained when the section plane makes a smaller angle with its axis than that of the angle made by the generator of a cone is called

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

42. The proportion by which the actual size of an object is enlarged or reduced in drawing is simply known as

a. multiplication factor Dil

b. scale

c. reflection factor

d. none of the above

43. The area occupied by Ao (drawing sheet) is .......... times the area occupied by Az (drawing sheet).

a. 2

b. 4

c. 8

d. 16

44. Which pencil is used for thinner line?

a. H

b. B

c. 2B

d. 2H

45. The estimate based on a detailed quantity survey and furnishes the most accurate and reliable estimate possible is known as

a. Conceptual estimate

b. Definitive estimate

c. Probabilistic estimate

d. None of these

46. Pick up the correct statement from the following:

a. The ratio of current assests, loans and advances, and the current liquidity is called current ratio.

b. Larger the current ratio, larger is the margin of safety.

c. The operating profit is the difference between gross profit and operating expenses.

d. All of these

47. The CRF (ep) is also known as: [CRF(EP) - 8% - 7], where

a. 8% is the rate of interest per year

b. money is borrowed for n = 7 years

c. both (a) and (b)

d. Neither (a) nor (b)

48. The minimum and maximum no. of members in the user's committee should be

a. 15&17

b. 17&19

c. 9& 15

d. 7&9

49. Which is the professional body of architectural and engineering firms is Nepal?

a. Society of Nepalese Architects (SONA)

b. Society of Public Health Engineers (SOPHEN)

c. Sociality of Consulting Engineering and Architectural firms

d. None of above

50. According to the Public Procurement Regulation, 2064, Who approves the prequalification criteria ?

a. concerned project chief

b. concerned department chief

c. concerned secretary

d. concerned minister