***Engineer***

1. The eccentricity (e) of a hollow circular column, external diameter 25 cm, internal diameter 15 cm for an eccentric load 100 t for non-development of tension, is

a. 3.00 cm

b. 3.50 cm

c. 4.25 cm

d. 5.0 cm

2. If a three hinged parabolic arch, (span l, rise h) is carrying a uniformly distributed load w/unit length over the entire span,

a. horizontal thrust is wl2/8h

b. S.F. will be zero throughout

c. B.M. will be zero throughout

d. all the above.

3. In the truss, the force in the member AC is

a. 6.25 t compressive

b. 8.75 t tentile

c. t tensile

d. t compressive.

4. In a three hinged arch, the bending moment will be zero

a. at right hinge only

b. at left hinge only

c. at both right and left hinges

d. at all the three hinges.

5. The property of a material by which it can be drawn to a smaller section, due to tension, is called

a. plasticity

b. ductility

c. elasticity

d. malleability.

6. Pick up the correct statement from the following :

a. the apparent error on reversal is twice the actual error

b. the correction may be made equal to half the observed discrepancy.

c. the good results may be obtained from a defective instrument by reversing and taking the mean of two erroneous results

d. all the above.

7. Pantagraph is used for

a. measuring distances

b. measuring areas

c. enlarging or reducing plans

d. setting out right angles

8. The main plate of a transit is divided into 1080 equal divisions. 60 divisions of the vernier coincide exactly with 59 divisions of the main plate. The transit can read angles accurate upto

a. 10"

b. 15"

c. 20"

d. 30"

9. Ramsden eye-piece consists of

a. two convex lenses short distance apart

b. two concave lenses short distance apart

c. one convex lens and one concave lens short distance apart

d. two plano-convex lenses short distance apart, with the convex surfaces facing each other.

10. In a constant level tube, size of the bubble remains constant because upper wall is

a. of relatively larger radius

b. of relatively smaller radius

c. flat

d. convex downwards.

11. While working on a plane table, the correct rule is :

a. Draw continuous lines from all instrument stations

b. Draw short rays sufficient to contain the points sought

c. Intersection should be obtained by actually drawing second rays

d. Take maximum number of sights as possible from each station to distant objects.

12. The diaphragm of a stadia theodolite is fitted with two additional

a. horizontal hairs

b. vertical hairs

c. horizontal and two vertical hairs

d. none of these.

13. The process of manufacturing steel by heating short lengths of wrought iron bars mixed with charcoal in fire clay crucibles and collecting the molten iron into moulds, is known as

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

14. Pick up the correct statement from the following:

a. The low voltage porcelain is prepared by wet process

b. The high voltage porcelain is prepared by dry process

c. The low voltage porcelain is prepared by dry process

d. None of the above.

15. Kaolin is chemically classified as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

16. Asbestos is

a. an incombustible fire proof material

b. an organic substance

c. a bad insulator for sound and heat

d. all the above.

17. The cast iron when heated to red heat with powdered red haemetite in an oven for increasing its toughness, is converted to

a. grey cast iron

b. white cast iron

c. mottled cast iron

d. toughed cast iron.

18. Expanded metal is

a. manufactured from steel sheets

b. used for reinforced concrete in road pavements

c. measured in term of SWM (shortway mesh) and LWM (long way mesh)

d. all the above.

19. The specifications of a cement bag for storage, are

a. height 18 cm

b. plan area 3000 sq. cm

c. volume 35 litres

d. all the above.

20. Non-uniform compaction may cause the concrete

a. porous

b. non-homogeneous

c. reduced strength

d. all the above.

21. Pick up the correct statement from the following:

a. Water lubricates the mixture of gravel, sand and cement

b. Only a small quantity of water is required for hydration of cement

c. Strength of concrete structure largely depends upon its workability

d. All the above.

22. Pick up the correct statement from the following:

a. The maximum size of the fine aggregate, is 4.75 mm and minimum 0.075 mm

b. The material having particles of size varying from 0.06 mm to 0.002 mm, is known as silt

c. The material having particles of size less than 0.002 mm, is known as clay

d. All the above.

23. The maximum amount of dust which may be permitted in aggregates is

a. 5% of the total aggregates for low workability with a coarse grading

b. 10% of the total aggregates for low workability with a fine grading

c. 20% of the total aggregates for a mix having high workability with fine grading

d. all the above.

24. Pick up the correct statement from the following:

a. An unconfined compression test is a special case of triaxial compression test

b. An unconfined compression test is a special case of direct shear test

c. The confining pressure is maximum during an unconfined compression test

d. The cylindrical specimen of a soil is subjected to major principal stress till it fails due to shearing along the plane of the failure.

25. The consolidation time for soils

a. decreases with increasing permeability

b. increases rapidly with increasing size of soil mass

c. is independent of the magnitude of the stress change.

d. All the above.

26. The bearing capacity of a soil depends upon

a. shape of the particles

b. cohesive properties of particles

c. internal frictional resistance of particles

d. all the above.

27. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

28. The minimum centre to centre distance of friction piles of 1 m diameter, is

a. 2 m

b. 2 m to 3 m

c. 3 m to 4 m

d. 5 m

29. The shear resistance of a soil is con-tituted basically of the following component.

a. The frictional resistance to translocation between the individual soil particles at their contact point

b. To the structural relation to displacement of the soil because of the interlocking of the particles

c. Cohesion and adhesion between the surfaces of the soil particles

d. All the above.

30. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

31. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

32. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

33. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

34. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

35. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

36. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

37. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

38. The expected out turn of half brick partition wall per mason per day is

a. 1.5 m3

b. 2.0 m3

c. 4.0 m2

d. 5.0 m2

39. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

40. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

41. For drawing thin lines of uniform thickness the pencil should be sharpened in the form of

a. chisel edge

b. conical

c. pointed

d. circular

42. The space covered by a drawing in a paper after changing the scale from 1:100 to 1:50 will:

a. increase 400%

b. increase 200%

c. decrease 200

d. decrease 400%

43. The angle between isometric axes is

a. 30°

b. 60°

c. 90°

d. 120°

44. Select the incorrect statement about orthographic projections :

a. the reference planes are also called the principle planes

b. the object must the situated in the first quadrant

c. the planes of projection and assumed to be transparent

d. projections are obtained by drawing perpendicular s from the object to the planes

45. Current assests less inventories divided by current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

46. If P is principal amount, i is the rate of interest and n is the number of periods in years, then the interest factor is :

a. (1 + ni)

b. (ni - 1)

c. ni

d. None of these

47. Which one of the following is not a construction estimate ?

a. Initial feasibility estimate

b. Conceptual preliminary budget

c. Definite estimate

d. None of these

48. The number of members in Nepal Engineers' Association are

a. 5

b. 10

c. 15

d. 20

49. Provision for professional code of conduct of engineers in Nepal is made in detail in:

a. NEC Act

b. NEC Rules

c. NEC bye-laws

d. NEC ordience

50. The minimum qulification & experience required for the registration of foreigner engineers is

a. bacheclor in engg. and 5 years experience

b. bachelor in engg. and 10yrs experience

c. master in engg. and 5 yrs experience

d. master in engg. and 10yrs experience