***Engineer***

1. A body is said to be in equilibrium if

a. it moves horizontally

b. it moves vertically

c. it rotates about its C.G.

d. none of these.

2. The ratio of maximum and average shear stresses on a rectangular section, is

a. 1.25

b. 1.5

c. 2.0

d. 2.5

3. Pick up the correct statement from the following:

a. In a loaded beam, the moment at which the entire section of the beam becomes fully plastic, is called plastic moment

b. In a fully plastic stage of the beam, the neutral axis divides the section in two sections of equal area

c. The ratio of plastic moment to the yield moment, is called shape factor

d. All the above.

4. A cast iron T section beam is subjected to pure bending. For maximum compressive stress to be three times the maximum tensile stress, centre of gravity of the section from flange side is

a. h/4

b. h/3

c. h/2

d. 2/3 h.

5. The stress necessary to initiate yielding, is considerably

a. more than that necessary to continue it

b. less than that necessary to continue it

c. more than that necessary to stop it

d. less than that necessary to stop it.

6. In a lemniscate curve the ratio of the angle between the tangent at the end of the polar ray and the straight, and the angle between the polar ray and the straight, is

a. 3

b. 4

c. 5

d. 3/2

7. The additional lines which are measured to show the correctness of the chain surveying are called:

a. check clines

b. proof lines

c. tie lines

d. all of these.

8. The sum of the interior angles of a geometrical figure laid on the surface of the earth differs from that of the corresponding plane figure only to the extent of one second for every

a. 100 sq. km of area

b. 150 sq. km of area

c. 200 sq. km of area

d. none of these.

9. The real image of an object formed by the objective, must lie

a. in the plane of cross hairs

b. at the centre of the telescope

c. at the optical centre of the eye-piece

d. anywhere inside the telescope.

10. If θ is the slope of the ground and l is the measured distance, the correction is

a. 2l sin2 θ/2

b. 2l cos2 θ/2

c. 2l tan2 θ/2

d. 2l cot2 θ/2.

11. A theodolite is said to be in perfect adjustment if

a. rotation axis is vertical to the transit axis

b. transit axis is perpendicular to line of collimation

c. line of collimation sweeps out a vertical plane while the telescope is elevated or depressed

d. all the above.

12. The area of a plane triangle ABC, having its base AC and perpendicular height h, is

a.  ba sin C

b.  bc sin A

c. S(S - a)(S - b)(S - c) where S is

d. all the above

13. Sewer pipes are made of

a. stone ware

b. refractory clay

c. terracota

d. all the above.

14. Spirit varnish generally consists of

a. alcohol, wax and turpentine

b. pigment and synthetic resin

c. spirit and shellac

d. none of these.

15. Quick setting cement is produced by adding

a. more amount of gypsum in very fine powdered form

b. aluminium sulphate in very fine powdered form

c. pozzolana in very fine powdered form

d. none of these.

16. The initial setting time of hydraulic lime, is

a. 60 minutes

b. 90 minutes

c. 120 minutes

d. 150 minutes.

17. Cast iron

a. is manufactured in required shapes

b. may contain 2 to 5 per cent of carbon with other impurities

c. is remelted in a cupola furnace

d. all the above.

18. The initial setting time of lime-pozzolana, is

a. 30 minutes

b. 60 minutes

c. 90 minutes

d. 120 minutes.

19. The 28 days cube strength of mass concrete using aggregates of maximum size 5 cm for gravity dams should be

a. between 350 to 600 kg/cm2

b. between 150 to 500 kg/cm2

c. below 200 kg/cm2

d. none of these.

20. The process of proper and accurate measurement of concrete ingredients for uniformity of proportion, is known

a. curing

b. mixing

c. batching

d. none of these.

21. Pick up the incorrect statement from the following:

a. Admixtures accelerate hydration

b. Admixtures make concrete water proof

c. Admixtures make concrete acid proof

d. Admixtures give high strength

22. If fineness modulus of sand is 2.5, it is graded as

a. fine sand

b. medium sand

c. coarse sand

d. very coarse sand.

23. Construction joints are generally provided in concrete

a. retaining walls

b. lining of canals

c. lining of tunnels

d. all the above.

24. If there is no impervious boundary at the bottom of a hydraulic structure, stream lines tend to follow :

a. a straight line

b. a parabola

c. a semi-ellipse

d. a semi-circle.

25. Soils containing organic matters

a. are of spongy nature

b. swell with decrease of moisture

c. shrink with increase of moisture content

d. none of these.

26. The compression index of a soil

a. decreases with an increase in the liquid limit

b. increases with an increase in the liquid limit

c. decreases with an increase in the plastic limit

d. is not related with plastic limit.

27. The equation τ = C + σ tan φ is given by

a. Rankine

b. Coulomb

c. Culaman

d. Mohr.

28. If G is specific gravity of sand particles, e is porosity, the critically hydraulic gradient

a. ic =

b. ic =

c. ic =

d. ic =

29. The general relationship between specific gravity (G), weight of water (γω), degree of saturation (Sr), void ratio (e) and bulk density (γ), is

a. γ =

b. γ =

c. γ =

d. γ =

30. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

31. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

32. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

33. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

34. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

35. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

36. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

37. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

38. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

39. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

40. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

41. The art of representation of an object by systemic lines on a paper is known as

a. drawing

b. art

c. dimensioning

d. profession

42. In the first angle projection the object is assumed to be placed in

a. fourth quadrant

b. third quadrant

c. second quadrant

d. first quadrant

43. The space between two sentences should be left equal to

a. height of letter

b. twice the ht. of letter

c. 1.5 times ht. of letter

d. none of the above

44. The other name of axonometric projection is

a. one plane projection

b. two plane projection

c. three plane projection on

d. all of the above

45. The estimator for definitive estimates must be able :

a. and an all-around construction expert.

b. to read the plans and specifications to determine accurate quantities of permanent materials and installed Equipment.

c. to express the job material requirements in dimensions suitable for costing and construction supplies.

46. If P is principal amount, i is the rate of interest per annum and n is the number of periods in years, the compound amount factor (CAF) is :

a. (1 + i)n

b. (1 + i)(1/2n)

c. (n + i)

d. None of these

47. The owner of the construction company makes use of the estimate :

a. to assist in financial arrangements

b. to determine economic feasibility of the project.

c. to determine the tax, insurance and evaluation purpose.

d. All of these

48. According to the Public Procurement Regulation, 2064, Who approves the prequalification criteria ?

a. concerned project chief

b. concerned department chief

c. concerned secretary

d. concerned minister

49. As per the Public Procurement Regulation of Nepal, 2064, What is the maximum consulting services that can be procured directly by a public agency?

a. Rs. 200,000/-

b. Rs. 300,000/-

c. Rs. 500,000/-

d. none of the above

50. Provision for professional code of conduct of engineers in Nepal is made in detail in:

a. NEC Act

b. NEC Rules

c. NEC bye-laws

d. NEC ordience