***Engineer***

1. Slenderness ratio of a long column, is

a. area of cross-section divided by radius of gyration

b. area of cross-section divided by least radius of gyration

c. radius of gyration divided by area of cross-section

d. length of column divided by least radius of gyration.

2. A material may fail if

a. maximum strain exceeds

b. maximum shear stress exceeds

c. total strain energy exceeds

d. all the above.

3. At yield point of a test piece, the material

a. obeys Hooke's law

b. behaves in an elastic manner

c. regains its original shape on removal of the load

d. undergoes plastic deformation.

4. The distance between the centres of adjacent rivets in the same row, is called

a. pitch

b. lap

c. gauge

d. staggered pitch.

5. The property of a material by which it can be drawn to a smaller section, due to tension, is called

a. plasticity

b. ductility

c. elasticity

d. malleability.

6. In a telescope the object glass of focal length 14 cm, is located at 20 cm from the diaphragm. The focussing lens is midway between them when a staff 16.50 m away is focussed. The focal length of the focussing lens, is

a. 5.24 cm

b. 6.24 cm

c. 7.24 cm

d. 8.24 cm

7. If the radius of a simple curve is 600 m, the maximum length of the chord for calculating offsets, is taken

a. 15 m

b. 20 m

c. 25 m

d. 30 m.

8. The bearing of line AB is 152° 30' and angle ABC measured clockwise is 124° 28'. The bearing of BC is

a. 27° 52'

b. 96° 58'

c. 148° 08'

d. 186° 58'

9. If h1 and h2 are the differences in level between ground and the formation levels, m is the slope of the sloping sides. D is the distance between the cross sections then, prismoidal correction for a level section is

a. D/3m(h1 - h2)

b. D/6m(h1 - h2)2

c. D/6m(h1 - h2)3

d. D/6m(h1 + h2)2

10. ABCD is a regular parallelogram plot of land whose angle BAD is 60°. If the bearing of the line AB is 30°, the bearing of CD, is

a. 90°

b. 120°

c. 210°

d. 270°

11. A relatively fixed point of known elevation above datum, is called

a. bench mark

b. datum point

c. reduced level

d. reference point.

12. In chain surveying field work is limited to

a. linear measurements only

b. angular measurements only

c. both linear and angular measurements

d. all the above.

13. The timber having maximum resistance against white ants, is obtained from

a. chir

b. shisham

c. sal

d. teak.

14. Pick up the correct statement from the following:

a. The bulking of fine sand is more than that of coarse sand

b. If the percentage content of moisture exceeds 10%, increase in bulk of sand starts increasing

c. The volume of fully saturated sand equals that of dry sand

d. All the above.

15. For filling cracks in masonry structures, the type of bitumen used, is

a. cut-back bitumen

b. bitumen-emulsion

c. blown bitumen

d. plastic bitumen.

16. Pick up the correct statement from the following:

a. For mass concrete structures subjected to wetting and drying, the water ratio should be 0.55

b. For thin structures which remain continuously under water, the water-cement ratio by weight should be 0.55

c. For massive concrete structures which remain continuously under water, the water cement ratio by weight should be 0.65

d. All the above.

17. Slump test for concrete is carried out, to determine

a. strength

b. durability

c. workability

d. water content.

18. Which one of the following is an air binding material ?

a. Gypsum

b. Acid-resistant cement

c. Quick lime

d. All of these.

19. The produce impermeable concrete

a. proper compaction of concrete is required

b. proper curing of concrete is required

c. properly graded and non-porous aggregates are requied

d. all the above.

20. The grade of concrete not recommended by I.S. : 456, is

a. M 200

b. M 300

c. M 400

d. M 500

21. For an ordinary Portland cement

a. soundness varies from 5 to 10 mm

b. initial setting time is not less than 30 minutes

c. compressive stress after 7 days, is not less than 175 kg/cm2

d. all the above.

22. Permissible compressive strength of M 150 concrete grade is

a. 150 kg/cm2

b. 200 kg/cm2

c. 250 kg/cm2

d. 300 kg/cm2

23. If the average compressive strength is 4000 kg/cm2 and standard deviation is 500, the co-efficient of variation is

a. 12.5%

b. 15%

c. 18.5 %

d. 20%

24. The expression  ρω is used for

a. dry density

b. bulk density

c. degree of saturation

d. optimum water content.

25. Pick up the correct statement from the following:

a. Isotropic consolidation of clay can be obtained in the triaxial apparatus under equal all-round pressure.

b. If the present effective stress is the maximum to which the clay has ever been subjected, it is called normally consolidated clay

c. If the present effective stress in the past was more than present effective stress, it is called over-consolidated clay

d. All the above.

26. The shear strength of a soil

a. is proportional to the cohesion of the soil

b. is generally known as the strength of the soil

c. is proportional to the tangent of the angle of internal friction

d. all the above.

27. The zero atmospheric pressure is at

a. mean sea level

b. water table

c. phreatic surface

d. both (c) and (d) of the above.

28. The minimum depth of building foundations on

a. sandy soils is 80 cm to 100 cm

b. clay soils is 90 cm to 160 cm

c. rocky soils is 5 cm to 50 cm

d. all the above.

29. Pick up the correct statement from the following:

a. In hydrometer method, weight Wd per ml of suspension is found directly

b. In pipette analysis, weight Wd per ml of suspension is found indirectly

c. In pipette analysis, weight Wd per ml of suspension is found directly

d. None of these.

30. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

31. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

32. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

33. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

34. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

35. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

36. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

37. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

38. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

39. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

40. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

41. The section obtained when the section plane is inclined to the axis of cone, but parallel to one of the generators of a cone is called ......... ......

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

42. Which of the the following is not intended while making a 'sketch' in the context of technical drawings?

a. a fished product

b. free hand sketching

c. a preliminary product

d. a quick way to record an idea for work

43. One of the characteristics features of a perspective drawing is that objects are drawn:

a. smaller as their distance from the observer increases

b. same size irrespective of distance

c. larger as their distance from the observer increases

d. none of the above

44. The angle between two perpendicular line is

a. 0°

b. 45°

c. 90°

d. 180°

45. The sunk costs include :

a. a past expenditure

b. an unrecovered balance

c. an invested capital that cannot be retreived

d. All of these

46. The CRF (ep) is also known as: [CRF(EP) - 8% - 7], where

a. 8% is the rate of interest per year

b. money is borrowed for n = 7 years

c. both (a) and (b)

d. Neither (a) nor (b)

47. The key to profitable operation for project cost control, is :

a. To keep the project cost equal to original cost estimate.

b. To keep the project cost equal to subsequent construction budget.

c. To keep the project cost within the cost budget and knowing when and where job costs are deviating.

d. None of these

48. Labour act was introduced /founded in

a. 2038

b. 2048

c. 2043

d. 2053

49. Maximum working hour for a labour per week should be

a. 42hr

b. 45hr

c. 48hr

d. 53hr

50. Procurement of services means:

a. buying logistics

b. hiring consultant

c. renting house

d. none of the above