***Engineer***

1. The tangential component of stress on an plane inclined θ° to the direction of the force, may be obtained by multiplying the normal stress by

a. cos θ

b. tan θ

c. sin θ cos θ

d. sin2θ

2. A shaft subjected to a bending moment M and a torque T, experiences

a. maximum bending stress =

b. maximum shear stress =

c. both (a) and (b)

d. neither (a) nor (b)

3. The assumption in the theory of bending of beams, is :

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

4. For a beam, if fundamental equations of statics are not sufficient to determine all the reactive forces at the supports, the structure is said to be

a. determinate

b. statically determinate

c. statically indeterminate

d. none of these.

5. The type of butt joints in common use, is :

a. single inverted V-butt joint

b. double V-butt joint

c. double U-butt joint

d. single V-butt joint.

6. An internal focussing type surveying telescope, may be focussed by the movement of

a. objective glass of the telescope

b. convex-lens in the telescope

c. concave lens in the telescope

d. plano-convex lens in the telescope.

7. The method of finding out the difference in elevation between two points for eliminating the effect of curvature and refraction, is

a. reciprocal levelling

b. precise levelling

c. differential levelling

d. flying levelling

8. Pick up the correct statement from the following :

a. spherical aberration may be minimised by replacing the single lens by a combination of the lenses.

b. in telescope objectives, a combination of convex lens and concave lens is used.

c. in eyepieces, two plano-convex lenses placed at a certain distance apart are used

d. all the above.

9. Pick up the correct statement from the following :

a. the lines of sight while observing back sight and fore  sight lie in the same horizontal plane

b. the   staff readings are measurements made vertically downwards from a horizontal plane

c. the horizontal plane with reference to which staff readings are taken, coincides with the level surface through the telescope axis

d. all the above.

10. Ranging in chain survey means

a. establishing an intermediate point on the line

b. determining the distance between end points

c. determining the offset distance

d. none of these

11. Pick up the wrong statement from the following :

a. the diaphragm is placed between the eyepiece and the objective but nearer to the later

b. the outer component of the objective is a double-convex lens of crown glass

c. the inner component of the objective is a flint glass, convexo-concave

d. all the above.

12. For taking offsets with an optical square on the right hand side of the chain line, it is held

a. by right hand upside down

b. by left hand upright

c. by right hand upright

d. by left hand up side down.

13. Brittleness of cold is due to an excess of

a. sulphur

b. carbon

c. phosphorus

d. silicon.

14. Pick up the correct statement from the following:

a. Strength and stiffness of corrugated sheets are considerably increased

b. Corrugated sheets are generally gal-venised to protect iron from corrosion by rust

c. Corrugated sheets are generally used on slanting roofs

d. all the above.

15. Chemically, marble is known as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

16. Lime mortar is generally made with

a. fat lime

b. hydraulic lime

c. plain lime

d. none of these.

17. Portland pozzolana cement possesses

a. lower heat of hydration

b. lower shrinkage on drying

c. water tightness

d. all the above.

18. Pick up the correct statement from the following: Method of sawing timber

a. tangentially to annual rings, is known as tangential method.

b. in four quarters such that each board cuts annual rings at angles not less than 45°, is known as quarter sawing method.

c. cut out of quarter logs, parallel to the medullary rays and perpendicular to annual rings, is known as radial sawing.

d. all the above.

19. Specified compressive strengh of concrete is obtained from cube tests at the end of

a. 7 days

b. 14 days

c. 21 days

d. 28 days.

20. Pick up the correct statement from the following:

a. High percentage of C3S and low percentage of C2S make the cement less resistive to chemical attack

b. Low percentage of C3S and high percentage of C2S contribute to slow hardening

c. Low percentage of C3S and high percentage of C2S provide greater resistance to chemical attack

d. All the above.

21. Gypsum is added for

a. colour

b. strength

c. controlling setting time

d. none of these.

22. Permissible compressive strength of M 200 concrete grade is

a. 150 kg/cm2

b. 200 kg/cm2

c. 250 kg/cm2

d. 300 kg/cm2

23. The cement whose strength is a little lower than the ordinary cement during the first three months but attains afterwards the same strength, is known as

a. low-heat Portland cement

b. rapid hardening Portland cement

c. Portland blast slag cement

d. Portland pozzolana cement

24. The specific yield of soil depends upon

a. compaction of stratum

b. distribution of pores

c. shape and size of particles

d. all the above.

25. A clay subjected to pressure in excess to its present over-burden, is said to be

a. pre-compressed

b. pre-consolidated

c. over-consolidated

d. all the above.

26. A pycnometer is used to determine

a. voids ratio

b. dry density

c. water content

d. density index.

27. The specific gravity of Calcite is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

28. The neutral stress in a soil mass is

a. force per neutral area

b. force per effective area

c. stess taken up by the pore water

d. stress taken up by solid particles.

29. The maximum value of effective stress in the past divided by the present value, is defined as over consolidation ratio (OCR). The O.C.R. of an over consolidated clay is

a. less than 1

b. 1

c. more than 1

d. None of these.

30. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

31. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

32. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

33. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

34. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

35. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

36. The item of the brick structure measured in sq.m, is

a. Reinforced brick work

b. Broken glass coping

c. Brick edging

d. Brick work in arches.

37. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

38. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

39. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

40. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

41. Bisecting a line means, divide a line into..... equal part.

a. two

b. three

c. four

d. none of the above

42. Tracing paper is also named as

a. drafting vellum

b. drafting film

c. drafting media

d. drafting line

43. As built drawing is normally constructed................the construction.

a. before construction

b. simultaneously with construction

c. after construction

d. all of the above

44. In orthographic projection which one have all three views same

a. cube

b. circle

c. sphere

d. both (a) and (c) of above

45. Which one of the following statements is correct?

a. The number of years required to recover the initial cash investment in a project, is called Pay Back period (PBP).

b. The discount rate that equates the present value of the expected Net Cash Flows (CFs) with the Initial Cash Outflow (ICO) is known as internal rate of return.

c. The present value of the proposal's net cash flows, less the proposal's initial cash outflow is known as the Net Present Value (NPV)

d. All of these

46. Liquidity ratios are used :

a. to measure a firms ability to meet short-cut obligations.

b. to compare short term obligations to short-term resources available to meet these obligations.

c. to obtain much insight into the present cash solvency of the firm and the firm's ability to remain solvent in the event of adversity.

d. All of these

47. Pick up the correct statement from the following:

a. A NPV profile graph shows the curvilinear relationship between the net present value of the project and discount rate employed.

b. In a NPV profile, if discount rate is zero, then net present value is simply total cash inflows less the total cash outflows of the project.

c. As the discount rate increases, the net present value profile slopes downward to the right.

d. All of these

48. Who is the Chairperson of Project Co-ordination Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer

49. What is the provision in Engineer Council Act and Regulation in Nepal for the punishment na person practices Engineering work without registering into the Nepal Engineering Council?

a. Rs 1000 find or imprisonment of 1 month or both

b. Rs 5000 fine

c. Rs10000 fine

d. Rs3000 fine or Imprisonment of 3 month of both

50. The space required for living the worker is 15m with considering the maximum height of room above floor level.

a. 3m

b. 3.5m

c. 4m

d. 5 m