***Engineer***

1. A steel rod of sectional area 250 sq. mm connects two parallel walls 5 m apart. The nuts at the ends were tightened when the rod was heated to 100°C. If αsteel = 0.000012/C°, Esteel = 0.2 MN/mm2, the tensile force developed at a temperature of 50°C, is

a. 80 N/mm2

b. 100 N/mm2

c. 120 N/mm2

d. 150 N/mm2

2. Gradually applied static loads do not change with time their

a. magnitude

b. direction

c. point of application

d. all the above.

3. A material may fail if

a. maximum strain exceeds

b. maximum shear stress exceeds

c. total strain energy exceeds

d. all the above.

4. Struts are load carrying members of a frame structure which are subjected to

a. axial tension loads

b. axial compressive loads

c. torsional loads

d. transverse loads.

5. The shape of the bending moment diagram over the length of a beam, having no external load, is always

a. linear

b. parabolic

c. cubical

d. circular.

6. Simpson's rule for calculating areas states that the area enclosed by a curvilinear figure divided into an even number of strips of equal width, is equal to

a. one third the width of a strip, multiplied by the sum of two extreme offsets, twice the sum of remaining odd offsets and four times the sum of the even offsets

b. one third the width of a strip, multiplied by the sum of two extreme offsets, four times the sum of the remaining odd offsets, and twice the sum of the even offsets

c. one sixth the width of a strip, multiplied by the sum of the two extreme offsets, twice the sum of remaining odd offsets and four times the sum of the even offsets

d. none of these

7. Planimeter is used for measuring

a. area

b. contour gradient

c. slope angle

d. none of these

8. If + 0.8% grade meets - 0.7% grade and the rate of change of grade for 30 m distance is 0.05, the length of the vertical curve will be

a. 700 m

b. 800 m

c. 900 m

d. 1000 m

9. The line of collimation method of reduction of levels, does not provide a check on

a. intermediate sights

b. fore sights

c. back sights

d. reduced levels.

10. A lemniscate curve will not be transitional throughout, if its deflection angle, is

a. 45°

b. 60°

c. 90°

d. 120°

11. The best method of interpolation of contours, is by

a. estimation

b. graphical means

c. computation

d. all of these.

12. Probable accidental error in precise levelling as recommended by International Geodetic Association, should not exceed (where k is in kilometers.)

a. ± 0.5 k mm

b. ± 1 k mm

c. ± 2 k mm

d. ± 5 k mm.

13. Pozzolana (or surkhi) is used in lime

a. to impart hydraulicity

b. to prevent shrinkage

c. to decrease the cost of construction

d. to decrease the setting time.

14. The weight of a good quality brick when immersed in water for a period of 16 hours should not exceed the weight of dry brick

a. 20%

b. 15%

c. 10%

d. None of these.

15. In stone masonry, if stones are so placed that their layers are parallel to the direction of load, they

a. split easily

b. are affected by moisture

c. both (a) and (b)

d. none of these.

16. The melting point of silica is :

a. 1570°C

b. 1630°C

c. 1730°C

d. 1850°C.

17. Refractory bricks are

a. neutral refractory bricks

b. acid refractory bricks

c. basic refractory bricks

d. all the above.

18. Pick up the correct statement from the following:

a. In basic Bessemer process, the steel heats the converter

b. In open-hearth process, the furnace heats the steel

c. In Siemens process, the impurities of pig iron are oxidised by the oxygen of the ore

d. all the above.

19. The aggregate impact value of the aggregate used in

a. building concrete is less than 45

b. road pavement concrete is less than 30

c. runway concrete is less than 30

d. all the above.

20. According to IS : 382-1963, a good aggregate should be

a. chemically inert

b. sufficiently strong

c. hard and durable

d. all the above.

21. The main object of compaction of concrete, is:

a. to eliminate air holes

b. to achieve maximum density

c. to provide initimate contact between the concrete and embedded materials

d. all the above.

22. Wp and Wf are the weights of a cylinder containing partially compacted and fully compacted concrete. If the compaction factor  is 0.95, the workability of concrete is

a. very low

b. low

c. high

d. none of these.

23. Curing of pavements, floors, roofs and slabs, is done by

a. ponding method

b. covering surface with bags

c. sprinkling water method

d. shading concrete method.

24. The expression  ρω is used for

a. dry density

b. bulk density

c. degree of saturation

d. optimum water content.

25. The plasticity of fine soils may be assessed by means of

a. dry strength test

b. toughness test

c. dilatancy test

d. all of these.

26. For a clay slope of height of 10 m, the stability number is 0.05, γ = 2.0 t/m3, C = 2.5 t/m2, the critical height of the slope of the soil, is

a. 4.0 m

b. 12.5 m

c. 25.0 m

d. 15.0 m

27. Transporting and redepositing soils, is done by

a. glacier

b. gravity

c. wind

d. all the above.

28. The maximum value of effective stress in the past divided by the present value, is defined as over consolidation ratio (OCR). The O.C.R. of an over consolidated clay is

a. less than 1

b. 1

c. more than 1

d. None of these.

29. The slip at critical angle, is generally known

a. δ 1-line

b. rupture plane

c. slip plane

d. all the above.

30. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

31. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

32. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

33. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

34. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

35. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

36. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

37. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

38. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

39. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

40. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

41. The view actually appears to the observer when viewed from a fixed position is called

a. isometric drawing

b. orthographic projection

c. perspective drawing

d. none of the above

42. The drawing for municipality is done at scale

a. 1" = 8"

b. 1:50

c. 1:100

d. both (a) and (c)

43. As built drawing is normally constructed................the construction.

a. before construction

b. simultaneously with construction

c. after construction

d. all of the above

44. The area occupied by Ao (drawing sheet) is ........... times the area occupied by A1 (drawing sheet).

a. 2

b. 4

c. 8

d. 16

45. Pick up the correct method adopted for developing the approximate or conceptual estimates from the following :

a. Cost per function method

b. Cost per square metre

c. Cost per linear unit

d. All of these

46. Keeping in view, the feasibility order of magnitude, the preliminary, conceptual or budget estimates, are prepared by :

a. construction manager

b. owner himself/herself

c. construction manager

d. None of these

47. Pick up the correct statement from the following:

a. The capital required to get a project started is the first cost.

b. The first cost is a single cash flow or a series of cash flows that are made in the beginning of the activity's life span

c. The first cost of purchasing a car is the sum of the down payment, taxes and dealers charges.

d. All of these

48. Provision for professional code of conduct of engineers in Nepal is made in detail in:

a. NEC Act

b. NEC Rules

c. NEC bye-laws

d. NEC ordience

49. The variation exceeding 15 % of the contract amount is approved by

a. gazetted 2nd class

b. gazetted 1st class

c. deaprtment head

d. none of the above

50. During construction price variation clause in a contract applies to

a. increase in cost of important materials only

b. variation in the cost of material labour and fuel

c. variation in total cost of the project estimated as lump sum figure

d. the rate of inflation