***Engineer***

1. For a strongest rectangular beam cut from a circular log, the ratio of the width and depth, is

a. 0.404

b. 0.505

c. 0.606

d. 0.707

2. Pick up the correct statement from the following:

a. The pitch of a close coil spring, is very small

b. The angle made by the coil with horizontal, is called the angle of helix

c. In the open coil helical spring, the angle of helix is comparatively large

d. All the above.

3. Pick up the correct statement from the following:

a. For a uniformly distributed load, B.M. curve is a parabola

b. For a load varying linearly, the shear force curve is a parabola

c. For a load varying linearly, the B.M. curve is a cubic parabola

d. All the above.

4. The maximum compressive stress at the top of a beam is 1600 kg/cm2 and the corresponding tensile stress at its bottom is 400 kg/cm2 . If the depth of the beam is 10 cm, the neutral axis from the top, is

a. 4 cm

b. 6 cm

c. 8 cm

d. 10 cm.

5. In a three hinged arch, the bending moment will be zero

a. at right hinge only

b. at left hinge only

c. at both right and left hinges

d. at all the three hinges.

6. Perpendicularity of an offset may be judged by eye, if the length of the offset is

a. 5 m

b. 10 m

c. 15 m

d. 20 m.

7. The angle of intersection of a curve is the angle between

a. back tangent and forward tangent

b. prolongation of back tangent and forward tangent

c. forward tangent and long chord

d. back tangent and long chord.

8. Cross hairs in surveying telescopes, are fitted

a. in the objective glass

b. at the centre of the telescope

c. at the optical centre of the eye piece

d. in front of the eye piece.

9. In a precision traverse, included angles are measured by setting the vernier

a. to read zero exactly on back station

b. to read 5° exactly on back station

c. some where near zero and reading both verniers on back station

d. all the above.

10. The representation of general topography of a very flat terrain is possible only

a. by drawing contours at large interval

b. by drawing contours at small interval

c. by giving spot levels at large interval

d. by giving spot levels to salient features at close interval.

11. Pick up the correct statement from the following :

a. the principal axes of both the lenses coincide the optical axis of the telescope

b. the lens nearer the object to be viewed is convex and is called objective

c. the lens nearer the eye is called eyepiece

d. all the above.

12. A theodolite is said to be in perfect adjustment if

a. rotation axis is vertical to the transit axis

b. transit axis is perpendicular to line of collimation

c. line of collimation sweeps out a vertical plane while the telescope is elevated or depressed

d. all the above.

13. Good quality sand is never obtained from

a. river

b. nala

c. sea

d. gravel powder.

14. The standard size of masonry bricks, is

a. 19 cm x 9 cm x 9 cm

b. 20 cm x 10 cm x 10 cm

c. 21 cm x 11 cm x 11 cm

d. none of these.

15. Knots in timber are

a. defects caused by crushing fibres

b. splits radiating from the centre

c. speckled strains

d. signs of branches cut off.

16. Quartzite is a

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

17. Bitumen completely dissolves in

a. chloroform

b. benzol

c. coaltar

d. All of these.

18. Second class bricks

a. are of dark brown colour

b. produce a metallic sound when struck

c. are well burnt

d. are under burnt.

19. The ratio of various ingredients (cement, sand, aggregates) in concrete of grade M 200, is

a. 1 : 3 : 6

b. 1 :   : 3

c. 1 : 1 : 2

d. 4 : 2 : 1

20. For the construction of thin R.C.C. structures, the type of cement to be avoided, is

a. rapid hardening cement

b. low heat cement

c. blast furnace slag cement

d. sulphate resisting cement.

21. The dimensions of a 35 litre forma for measuring aggregates by volume, are :

a. length 39 cm, breadth 25 cm, height 32 cm

b. length 27 cm, breadth 27 cm, height 48 cm

c. length 220 cm, breadth 25 cm, height 40 cm

d. none of these.

22. Di-calcium silicate (C2S)

a. generates less heat of hydration

b. hardens rapidly

c. provides less ultimate strength to cement

d. has less resistance to sulphate attack.

23. If the slump of a concrete mix is 60 mm, its workability is

a. low

b. medium

c. high

d. none of these.

24. A soil sample of mass specific gravity 1.92, has a moisture content 30%. If the specific gravity of solids is 2.75, the degree of saturation, is

a. 95.4%

b. 95.5%

c. 95.6%

d. 95.7%

25. The maximum water content at which a reduction in water content does not cause a decrease in volume of a soil mass, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

26. Negative skin friction on piles

a. is caused due to relative settlement of the soil

b. is caused in soft clays

c. decreases the pile capacity

d. all of the above.

27. Pick up the correct statement from the following:

a. The permeability of the coarse-grained soils may be reduced by grouting.

b. The process of injecting fluids (i.e. grouts) into the pores space of the soil, is called grouting.

c. The grouting increases the soil strength.

d. All the above.

28. For determining the moisture content of a soil sample, the following data is available Weight of container = 260 g, Weight of soil sample and = 320 g container, Weight of soil sample (dried) and = 310 g container. The moisture content of the soil sample, is

a. 15%

b. 18%

c. 20%

d. 25%

29. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

30. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

31. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

32. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

33. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

34. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

35. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

36. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

37. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

38. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

39. Referring of given figure, pick up the correct statement from the following:

a. The total length of centre line of four walls is 20 m

b. Length of long wall out-to-out is 6.80 m

c. Length of short walls in-to-in is 3.20 m

d. All the above.

40. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

41. The purpose of making the drawing board with seasoned soft wood

a. easy to carry

b. for fixing of drawing paper with the help of drawing pin

c. to match with T-square

d. all of the above

42. Axonometric drawing is a drawing like

a. parallel to the plane

b. perpendicular to the plane

c. inclined to the plane

d. all of the above

43. To draw very light line which pencil is used

a. B

b. HB

c. H

d. 2H

44. The main purpose of leader line is

a. show dimension

b. to connect a note with the feature

c. to show beauty

d. to show hatch

45. Current assests less inventories divided by current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

46. In a cash-flow diagram :

a. Time 1 is considered to be the end of time period 1

b. A vertical arrow pointing up indicates a positive cash flow

c. An arrow pointing downward indicates a negative cash flow

d. All of these

47. The sunk costs include :

a. a past expenditure

b. an unrecovered balance

c. an invested capital that cannot be retreived

d. All of these

48. Fundamental principle of Professional Engineering Ethics is:

a. uploading and advancing Engineering Profession

b. maintain high level of ethical conduct

c. both of the above

d. none of the above

49. Code of ethics should be

a. comprehensive and precise

b. measure of integrity perfomance

c. both a) and b) of above

d. none of the above

50. Which one type of membership is not categorized by NEC

a. general engineer

b. foreign engineer

c. professional engineer

d. supreintendent engineer