***Engineer***

1. There are two hinged semicircular arches A, B and C of radii 5 m, 7.5 m and 10 m respectively and each carries a concentrated load W at their crowns. The horizontal thrust at their supports will be in the ratio of

a. 1 : 1 1/2 : 2

b. 2 : 1 1/2 : 1

c. 1 : 1 : 2

d. none of these.

2. In plastic analysis, the shape factor for a triangular section, is

a. 1.5

b. 1.34

c. 2.34

d. 2.5

3. A material which obeys Hook's law, is subjected to direct stress σ0. At its elastic limit, the following statement is true,

a. Maximum shear stress =

b. Strain energy =

c. Shear strain energy =

d. All the above.

4. If the shear force along a section of a beam is zero, the bending moment at the section is

a. maximum

b. minimum

c. average of maximum-minimum

d. none of these.

5. A simply supported wooden beam 150 cm long and having a cross section 16 cm x 24 cm carries a concentrated load, at the centre. If the permissible stress ft = 75 kg/cm2 and fs = 10 kg/cm2 the safe load is

a. 3025 kg

b. 3050 kg

c. 3075 kg

d. 3100 kg.

6. An imaginary line joining the points of equal elevation on the surface of the earth, represents

a. contour gradient

b. contour line

c. level line

d. none of these.

7. When the bubble of the level tube of a level, remains central

a. line of sight is horizontal

b. axis of the telescope is horizontal

c. line of collimation is horizontal

d. geometrical axis of the telescope is horizontal.

8. For preparation of a contour plan for a route survey

a. method of squares is used

b. method of trace contour is used

c. method of cross profile is used

d. indirect method of contouring is used.

9. If + 0.8% grade meets - 0.7% grade and the rate of change of grade for 30 m distance is 0.05, the length of the vertical curve will be

a. 700 m

b. 800 m

c. 900 m

d. 1000 m

10. In geodetic surveys higher accuracy is achieved, if

a. curvature of the earth surface is ignored

b. curvature of the earth surface is taken into account

c. angles  between the curved lines are treated as plane angles

d. none of these.

11. Centering error of a theodolite produces an error

a. in all angles equally

b. which does not vary with the direction or pointing

c. which varies with the direction of pointing and inversely with the length of sight

d. none of these.

12. In a constant level tube, size of the bubble remains constant because upper wall is

a. of relatively larger radius

b. of relatively smaller radius

c. flat

d. convex downwards.

13. The rocks having alumina or clay as their major constituents, are known as

a. argillaceous rocks

b. calcareous rocks

c. sedimentary rocks

d. igneous rocks.

14. Water paint is a

a. colour wash

b. whiting

c. distemper

d. all the above.

15. The slag which floats on the surface of the molten iron generally contains

a. Silica (SiO2) 35%

b. Alumina (Al2O3) 12%

c. MgO, CaSO4, KMnO2 and FeO 8%

d. all the above.

16. For the manufacture of Portland cement, the proportions of raw materials used, are

a. lime 63% ; silica 22% ; other ingredients 15%

b. lime 22% ; silica 63% ; other ingredients 15%

c. silica 40% ; lime 40% ; other ingredients 20%

d. silica 70% ; lime 20% ; other ingredients 10%.

17. The hardest rock is

a. marble

b. diamond

c. talc

d. quartz.

18. Asphalt is obtained from

a. petroleum distillation

b. bitumen distillation

c. plastic distillation

d. none of these.

19. For road pavements, the cement generally used, is

a. rapid hardening cement

b. low heat cement

c. blast furnace slag cement

d. none of these.

20. Particles of 0.002 mm size are that of

a. clay

b. sand

c. gravel

d. none of these.

21. The maximum amount of dust which may be permitted in aggregates is

a. 5% of the total aggregates for low workability with a coarse grading

b. 10% of the total aggregates for low workability with a fine grading

c. 20% of the total aggregates for a mix having high workability with fine grading

d. all the above.

22. Concrete gains strength due to

a. chemical reaction of cement with sand and coarse aggregates

b. evaporation of water from concrete

c. hydration of cement

d. All the above.

23. The high strength of rapid hardening cement at early stage, is due to its

a. finer grinding

b. burning at high temperature

c. increased lime cement

d. higher content of tricalcium.

24. The maximum load carried by a pile, when it continues to sink without further increase of load, is known as

a. ultimate load carrying capacity

b. ultimate bearing capacity

c. ultimate bearing resistant

d. all the above.

25. The pressure that builds up in pore water due to load increment on the soil, is termed

a. excess pore pressure

b. excess hydrostatic pressure

c. hydrodynamic pressure

d. all the above.

26. Pick up the incorrect statement from the following:

a. Below the water table, the pore water may be static.

b. The hydrostatic pressure depends on the depth below the water level.

c. The attractive forces between the particles, caused due to negative pressure of water held above the water table is called soil suction.

d. None of these.

27. A compacted soil sample using 10% moisture content has a weight of 200 g and mass unit weight of 2.0 g/cm3. If the specific gravity of soil particles and water are 2.7 and 1.0, the degree of saturation of the soil is

a. 11.1%

b. 55.6%

c. 69.6%

d. none of these.

28. The Westergaard analysis is used for

a. sandy soils

b. cohesive soils

c. stratified soils

d. clayey soils.

29. Pick up the correct statement from the following:

a. In hydrometer method, weight Wd per ml of suspension is found directly

b. In pipette analysis, weight Wd per ml of suspension is found indirectly

c. In pipette analysis, weight Wd per ml of suspension is found directly

d. None of these.

30. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

31. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

32. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

33. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

34. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

35. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

36. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

37. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

38. The item of the brick structure measured in sq.m, is

a. Reinforced brick work

b. Broken glass coping

c. Brick edging

d. Brick work in arches.

39. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

40. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

41. The maps which are on sufficient large scale to enable the individual features shown on the map to be identified on the ground by their shape and position are called :

a. topographical maps

b. geographical maps

c. geological maps

d. site plan

42. The equipment mainly used to transfer the measurements is

a. scale

b. divider

c. compass

d. drawing ping

43. Dimension lines and projection lines are drawn as:

a. continuous thick lines

b. continuous thin line with zig zags

c. continuous thin line

d. continuous thin and wary lines

44. Which one is not drafting equipment ?

a. drafter

b. dust cloth

c. scale

d. None of the above

45. Keeping in view, the feasibility order of magnitude, the preliminary, conceptual or budget estimates, are prepared by :

a. construction manager

b. owner himself/herself

c. construction manager

d. None of these

46. The financial analysis :

a. helps a share holder to compare the expected return on his investment in the firm against the expected return from other alternative investment.

b. helps a bank to know the financial position of the firm for granting a loan to the firm.

c. helps to judge the success of the firm's financial plans.

d. All of these.

47. If P is principal amount, i is the rate of interest per annum and n is the number of periods in years, the compound amount factor (CAF) is :

a. (1 + i)n

b. (1 + i)(1/2n)

c. (n + i)

d. None of these

48. The variation exceeding 15 % of the contract amount is approved by

a. gazetted 2nd class

b. gazetted 1st class

c. deaprtment head

d. none of the above

49. The minimum qulification required for a general engineer is

a. diploma ine engg.

b. bachelor in engg.

c. master in engg.

d. all of the above

50. Prequalification is done

a. after the invitation of bids

b. before the invitation of bids

c. during the invitation of bids

d. none of the above