***Engineer***

1. The assumption in the theory of bending of beams, is :

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

2. The ratio of maximum and average shear stresses on a rectangular section, is

a. 1.25

b. 1.5

c. 2.0

d. 2.5

3. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

4. Strain energy of any member may be defined as work done on it

a. to deform it

b. to resist elongation

c. to resist shortening

d. all the above.

5. If all the dimensions of a bar are increased in the proportion n : 1, the proportion with which the maximum stress produced in the prismatic bar by its own weight, will increase in the ratio

a. n : 1

b. 1 :

c.  : 1

d. 1 : n.

6. The representation of general topography of a very flat terrain is possible only

a. by drawing contours at large interval

b. by drawing contours at small interval

c. by giving spot levels at large interval

d. by giving spot levels to salient features at close interval.

7. Pick up the correct statement from the following :

a. if the image of the object does not fall on the plane of the cross-lines, parallax exists

b. parallax has nothing to do with the eyepiece

c. the eyepiece is adjusted for clear vision of the cross hairs

d. all the above.

8. Orientation of a plane table by solving two point problem is only adopted when

a. saving of time is a main factor

b. better accuracy is a main factor

c. given points are inaccessible

d. none of these.

9. The bearing of line AB is 152° 30' and angle ABC measured clockwise is 124° 28'. The bearing of BC is

a. 27° 52'

b. 96° 58'

c. 148° 08'

d. 186° 58'

10. An imaginary line joining the points of equal elevation on the surface of the earth, represents

a. contour gradient

b. contour line

c. level line

d. none of these.

11. The bearings of the lines AB and BC are 146° 30' and 68° 30'. The included angle ABC is

a. 102°

b. 78°

c. 45°

d. none of these.

12. The branch of surveying in which both horizontal and vertical positions of a point, are determined by making instrumental observations, is known

a. tacheometry

b. tachemetry

c. telemetry

d. all the above.

13. Plywood is normally available

a. 1 mm thick

b. 2 mm thick

c. 2 to 3 mm thick

d. 3 mm to 4 mm thick.

14. Which one of the following is acid resistant asbestos:

a. amosite asbestos

b. anthophylite asbestos

c. crocidolite asbestos

d. All the above.

15. The colour of statuary marble used for sculptor's work, is

a. blue

b. white

c. green

d. yellow

16. Non acid-resistant asbestos is :

a. tremolite asbestos

b. chrysolite abestos

c. amosite absestos

d. none of these.

17. The variety of pig iron used for the manufacture of steel by Bessemer process, is

a. Bessemer pig

b. Grey pig

c. White forge pig

d. Mottled pig.

18. Refractory bricks resist

a. high temperature

b. chemical action

c. dampness

d. all the above.

19. If 20 kg of coarse aggregate is sieved through 80 mm, 40 mm, 20 mm, 10 mm, 4.75 mm, 2.36 mm, 1.18 mm, 600 micron, 300 micron and 150 micron standard sieves and the weights retained are 0 kg, 2 kg, 8 kg, 6 kg, 4 kg respectively, the fineness modulus of the aggregate, is

a. 7.35

b. 7.40

c. 7.45

d. none of these.

20. Gypsum is added for

a. colour

b. strength

c. controlling setting time

d. none of these.

21. For ensuring quality of concrete, use

a. single sized aggegates

b. two sized aggregate

c. graded aggregates

d. coarse aggregates.

22. According to Water-Cement Ratio Law, the strength of workable plastic concrete

a. depends upon the amount of water used in the mix

b. does not depend upon the quality of cement mixed with aggregates

c. does not depend upon the quanity of cement mixed with aggregates

d. all the above.

23. The high strength of rapid hardening cement at early stage, is due to its

a. finer grinding

b. burning at high temperature

c. increased lime cement

d. higher content of tricalcium.

24. The capillary rise of water

a. depends upon the force responsible

b. increases as the size of the soil particles increases

c. decreases as the size of the soil particles decreases

d. is less in wet soil than in dry soil.

25. Rankine's theory of active earth pressure assumes

a. ground surface is a plane which may be horizontal or inclined

b. back of the wall is vertical and smooth

c. wall yields about the base

d. all the above.

26. Fundamental relationship between dry density (γd), specific gravity (G), water content (ω) and percentage of air voids (na) is :

a. γd =

b. γd =

c. γd =

d. γd =

27. Pick up the in-correct statement from the following: The soils which contain montmorillonite minerals

a. shrink more when dry

b. possess high plasticity

c. possess high coefficient of internal coefficient

d. None of these.

28. The minimum depth of building foundations on

a. sandy soils is 80 cm to 100 cm

b. clay soils is 90 cm to 160 cm

c. rocky soils is 5 cm to 50 cm

d. all the above.

29. For a clay slope of height of 10 m, the stability number is 0.05, γ = 2.0 t/m3, C = 2.5 t/m2, the critical height of the slope of the soil, is

a. 4.0 m

b. 12.5 m

c. 25.0 m

d. 15.0 m

30. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

31. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

32. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

33. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

34. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

35. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

36. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

37. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

38. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

39. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

40. Due to change in price level, a revised estimate is prepared if the sanctioned estimate exceeds

a. 2.0%

b. 2.5%

c. 4.0%

d. 5.0%

41. The equipment mainly used to transfer the measurements is

a. scale

b. divider

c. compass

d. drawing ping

42. In case of aligned method of dimensioning, they can be readable form

a. bottoms and right side of drawing sheet

b. bottoms and left side of drawing sheet

c. bottoms and top side of drawing sheet

d. all of the above

43. Circles of small radii are drawn by means of a

a. straight bar

b. bow compass

c. lengthening bar

d. drop compass

44. The angle between two horizontal line is

a. 0°

b. 180°

c. both (a) and (b) of above

d. none of the above

45. Current assests less inventories divided by current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

46. The project contractor relies on the cost of the estimate :

a. for submission of a competitive bid for a lumpsum contract

b. for a unit price contract

c. for preparation of a definitive estimate to help negotiate contract.

d. All of these

47. The financial analysis helps to judge:

a. The operational efficiency of the firm

b. The financial position of the firm.

c. Both (a) and (b)

d. Neither (a) nor (b)

48. For the registiration of professional engineer amount fee required is:

a. NRS 5000/-

b. NRS 10000/-

c. NRS 15000/-

d. NRS 20000/-

49. The Engineering Council members consist of the following members now more

a. 12

b. 14

c. 16

d. 18

50. The user's committee method of construction is labour intensive with a cost estimate upto:

a. 4 million

b. 6 million

c. 8 million

d. 10 million