***Engineer***

1. By applying the static equations i.e. ΣH = 0, ΣV = 0 and ΣM = 0, to a determinate structure, we may determine

a. shear forces only

b. bending moments only

c. internal forces only

d. all the above.

2. There are two hinged semicircular arches A, B and C of radii 5 m, 7.5 m and 10 m respectively and each carries a concentrated load W at their crowns. The horizontal thrust at their supports will be in the ratio of

a. 1 : 1 1/2 : 2

b. 2 : 1 1/2 : 1

c. 1 : 1 : 2

d. none of these.

3. In plastic analysis, the shape factor for rectangular section, is

a. 1.4

b. 1.5

c. 1.6

d. 1.7

4. In a loaded beam, the point of con-traflexture occurs at a section where

a. bending moment is zero or changes sign

b. bending moment is maximum

c. shearing force is maximum

d. shearing force is minimum.

5. Reactions at the supports of a structure can be determined by equating the algebraic sum of

a. horizontal forces to zero

b. vertical forces to zero

c. moment about any point to zero

d. all the above.

6. The ratio of the linear displacement at the end of a line, subtended by an arc of one second to the length of the line, is

a. 1:206 300

b. 1:3440

c. 1:57

d. 1:100.

7. In precision theodolite traverse if included angles are read twice and the mean reading accepted using both verniers having a least count of 30". Assuming the instrument to be in perfect adjustment, linear measurements correct to 6 mm per 30 metre tape duly corrected for temperature, slope and sag, the angular error of closure not to exceed (where n is the number of traverse legs)

a. 50" n

b. 30" n

c. 60" n.

8. If α is the angle between the polar ray and the tangent at the point of commencement of a lemniscate curve, the equation of the curve, is

a. l = ksin 2α

b. l = ksin 3α

c. l = ktan 2α

d. l = kcos 2α

9. A lens or combination of lenses in which the following defect is completely eliminated is called aplanatic

a. spherical aberration

b. chromatic aberration

c. coma

d. astigmatism.

10. If θ is the slope of the ground and l is the measured distance, the correction is

a. 2l sin2 θ/2

b. 2l cos2 θ/2

c. 2l tan2 θ/2

d. 2l cot2 θ/2.

11. If the length of a transition curve to be introduced between a straight and a circular curve of radius 500 m is 90 m, the maximum perpendicular offset for the transition curve, is

a. 1.70 m

b. 2.70 m

c. 3.70 m

d. 4.70 m

12. The limiting length of an offset does not depend upon

a. accuracy of the work

b. method of setting out perpendiculars

c. scale of plotting

d. indefinite features to be surveyed.

13. Rapid hardening cement attains early strength due to

a. larger proportion of lime grounded finer than normal cement

b. lesser proportion of lime grounded coarser than normal cement

c. lesser proportion of lime grounded finer than normal cement

d. larger proportien of lime grounded coarser than normal cement

14. Pick up the correct statement from the following:

a. Horn blende mineral is brittle

b. Muscovite is also known as white mica and potashmica

c. Biotite is also known as blackmica

d. All the above.

15. Blister steel

a. is full of fissures and cavities

b. can not be forged

c. can be easily welded

d. all the above.

16. Soundness test of cement determines

a. quality of free lime

b. ultimate strength

c. durability

d. initial setting

17. Bitumen completely dissolves in

a. chloroform

b. benzol

c. coaltar

d. All of these.

18. The cracks caused by shrinkage of the exterior surface of the wood exposed to atmosphere, are called:

a. radial shakes

b. heart shakes

c. wind cracks

d. twisted fibres.

19. The bulk density of aggregates, depends upon

a. shape

b. grading

c. compaction

d. all the above.

20. Concrete gains strength due to

a. chemical reaction of cement with sand and coarse aggregates

b. evaporation of water from concrete

c. hydration of cement

d. All the above.

21. If P, Y and Z are the weights of cement, fine aggregates and coarse aggregates respetively and W/C is the water cement ratio, the minimum quantity of water to be added to first batch, is obtained by the equation

a. 0.3P + 0.1Y + 0.01Z = W/C x P

b. 0.4P + 0.2Y + 0.01Z = W/C x P

c. 0.5P + 0.3Y + 0.01Z = W/C x P

d. 0.2P + 0.5Y + 0.1Z = W/C x P

22. For construction of structures in sea water, the cement generally preferred to, is

a. Portland-pozzolana cement

b. quick setting cement

c. low heat Portland cement

d. rapid hardening cement

23. Proper batching ensures

a. durability

b. workability

c. strength

d. all the above.

24. During seepage through a soil, direction of seepage is always

a. parallel to equipotential lines

b. perpendicular to stream lines

c. perpendicular to equipotential lines

d. none of these.

25. Si particles

a. show dilatancy

b. swell when moist

c. possess high strength when dry

d. disintegrate easily.

26. Plasticity index is defined as the range of water content between

a. liquid and plastic limit

b. plastic limit and semi solid limit

c. semi-solid limit and liquid limit

d. liquid limit and solid limit.

27. The direct shear test suffers from the following disadvantage:

a. Pore water pressure cannot be measured

b. Shear stress on the failure plane is not uniform.

c. The area under the shear and vertical loads does not remain constant throughout the test

d. All the above.

28. Pick up the correct statement from the following:

a. The ratio of shear strength in natural state to the remoulded shear strength under undrained conditions, is called degree of sensitivity.

b. The difference between the undisturbed shear strength and remoulded shear strength is known remoulding loss

c. The tendency of dense sand to expand on application of shearing load, is known as dilatancy

d. All the above.

29. A coarse-grained soil has a voids ratio 0.75, and specific gravity as 2.75. The critical gradient at which quick sand condition occurs, is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

30. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

31. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

32. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

33. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

34. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

35. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

36. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

37. The rate of payment is made for 100 cu m (per % cu m) in case of

a. Rock cutting

b. Excavation in trenches for foundation

c. Earth work in filling the plinth

d. all the above.

38. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

39. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

40. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

41. Set squares are not used to draw

a. straight line

b. vertical line with T-square

c. horizontal line

d. all of the above

42. The proportion by which the actual size of an object is enlarged or reduced in drawing is simply known as

a. multiplication factor Dil

b. scale

c. reflection factor

d. none of the above

43. As built drawing is normally constructed................the construction.

a. before construction

b. simultaneously with construction

c. after construction

d. all of the above

44. In the first angle method of orthographic projection

a. the object comes between the observer and the plane

b. the plane comes between the object and the observer

c. the observer comes between the object and the plane

d. none of the above

45. Pick up the main purpose of project cost control from the following :

a. To signal immediate warning of uneconomic operations

b. To provide a feed back to the estimator

c. To promote cost consciousness

d. All of these

46. Present worth Annuity (PWA) is generally known as

a. Premium annuities

b. Income annuities

c. Future annuities

d. All of these

47. Pick up the correct statement from the following:

a. Ratio analysis is the procedure of determining and interpreting numerical relationship of various items of the financial statement.

b. All financial ratios are obtained by relating two sets of information contained in a Single financial statement.

c. The relationship between two accounting figures expressed mathematically, is known as a financial ratio.

d. All of these

48. Integrity is

a. state or quality of being complete, undivided or unbroken moral soundness, honesty & uprightness

b. the wholeness of deriving quality

c. both a) and b) of above

d. none of the above

49. NEC publishes the name of registered engineers each year as well as every forth year. The name of foreigner engineers publish

a. each year

b. every forth year

c. both (a) and (b)

d. none of the above

50. Five members are elected from Nepal Engineers Association members to represent in Nepal Engineering Council every:

a. 2 years

b. 3 years

c. 4years

d. 5years