***Engineer***

1. Stress may be defined as

a. force per unit length

b. force per unit volume

c. force per unit area

d. none of these.

2. A steel bar 5 m x 50 mm is loaded with 250, 000 N. If the modulus of elasticity of the material is 0.2 MN/mm2 and Poisson's ratio is 0.25, the change in the volume of the bar is :

a. 1.125 cm3

b. 2.125 cm3

c. 3.125 cm3

d. 4.125 cm2

3. Beams composed of more than one material, rigidly connected together so as to behave as one piece, are known as

a. Compound beams

b. Indeterminate beams

c. Determinate beams

d. Composite beams.

4. Hooke's law states that stress and strain are

a. directly proportional

b. inversely proportional

c. curvilinearly related

d. none of these.

5. Failure of riverted joints is due to

a. Tearing of plates between rivets

b. Shearing of rivets

c. Crushing of rivets

d. All the above.

6. Setting out a curve by two theodolite method, involves

a. linear measurements only

b. angular measurements only

c. both linear and angular measurements

d. none of these.

7. Tilt of the staff in stadia tacheometry increases the intercept if it is

a. away from the telescope pointing down hill

b. towards the telescope pointing up-hill

c. away from the telescope pointing up-hill

d. none of these.

8. For true difference in elevations between two points A and B, the level must be set up

a. at any point between A and B

b. at the exact mid point of A and B

c. near the point A

d. near the point B.

9. If the length of a transition curve to be introduced between a straight and a circular curve of radius 500 m is 90 m, the maximum deflection angle to locate its junction point, is

a. 1°43' 08"

b. 1°43' 18"

c. 1°43' 28"

d. 1°43' 38"

10. If the plane table is not horizontal in a direction at right angles to the alidade, the line of sight is parallel to the fiducial edge only for

a. horizontal sights

b. inclined sights upward

c. inclined sight downward

d. none of these.

11. In geodetic surveys higher accuracy is achieved, if

a. curvature of the earth surface is ignored

b. curvature of the earth surface is taken into account

c. angles  between the curved lines are treated as plane angles

d. none of these.

12. The operation of resection involves the following stepsrough orientation of the plane tablethe three lines form a triangle of errordrawing lines back through the three control pointsselect a point in the triangle of error such that each ray is equally rotated either clockwise or anti clockwisethe points obtained by three rays is the correct location. The correct sequence is

a. 1, 3, 2, 4, 5

b. 1, 2, 3, 4, 5

c. 1, 4, 3, 2, 5

d. 1, 4, 2, 3, 5

13. The main constituent of fly-ash, is

a. aluminium oxide

b. silica

c. ferrous oxide

d. All of these.

14. Pick up the plutonic rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

15. The most important constituent of varnish, is

a. drier

b. solvent

c. resin

d. all the above.

16. Pick up the correct statement from the following:

a. The commercial plywoods are available upto 150 cm wide and upto 300 cm long

b. The plywoods possess uniform tensile strength in all directions

c. The plywoods are light in weight

d. All the above.

17. The weight of a good quality brick when immersed in water for a period of 16 hours should not exceed the weight of dry brick

a. 20%

b. 15%

c. 10%

d. None of these.

18. For the manufacture of plywood, veneers are placed so that grains of adjacent veneers

a. run at right angles

b. parallel

c. inclined at 45°

d. inclined at 60°.

19. The aggregate impact value of the aggregate used in

a. building concrete is less than 45

b. road pavement concrete is less than 30

c. runway concrete is less than 30

d. all the above.

20. The high strength of rapid hardening cement at early stage, is due to its

a. finer grinding

b. burning at high temperature

c. increased lime cement

d. higher content of tricalcium.

21. Horizontal construction joints in concrete walls are generally provided at

a. soffit level

b. window sill level

c. floor level

d. all the above.

22. Saw dust can be rendered chemically inert by boiling it in water containing

a. ferrous sulphate

b. potassium chloride

c. ammonia

d. nitric acid

23. Workability improved by adding

a. foaming agent

b. oily-agent

c. aluminium compound

d. all the above.

24. A pile is being driven with a drop hammer weighing 1800 kg and having a free fall of 1.00 m. If the penetration with last blow is 5 mm, the load carrying capacity of the pile, -according to the Engineering News formula, is

a. 100 tonnes

b. 50 tonnes

c. 20 tonnes

d. 10 tonnes.

25. Pick up the correct statement from the following:

a. An unconfined compression test is a special case of triaxial compression test

b. An unconfined compression test is a special case of direct shear test

c. The confining pressure is maximum during an unconfined compression test

d. The cylindrical specimen of a soil is subjected to major principal stress till it fails due to shearing along the plane of the failure.

26. Pick up the correct statement from the following:

a. Isotropic consolidation of clay can be obtained in the triaxial apparatus under equal all-round pressure.

b. If the present effective stress is the maximum to which the clay has ever been subjected, it is called normally consolidated clay

c. If the present effective stress in the past was more than present effective stress, it is called over-consolidated clay

d. All the above.

27. The length/diameter ratio of cylindrical specimens used in triaxial test, is generally

a. 1.5

b. 2

c. 2.5

d. 3

28. The compression index of a soil

a. decreases with an increase in the liquid limit

b. increases with an increase in the liquid limit

c. decreases with an increase in the plastic limit

d. is not related with plastic limit.

29. Depending upon the properties of a material, the failure envelope may

a. be either straight or curved

b. pass through the origin of stress

c. intersect the shear stress axis

d. all the above.

30. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

31. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

32. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

33. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

34. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

35. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

36. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

37. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

38. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

39. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

40. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

41. The section obtained when the section plane is inclined to the axis of cone, but parallel to one of the generators of a cone is called ......... ......

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

42. Bisecting a line means, divide a line into..... equal part.

a. two

b. three

c. four

d. none of the above

43. The best drawing paper used for water colour is :

a. hand made paper (hand made drawing sheet)

b. tracing drawing sheet

c. ordinary drawing sheet

d. ammonia paper

44. The area occupied by Ao (drawing sheet) is ......... times the area occupied by A3 (drawing sheet).

a. 2

b. 4

c. 8

d. 16

45. Pick up the correct method adopted for developing the approximate or conceptual estimates from the following :

a. Cost per function method

b. Cost per square metre

c. Cost per linear unit

d. All of these

46. Pick up the correct statement from the following:

a. Uniform series present worth factor =

b. Sinking fund factor =

c. Capital recovery factor =  where letters carry their usual meanings.

d. All of these

47. The key to profitable operation for project cost control, is :

a. To keep the project cost equal to original cost estimate.

b. To keep the project cost equal to subsequent construction budget.

c. To keep the project cost within the cost budget and knowing when and where job costs are deviating.

d. None of these

48. What is the abbreviation used for German Technical Co-operation?

a. GTZ

b. GTC

c. GTCO

d. all of the above

49. Code of conduct defines

a. what should be

b. what should not be

c. what had been

d. what will be

50. The bid security shall be forfeited if

a. Bidder withdraws its bid during the period of bid validity specified by the bidder on the Bid form

b. in case of successful bidder, if the bidder fails within the specified time limit either sign the contract or furnish performance security

c. both (a) and (b) of above

d. none of the above