***Engineer***

1. A close coil helical spring when subjected to a moment M having its axis along the axis of the helix

a. it is subjected to pure bending

b. its mean diameter will decrease

c. its number of coils will increase

d. all the above.

2. The tangential component of stress on an plane inclined θ° to the direction of the force, may be obtained by multiplying the normal stress by

a. cos θ

b. tan θ

c. sin θ cos θ

d. sin2θ

3. Total strain energy theory for the failure of a material at elastic limit, is known

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

4. The rise of a parabolic arch at quarter points, is equal to

a.  times the rise of the crown

b.  times the rise of the crown

c.  times the rise of the crown

d.  times the rise of the crown.

5. Reactions at the supports of a structure can be determined by equating the algebraic sum of

a. horizontal forces to zero

b. vertical forces to zero

c. moment about any point to zero

d. all the above.

6. There are two stations A and B. Which of the following statements is correct :

a. the fore bearing of AB is AB

b. the back bearing of AB is BA

c. the fore and back bearings of AB differ by 180°

d. all the above.

7. The real image of an object formed by the objective, must lie

a. in the plane of cross hairs

b. at the centre of the telescope

c. at the optical centre of the eye-piece

d. anywhere inside the telescope.

8. A traverse deflection angle is

a. less than 90°

b. more than 90° but less than 180°

c. the difference between the included angle and 180°

d. the difference between 360° and the included angle.

9.  is the conventional sign of

a. temple

b. mosque

c. idgah

d. church

10. Removal of parallax, may be achieved by focussing

a. the objective

b. the eye-piece

c. the objective and the eye-piece

d. none of these.

11. In tangential tacheometry, an ordinary level staff is used

a. leaning towards the instrument for inclined sights upward

b. leaning away from the instrument for inclined sights downwards

c. vertical in all cases

d. none of these.

12. Accurate measurement of deflection angles with a transit not properly adjusted may be made by

a. setting the vernier A at zero at back station and then plunging the telescope

b. setting the vernier A at zero at back station and turning the instrument to the forward station

c. taking two back sights one with the telescope normal and the other with telescope inverted

d. none of these.

13. The most durable varnish is

a. spirit varnish

b. turpentine varnish

c. oil varnish

d. none of these.

14. Mastic asphalt is

a. acid resisting material

b. non-corrosive material

c. corrosive material

d. heating-resisting material.

15. Which one of the following is an air binding material ?

a. Gypsum

b. Acid-resistant cement

c. Quick lime

d. All of these.

16. The normal curing period for lime mortar, is:

a. 3 days

b. 7 days

c. 10 days

d. 14 days.

17. Based on flow quality, the sequence of pipes is

a. A.C. pipes, G.I. pipes, C.I. pipes, PVC pipes

b. C.I. pipes, G.I. pipes, A.C. pipes, PVC pipes

c. C.I. pipes, G.I. pipes, PVC pipes, A.C. pipes

d. None of these.

18. Bitumen in

a. solid state, is called asphalt

b. semi fluid state, is called mineral tar

c. fluid state, is called petroleum

d. all the above.

19. After casting, an ordinary cement concrete on drying

a. expands

b. mix

c. shrinks

d. none of these.

20. If 20 kg of coarse aggregate is sieved through 80 mm, 40 mm, 20 mm, 10 mm, 4.75 mm, 2.36 mm, 1.18 mm, 600 micron, 300 micron and 150 micron standard sieves and the weights retained are 0 kg, 2 kg, 8 kg, 6 kg, 4 kg respectively, the fineness modulus of the aggregate, is

a. 7.35

b. 7.40

c. 7.45

d. none of these.

21. If P, Y and Z are the weights of cement, fine aggregates and coarse aggregates respetively and W/C is the water cement ratio, the minimum quantity of water to be added to first batch, is obtained by the equation

a. 0.3P + 0.1Y + 0.01Z = W/C x P

b. 0.4P + 0.2Y + 0.01Z = W/C x P

c. 0.5P + 0.3Y + 0.01Z = W/C x P

d. 0.2P + 0.5Y + 0.1Z = W/C x P

22. Pozzolana cement is used with confidence for construction of

a. massive foundations

b. abutments

c. R.C.C. structures

d. all the above.

23. For the construction of the retaining structures, the type of concrete mix to be used, is

a. 1 : 2 :4

b. 1 :  : 3

c. 1 : l : 2

d. 1 : 4 : 8.

24. The property of a soil which permits water to percolate through it, is called

a. moisture content

b. permeability

c. capillarity

d. none of these.

25. A soil mass coated with a thin layer of paraffin weighs 460 g. When immersed, it displaces 299 cc of water. The weight of paraffin is 10 g. If specific gravity of solids is 2.5 and that of paraffin 0.9, the void ratio of soil, is

a. 0.55

b. 0.60

c. 0.65

d. 0.70

26. Pick up the correct statement applicable to plate load test

a. At the centre of the test pit, a hole is dug out whose size is kept equal to the size of the test plate

b. Bottom level of the hole dug at the centre of the test pit, is kept at the level of the actual formation

c. Ratio of the depth of the hole to the width of the test plate is the same as the ratio of the depth of the actual formation to the width of the formation

d. All the above.

27. In active state of plastic equilibrium in a non cohesive soil with horizontal ground surface

a. major principal stress is horizontal

b. minor principal stress is vertical

c. major principal stress is vertical

d. minor and major principal stresses are equally inclined to horizontal.

28. The compression resulting from a long term static load and consequent escape of pore water, is known as

a. compaction

b. consolidation

c. swelling

d. none of these.

29. The triaxial apparatus is usually used for

a. unconsolidated-undrained test

b. consolidated-undrained test

c. drained test

d. all the above tests.

30. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

31. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

32. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

33. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

34. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

35. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

36. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

37. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

38. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

39. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

40. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

41. The main objective of writing letters/alphabets on the drawing is to make the drawing

a. more fantastic

b. more informative

c. more expensive

d. all of the above

42. Which of the following regarding scales in drawing is NOT true?

a. the name of the scale (e.g. scale 1:10) should be mentioned below the scale

b. a diagonal scale consists of a primary scale and a venirer

c. comparative scales may be plain or diagonal

d. when an unusual scale is used it is constructed on the drawing sheet

43. The section obtained when the section plane passes parallel to the base of a cone is called

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

44. Angles in multiples of 15 degree are constructed by the combined use of

a. T-square and set-square

b. set-square and scale

c. scale and french curve

d. french curve and erasing shield

45. Present worth Annuity (PWA) is generally known as

a. Premium annuities

b. Income annuities

c. Future annuities

d. All of these

46. The more critical (or severe) test of the firm's liquidity can be judged by :

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

47. Pick up the ratio which gives us sufficient information by which to judge the financial condition and performance of the firm, from the following:

a. Financial leverage ratio

b. Activity ratio

c. Portability

d. None of these

48. Procurement of services means:

a. buying logistics

b. hiring consultant

c. renting house

d. none of the above

49. In Nepal, which of the following act contains the provisions on health and safety of the workers?

a. contract act

b. industry act

c. labour act

d. all of the above

50. To work for survey, design estimate & supervision of a road project, the consultant has to submit

a. technical proposal

b. financial proposal

c. technical and financial proposal

d. none of the above