***Engineer***

1. Pick up the correct statement from the following:

a. For I sections, the shear centre coincides with the centroid of the cross section of the beam

b. For channels, the shear centre does not concide its centroid

c. Bending loads should pass through the shear centre to avoid twisting

d. All the above.

2. Stress may be expressed in Newtons

a. per millimetre square (N/mm2)

b. per centimetre square (N/cm2)

c. per metre square (N/m2)

d. All of the above.

3. In plastic analysis, the shape factor for a circular section, is

a. 1.5

b. 1.6

c. 1.7

d. 1.75

4. An arch with three hinges, is a structure

a. statically determinate

b. statically indeterminate

c. geometrically unstable

d. structurally sound but indeterminate

5. Strain energy of a member may be equated to

a. average resistance x displacement

b.  stress x strain x area of its cross-section

c.  stress x strain x volume of the member

d.  (stress)2 x volume of the member + Young's modulus E.

6. Number of subdivisions per metre length of a levelling staff is

a. 100

b. 200

c. 500

d. 1000

7. Keeping the instrument height as 1.5 m, length of staff 4 m, the slope of the ground as 1 in 10, the sight on the down-slope, must be less than

a. 30 m

b. 25 m

c. 20 m

d. 15 m

8. For a closed traverse the omitted measurements may be calculated

a. bearing of one side only

b. both length and bearing of one side

c. length or bearing of adjacent side

d. all the above.

9. Centering error of a theodolite produces an error

a. in all angles equally

b. which does not vary with the direction or pointing

c. which varies with the direction of pointing and inversely with the length of sight

d. none of these.

10. The staff reading at a distance of 80 m from a level with the bubble at its centre is 1.31 m. When the bubble is moved by 5 divisions out of the centre, the reading is 1.39 m. The angular value of the one division of the bubble, is

a. 41.25 secs

b. 14.52 secs

c. 25.05

d. none of these

11. A clinometer is used for

a. measuring angle of slope

b. correcting line of collimation

c. setting out right angles

d. defining natural features.

12. Systematic errors are those errors

a. which cannot be recognised

b. whose character is understood

c. whose effects are cumulative and can be eliminated

d. none of these.

13. Pick up the compound responsible for early strength of cement from the following:

a. Tetra-calcium alumino-ferrite

b. Tricalcium silicate

c. Tricalcium aluminate

d. Dicalcium silicate.

14. Seasoning is

a. a process of removing sap

b. creosoting

c. painting with sodium silicate

d. coating with tar.

15. The commonly used thinner in oil paints, is

a. turpentine

b. both (a) and (b)

c. neither (a) nor (b)

d. none the these.

16. Seasoning of timber is essential to remove

a. knots from timber

b. sap from timber

c. twisted fibre from timber

d. roughness of timber.

17. The frog of a brick is normally made on its

a. top face

b. bottom face

c. longer face

d. shorter side.

18. Invar contains

a. 12% of nickel

b. 24% of nickel

c. 30% to nickel

d. 36% of nickel.

19. According to IS : 382-1963, a good aggregate should be

a. chemically inert

b. sufficiently strong

c. hard and durable

d. all the above.

20. To obtain cement dry powder, lime stones and shales or their slurry, is burnt in a rotary kiln at a temperature between

a. 1200° and 1300°C

b. 1300° and 1400°C

c. 1400° and 1500°C

d. 1500° and 1600°C

21. Pick up the correct proportions of chemical ingredients of cement

a. Lime : Silica : Alumina : Iron oxide : 63 : 22 : 6 : 3

b. Silica : Lime : Alumina : Iron oxide : 63 : 22 : 6 : 3

c. Alumina : Silica : Lime : Iron oxide : 63 : 22 : 6 : 3

d. Iron oxide : Alumina : Silica : Lime : 63 : 22 : 6 : 3

22. While compacting the concrete by a mechanical vibrator, the slump should not exceed

a. 5.0 cm

b. 7.5 cm

c. 10 cm

d. 15 cm

23. For a concrete mix 1:3:6 and water cement ratio 0.6 both by weight, the quantity of water required per bag, is

a. 12 kg

b. 14 kg

c. 16 kg

d. 20 kg

24. Pick up the correct statement from the following:

a. A soil classification system is meant to provide an accepted and systematic method of describing the various types of soils eliminating personal factors

b. The first category of soil classification is based on grain size of the soil

c. The second category of soil classification is based on fine as well as coarse grains

d. All the above.

25. A plane inclined at an angle φ to the horizontal at which the soil is expected to stay in the absence of any lateral support, is known as

a. natural slope line

b. repose line

c. the φ line

d. all the above.

26. If the back fill is having a uniform surcharge of intensity q per unit area, the lateral pressure will be

a. q times the lateral pressure within the surface

b. 1/q times the lateral pressure within the surface

c. equal to a fill of height Z equal to q/r, where r is the density of the backfill

d. none of these.

27. Pick up the correct statement applicable to plate load test

a. At the centre of the test pit, a hole is dug out whose size is kept equal to the size of the test plate

b. Bottom level of the hole dug at the centre of the test pit, is kept at the level of the actual formation

c. Ratio of the depth of the hole to the width of the test plate is the same as the ratio of the depth of the actual formation to the width of the formation

d. All the above.

28. The maximum possible value nf dry density is referred to as

a. dry density

b. zero air voids

c. saturation dry density

d. all the above.

29. Buoyant unit weight equals the saturated density

a. multiplied by unit weight of water

b. divided by unit weight of water

c. plus unit weight of water

d. minus unit weight of water.

30. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

31. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

32. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

33. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

34. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

35. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

36. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

37. Pick up the item whose weight is added to the weight of respective item, is

a. Brackets

b. Bolts

c. Distance separators

d. All the above.

38. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

39. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

40. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

41. Drawing board is made of

a. well seasoned soft wood

b. well seasoned hard wood

c. moisted soft wood

d. moisted hard wood

42. In case of aligned method of dimensioning, they can be readable form

a. bottoms and right side of drawing sheet

b. bottoms and left side of drawing sheet

c. bottoms and top side of drawing sheet

d. all of the above

43. Working drawing space on the paper is determined by

a. working line

b. drawing line

c. border line

d. deader line

44. The solid which is generated by the revolution of a right angle triangle about one of its perpendicular sides which remains fixed is called ....

a. a prism

b. a pyramid

c. a cylinder

d. a cone

45. Pick up the correct statement from the following:

a. Ratio analysis is the procedure of determining and interpreting numerical relationship of various items of the financial statement.

b. All financial ratios are obtained by relating two sets of information contained in a Single financial statement.

c. The relationship between two accounting figures expressed mathematically, is known as a financial ratio.

d. All of these

46. Pick up the ratio which gives us sufficient information by which to judge the financial condition and performance of the firm, from the following:

a. Financial leverage ratio

b. Activity ratio

c. Portability

d. None of these

47. Pick up the correct statement from the following:

a. Engineering economy is a collection of mathematical techniques which simplify economic comparisons

b. Engineering economy is a decision assistance tool by which one method will be chosen as the most economically one.

c. For understanding the engineering economy, one should be able to classify the basic terminology and fundamental concepts of economy.

d. All of these.

48. For the registration of general engineer tha amount of fee required is:

a. NRS 2000/-

b. NRS 3000/-

c. NRS 3500/-

d. NRS 4000/-

49. In Nepal, which of the following Act contains the provisions o health and safety of the workers ?

a. contract act

b. Industry act

c. labour act

d. all of the above

50. NEC Regulation, 2057 is effective from

a. 2057/3/8

b. 2057/4/8

c. 2057/5/8

d. 2057/6/8