***Engineer***

1. The general expression for the B.M. of a beam of length l is the beam carries

a. a uniformly distributed load w/unit length

b. a load varying linearly from zero at one end to w at the other end

c. an isolated load at mid span

d. none of these.

2. A three hinged arch is generally hinged at its supports and

a. at one quarter span

b. at the crown

c. any where in the rib

d. none of these.

3. If a concrete column 200 x 200 mm in cross-section is reinforced with four steel bars of 1200 mm2 total cross-sectional area. Calculate the safe load for the column if permissible stress in concrete is 5 N/mm2 and Es is 15 Ec

a. 274 MN

b. 284 MN

c. 294 MN

d. None of these.

4. If a rectangular beam measuring 10 x 18 x 400 cm carries a unformly distributed load such that the bending stress developed is 100 kg/cm2. The intensity of the load per metre length, is

a. 250 kg

b. 260 kg

c. 270 kg

d. 280 kg.

5. A simply supported beam (l + 2a) with equal overhangs (a) carries a uniformly distributed load over the whole length, the B.M. changes sign if

a. l > 2a

b. l < 2a

c. l = 2a

d. l = 4a

6. If the radius of a simple curve is R, the length of the chord for calculating offsets by the method of chords produced, should not exceed.

a. R/10

b. R/15

c. R/20

d. R/25.

7. A sewer is laid from a manhole A to a manhole B, 250 m away along a gradient of 1 in 125. If the reduced level of the invert at A is 205.75 m and the height of the boning rod is 3 m, the reduced level of the sight rail at B, is

a. 208.75 m

b. 202.75 m

c. 206.75 m

d. 211.75 m

8. If d is the distance between equidistant odd ordinates, the Simpson's rule for the areas, is

a. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

b. [h1 + hn x 2(h2 + h4 + ... + hn - 1) + 4(h3 + h5 + ... + hn - 2)]

c. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

d. none of these.

9. The sag of 50 m tape weighing 4 kg under 5 kg tension is roughly

a. 0.053 m

b. 0.063 m

c. 0.073 m

d. 0.083 m

10. The ratio of the distances at which a stated length can be distinguished by the telescope and the human eye, respectively, is called

a. brightness of telescope

b. magnification of telescope

c. resolving power of telescope

d. none of these.

11. The intercept of a staff

a. is maximum if the staff is held truly normal to the line of sight.

b. is minimum if the staff is held truly normal to the line of sight.

c. decreases if the staff is tilted away from normal

d. increases if the staff is tilted towards normal.

12. The least count of a vernier scale is

a. sum of the smallest divisions of main and vernier scales

b. value of one division of the primary scale divided by total number of divisions of vernier scale

c. value of one division of vernier scale divided by total number of divisions of primary scale

d. none of these.

13. The initial setting time of lime-pozzolana, is

a. 30 minutes

b. 60 minutes

c. 90 minutes

d. 120 minutes.

14. Generally wooden moulds are made from

a. shisham wood

b. deodar wood

c. teak wood

d. hard wood.

15. The silica is used for preparing

a. silica bricks

b. coke oven

c. lining for glass furnaces

d. all of these.

16. In order of increasing percentage of silica, the correct sequence is

a. sandy clay, calcareous clay, pure clay

b. calcareous clay, pure clay, sandy clay

c. pure clay, sandy clay, calcareous clay

d. None of these.

17. Stuco paints are suitable for

a. stone masonry

b. brick walls

c. both (a) and (b)

d. neither (a) nor (b).

18. Refractory bricks are

a. neutral refractory bricks

b. acid refractory bricks

c. basic refractory bricks

d. all the above.

19. If the effective plan area of a warehouse is 54 sq. m, and maximum height of piles permitted is 270 cm, the number of cement bags to be stored, is

a. 2200 bags

b. 2400 bags

c. 2700 bags

d. 3000 bags

20. Internal friction between the ingredients of concrete, is decreased by using

a. fine aggregates

b. rich mix

c. more water and coarse aggregates

d. none of these.

21. The commonly used material in the manufacture of cement is

a. sand stone

b. slate

c. lime stone

d. graphite.

22. Pick up the correct statement from the following:

a. Water cement paste hardens due to hydration

b. During hardening cement binds the aggregates together

c. Cement provides strength, durability and water tightness to the concrete

d. All the above.

23. M 150 grade of concrete approximates

a. 1 : 1 :2 mix

b. 1 : 2 : 4 mix

c. 1 : 1.5 : 3 mix

d. none of these.

24. A soil not fully consolidated under the existing over-burden pressure, is called

a. pre-consolidated

b. normally consolidated

c. under-consolidated

d. none of these.

25. The ratio of settlement at any time 't' to the final settlement, is known as

a. co-efficient of consolidation

b. degree of consolidation

c. consolidation index

d. consolidation of undisturbed soil.

26. Pick up the correct statement from the following :

a. Liquid limit minus plastic limit, is known as plasticity index of the soil

b. Plastic limit minus shrinkage limit, is known as shrinkage index of the soil

c. The ratio of the plasticity index to the flow limit, is known as toughness index of the soil

d. All the above.

27. If drainage is permitted throughout the test, during the application of both normal, and shear stresses so that full consolidation occurs and no excess pore pressure is set up at any stage of the test, is known as

a. quick test

b. drained test

c. consolidated undrained test

d. none of these.

28. The earth pressure of a soil at rest, is proportional to (where φ is the angle of internal friction of the soil)

a. tan (45° + φ)

b. tan2 (45° - φ)

c. tan2 (45° + φ)

d. none of these

29. A partially saturated soil is classified as

a. one phase soil

b. two phase soil

c. three phase soil

d. four phase soil.

30. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

31. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

32. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

33. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

34. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

35. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

36. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

37. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

38. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

39. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

40. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

41. The most suitable angle for the cone of the vision for drawing perspective is

a. 60°

b. 45°

c. 30°

d. 150

42. The two parts of T-square are....... and .......

a. vertical and horizontal edge

b. straight and slant edge

c. stock and blade

d. blade and batten

43. The point in space where parallel lines meet are called

a. station point

b. vanishing point

c. origin

d. all of the above

44. The maps which are on sufficient large scale to enable the individual features shown on the map to be identified on the ground by their shape and position are called :

a. topographical maps

b. geographical maps

c. geological maps

d. site plan

45. The owner of the construction company makes use of the estimate :

a. to assist in financial arrangements

b. to determine economic feasibility of the project.

c. to determine the tax, insurance and evaluation purpose.

d. All of these

46. The ratio obtained by dividing 'quick assests' by current liabilities is called

a. Turnover ratio

b. Acid test ratio

c. Solvency ratio

d. None of these.

47. Pick up the correct statement from the following:

a. A NPV profile graph shows the curvilinear relationship between the net present value of the project and discount rate employed.

b. In a NPV profile, if discount rate is zero, then net present value is simply total cash inflows less the total cash outflows of the project.

c. As the discount rate increases, the net present value profile slopes downward to the right.

d. All of these

48. Code of conduct is not related with

a. making the advice of the enginerr impartial

b. building code

c. ensuring the quality of advice

d. making the advice untained by any personal consideration

49. For procurement of works up to 2 million, buy which of the following method is to be considered?

a. by sealed tender

b. by direct negotiation

c. by bidding

d. by direct labour or direct negotiation

50. Fundamental principle of Professional Engineering Ethics is:

a. uploading and advancing Engineering Profession

b. maintain high level of ethical conduct

c. both of the above

d. none of the above