***Engineer***

1. A three hinged arch is generally hinged at its supports and

a. at one quarter span

b. at the crown

c. any where in the rib

d. none of these.

2. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

3. A steel bar 20 mm in diameter simply-supported at its ends over a total span of 40 cm carries a load at its centre. If the maximum stress induced in the bar is limited to N/mm2, the bending strain energy stored in the bar, is

a. 411 N mm

b. 511 N mm

c. 611 Nmm

d. 711 N mm

4. The materials which have the same elastic properties in all directions, are called

a. isotropic

b. brittle

c. homogeneous

d. hard.

5. A cylinder is said to be thin if the ratio of its thickness and diameter, is less than

a. 1/20

b. 1/15

c. 1/10

d. 1/5

6. Ramsden eye-piece consists of

a. two convex lenses short distance apart

b. two concave lenses short distance apart

c. one convex lens and one concave lens short distance apart

d. two plano-convex lenses short distance apart, with the convex surfaces facing each other.

7. During secular variation of magnetic meridian at different places

a. range of oscillations is constant

b. period of oscillation is constant

c. range and period of oscillation both vary

d. period of oscillation only varies.

8. If the declination of the needle is 10° W

a. each of the whole circle reckoning has to be micros by 10°

b. in the quadrantal method, the correction is positive in the 1st and 3rd quadrants

c. in the quadrantal method, the corrections is negative in 2nd and 4th quadrants

d. all the above.

9. Closed contours of decreasing values towards their centre, represent

a. a hill

b. a depression

c. a saddle or pass

d. a river bed.

10. Check lines (or proof lines) in Chain Surveying, are essentially required

a. to plot the chain lines

b. to plot the offsets

c. to indicate the accuracy of the survey work

d. to increase the out-turn

11. Diopter is the power of a lens having a focal length of

a. 50 cm

b. 75 cm

c. 100 cm

d. 125 cm

12. Correction per chain length of 100 links along a slope of α radians, is

a. 100 α2

b. 100 α

c. 100 α3

d. 100 α-1.

13. For the manufacture of Portland cement, the proportions of raw materials used, are

a. lime 63% ; silica 22% ; other ingredients 15%

b. lime 22% ; silica 63% ; other ingredients 15%

c. silica 40% ; lime 40% ; other ingredients 20%

d. silica 70% ; lime 20% ; other ingredients 10%.

14. The rocks which are formed due to cooling of magma at a relatively shallow depth from the earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Ignoeous rocks.

15. Shingle is

a. decomposed laterite

b. crushed granite

c. water bound pebbles

d. air weathered rock.

16. The weight of a good quality brick when immersed in water for a period of 16 hours should not exceed the weight of dry brick

a. 20%

b. 15%

c. 10%

d. None of these.

17. The rocks which are formed due to cooling of magma at a considerable depth from earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Igneous rocks.

18. Pick up the correct characteristic of Pyroxene from the following:

a. It converts to chlorine by hydration

b. Its density is 2.3 to 3.6 g/cm2

c. Its hardness is 5 to 6

d. All the above.

19. Pick up the correct statement from the following:

a. According to the petrological characteristics, concrete aggregates are classified as heavy weight, normal weight and light weight

b. According to the shape of the particles, concrete aggregates are classified as rounded irregular, angular and flaky

c. According to the surface texture of the particles, the concrete aggregates are classified as glassy, smooth, granular, rough, crystalline, honey combed and porous

d. All the above.

20. Pick up the correct statement from the following:

a. The specific gravity of aggregate is important for the determination of the moisture content

b. The absorption and porosity of an aggregate influence the property of the concrete

c. A highly absorptive aggregate reduces the workability of concrete considerably

d. All the above.

21. Allowable shear strength of concrete, depends upon

a. shear strength

b. tensile strength

c. compressive strength

d. none of these.

22. The rock which is not calcareous, is :

a. macl

b. chalk

c. laterite

d. none of these.

23. An excess of flaky particles in concrete aggregates

a. increases the quantity of water and sand

b. affects the durability of concrete

c. more than 15% are not desirable

d. all the above.

24. Pick up the correct statement from the following:

a. The percent settlement at any time is called degree of consolidation

b. Time factor is a dimensionless quantity

c. The initial curve on either side of the point of unloading and reloading is called 'virgin curve'

d. All the above.

25. A triaxial shear test is prefered to direct shear test, because

a. it can be performed under all three driange conditions with complete control

b. precise measurement of pore pressure and change in volume during test, is not posible

c. stress distribution on the failure plane, is non uniform

d. none of these.

26. If dry density, water density and specific gravity of solids of a given soil sample are 1.6 g/cc, 1.84 g/cc and 2.56 respectively, the porosity of the soil sample, is

a. 0.375

b. 0.370

c. 0.380

d. 0.390

27. The passive earth pressure of a soil, is proportional to (where φ is the angle of internal friction of the soil.)

a. tan (45° - φ)

b. tan (45° + φ)

c. tan2 (45° - φ

d. tan2 (45° + φ)

28. For a clay slope of height of 10 m, the stability number is 0.05, γ = 2.0 t/m3, C = 2.5 t/m2, the critical height of the slope of the soil, is

a. 4.0 m

b. 12.5 m

c. 25.0 m

d. 15.0 m

29. Darcy's law is applicable to seepage if a soil is

a. homogeneous

b. isotropic

c. incompressible

d. all the above.

30. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

31. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

32. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

33. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

34. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

35. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

36. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

37. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

38. Pick up the item whose weight is added to the weight of respective item, is

a. Brackets

b. Bolts

c. Distance separators

d. All the above.

39. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

40. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

41. The edge of drawing board where T-square is made to slide is called

a. sliding edge

b. fixing edge

c. working edge

d. all of the above

42. Free hand sketch is normally used for

a. concept

b. final

c. discussion

d. none of the above

43. The selection of suitable scale and allotment of proper space for margin, title block, revision panel, folding marks, etc. on the drawing sheet is known as:

a. formatting

b. layout

c. folding

d. centering

44. The most of the texture can be drawn with the help of drawing pen with line thickness

a. 0.1mm

b. 0.5mm

c. 0.8mm

d. 1.0mm

45. Both architect and engineer make use of the cost estimate of the project:

a. for site selection

b. for designing of the project

c. for choosing alternatives

d. All of these

46. Which one of the following statements is correct?

a. The number of years required to recover the initial cash investment in a project, is called Pay Back period (PBP).

b. The discount rate that equates the present value of the expected Net Cash Flows (CFs) with the Initial Cash Outflow (ICO) is known as internal rate of return.

c. The present value of the proposal's net cash flows, less the proposal's initial cash outflow is known as the Net Present Value (NPV)

d. All of these

47. Pick up the correct statement from the following:

a. An annuity is a series of equal payments occurring at equal period of time.

b. Annuity is called an equal payment or uniform payment series.

c. An annuity may have periods of time of any length but should always be of equal length.

d. All the above.

48. Who is the Chairperson of Project Co-ordination Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer

49. NEC publishes the name of registered engineers each year as well as every forth year. The name of foreigner engineers publish

a. each year

b. every forth year

c. both (a) and (b)

d. none of the above

50. "Fraudulent practice” means

a. offering anything to influence the action of public official

b. buy more than one bid document

c. mis representation of facts in order to influence a procurement process

d. all of the above