***Engineer***

1. A close coil helical spring when subjected to a moment M having its axis along the axis of the helix

a. it is subjected to pure bending

b. its mean diameter will decrease

c. its number of coils will increase

d. all the above.

2. A steel rod 1 metre long having square cross section is pulled under a tensile load of 8 tonnes. The extension in the rod was 1 mm only. If Esteel = 2 x 106 kg/cm2, the side of the rod, is

a. 1 cm

b. 1.5 cm

c. 2 cm

d. 2.5 cm

3. The shape factor of standard rolled beam section varies from

a. 1.10 to 1.20

b. 1.20 to 1.30

c. 1.30 to 1.40

d. 1.40 to 1.50

4. As the elastic limit reaches, tensile strain

a. increases more rapidly

b. decreases more rapidly

c. increases in proportion to the stress

d. decreases in proportion to the stress.

5. If the length of a cantilever carrying an isolated load at its free end is doubled, the deflection of the free end will increase by

a. 8

b. 1/8

c. 1/3

d. 2

6. If the whole circle bearing of a line is 180°, its reduced bearing is

a. S 0° E

b. S 0° W

c. S

d. N.

7. Pick up the correct statement from the following :

a. a back sight is the first sight taken after setting up the instrument in any position

b. the first sight on each change point is a fore sight

c. the second sight on each change point is a back sight

d. all the above.

8. The main principle of surveying is to work

a. from part to the whole

b. from whole to the part

c. from higher level to the lower level

d. from lower level to higher level.

9. For true difference in elevations between two points A and B, the level must be set up

a. at any point between A and B

b. at the exact mid point of A and B

c. near the point A

d. near the point B.

10. A dumpy level is set up with its eye-piece vertically over a peg A. The height from the top of peg A to the centre of the eye-piece is 1.540 m and the reading on peg B is 0.705 m. The level is then setup over B. The height of the eye-piece above peg B is 1.490 m and a reading on A is 2.195 m. The difference in level between A and B is

a. 3.030 m

b. 0.770 m

c. 0.785 m

d. 1.770 m.

11. If the length of a transition curve to be introduced between a straight and a circular curve of radius 500 m is 90 m, the maximum perpendicular offset for the transition curve, is

a. 1.70 m

b. 2.70 m

c. 3.70 m

d. 4.70 m

12. In a theodolite

a. the telescope axis is perpendicular to transit axis

b. the axis of rotation is perpendicular to transit axis

c. the telescope axis, the transit axis and the rotation axis pass through the centre of theodolite

d. all the above.

13. A pug mill is used for

a. moulding brick earth

b. tempering brick earth

c. providing brick earth

d. all the above.

14. Teak wood is suitable for

a. sports articles

b. furnitures

c. railway sleepers

d. all the above.

15. Apiece of sawn timber whose cross-sectional dimensions exceed 5 cm, in one direction and 20 cm in the other direction, is called a

a. cant

b. deal

c. baulk

d. strip.

16. Chlorite, a green colour mineral is mainly derived from the decomposition of

a. augite

b. biotite

c. horn blende

d. All of these.

17. Slacking of lime is affected by

a. keeping it exposed to air

b. immersing the lime in water

c. crushing the lime lumps

d. none of these.

18. The proportions of charcoal, saltpetre and sulphur in gun powder by weight, are respectively:

a. 15, 75, 10

b. 75, 10, 15

c. 10, 15, 75

d. 10, 75, 15.

19. The main object of compaction of concrete, is:

a. to eliminate air holes

b. to achieve maximum density

c. to provide initimate contact between the concrete and embedded materials

d. all the above.

20. The strength and quality of concrete, depend upon:

a. surface area of aggregates

b. shape of aggregates

c. surface texture of aggregates

d. all the above.

21. Batching error means inaccuracy in the quantity of

a. aggregates

b. cement

c. water

d. all the above.

22. Shrinkage in concrete can be reduced by using

a. less cement in the concrete

b. proper concrete mix

c. presaturated aggregates

d. all the above.

23. Hardening of cement occurs at

a. rapid rate during the first few days and afterwards it continues to increase at a decreased rate

b. slow rate during the first few days and afterwards it continues to increase at a rapid rate

c. uniform rate throughout its age

d. none of these.

24. The specific gravity of Calcite is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

25. A structure is erected on an impervious clay whose thickness is 12 m. Drainage is possible both at upper and lower surfaces. Coefficient of consolidation is 0.015 cm2 per minute. For attaining 50% consolidation with a time factor 0.20, the number of days required

a. 3233

b. 3123

c. 33331

d. 3313

26. A soil sample of mass specific gravity 1.92, has a moisture content 30%. If the specific gravity of solids is 2.75, the degree of saturation, is

a. 95.4%

b. 95.5%

c. 95.6%

d. 95.7%

27. Pick up the correct statement from the following:

a. The freezing of water is accompained by a volume increase of 9%.

b. Below freezing point, higher soil suction develops.

c. The magnitude of frost heave decreases as the degree of saturation of soil decreases

d. All the above.

28. A compacted soil sample using 10% moisture content has a weight of 200 g and mass unit weight of 2.0 g/cm3. If the specific gravity of soil particles and water are 2.7 and 1.0, the degree of saturation of the soil is

a. 11.1%

b. 55.6%

c. 69.6%

d. none of these.

29. The void ratio of a soil sample decreases from 1.50 to 1.25 when the pressure is increased from 25 tonnes/m2 to 50 tonnes/m2, the coefficient of compressibility is

a. 0.01

b. 0.02

c. 0.05

d. 0.001

30. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

31. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

32. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

33. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

34. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

35. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

36. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

37. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

38. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

39. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

40. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

41. The main purpose of as built drawing is

a. maintenance of service work '

b. dimensioning

c. profession

d. to award tender

42. Working drawings:

a. must provide complete information to produce the object

b. need not provide complete information

c. should be supplemented by further details TOPS

d. should be supplemented by specifications

43. The principal of oblique view drawing is to keep:

a. width axes at 90°

b. width and height axes at 90°

c. width, height and depth axes at 90°

d. all of the above

44. The drawing prepared by a pencil and without use of drawing instrument is called

a. isometric drawing

b. orthographic projection

c. cadastral

d. free hand sketching

45. Pick up the correct method adopted for developing the approximate or conceptual estimates from the following :

a. Cost per function method

b. Cost per square metre

c. Cost per linear unit

d. All of these

46. Which one of the following definitions, is correct ?

a. The ratio of total debt to share holder's equity is called 'debt ratio'.

b. The ratio debt-to-total assests is called Debt-to-total assest ratio.

c. The ratio of earnings before interest and taxes for a particular reporting period to the amount of interest charges for the period, is called interest coverage ratio.

d. All of these

47. Pick up the correct statement from the following:

a. The ratio of current assests, loans and advances, and the current liquidity is called current ratio.

b. Larger the current ratio, larger is the margin of safety.

c. The operating profit is the difference between gross profit and operating expenses.

d. All of these

48. Prequalification is done

a. after the invitation of bids

b. before the invitation of bids

c. during the invitation of bids

d. none of the above

49. As per the prevalling Public Procurement Acts & Regulations of Nepal, provision of price adjustments is normally to inculpated in civil works contract having project duration ?

a. more than 12 months

b. more than 15 months

c. more than 18 months

d. more than 24 months

50. Code of conduct is not related with

a. making the advice of the enginerr impartial

b. building code

c. ensuring the quality of advice

d. making the advice untained by any personal consideration