***Engineer***

1. If a solid shaft (diameter 20 cm, length 400 cm, N = 0.8 x 105 N/mm2) when subjected to a twisting moment, produces maximum shear stress of 50 N/mm2, the angle of twist in radians, is

a. 0.002

b. 0.0025

c. 0.003

d. 0.005

2. For determining the support reactions at A and B of a three hinged arch, points B and C are joined and produced to intersect the load line at D and a line parallel to the load line through A at D'. Distances AD, DD' and AD' when measured were 4 cm, 3 cm and 5 cm respectively. The angle between the reactions at A and B is

a. 30°

b. 45°

c. 60°

d. 90°

3. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

4. The ratio of the flexural strengths of two square beams one placed with its two sides horizontal and the other placed with one diagonal vertical, diagonal, is

a. 2

b. 2

c. 5

d. 7

5. Rankine-Golden formula accounts for direct as well as buckling stress and is applicable to

a. long columns

b. short columns

c. intermediate columns

d. all the above.

6. A well conditioned triangle has no angle less than

a. 20°

b. 30°

c. 45°

d. 60°.

7. If D is the degree of the curve of radius R, the exact length of its specified chord, is

a. radius of the curve x sine of half the degree

b. diameter of the curve x sine of half the degree

c. diameter of the curve x cosine of half the degree

d. diameter of the curve x tangent of half the degree.

8. Permanent adjustments of a level are

a. 2 in number

b. 3 in number

c. 4 in number

d. 6 in number

9. The  lens  equation   is applicable

a. when the thickness of the lens is small

b. only to conjugate distances along the principal axis

c. the computed quantities are accurate enough for geometrical optics of simple distances

d. all the above.

10. The included angles of a theodolite traverse, are generally measured

a. clockwise from the forward station

b. anti-clockwise from the back station

c. anti-clockwise from the forward station

d. clockwise from the back station.

11. The theodolites used for making tacheometric observations by optical wedge system, are

a. provided with stadia hairs in front of eye piece

b. not provided with stadia hairs at all

c. fitted with a glass wedge inside the telescope

d. fitted with a glass wedge in front of telescope.

12. ABCD is a regular parallelogram plot of land whose angle BAD is 60°. If the bearing of the line AB is 30°, the bearing of CD, is

a. 90°

b. 120°

c. 210°

d. 270°

13. Forge pig may be converted to wrought iron by

a. rolling

b. pudding

c. shingling

d. refining.

14. Pick up the correct statement from the following:

a. Bull's trench kiln a trench excavated in ground

b. Hoffman's kiln is constructed overground

c. Tunnel Kiln is constructed as a tunnel

d. All the above.

15. Based on its dry weight, a freshly felled tree may contain water

a. 25%

b. 50%

c. 75%

d. 100%.

16. Portland pozzolana cement possesses

a. lower heat of hydration

b. lower shrinkage on drying

c. water tightness

d. all the above.

17. Gniess is obtained from

a. igneous rocks

b. metamorphic rocks

c. sedimentary rocks

d. sedimentary metamorphic rocks.

18. Name the type of cement from the following for canal linings :

a. sulphate resisting cement

b. rapid hardening cement

c. quick setting cement

d. pozzuolana cement.

19. According to IS : 382-1963, a good aggregate should be

a. chemically inert

b. sufficiently strong

c. hard and durable

d. all the above.

20. If aggregates completely pass through a sieve of size 75 mm and are retained on a sieve of size 60 mm, the particular aggregate will be flaky if its minimum dimension is less than

a. 30.5 mm

b. 40.5 mm

c. 50.5 mm

d. none of these.

21. For given water content, workability decreases if the concrete aggregates contain an excess of

a. flat particles

b. elongated particles

c. flaky paticles

d. all the above.

22. Placing of concrete should preferably be done at a temperature of

a. 10°C

b. 20°C

c. 23°C

d. 27 ± 2°C

23. If a grading curve is horizontal bet-wen the portions of 20 mm I.S. Sieve and 4.75 mm I.S. Sieve, the graded aggregates do not contain

a. 20 mm particles

b. 10 mm particles

c. 4.75 mm particles

d. all the above.

24. The ratio of emax and emin of silty sand, is

a. 5

b. 3.0

c. 3.5

d. 4.0

25. If the failure of a finite slope occurs through the toe, it is known as

a. slope failure

b. face failure

c. base failure

d. toe failure.

26. Soils containing organic matters

a. are of spongy nature

b. swell with decrease of moisture

c. shrink with increase of moisture content

d. none of these.

27. 'Drift' is the material picked up, mixed, disintegrated, transported and redeposited by

a. wind

b. gravitational force

c. glaciated water

d. all the above.

28. If W1, W2, W3 and W4 are the sequential weights obtained during observations in pycnometer method for determining water content, the formula to be used, is

a. W =

b. W =

c. W =

d. W =

29. Terzaghi's analysis assumes :

a. soil is homogeneous and isotropic

b. elastic zone has straight boundaries inclined at ψ = φ to the horizontal and plastic zones fully developed

c. failure zones do not extend above the horizontal plane through the base of the footing

d. all the above.

30. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

31. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

32. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

33. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

34. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

35. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

36. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

37. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

38. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

39. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

40. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

41. The section obtained when the section plane is inclined to the axis of cone, but parallel to one of the generators of a cone is called ......... ......

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

42. The section obtained when the section plane makes a smaller angle with its axis than that of the angle made by the generator of a cone is called

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

43. The proportion by which the actual size of an object is enlarged or reduced in drawing is simply known as

a. multiplication factor Dil

b. scale

c. reflection factor

d. none of the above

44. One of the characteristics features of a perspective drawing is that objects are drawn:

a. smaller as their distance from the observer increases

b. same size irrespective of distance

c. larger as their distance from the observer increases

d. none of the above

45. Which method is adopted to develop an approximate or conceptual estimate for perimeter works for buildings from the following:

a. Cost per function method

b. Cost per square metre method

c. Cost per cubic metre method

d. Cost per linear metre method

46. Pick up the correct statement from the following:

a. Engineering economy is a collection of mathematical techniques which simplify economic comparisons

b. Engineering economy is a decision assistance tool by which one method will be chosen as the most economically one.

c. For understanding the engineering economy, one should be able to classify the basic terminology and fundamental concepts of economy.

d. All of these.

47. The sunk costs include :

a. a past expenditure

b. an unrecovered balance

c. an invested capital that cannot be retreived

d. All of these

48. NEC Regulation, 2057 is effective from

a. 2057/3/8

b. 2057/4/8

c. 2057/5/8

d. 2057/6/8

49. The Nepal Engineers' Association was established in

a. 2014 BS

b. 2024 BS

c. 2034 BS

d. 2044 BS

50. Profession is defined as

a. systematic knowledge through specialized training

b. synonym to job or occupation

c. types of service to needy people

d. all of above