***Engineer***

1. Slenderness ratio of a long column, is

a. area of cross-section divided by radius of gyration

b. area of cross-section divided by least radius of gyration

c. radius of gyration divided by area of cross-section

d. length of column divided by least radius of gyration.

2. A compound bar consists of two bars of equal length. Steel bar cross-section is 3500 mm2 and that of brass bar is 3000 mm2. These are subjected to a compressive load 100,000 N. If Eb = 0.2 MN/mm2 and Eb = 0.1 MN/mm2, the stresses developed are:

a. σb = 10 N/mm2, σs = 20 N/mm2

b. σb = 8 N/mm2, σs = 16 N/mm2

c. σb = 6 N/mm2, σs = 12 N/mm2

d. σb = 5 N/mm2, σs = 10 N/mm2

3. The eccentricity (e) of a hollow circular column, external diameter 25 cm, internal diameter 15 cm for an eccentric load 100 t for non-development of tension, is

a. 3.00 cm

b. 3.50 cm

c. 4.25 cm

d. 5.0 cm

4. The under mentioned type is simple strain

a. compressive strain

b. shear strain

c. volumetric strain

d. all the above.

5. The law which states, "within elastic limits strain produced is proportional to the stress producing it", is known as

a. Stress law

b. Hooke's law

c. Poisson's law

d. none of these.

6. Pick up the correct statement from the following:

a. the theodolite in which telescope can be rotated in vertical plane is called a transit

b. when the vertical circle is to the left of the telescope during observation, it is called to be in left face

c. when the vertical circle is to the right of the telescope during observation, it is called to be in right face

d. all the above.

7. A dumpy level was set up at mid-point between pegs A and B, 80 m apart and the staff readings were 1.32 and 1.56. When the level was set up at a point 10 m from A on BA produced, the staff readings obtained at A and B were 1.11 and 1.39. The correct staff reading from this set up at S should be

a. 1.435

b. 1.345

c. 1.425

d. none of these.

8. If the radius of a simple curve is R, the length of the chord for calculating offsets by the method of chords produced, should not exceed.

a. R/10

b. R/15

c. R/20

d. R/25.

9. The true meridian of a place is the line in which earth's surface is intersected by a plane through

a. east and west points

b. zenith and nadir points

c. north and south geographical poles

d. north and south magnetic poles.

10. If a tacheometer is fitted with an anal-latic lens

a. additive constant is 100, multiplying constant is zero

b. multiplying constant is 100, additive constant is zero

c. both multiplying and additive constants are 100

d. both multiplying and additive constants are 50.

11. The 10 mm markings on a levelling staff placed at 20 m are separated by

a.  radian

b.  radian

c.  radian

d.  radian.

12. In chain surveying, perpendiculars to the chain line, are set out by

a. a theodolite

b. a prismatic compass

c. a level

d. an optical square

13. A volatile substance added to a paint to make its application easy and smooth, is known as

a. base

b. solvent

c. vehicle

d. none to these.

14. For the manufacture of Portland cement, the proportions of raw materials used, are

a. lime 63% ; silica 22% ; other ingredients 15%

b. lime 22% ; silica 63% ; other ingredients 15%

c. silica 40% ; lime 40% ; other ingredients 20%

d. silica 70% ; lime 20% ; other ingredients 10%.

15. Pick up the correct statement from the following:

a. The bulking of sand is taken into account while volumetric proportioning of the aggregates

b. The dry sand and the sand completely flooded with water, have practically the same volume

c. The expansion and contraction joints are provided if concrete structures exceed 12 m in length

d. All the above.

16. The steel used for the manufacture of rails, is

a. Bessemer steel

b. mild steel

c. cast steel

d. stainless steel.

17. The process of decarbonising the pig iron completely and then adding proper percentage of carbon for manufacturing steel, is called

a. Cementation process

b. crucible process

c. Bessemer process

d. Open hearth process.

18. Asbestos is

a. an incombustible fire proof material

b. an organic substance

c. a bad insulator for sound and heat

d. all the above.

19. Pick up the incorrect statement from the following:

a. Concrete for which preliminary tests are conducted, is called controlled concrete

b. Bulking of sand depends upon the fineness of grains

c. Concrete mix 1 : 6 : 12, is used for mass concrete in piers.

d. All the above.

20. For given workability the grading requiring the least amount of water is one that gives

a. greatest surface area for the given cement and aggregates

b. least surface area for the given cement and aggregates

c. least weight for the given cement and aggregates

d. greatest weight for the given cement and aggregates

21. Slump test of concrete is a measure ofits

a. consistency

b. compressive strength

c. tensile strength

d. impact value.

22. For a good concrete

a. cement should be sufficient to produce the required strength

b. water should be free from organic materials

c. mixing of ingredients should be done thoroughly so as to produce homogeneity

d. All the above.

23. Pick up the incorrect statement from the following:

a. With passage of time, the strength of cement increases

b. With passage of time, the strength of cement decreases

c. After a period of 24 months, the strength of cement reduces to 50%

d. The concrete made with storage deteriorated cement, gains strength with time

24. For shear strength, triaxial shear test is suitable because

a. precise measurement of the pore pressure and volume change during the test is possible

b. stress distribution on the failure plane, is uniform

c. state of stress within the specimen during any stage of the test as well as at failure, is completely determined

d. all the above.

25. If a soil undergoes a change in shape and volume by application of external loads over it, but recovers its shape and volume immediately after removal of the load, the property of the soil is said to be

a. Resilience of soils

b. Elasticity of soils

c. Compressibility of soils

d. None of these.

26. Transporting and redepositing soils, is done by

a. glacier

b. gravity

c. wind

d. all the above.

27. Negative skin friction on piles

a. is caused due to relative settlement of the soil

b. is caused in soft clays

c. decreases the pile capacity

d. all of the above.

28. Pick up the correct statement from the following:

a. Isotropic consolidation of clay can be obtained in the triaxial apparatus under equal all-round pressure.

b. If the present effective stress is the maximum to which the clay has ever been subjected, it is called normally consolidated clay

c. If the present effective stress in the past was more than present effective stress, it is called over-consolidated clay

d. All the above.

29. Pick up the correct statement from the following:

a. The void space between the soil grains, is filled partly with air and partly with water

b. In perfectly saturated soil, the voids are completely filled with water

c. In dry soil, the voids are completely filled with air

d. all the above.

30. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

31. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

32. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

33. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

34. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

35. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

36. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

37. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

38. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

39. Pick up the item whose weight is added to the weight of respective item, is

a. Brackets

b. Bolts

c. Distance separators

d. All the above.

40. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

41. Tracing paper is also named as

a. drafting vellum

b. drafting film

c. drafting media

d. drafting line

42. Measurements from scale to the drawing are transferred with the aid of 19

a. scale

b. compass

c. divider

d. plumbing fork

43. The area occupied by Ao (drawing sheet) is .......... times the area occupied by Az (drawing sheet).

a. 2

b. 4

c. 8

d. 16

44. Picture plane is

a. transparent

b. opaque

c. semi transparent

d. all of the above

45. Which one of the following definitions, is correct ?

a. The ratio of total debt to share holder's equity is called 'debt ratio'.

b. The ratio debt-to-total assests is called Debt-to-total assest ratio.

c. The ratio of earnings before interest and taxes for a particular reporting period to the amount of interest charges for the period, is called interest coverage ratio.

d. All of these

46. Which one of the following statements is correct?

a. The number of years required to recover the initial cash investment in a project, is called Pay Back period (PBP).

b. The discount rate that equates the present value of the expected Net Cash Flows (CFs) with the Initial Cash Outflow (ICO) is known as internal rate of return.

c. The present value of the proposal's net cash flows, less the proposal's initial cash outflow is known as the Net Present Value (NPV)

d. All of these

47. The annuity which refers to a debt payment for recovering the initial amount or capital in equal periodical payments, is known as;

a. Present Worth Annuity

b. Sinking fund annuity

c. Compound annuity

d. Capital recovery annuity

48. Procurement of services means:

a. buying logistics

b. hiring consultant

c. renting house

d. none of the above

49. What is the abbreviation used for German Technical Co-operation?

a. GTZ

b. GTC

c. GTCO

d. all of the above

50. Code of conduct defines

a. what should be

b. what should not be

c. what had been

d. what will be