***Engineer***

1. The assumption in the theory of bending of beams, is :

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

2. A steel bar 5 m x 50 mm is loaded with 250, 000 N. If the modulus of elasticity of the material is 0.2 MN/mm2 and Poisson's ratio is 0.25, the change in the volume of the bar is :

a. 1.125 cm3

b. 2.125 cm3

c. 3.125 cm3

d. 4.125 cm2

3. The eccentricity (e) of a hollow circular column, external diameter 25 cm, internal diameter 15 cm for an eccentric load 100 t for non-development of tension, is

a. 3.00 cm

b. 3.50 cm

c. 4.25 cm

d. 5.0 cm

4. If the depth of a simply supported beam carrying an isolated load at its centre, is doubled, the deflection of the beam at the centre will be changed by a factor of

a. 1/2

b. 8

c. 1/8

d. 4

5. In a shaft rotated by a couple, the shear force varies

a. from zero at the centre to a maximum at the circumference

b. from minimum at the centre of maximum at the circumference

c. from maximum at the centre to zero at the circumference

d. equally throughout the section

6. The operation of making the algebraic sum of latitudes and departures of a closed traverse, each equal to zero, is known

a. balancing the sights

b. balancing the departures

c. balancing the latitudes

d. balancing the traverse.

7. The method of finding out the difference in elevation between two points for eliminating the effect of curvature and refraction, is

a. reciprocal levelling

b. precise levelling

c. differential levelling

d. flying levelling

8. The reduced bearing of a line is N 87° W. Its whole circle bearing is

a. 87°

b. 273°

c. 93°

d. 3°

9. The magnetic meridian at any point, is the direction indicated by a freely suspended

a. magnetic needle

b. and properly balanced magnetic needle

c. properly balanced and uninfluenced by local attractive force

d. magnetic needle over an iron pivot.

10. The diaphragm of a stadia theodolite is fitted with two additional

a. horizontal hairs

b. vertical hairs

c. horizontal and two vertical hairs

d. none of these.

11. In a precision traverse, included angles are measured by setting the vernier

a. to read zero exactly on back station

b. to read 5° exactly on back station

c. some where near zero and reading both verniers on back station

d. all the above.

12. Pick up the correct statement from the following :

a. the tangent screw enables to give small movement under conditions of smooth and positive control

b. standing on the tripod is the levelling head or trib arch

c. the levelling screws are used to tilt the instrument so that its rotation axis is truly vertical

d. all the above.

13. Pick up the polymineralic rock from the following:

a. Pure gypsum

b. Magnesite

c. Granite

d. None of these.

14. Soundness test of cement determines

a. quality of free lime

b. ultimate strength

c. durability

d. initial setting

15. Pick up the correct statement from the following:

a. The heating of a material to redness in contact with air, is known as calcination

b. The property of lime by which it sets or hardens in damp places having no free circulation of air is called setting

c. The product that remains after calcination of limestone, is called lime

d. All the above.

16. A stone is rejected if it absorbs water more than

a. 10%

b. 15%

c. 20%

d. 25%.

17. Polymerization helps to improve the property of

a. strength

b. rigidity

c. elasticity

d. all of these.

18. With storage, strength of cement

a. increases

b. decreases

c. remains the same

d. none to these.

19. Workability of concrete is measured by

a. Vicat apparatus test

b. Slump test

c. Minimum void method

d. Talbot Richard test.

20. Saw dust can be rendered chemically inert by boiling it in water containing

a. ferrous sulphate

b. potassium chloride

c. ammonia

d. nitric acid

21. Pick up the correct statement from the following:

a. Cement should be mixed for at least one minute

b. 10% of water is placed in the rotating drum before adding dry material

c. 10% of water is added after placing the other ingredients in the drum

d. All the above.

22. An aggregate which passes through 25 mm I.S. sieve and is retained on 20 mm sieve, is said to be flaky if its least dimension is less than

a. 18.5 mm

b. 16.5 mm

c. 15.5 mm

d. 13.5 mm.

23. Pick up the incorrect statement from the following:

a. With passage of time, the strength of cement increases

b. With passage of time, the strength of cement decreases

c. After a period of 24 months, the strength of cement reduces to 50%

d. The concrete made with storage deteriorated cement, gains strength with time

24. A structure is erected on an impervious clay whose thickness is 12 m. Drainage is possible both at upper and lower surfaces. Coefficient of consolidation is 0.015 cm2 per minute. For attaining 50% consolidation with a time factor 0.20, the number of days required

a. 3233

b. 3123

c. 33331

d. 3313

25. The capillary rise of water

a. depends upon the force responsible

b. increases as the size of the soil particles increases

c. decreases as the size of the soil particles decreases

d. is less in wet soil than in dry soil.

26. According to IS : 2720 - 1965, the composition of a dispersing solution used in pipette analysis for determining the size of particles, is

a. sodium-hexametaphosphate 33 g, sodium carbonate 7 g and distilled water one litre

b. sodium-hexametaphosphate 7 g, sodium carbonate 33 g and distilled water one litre

c. sodium-hexametaphosphate 23 g, sodium carbonate 17 g and distilled water one litre

d. none of these.

27. The coefficient of compressibility of soil, is the ratio of

a. stress to strain

b. strain to stress

c. stress to settlement

d. rate of loading to that of settlement.

28. A sample of saturated soil has 30% water content and the specific gravity of soil grains is 2.6. The dry density of the soil mass in g/cm3, is

a. 1.47

b. 1.82

c. 1.91

d. none of these.

29. For determining the moisture content of a soil sample, the following data is available Weight of container = 260 g, Weight of soil sample and = 320 g container, Weight of soil sample (dried) and = 310 g container. The moisture content of the soil sample, is

a. 15%

b. 18%

c. 20%

d. 25%

30. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

31. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

32. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

33. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

34. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

35. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

36. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

37. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

38. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

39. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

40. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

41. To remove unnecessary lines ....... is used

a. duster

b. chalk

c. sand box

d. eraser

42. In engineering drawing, auxiliary views are used to show details of

a. incline surface

b. horizontal surface

c. vertical surface

d. none of the above

43. In case of perspective projection, picture plane is placed

a. between eye and object

b. outside of eye towards eye

c. outside of object toward object

d. all of above

44. Dashed (dotted) line is used for indication

a. edge of the object

b. hidden edge of the object

c. centre line of the object

d. all of the above

45. Pick up the correct statement from the following:

a. The capital required to get a project started is the first cost.

b. The first cost is a single cash flow or a series of cash flows that are made in the beginning of the activity's life span

c. The first cost of purchasing a car is the sum of the down payment, taxes and dealers charges.

d. All of these

46. The construction estimate of a project is used by :

a. the owner of the facility

b. the consulting architect/engineer

c. the contractor of the project

d. All of these

47. Pick up the correct reason for making conceptual (or preliminary) estimate from the following:

a. To have a check on a definitive cost estimate.

b. To check qoutations from contractors and/or sub contractors.

c. To compute target estimate for the owner while drawings and specifications are in initial stage.

d. All of these.

48. Code of conduct is not related with

a. making the advice of the enginerr impartial

b. building code

c. ensuring the quality of advice

d. making the advice untained by any personal consideration

49. Labour act was introduced /founded in

a. 2038

b. 2048

c. 2043

d. 2053

50. The number of countries in which Nepal Engineering Council has recognized the academic institutions are (September, 2013).

a. 20

b. 30

c. 40

d. 50