***Engineer***

1. Shear strain energy theory for the failure of a material at elastic limit, is due to

a. Guest or Trecas

b. St. Venant

c. Haig

d. Von Mises.

2. Pick up the correct statement from the following:

a. The pitch of a close coil spring, is very small

b. The angle made by the coil with horizontal, is called the angle of helix

c. In the open coil helical spring, the angle of helix is comparatively large

d. All the above.

3. In case of principal axes of a section

a. sum of moment of inertia is zero

b. difference of moment inertia is zero

c. product of moment of inertia is zero

d. none of these.

4. The rise of a parabolic arch at quarter points, is equal to

a.  times the rise of the crown

b.  times the rise of the crown

c.  times the rise of the crown

d.  times the rise of the crown.

5. The law which states, "within elastic limits strain produced is proportional to the stress producing it", is known as

a. Stress law

b. Hooke's law

c. Poisson's law

d. none of these.

6. The method of finding out the difference in elevation between two points for eliminating the effect of curvature and refraction, is

a. reciprocal levelling

b. precise levelling

c. differential levelling

d. flying levelling

7. The distances AC and BC are measured from two fixed points A and B whose distance AB is known. The point C is plotted by intersection. This method is generally adopted in

a. chain surveying

b. traverse method of surveys

c. triangulation

d. none of these.

8. A lens or combination of lenses in which the following defect is completely eliminated is called aplanatic

a. spherical aberration

b. chromatic aberration

c. coma

d. astigmatism.

9. Centering error of a theodolite produces an error

a. in all angles equally

b. which does not vary with the direction or pointing

c. which varies with the direction of pointing and inversely with the length of sight

d. none of these.

10. The radius of curvature of the arc of the bubble tube is generally kept

a. 10 m

b. 25 m

c. 50 m

d. 100 m

11. In geodetic surveys higher accuracy is achieved, if

a. curvature of the earth surface is ignored

b. curvature of the earth surface is taken into account

c. angles  between the curved lines are treated as plane angles

d. none of these.

12. For setting out a simple curve, using two theodolites.

a. offsets from chord produced are required

b. offsets from long chord are required

c. deflection angles from Rankine's formula are required

d. none of these.

13. Pick up the correct statement from the following:

a. Quick lime contains calcium oxide

b. Slaked lime may be obtained from quick lime

c. Slaked lime is obtained by adding water to quick lime

d. All the above.

14. Ground glass

a. is made by grinding its one side

b. is made by melting powdered glass paints surface

c. is used for getting light without transparency

d. all the above.

15. The commonly used thinner in oil paints, is

a. turpentine

b. both (a) and (b)

c. neither (a) nor (b)

d. none the these.

16. Bitumen felt

a. is used as water proofing material

b. is used as damp proofing material

c. is made from bitumen and hessian fibres

d. all the above.

17. When a brick is immersed in water for 24 hours and then dried, if

a. no grey or white deposits appear on the surface, the brick is free from soluble salts

b. 10 per cent surface is covered with grey or white deposits, the brick has slight efflorescence

c. 50 per cent surface is covered with grey or white deposits, the brick has serious efflorescence

d. All the above.

18. The most commonly used synthetic abrasive is

a. aluminium carbide

b. boric acid

c. silicon

d. All of these.

19. Separation of coarse aggregates from mortar during transportation, is known

a. creeping

b. segregation

c. shrinkage

d. none of these.

20. Pick up the correct statement from the following:

a. Consistency does not affect the workability of concrete

b. If the slump increases, workability decreases

c. If the concrete mix is dry, the slump is maximum

d. None of these.

21. Pick up the incorrect statement from the following:

a. The process of changing cement paste into hard mass, is known as setting of cement

b. The phenomenon by virtue of which cement does not allow transmission of sound, is known as soundness of cement

c. The heat generated during chemical reaction of cement with water, is known as heat of hydration

d. None of these.

22. The shuttering of a hall measuring 4 m x 5 m, can be removed after

a. 7 days

b. 10 days

c. 14 days

d. 21 days

23. For the construction of cement concrete floor, the maximum permissible size of aggregate, is

a. 6 mm

b. 8 mm

c. 10 mm

d. 12 mm

24. Water content of a soil sample is the difference of the weight of the given sample at the given temperature and the weight determined after drying it for 24 hours at temperature ranging from

a. 90° to 95°C

b. 95° to 100°C

c. 103° to 105°C

d. 105° to 110°C

25. Cohesive soils are generally

a. plastic and also compressible

b. elastic and also compressible

c. plastic but incompressible

d. none of these.

26. A soil not fully consolidated under the existing over-burden pressure, is called

a. pre-consolidated

b. normally consolidated

c. under-consolidated

d. none of these.

27. The total weight of a pycnometer with water and oven dried soil 20 (g) is 1600 g. The pycnometer filled with water alone weighs 1500 g. The specific gravity of the soil, is

a. 1.0

b. 1.5

c. 2.0

d. 2.5

28. The internal molecular attraction of a soil, the cohesion

a. decreases as the moisture content increases

b. increases as the moisture content decreases

c. is more in well compacted clays

d. depends upon the external applied load.

29. Minimum size of the particles of silt soil, is

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm.

d. 0.08 mm

30. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

31. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

32. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

33. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

34. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

35. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

36. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

37. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

38. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

39. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

40. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

41. In case of third angle projection, plan is kept at

a. top

b. below

c. side

d. none

42. The scale should never be used as a ........ for drawing straight lines.

a. set-square

b. working edge

c. straight edge

d. none of the above

43. The shape of scales are

a. flat and triangular

b. flat and rectangular

c. flat and square

d. all of the above

44. The space covered by a drawing in a paper after changing the scale from 1:100 to 1:50 will:

a. increase 400%

b. increase 200%

c. decrease 200

d. decrease 400%

45. Current assests less inventories divided by current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

46. Keeping in view, the feasibility order of magnitude, the preliminary, conceptual or budget estimates, are prepared by :

a. construction manager

b. owner himself/herself

c. construction manager

d. None of these

47. Earning per share is the most important ratio for

a. share holders

b. banks

c. company's management

d. All of these

48. The time between ordering of the goods or materials and receiving and placing them into use at the point of need is called

a. order-interval

b. lead time

c. usage time

d. delayed time

49. Which is the professional body of architectural and engineering firms is Nepal?

a. Society of Nepalese Architects (SONA)

b. Society of Public Health Engineers (SOPHEN)

c. Sociality of Consulting Engineering and Architectural firms

d. None of above

50. What is EPC contract ?

a. Engineering, Planning and costing

b. Evaluation, Planning and consulting

c. Engineering, procurement and Construction

d. none of above