***Engineer***

1. For calculating the allowable stress of long columns. The empirical formula , is known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

2. A close coil helical spring when subjected to a moment M having its axis along the axis of the helix

a. it is subjected to pure bending

b. its mean diameter will decrease

c. its number of coils will increase

d. all the above.

3. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

4. Maximum deflection of a

a. simply supported beam carrying a concentrated load W at mid-span is

b. cantilever beam, carrying a uniformly distributed load over span is

c. simply supported beam carrying a uniformly distributed load over the span is

d. All the above.

5. If a steel rod of 20 mm diameter and 5 metres long elongates by 2.275 mm when subjected to an axial pull of 3000 kg, the stress developed, is

a. 9.5541 kg/cm2

b. 95.541 kg/cm2

c. 955.41 kg/cm2

d. 9554.1 kg/cm2.

6. In precision theodolite traverse if included angles are read twice and the mean reading accepted using both verniers having a least count of 30". Assuming the instrument to be in perfect adjustment, linear measurements correct to 6 mm per 30 metre tape duly corrected for temperature, slope and sag, the angular error of closure not to exceed (where n is the number of traverse legs)

a. 50" n

b. 30" n

c. 60" n.

7. The chaining on sloping ground is

a. easier along the falling gradient

b. easier along the up gradient

c. equally convenient along falling as well as up gradient

d. all the above.

8. If S is the length of a subchord and R is the radius of simple curve, the angle of deflection between its tangent and sub-chord, in minutes, is equal to

a. 573 R/S

b. 171.9 S/R

c. 1718.9 R/S

d. 1718.9 S/R.

9. If the length of a transition curve to be introduced between a straight and a circular curve of radius 500 m is 90 m, the maximum deflection angle to locate its junction point, is

a. 1°43' 08"

b. 1°43' 18"

c. 1°43' 28"

d. 1°43' 38"

10. Which one of the following statements is correct ?

a. when the axes of rotation of the graduated circle and the verniers are not coincident, the instrument possesses eccentricity

b. the mean of the readings of the two verniers gives correct reading free from the eccentricity

c. one vernier may be used if the readings of two verniers differ by a constant

d. all the above.

11. For the construction of highway (or railway)

a. longitudinal sections are required

b. cross sections are required

c. both longitudinal and cross sections are required

d. none of these.

12. Volume of the earth work may be calculated by

a. end areas

b. Prismoidal formula

c. Trapezoidal

d. all the above.

13. The specific gravity of marble, is

a. 2.60

b. 2.66

c. 2.72

d. 3.00.

14. Dry rot

a. reduces the strength of timber

b. reduces the timber to powder

c. shrinks the timber

d. spoils the appearance of timber.

15. Cast iron is used for

a. structural works in beams

b. small sized water pipes

c. columns and struts

d. none to these.

16. For one cubic metre of brick masonry, number of bricks required, is

a. 425

b. 450

c. 500

d. 550.

17. If the iron ore contains clay as an impurity, the flux added during calcination, is

a. clay

b. lime stone

c. argillaceous iron ore

d. all the above.

18. The steel which contains fissures and cavities, is manufactured by

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

19. Pick up the correct proportions of chemical ingredients of cement

a. Lime : Silica : Alumina : Iron oxide : 63 : 22 : 6 : 3

b. Silica : Lime : Alumina : Iron oxide : 63 : 22 : 6 : 3

c. Alumina : Silica : Lime : Iron oxide : 63 : 22 : 6 : 3

d. Iron oxide : Alumina : Silica : Lime : 63 : 22 : 6 : 3

20. The maximum percentage of chemical ingredient of cement is that of

a. iron oxide

b. aluminium

c. lime

d. silica.

21. The void ratio of

a. single size coarse aggregate is roughly 0.45.

b. graded coarse aggregate is roughly 0.040

c. fine aggregate is roughly 0.45

d. all the above.

22. The minimum percentage of chemical ingredient of cement is that of

a. magnesium oxide

b. iron oxide

c. alumina

d. lime

23. Vicat's apparatus is used for

a. consistency test

b. setting time test

c. soundness test

d. compressive strength test.

24. Pick up the correct statement from the following:

a. A soil having pH value more than 7 is an acidic soil

b. A soil having pH value less than 7 is an acidic soil

c. A soil having pH value more than 7 is an alkaline soil

d. A soil containing chemicals for the manufacture of portland cement is preferred.

25. A pycnometer is used to determine

a. voids ratio

b. dry density

c. water content

d. density index.

26. The shearing force acting along the slice of a curved surface of slippage, causes the soil to slide

a. down at the centre

b. down at the toe

c. upward at the centre

d. none of these.

27. For determining the ultimate bearing capacity of soil, the recommended size of a square bearing plate to be used in load plate test should be 30 to 75 cm square with a minimum thickness of

a. 10 mm

b. 15 mm

c. 20 mm

d. 25 mm

28. Cohesive soils are generally

a. plastic and also compressible

b. elastic and also compressible

c. plastic but incompressible

d. none of these.

29. Pick up the correct statement from the following:

a. The void space between the soil grains, is filled partly with air and partly with water

b. In perfectly saturated soil, the voids are completely filled with water

c. In dry soil, the voids are completely filled with air

d. all the above.

30. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

31. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

32. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

33. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

34. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

35. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

36. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

37. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

38. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

39. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

40. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

41. In one plane drawing, which one does not falls under

a. orthographic

b. axonometric

c. oblique

d. dimetric

42. The paper size of A, is

a. 297 x 420

b. 210 x 297

c. 420x 850

d. none of the above

43. Set squares are used for drawing ........ ....... .. and ........... lines.

a. vertical

b. inclined

c. parallel

d. all of the above

44. Working drawing is prepared not for

a. bill of quantities

b. estimation

c. layout

d. specification

45. The ratio of current assests to current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

46. Annuities involve:

a. all payments of equal amount

b. payment at equal time intervals

c. payments at the end of periods.

d. All of these

47. If a is the base amount expenditure, b is the increase in the operation cost each year over a period of n years, the total cost of maintenance is :

a. a + (n + 1) b

b. a + (n - 1) b

c. a x (n - 1) b

d. a - (n - 1) b

48. Collusive practice" means

a. offering anything to influence the action of public official

b. arrangement between two or more parties to achieve an improper purpose including influencing improperly the action of another party

c. mis representation of facts in order to influence a procurement process

d. all of the above

49. Prequalification is done

a. after the invitation of bids

b. before the invitation of bids

c. during the invitation of bids

d. none of the above

50. What is EPC contract ?

a. Engineering, Planning and costing

b. Evaluation, Planning and consulting

c. Engineering, procurement and Construction

d. none of above