***Engineer***

1. The ratio of the area of cross-section of a circular section to the area of its core, is

a. 4

b. 8

c. 12

d. 16

2. A three hinged arch is generally hinged at its supports and

a. at one quarter span

b. at the crown

c. any where in the rib

d. none of these.

3. A body is said to be in equilibrium if

a. it moves horizontally

b. it moves vertically

c. it rotates about its C.G.

d. none of these.

4. If the width b and depth d of a beam simply supported with a central load are interchanged, the deflection at the centre of the beam will be changed in the ratio of

a. d/b

b. (d/b)2

c. (b/d)2

d. (b/d)3.

5. Pick up the correct assumption of the theory of simple bending

a. Transverse section of a beam remains plane before and after bending

b. The material of the beam is homogeneous and isotropic

c. The resultant pull or thrust on transverse section of a beam is zero

d. All the above.

6. The horizontal angle between true meridian and magnetic meridian, is known

a. bearing

b. magnetic declination

c. dip

d. convergence.

7. The probable error of the adjusted bearing at the middle is

a.  rn

b.  rn

c.  rn

d.  rn.

8. The power of a lens

a. is reciprocal of its focal length

b. is positive if it is a convex lens

c. is negative if it is a concave lens

d. is measured in diopter.

9. Profile levelling is usually done for determining

a. contours of an area

b. capacity of a reservoir

c. elevations along a straight line

d. boundaries of property

10. Hydrographic surveys deal with the mapping of

a. large water bodies

b. heavenly bodies

c. mountaineous region

d. canal system

11. Two contour lines, having the same elevation

a. cannot cross each other

b. can cross each other

c. cannot unite together

d. can unite together.

12. The ratio of the length of long chord and the tangent length of a circular curve of radius R deflecting through angle Δ, is

a. cos

b. tan

c. 2 sin

d. 2 cos .

13. Water paint is a

a. colour wash

b. whiting

c. distemper

d. all the above.

14. The plastics prepared from Vinyl resin are

a. non-toxic

b. transparent

c. colourless

d. all of these.

15. Ultimate strength to cement is provided by

a. Tricalcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

16. In a mortar, the binding material is

a. cement

b. sand

c. surkhi

d. cinder.

17. Bitumen paints offer

a. pleasing surface

b. hard surface

c. smooth surface

d. protective surface

e. rough surface.

18. The minimum compressive strength of 1st class bricks should be

a. 90 kg/cm2

b. 100 kg/cm2

c. 120 kg/cm2

d. 130 kg/cm2.

19. The main object of compaction of concrete, is:

a. to eliminate air holes

b. to achieve maximum density

c. to provide initimate contact between the concrete and embedded materials

d. all the above.

20. The ratio of various ingredients (cement, sand, aggregates) in concrete of grade M 200, is

a. 1 : 3 : 6

b. 1 :   : 3

c. 1 : 1 : 2

d. 4 : 2 : 1

21. According to Water-Cement Ratio Law, the strength of workable plastic concrete

a. depends upon the amount of water used in the mix

b. does not depend upon the quality of cement mixed with aggregates

c. does not depend upon the quanity of cement mixed with aggregates

d. all the above.

22. Water cement ratio is generally expressed in volume of water required per

a. 20 kg

b. 30 kg

c. 40 kg

d. 50 kg

23. If aggregates completely pass through a sieve of size 75 mm and are retained on a sieve of size 60 mm, the particular aggregate will be flaky if its minimum dimension is less than

a. 30.5 mm

b. 40.5 mm

c. 50.5 mm

d. none of these.

24. Pick up the correct statement from the following:

a. Isotropic consolidation of clay can be obtained in the triaxial apparatus under equal all-round pressure.

b. If the present effective stress is the maximum to which the clay has ever been subjected, it is called normally consolidated clay

c. If the present effective stress in the past was more than present effective stress, it is called over-consolidated clay

d. All the above.

25. A stratum of clay 2 m thick will get consolidated 80% in 10 years. For the 80% consolidation of 8 m thick stratum of the same clay, the time required is

a. 120 years

b. 140 years

c. 160 years

d. 180 years.

26. Failure of a slope occurs only when total shear force is

a. equal to total shearing strength

b. greater than total shearing strength

c. less than total shearing strength

d. none of these.

27. The liquid limit and plastic limit exist in

a. sandy soils

b. silty soils

c. gravel soils

d. clay soils.

28. If the failure of a finite slope occurs through the toe, it is known as

a. slope failure

b. face failure

c. base failure

d. toe failure.

29. Maximum size of clay particles, is :

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm

d. 0.08 mm

30. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

31. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

32. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

33. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

34. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

35. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

36. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

37. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

38. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

39. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

40. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

41. The edge of the board on which T-square is made to slide is called

a. working edge

b. drawing line

c. border line

d. deader line

42. Select the incorrect statement about orthographic projections :

a. the reference planes are also called the principle planes

b. the object must the situated in the first quadrant

c. the planes of projection and assumed to be transparent

d. projections are obtained by drawing perpendicular s from the object to the planes

43. Which of the the following is not intended while making a 'sketch' in the context of technical drawings?

a. a fished product

b. free hand sketching

c. a preliminary product

d. a quick way to record an idea for work

44. Which one is not drafting equipment ?

a. drafter

b. dust cloth

c. scale

d. None of the above

45. Which one of the following statements is correct?

a. The number of years required to recover the initial cash investment in a project, is called Pay Back period (PBP).

b. The discount rate that equates the present value of the expected Net Cash Flows (CFs) with the Initial Cash Outflow (ICO) is known as internal rate of return.

c. The present value of the proposal's net cash flows, less the proposal's initial cash outflow is known as the Net Present Value (NPV)

d. All of these

46. The construction estimate of a project is used by :

a. the owner of the facility

b. the consulting architect/engineer

c. the contractor of the project

d. All of these

47. Present worth Annuity (PWA) is generally known as

a. Premium annuities

b. Income annuities

c. Future annuities

d. All of these

48. Who is the Chairperson of House Rent Fixation Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer

49. Prequalification is done

a. after the invitation of bids

b. before the invitation of bids

c. during the invitation of bids

d. none of the above

50. The user's committee method of construction is labour intensive with a cost estimate upto:

a. 4 million

b. 6 million

c. 8 million

d. 10 million