***Engineer***

1. Slenderness ratio of a long column, is

a. area of cross-section divided by radius of gyration

b. area of cross-section divided by least radius of gyration

c. radius of gyration divided by area of cross-section

d. length of column divided by least radius of gyration.

2. A cantilever of length 2 cm and depth 10 cm tapers in plan from a width 24 cm to zero at its free end. If the modulus of elasticity of the material is 0.2 x 106 N/mm2, the deflection of the free end, is

a. 3 mm

b. 4 mm

c. 5 mm

d. 6 mm

3. A load of 1960 N is raised at the end of a steel wire. The minimum diameter of the wire so that stress in the wire does not exceed 100 N/mm2 is :

a. 4.5 mm

b. 5.0 mm

c. 5.5 mm

d. 6.0 mm

4. In a three hinged arch, the third hinge is generally kept at

a. crown of the arch

b. midpoint of the crown and left support hinge

c. midpoint of the crown and right support hinge

d. none of these.

5. In a square beam loaded longitudinally, shear develops

a. on lower fibre along horizontal plane

b. on top fibre along vertical plane

c. equally on each fibre along horizontal plane

d. none of these.

6. Check lines (or proof lines) in Chain Surveying, are essentially required

a. to plot the chain lines

b. to plot the offsets

c. to indicate the accuracy of the survey work

d. to increase the out-turn

7. The curve composed of two arcs of different radii having their centres on the opposite side of the curve, is known

a. a simple curve

b. a compound curve

c. a reverse curve

d. a vertical curve.

8. A well conditioned triangle has no angle less than

a. 20°

b. 30°

c. 45°

d. 60°.

9. The longitudinal section of the surface of bubble tube is

a. straight

b. circular

c. parabolic

d. elliptic.

10. In chain surveying tie lines are primarily provided

a. to check the accuracy of the survey

b. to take offsets for detail survey

c. to avoid long offsets from chain lines

d. to increase the number of chain lines.

11. Pick up the incorrect statement from the following :

a. while measuring a distance with a tape of length 100.005 m, the distance to be increasing by 0.005 m for each tape length

b. an increase in temperature causes a tape to increase in length and the measured distance is too large

c. the straight distance between end points of a suspended tape is reduced by an amount called the sag correction

d. a 100 m tape of cross section 10 mm x 0.25 mm stretches about 10 mm under 5 kg pull.

12. For indirect ranging, number of ranging rods required, is

a. 2

b. 3

c. 4

d. 5

13. Steel contains carbon approximately

a. 1.50% to 5.6%

b. 0.05% to 1.75%

c. 0.25%

d. none to these.

14. Percentage content of silica in window glass, is

a. 40 to 45

b. 50 to 55

c. 60 to 65

d. 70 to 75.

15. Asbestos

a. is fire-proof and acid proof

b. has sp. gravity equal to 3.10

c. is smooth like glass and silk

d. All the above.

16. Plaster of Paris is obtained by calcining

a. gypsum

b. kankar

c. lime stone

d. none of these.

17. Calcination of iron ores is done

a. to remove carbonic acid

b. by roasting in heaps

c. after dressing

d. all the above.

18. For obtaining vinyl chloride acetate, the method used, is

a. addition polymerization

b. condensation polymerization

c. co-polymerization

d. none of these.

19. Concrete mainly consists of

a. aggregates

b. admixture

c. water

d. all the above.

20. After casting, an ordinary cement concrete on drying

a. expands

b. mix

c. shrinks

d. none of these.

21. According to the recommendations of IS : 456-1978, the expansion joints

a. are provided to ensure minimum resistance

b. are supported on separate columns

c. do not carry reinforcement across them

d. all the above.

22. An aggregate is said to be flaky, if its least dimension is less than

a. 3/4 mean dimension

b. 3/5 mean dimension

c. 5/8 mean dimension

d. none of these.

23. Pick up the correct statement from the following:

a. Concrete using sand stones, cracks due to excessive shrinkage

b. Very hard and close grained crystallined lime stones are suitable aggregates but provide low strength

c. Broken bricks produce a concrete having good fire resisting qualities

d. All the above.

24. If a soil undergoes a change in shape and volume by application of external loads over it, but recovers its shape and volume immediately after removal of the load, the property of the soil is said to be

a. Resilience of soils

b. Elasticity of soils

c. Compressibility of soils

d. None of these.

25. Under-reamed piles are generally

a. driven piles

b. bored piles

c. precast piles

d. all the above.

26. If the back fill is having a uniform surcharge of intensity q per unit area, the lateral pressure will be

a. q times the lateral pressure within the surface

b. 1/q times the lateral pressure within the surface

c. equal to a fill of height Z equal to q/r, where r is the density of the backfill

d. none of these.

27. Darcy's law is applicable to seepage if a soil is

a. homogeneous

b. isotropic

c. incompressible

d. all the above.

28. Pick up the correct statement from the following:

a. Illite bond is stronger than montmorillonite bond

b. Illites do not swell when wet

c. Illites are composed of two silica tetrahedralsheets with a central octahedral sheet

d. All the above.

29. Stoke's law does not hold good if the size of particle is smaller than

a. 0.0002 mm

b. 0.002 mm

c. 0.02 mm

d. 0.2 mm

30. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

31. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

32. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

33. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

34. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

35. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

36. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

37. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

38. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

39. The expected out turn of half brick partition wall per mason per day is

a. 1.5 m3

b. 2.0 m3

c. 4.0 m2

d. 5.0 m2

40. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

41. Dashed (dotted) line is used for indication

a. edge of the object

b. hidden edge of the object

c. centre line of the object

d. all of the above

42. The equipment mainly used to transfer the measurements is

a. scale

b. divider

c. compass

d. drawing ping

43. The projection in which the length, breadth and height of an object is shown in one view is known as:

a. pictorial projection

b. isometric projection

c. cavalier projection

d. cabinet projection

44. Dimension lines and projection lines are drawn as:

a. continuous thick lines

b. continuous thin line with zig zags

c. continuous thin line

d. continuous thin and wary lines

45. The construction manager uses the estimate of the project

a. to advise the architect/engineer regarding design cost parameter especially in value engineering analysis.

b. to develop bids on the project.

c. to control the project during its construction.

d. All of these

46. Renu Bala deposits Rs 1200 now, Rs 800 two years from now and Rs 1000 five years from now. If the savings bank's rate of interest in 5%, she will receive an amount of Rs X 10 years from now, where X is

a. Rs 3415

b. Rs 4225

c. Rs 4413

d. Rs 4826

47. Pick up the correct statement from the following:

a. Ratio analysis is the procedure of determining and interpreting numerical relationship of various items of the financial statement.

b. All financial ratios are obtained by relating two sets of information contained in a Single financial statement.

c. The relationship between two accounting figures expressed mathematically, is known as a financial ratio.

d. All of these

48. The registered engineer in Nepal Engineering Council should have to renew their registration in:

a. every year

b. every two years

c. every four years

d. none of the above

49. Nepal engineering council is an autonomous body formed under NEC act in

a. 2056 BS

b. 2055 BS

c. 2054 BS

d. 2053 BS

50. Which Ministry of Government of Nepal deals with Civil Engineering group of Nepal Engineering service?

a. Ministry of General Administration

b. Ministry of Energy

c. Ministry of Irrigation

d. Ministry of Physical Infrastructure and Transport