***Engineer***

1. Pick up the correct statement from the following:

a. For a uniformly distributed load, B.M. curve is a parabola

b. For a load varying linearly, the shear force curve is a parabola

c. For a load varying linearly, the B.M. curve is a cubic parabola

d. All the above.

2. The normal component of a force inclined through θ° is obtained by multiplying the force by

a. cos θ

b. tan θ

c. sin θ cos θ

d. sin2θ

3. A composite beam is composed of two equal strips one of brass and other of steel. If the temperature is raised

a. brass experiences compressive force

b. composite beam gets subjected to a couple

c. composite beam bends

d. All the above.

4. The stress at which extension of a material takes place more quickly as compared to the increase in load, is called

a. elastic point

b. plastic point

c. breaking point

d. yielding point.

5. Maximum deflection of a

a. simply supported beam carrying a concentrated load W at mid-span is

b. cantilever beam, carrying a uniformly distributed load over span is

c. simply supported beam carrying a uniformly distributed load over the span is

d. All the above.

6. The line of sight is kept as high above ground surface as possible to minimise the error in the observed angles due to

a. shimmering

b. horizontal refraction

c. vertical refraction

d. both shimmering and horizontal refraction.

7. Deviation of the actual road gradient from the proposed contou'r gradient up hill side, involves

a. embankment on the centre line

b. excavation on the centre line

c. earth work on the centre line

d. none of these.

8. While measuring with a metallic tape of 30 m length pull should be applied

a. 1 kg

b. 2 kg

c. 3 kg

d. 4 kg

9. True meridians are generally preferred to magnetic meridians because

a. these converge to a point

b. these change due to change in time

c. these remain constant.

d. None of these.

10. The combined effect of curvature and refraction over a distance L kilometres is

a. 67.2 L2 mm

b. 76.3 L2 mm

c. 64.5 L2 mm

d. none of these.

11. If α is the angle between the polar ray and the tangent at the point of commencement of a lemniscate curve, the equation of the curve, is

a. l = ksin 2α

b. l = ksin 3α

c. l = ktan 2α

d. l = kcos 2α

12. In an adjusted level, when the bubble is central, the axis of the bubble tube becomes parallel to

a. line of sight

b. line of collimation

c. axis of the telescope

d. None of these.

13. If the iron ore contains clay as an impurity, the flux added during calcination, is

a. clay

b. lime stone

c. argillaceous iron ore

d. all the above.

14. The foliated structure is very common in

a. sedimentary rocks

b. igneous rocks

c. metamorphic rocks

d. none of these.

15. Whitworth compressed steel is obtained when molten steel is subjected to a pressure of

a. 9 kg/mm2

b. 13 kg/mm2

c. 15 kg/mm2

d. 10 kg/mm2.

16. Minimum required water cement ratio for a workable concrete, is

a. 0.40

b. 0.50

c. 0.60

d. 1.0.

17. The minimum compressive strength of Ilnd class bricks should be

a. 75 kg/cm2

b. 90 kg/cm2

c. 100 kg/cm2

d. 120 kg/cm2

18. The slag which floats on the surface of the molten iron generally contains

a. Silica (SiO2) 35%

b. Alumina (Al2O3) 12%

c. MgO, CaSO4, KMnO2 and FeO 8%

d. all the above.

19. The size of fine aggregates does not exceed

a. 3.00 mm

b. 3.75 mm

c. 4.75 mm

d. 5.75 mm

20. The maximum thickness of concrete floor of a cement warehouse, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

21. The maximum amount of dust which may be permitted in aggregates is

a. 5% of the total aggregates for low workability with a coarse grading

b. 10% of the total aggregates for low workability with a fine grading

c. 20% of the total aggregates for a mix having high workability with fine grading

d. all the above.

22. The concrete mix which causes difficulty in obtaining a smooth finish, possess

a. internal friction

b. harshness

c. bleeding

d. none of these.

23. The 28 days cube strength of mass concrete using aggregates of maximum size 5 cm for gravity dams should be

a. between 350 to 600 kg/cm2

b. between 150 to 500 kg/cm2

c. below 200 kg/cm2

d. none of these.

24. The active earth pressure of a soil is proportional to (where φ is the angle of friction of the soil)

a. tan (45° - φ)

b. tan2 (45° + φ/2)

c. tan2 (45° - φ/2)

d. tan (45° + φ)

25. The coefficient of curvature for a well graded soil, must be between

a. 0.5 to 1.0

b. 1.0 to 3.0

c. 3.0 to 4.0

d. 4.0 to 5.0

26. The method of the slices is applicable to

a. homogenous soils

b. stratified soils

c. saturated soils

d. non-uniform slopes

27. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

28. A critical hydraulic gradient may occur when

a. flow is in upward direction

b. seepage pressure is in upward direction

c. effective pressure is zero

d. all the above.

29. When a cohesionless soil attains quick condition, it looses

a. shear strength

b. bearing capacity

c. both (a) and (b)

d. neither (a) nor (6).

30. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

31. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

32. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

33. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

34. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

35. Optimistic time, most likely time and pessimistic times for the activities of a network in the given figure are written above their arrows. If the contractual obligation time for the project is 75, the latest occurrence time for the event 2, is

a. 20

b. 25

c. 35

d. 15

36. Referring of given figure, pick up the correct statement from the following:

a. The total length of centre line of four walls is 20 m

b. Length of long wall out-to-out is 6.80 m

c. Length of short walls in-to-in is 3.20 m

d. All the above.

37. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

38. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

39. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

40. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

41. Tracing paper is also named as

a. drafting vellum

b. drafting film

c. drafting media

d. drafting line

42. The techniques for free hand sketch to drawing straight line is

a. fixing two end points

b. fixing length of the line

c. fixing the center of line

d. all of the above

43. The size of the title block of all sizes of drawing sheet is :

a. 420 mm, 65 mm

b. 185 mm, 65 mm

c. 297 mm, 65 mm

d. 185 mm, 50 mm

44. Working drawings:

a. must provide complete information to produce the object

b. need not provide complete information

c. should be supplemented by further details TOPS

d. should be supplemented by specifications

45. The alternatives which are standalone solutions for given situations in engineering involve :

a. the anticipated life of the assest

b. the yearly costs of maintaining the assest (annual maintenance and operating cost)

c. the anticipated resaleable value (salvage value) and the interest return (rate of return)

d. All of these

46. Pick up the correct statement from the following:

a. Uniform series present worth factor =

b. Sinking fund factor =

c. Capital recovery factor =  where letters carry their usual meanings.

d. All of these

47. The CRF (ep) is also known as: [CRF(EP) - 8% - 7], where

a. 8% is the rate of interest per year

b. money is borrowed for n = 7 years

c. both (a) and (b)

d. Neither (a) nor (b)

48. NEC publishes the name of registered engineers each year as well as every forth year. The name of foreigner engineers publish

a. each year

b. every forth year

c. both (a) and (b)

d. none of the above

49. The number of countries in which Nepal Engineering Council has recognized the academic institutions are (September, 2013).

a. 20

b. 30

c. 40

d. 50

50. The registration book for general and professional engineers is written in Nepali language while for foreginer engineer

a. Nepali

b. English

c. Nepali and English

d. French