***Engineer***

1. If a concrete column 200 x 200 mm in cross-section is reinforced with four steel bars of 1200 mm2 total cross-sectional area. Calculate the safe load for the column if permissible stress in concrete is 5 N/mm2 and Es is 15 Ec

a. 274 MN

b. 284 MN

c. 294 MN

d. None of these.

2. A composite beam is composed of two equal strips one of brass and other of steel. If the temperature is raised

a. brass experiences compressive force

b. composite beam gets subjected to a couple

c. composite beam bends

d. All the above.

3. Slenderness ratio of a long column, is

a. area of cross-section divided by radius of gyration

b. area of cross-section divided by least radius of gyration

c. radius of gyration divided by area of cross-section

d. length of column divided by least radius of gyration.

4. In a simply supported beam (l + 2a) with equal overhangs (a) and carrying a uniformly distributed load over its entire length, B.M. at the middle point of the beam will be zero if

a. l = 2a

b. l = 4a

c. l < 2a

d. l > a

5. For a simply supported beam with a central load, the bending moment is

a. least at the centre

b. least at the supports

c. maximum at the supports

d. maximum at the centre.

6. Pick up the correct statement from the following :

a. the tangent screw enables to give small movement under conditions of smooth and positive control

b. standing on the tripod is the levelling head or trib arch

c. the levelling screws are used to tilt the instrument so that its rotation axis is truly vertical

d. all the above.

7. The method of finding out the difference in elevation between two points for eliminating the effect of curvature and refraction, is

a. reciprocal levelling

b. precise levelling

c. differential levelling

d. flying levelling

8. If the radius of a simple curve is 600 m, the maximum length of the chord for calculating offsets, is taken

a. 15 m

b. 20 m

c. 25 m

d. 30 m.

9. Prismatic compass is considered more accurate than a surveyor's compass, because

a. it is provided with a sliding glass in the object vane

b. its graduations are in whole circle bearings

c. it is provided with a prism to facilitate reading of its graduated circle

d. both (c) and (d).

10. Contours of different elevations may cross each other only in the case of

a. an over hanging cliff

b. a vertical cliff

c. a saddle

d. an inclined plane.

11. If the long chord and tangent length of a circular curve of radius R are equal the angle of deflection, is

a. 60°

b. 90°

c. 120°

d. 150°.

12. True meridians are generally preferred to magnetic meridians because

a. these converge to a point

b. these change due to change in time

c. these remain constant.

d. None of these.

13. The hardest rock is

a. marble

b. diamond

c. talc

d. quartz.

14. Cement paints usually

a. contain 5% to 10% colour pigments

b. are prepared with white cement

c. contain 5% sodium chloride

d. all the above.

15. Pick up the compound responsible for early strength of cement from the following:

a. Tetra-calcium alumino-ferrite

b. Tricalcium silicate

c. Tricalcium aluminate

d. Dicalcium silicate.

16. In the cement the compound quickest to react with water, is

a. Tricalcium aluminate

b. Tetra-calcium alumino-ferrite

c. Tricalcium silicate

d. Dicalcium silicate.

17. The slag which floats on the surface of the molten iron generally contains

a. Silica (SiO2) 35%

b. Alumina (Al2O3) 12%

c. MgO, CaSO4, KMnO2 and FeO 8%

d. all the above.

18. Rapid hardening cement contains

a. Tri-calcium silicate

b. Tri-calcium aluminate

c. Tetra-calcium alumino-ferrite

d. Dicalcium silicate.

19. Concrete gains strength due to

a. chemical reaction of cement with sand and coarse aggregates

b. evaporation of water from concrete

c. hydration of cement

d. All the above.

20. Water cement ratio is

a. weight of water to that of cement

b. weight of concrete to that of water

c. volume of concrete to that of water

d. both (a) and (b) of the above.

21. Vicat apparatus is used for

a. consistency test

b. test for setting time

c. test for tensile strength

d. none of these.

22. Addition of pozzolana to cement causes

a. loss of heat of hydration

b. reduction in bleeding

c. increase in curing time

d. all the above.

23. For ensuring quality of concrete, use

a. single sized aggegates

b. two sized aggregate

c. graded aggregates

d. coarse aggregates.

24. The maximum value of effective stress in the past divided by the present value, is defined as over consolidation ratio (OCR). The O.C.R. of an over consolidated clay is

a. less than 1

b. 1

c. more than 1

d. None of these.

25. A sample of saturated soil has 30% water content and the specific gravity of soil grains is 2.6. The dry density of the soil mass in g/cm3, is

a. 1.47

b. 1.82

c. 1.91

d. none of these.

26. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

27. The coefficient of curvature for a well graded soil, must be between

a. 0.5 to 1.0

b. 1.0 to 3.0

c. 3.0 to 4.0

d. 4.0 to 5.0

28. Compression of soil occurs rapidly if voids are filled with

a. air

b. water

c. partly with air and partly with water

d. none of these.

29. In non-cohesive soil in passive state of plastic equilibrium

a. major principal stress is horizontal

b. minor principal stress is vertical

c. major principal stress is vertical

d. minor and major principal stresses are equally inclined to the horizontal.

30. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

31. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

32. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

33. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

34. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

35. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

36. For the construction of buildings, the subheads of the estimate are

a. Brickwork, Stone work, Roofing

b. Brickwork Flooring, Wood work, Steel work

c. Plastering or pointing, finishing, water supply and sanitary work

d. All the above.

37. Due to change in price level, a revised estimate is prepared if the sanctioned estimate exceeds

a. 2.0%

b. 2.5%

c. 4.0%

d. 5.0%

38. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

39. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

40. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

41. Sketching is usually done:

a. with French curves

b. free hand

c. with drafting machine

d. with protractor

42. The shades and shadows are :

a. the same

b. the different

c. the result of object on the ground

d. the result of light on the ground

43. The angle between two perpendicular line is

a. 0°

b. 45°

c. 90°

d. 180°

44. Axonometric drawing is a drawing like

a. parallel to the plane

b. perpendicular to the plane

c. inclined to the plane

d. all of the above

45. Earning per share is the most important ratio for

a. share holders

b. banks

c. company's management

d. All of these

46. Pick up the correct statement from the following:

a. The ratio of current assests, loans and advances, and the current liquidity is called current ratio.

b. Larger the current ratio, larger is the margin of safety.

c. The operating profit is the difference between gross profit and operating expenses.

d. All of these

47. Liquidity ratios are used :

a. to measure a firms ability to meet short-cut obligations.

b. to compare short term obligations to short-term resources available to meet these obligations.

c. to obtain much insight into the present cash solvency of the firm and the firm's ability to remain solvent in the event of adversity.

d. All of these

48. For the registration of foregineer engineers the amount of fee required is:

a. NRS 3000/-

b. NRS 5000/-

c. NRS 10000/-

d. NRS 15000/-

49. Maximum working hour for a labour per week should be

a. 42hr

b. 45hr

c. 48hr

d. 53hr

50. Integrity is

a. state or quality of being complete, undivided or unbroken moral soundness, honesty & uprightness

b. the wholeness of deriving quality

c. both a) and b) of above

d. none of the above