***Engineer***

1. There are two hinged semicircular arches A, B and C of radii 5 m, 7.5 m and 10 m respectively and each carries a concentrated load W at their crowns. The horizontal thrust at their supports will be in the ratio of

a. 1 : 1 1/2 : 2

b. 2 : 1 1/2 : 1

c. 1 : 1 : 2

d. none of these.

2. The load on a spring per unit deflection, is called

a. stiffness

b. proof resilience

c. proof stress

d. proof load.

3. The general expression for the B.M. of a beam of length l is the beam carries

a. a uniformly distributed load w/unit length

b. a load varying linearly from zero at one end to w at the other end

c. an isolated load at mid span

d. none of these.

4. If a rectangular beam measuring 10 x 18 x 400 cm carries a unformly distributed load such that the bending stress developed is 100 kg/cm2. The intensity of the load per metre length, is

a. 250 kg

b. 260 kg

c. 270 kg

d. 280 kg.

5. For a simply supported beam of length L, the bending moment M is described as M = a (x - x3/L2), 0 ≤ x < L; where a is a constant. The shear force will be zero at

a. the supports

b. x = L/2

c. x = L/3

d. x = L/3

6. Pick up the correct statement from the following :

a. A level line lies in level surface

b. A horizontal surface is normal to the direction of gravity at only one point

c. A horizontal line is tangential to the level surface

d. All the above.

7. The apparent error on reversal is

a. equal to the actual error

b. twice the actual error

c. thrice the actual error

d. none of these.

8. Stadia techeometry was discovered by James Watt in the year.

a. 1670

b. 1770

c. 1870

d. 1900.

9. Mistakes which may produce a very serious effect upon the final results arise due to

a. in attention

b. in experience

c. carelessness

d. all of these.

10. The reduced bearing of a line is N 87° W. Its whole circle bearing is

a. 87°

b. 273°

c. 93°

d. 3°

11. The sum of the interior angles of a geometrical figure laid on the surface of the earth differs from that of the corresponding plane figure only to the extent of one second for every

a. 100 sq. km of area

b. 150 sq. km of area

c. 200 sq. km of area

d. none of these.

12. Pick up the correct statement from the following :

a. if the image of the object does not fall on the plane of the cross-lines, parallax exists

b. parallax has nothing to do with the eyepiece

c. the eyepiece is adjusted for clear vision of the cross hairs

d. all the above.

13. For high grade instruments the steel preferred to, is

a. cast steel

b. bessemer steel

c. mild steel

d. whitworth compressed steel.

14. The softest rock is

a. marble

b. diamond

c. talc

d. quartz.

15. In stone masonry, stones (stratified rocks) are so placed that the direction of pressure to the plane of bedding is

a. right angles

b. 45°

c. 60°

d. parallel

16. The initial setting time of lime-pozzolana, is

a. 30 minutes

b. 60 minutes

c. 90 minutes

d. 120 minutes.

17. The silica is used for preparing

a. silica bricks

b. coke oven

c. lining for glass furnaces

d. all of these.

18. The frog of a brick is normally made on its

a. top face

b. bottom face

c. longer face

d. shorter side.

19. Higher workability of concrete is required if the structure is

a. made with cement concrete

b. thick and reinfored

c. thin and heavily reinforced

d. thick and heavily reinfored.

20. Pick up the correct statement from the following:

a. Water and aggregates are measured in litres

b. The finished concrete is measured in cubic metres

c. 20 bags of cement make one tonne

d. All the above.

21. For preparing ordinary concrete, the quantity of water used, is

a. 10% by weight of aggregates plus 10% of weight of cement

b. 5% by weight of aggregates plus 30% of weight of cement

c. 30% by weight of aggregates plus 10% of weight of cement

d. none of these.

22. The process of mixing, transporting, placing and compacting concrete using Ordinary Port land Cement should not take more than

a. 30 minutes

b. 40 minutes

c. 60 minutes

d. 75 minutes

23. Pick up the incorrect statement from the following. While performing preliminary test on concrete

a. cement should be mixed by hand in order to maintain uniformity

b. concrete mix should be stored in air-tight containers

c. concrete ingredients should be kept at a temperature of 37° ± 2°C

d. none of these.

24. If a soil undergoes a change in shape and volume by application of external loads over it, but recovers its shape and volume immediately after removal of the load, the property of the soil is said to be

a. Resilience of soils

b. Elasticity of soils

c. Compressibility of soils

d. None of these.

25. According to IS : 2720 - 1965, the composition of a dispersing solution used in pipette analysis for determining the size of particles, is

a. sodium-hexametaphosphate 33 g, sodium carbonate 7 g and distilled water one litre

b. sodium-hexametaphosphate 7 g, sodium carbonate 33 g and distilled water one litre

c. sodium-hexametaphosphate 23 g, sodium carbonate 17 g and distilled water one litre

d. none of these.

26. Depending upon the properties of a material, the failure envelope may

a. be either straight or curved

b. pass through the origin of stress

c. intersect the shear stress axis

d. all the above.

27. Pick up the correct statement from the following:

a. When water table is above the base of a footing, the dry weight m should be used for soil below water table

b. When water table is located somewhat below the base of a footing, the elastic wedge is partly of moist soil and partly of submerged soil, and a suitable reduction factor is used

c. When water table is just at the base of the footing, no reduction factor is used

d. None of these.

28. The shear resistance of a soil is con-tituted basically of the following component.

a. The frictional resistance to translocation between the individual soil particles at their contact point

b. To the structural relation to displacement of the soil because of the interlocking of the particles

c. Cohesion and adhesion between the surfaces of the soil particles

d. All the above.

29. Tergazhi's theory of one dimensional consolidation assumes

a. water and soil particles are incompressible

b. deformation of the soil, is entirely due to change in volume

c. Darcey's law for the velocity of flow of water through soil, is perfectly valid

d. all the above.

30. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

31. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

32. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

33. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

34. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

35. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

36. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

37. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

38. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

39. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

40. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet.

41. The projection of any feature on the horizontal plane is called

a. plan

b. elevation

c. map

d. all of the above

42. In orthographic projection which one have all three views same

a. cube

b. circle

c. sphere

d. both (a) and (c) of above

43. The principal of oblique view drawing is to keep:

a. width axes at 90°

b. width and height axes at 90°

c. width, height and depth axes at 90°

d. all of the above

44. Picture plane is

a. transparent

b. opaque

c. semi transparent

d. all of the above

45. If a is the base amount expenditure, b is the increase in the operation cost each year over a period of n years, the total cost of maintenance is :

a. a + (n + 1) b

b. a + (n - 1) b

c. a x (n - 1) b

d. a - (n - 1) b

46. Pick up the correct statement from the following:

a. A NPV profile graph shows the curvilinear relationship between the net present value of the project and discount rate employed.

b. In a NPV profile, if discount rate is zero, then net present value is simply total cash inflows less the total cash outflows of the project.

c. As the discount rate increases, the net present value profile slopes downward to the right.

d. All of these

47. Annuities involve:

a. all payments of equal amount

b. payment at equal time intervals

c. payments at the end of periods.

d. All of these

48. The technology applicable for the development in developing countries is

a. appropriate technology

b. high technology

c. low technology

d. information technology

49. Time extension may need to be granted due to

a. causes by force majeure

b. causes by purchaser

c. both a) and b) of above

d. none of the above

50. Any Non-Nepali Engineer desiring to work in Nepal may apply for registration under:

a. Category A

b. Category B

c. Category C

d. Category D