***Engineer***

1. The greatest load which a spring can carry without getting permanently distorted, is called

a. stiffness

b. prof resilience

c. proof stress

d. proof load.

2. In plastic analysis, the shape factor for a triangular section, is

a. 1.5

b. 1.34

c. 2.34

d. 2.5

3. The ratio of shear stress and shear strain of an elastic material, is

a. Shear Modulus

b. Young's Modulus

c. Modulus of Elasticity

d. both (a) and (b).

4. For the beam shown in below figure, the maximum positive bending moment is nearly equal to negative bending moment when L1 is equal to

a. 1.0 L

b. 0.7 L

c. 0.5 L

d. 0.35 L.

5. The neutral axis of a beam cross-section must

a. pass through the centroid of the section

b. be equidistant from the top of bottom films

c. be an axis of symmetry of the section

d. none of these.

6. Prismatic compass is considered more accurate than a surveyor's compass, because

a. it is provided with a sliding glass in the object vane

b. its graduations are in whole circle bearings

c. it is provided with a prism to facilitate reading of its graduated circle

d. both (c) and (d).

7. The magnetic bearing of a line is 32° and the magnetic declination is 10° 15' W. The true bearing is

a. 21° 45'

b. 42° 15'

c. 42° 15' W

d. 21° 45' W

8. In chain surveying, perpendiculars to the chain line, are set out by

a. a theodolite

b. a prismatic compass

c. a level

d. an optical square

9. The bubble tube is nearly filled with

a. alcohol or chloroform

b. a liquid which is very mobile

c. a liquid having low freezing point

d. all the above.

10. The whole circle bearing of a line is 290°. Its reduced bearing is

a. N 20° E

b. N 20° W

c. N 70° W

d. S 70° E

11. For preparation of a contour plan for a route survey

a. method of squares is used

b. method of trace contour is used

c. method of cross profile is used

d. indirect method of contouring is used.

12. The most reliable method of plotting a theodolite traverse, is

a. by consecutive co-ordinates of each station

b. by independent co-ordinates of each station

c. by plotting included angles and scaling off each traverse leg

d. by the tangent method of plotting.

13. The PVC doors and windows are preferred as they are

a. rot proof

b. termile proof

c. water proof

d. all of these.

14. A piece of timber whose thickness and width are respectively 5 cm and 10 cm is called

a. slate

b. plank

c. board

d. strip.

15. The operation of removal of impurities or clay adhering to iron ores, is known as

a. dressing

b. calcination

c. roasting

d. smelting.

16. Knots in timber are

a. defects caused by crushing fibres

b. splits radiating from the centre

c. speckled strains

d. signs of branches cut off.

17. Refractory bricks are

a. neutral refractory bricks

b. acid refractory bricks

c. basic refractory bricks

d. all the above.

18. Pick up the correct statement from the following:

a. Blistering may be cured by applying water paint finished with oil paint dried with a little copal varnish

b. Cracked paints may be cured by removing paint and giving a fresh coat of paint

c. Crawling paints may be cured by sand preparing the surface and giving a fresh coat with plenty of turps

d. All the above.

19. For a concrete mix 1:3:6 and water cement ratio 0.6 both by weight, the quantity of water required per bag, is

a. 12 kg

b. 14 kg

c. 16 kg

d. 20 kg

20. Separation of water or water sand cement from a freshly concrete, is known

a. bleeding

b. creeping

c. segregation

d. flooding

21. Pick up the incorrect statement from the following:

a. Concrete for which preliminary tests are conducted, is called controlled concrete

b. Bulking of sand depends upon the fineness of grains

c. Concrete mix 1 : 6 : 12, is used for mass concrete in piers.

d. All the above.

22. The maximum amount of dust which may be permitted in aggregates is

a. 5% of the total aggregates for low workability with a coarse grading

b. 10% of the total aggregates for low workability with a fine grading

c. 20% of the total aggregates for a mix having high workability with fine grading

d. all the above.

23. The ratio of various ingredients (cement, sand, aggregates) in concrete of grade M 200, is

a. 1 : 3 : 6

b. 1 :   : 3

c. 1 : 1 : 2

d. 4 : 2 : 1

24. Pick up the incorrect definition from the following:

a. The rotation of soil particles into stable state while remoulding, is known as the thiostropy of soil

b. The water content at which a soil changes from the liquid state to solid state, is known liquid limit of the soil

c. The water content at which a soil flows, is known plastic limit of the soil

d. None of these.

25. If the coefficients of volume change and compressibility of a soil sample are respectively 6.75 x 10-2 and 3 x 10-2, the void ratio of the soil sample, is

a. 1.15

b. 1.20

c. 1.25

d. 1.30

26. A grillage foundation

a. is provided for heavily loaded isolated columns

b. is treated as spread foundation

c. consists of two sets of perpendicularly placed steel beams

d. all the above.

27. The critical exist gradient of seepage water in soils, is

a. inversely proportional to the specific gravity

b. directly proportional to the specific gravity

c. inversely proportional to the voids ratio

d. none of these.

28. If the cohesive force, (c), is 1.5 t/m2, the density (γ) of the soil is 2.0 t/m3, factor of safety (F) is 1.5 and stability factor (Sn) is 0.05, the safe height of the §lope, is

a. 5 metres

b. 8 metres

c. 10 metres

d. 12 metres

29. For slopes of limited extent the surface of slippage, is usually along

a. a parabolic arc

b. an elliptical arc

c. a straight line

d. a circular arc.

30. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

31. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

32. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

33. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

34. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

35. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

36. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

37. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

38. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

39. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

40. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

41. Angles in multiples of 15 degree are constructed by the combined use of

a. T-square and set-square

b. set-square and scale

c. scale and french curve

d. french curve and erasing shield

42. Ground plane is

a. horizontal plane

b. vertical plane

c. inclined plane

d. none of the above

43. The art of representation of an object by systemic lines on a paper is known as

a. drawing

b. art

c. dimensioning

d. profession

44. Dashed (dotted) line is used for indication

a. edge of the object

b. hidden edge of the object

c. centre line of the object

d. all of the above

45. The estimator for definitive estimates must be able :

a. and an all-around construction expert.

b. to read the plans and specifications to determine accurate quantities of permanent materials and installed Equipment.

c. to express the job material requirements in dimensions suitable for costing and construction supplies.

46. In a cash-flow diagram :

a. Time 1 is considered to be the end of time period 1

b. A vertical arrow pointing up indicates a positive cash flow

c. An arrow pointing downward indicates a negative cash flow

d. All of these

47. Which method is adopted to develop an approximate or conceptual estimate for perimeter works for buildings from the following:

a. Cost per function method

b. Cost per square metre method

c. Cost per cubic metre method

d. Cost per linear metre method

48. "Corrupt practice" means

a. offering/receiving anything to influence the action of public official

b. buy more than one bid document

c. mis representation of facts in order to influence a procurement process

d. all of the above

49. Fundamental principle of Professional Engineering Ethics is:

a. uploading and advancing Engineering Profession

b. maintain high level of ethical conduct

c. both of the above

d. none of the above

50. In Nepal, which of the following Act contains the provisions o health and safety of the workers ?

a. contract act

b. Industry act

c. labour act

d. all of the above