***Engineer***

1. At yield point of a test piece, the material

a. obeys Hooke's law

b. behaves in an elastic manner

c. regains its original shape on removal of the load

d. undergoes plastic deformation.

2. Gradually applied static loads do not change with time their

a. magnitude

b. direction

c. point of application

d. all the above.

3. The normal and tangential components of stress on an inclined plane through θ° to the direction of the force, will be equal if θ is

a. 45°

b. 30°

c. 60°

d. 90°

4. A reinforced concrete beam is assumed to be made of

a. homogeneous material

b. heterogeneous material

c. isotropic material

d. none of these.

5. The phenomenon of slow growth of strain under a steady tensile stress, is called

a. yielding

b. creeping

c. breaking

d. none of these.

6. Cross-staff is used for

a. setting out right angles

b. measuring contour gradient

c. taking levels

d. measuring distances

7. The difference of level between a point below the plane of sight and one above, is the sum of two staff readings and an error would be produced equal to

a. the distance between the zero of gradient and the foot of the staff

b. twice the distance between the zero of graduation and the foot of the staff

c. thrice the distance between the zero of graduation and the foot of the staff

d. none of the above.

8. The included angles of a theodolite traverse, are generally measured

a. clockwise from the forward station

b. anti-clockwise from the back station

c. anti-clockwise from the forward station

d. clockwise from the back station.

9. Diurnal variation of magnetic declination is

a. greater at equator than nearer the poles

b. less at equator than nearer the poles

c. less in summer than in winter

d. same at all latitudes and during different months.

10. The angle between two plane mirrors of optical square, is

a. 30°

b. 45°

c. 60°

d. 90°.

11. The ratio of the length of long chord and the tangent length of a circular curve of radius R deflecting through angle Δ, is

a. cos

b. tan

c. 2 sin

d. 2 cos .

12. The smaller horizontal angle between the true meridian and a survey line, is known

a. declination

b. bearing

c. azimuth

d. dip.

13. Teak wood is suitable for

a. sports articles

b. furnitures

c. railway sleepers

d. all the above.

14. Based on its dry weight, a freshly felled tree may contain water

a. 25%

b. 50%

c. 75%

d. 100%.

15. During pudding

a. carbon is converted into carbonic acid gas

b. silicon forms a slag

c. metal is heated by the burning of gases

d. all the above.

16. If the furnace is provided with insufficient fuel at low temperatures, the type of pig iron produced, is called

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

17. Second class bricks

a. are of dark brown colour

b. produce a metallic sound when struck

c. are well burnt

d. are under burnt.

18. PVC stands for

a. plastic very compact

b. polythene vinyl chloride

c. polythene vinyl carbon

d. polythene vanadium carbide.

19. If the average compressive strength is 4000 kg/cm2 and standard deviation is 500, the co-efficient of variation is

a. 12.5%

b. 15%

c. 18.5 %

d. 20%

20. The aggregate containing moisture in pores and having its suface dry, is known as

a. very dry aggregates

b. dry aggregates

c. saturated surface dry aggregate

d. none of these.

21. Addition of pozzolana to cement

a. increases strength

b. increases heat of hydration

c. decreases curing time

d. none of these.

22. For the construction of R.C.C. slabs, columns, beams, walls, etc. the grade of concrete mix used, is

a. 1 :  : 3

b. 1 : 2 : 4

c. 1 : 1 : 2

d. 1 : 4 : 8.

23. Concrete gains strength due to

a. chemical reaction of cement with sand and coarse aggregates

b. evaporation of water from concrete

c. hydration of cement

d. All the above.

24. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

25. Pick up the correct statement from the following:

a. The freezing of water is accompained by a volume increase of 9%.

b. Below freezing point, higher soil suction develops.

c. The magnitude of frost heave decreases as the degree of saturation of soil decreases

d. All the above.

26. Pick up the correct statement from the following:

a. The dry density reduces by addition of water after attaining optimum moisture content

b. The line joining the peak of three moisture content graphs obtained by using three compactive energies, is called line of optimus

c. Well graded coarse grained soils can be compacted to a very high density as compared to fine grained soils

d. All the above. .

27. A flow net may be utilised for the determination of

a. seepage

b. hydrostatic pressure

c. seepage pressure

d. all the above.

28. Pick up the correct statement from the following:

a. Kaolinite shows a very little sign of swelling on wetting

b. Kaolinite when wet, becomes moderately plastic because negative electro magnetic charges on platelets attrack water

c. Kaolinite is also called China clay

d. All the above.

29. Pick up the correct statement from the following:

a. The porosity of a soil is defined as the ratio of the volume of voids to the gross volume of the soil

b. The bulk density of a soil is defined as the unit weight of the soil

c. The dry density of a soil is defined as weight of solids to the total volume of the soil

d. All the above.

30. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

31. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

32. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

33. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

34. Pick up the correct statement from the following with regards to C.P.M. network analysis of projects

a. Earliest occurrence time of the event from which the activity arrow originates plus the duration of the activity, is called earliest finish time of the activity

b. The latest occurrence time of the node of which the activity arrow terminates minus the duration of the activity, is called latest start time

c. The latest occurrence time for the node at which the activity arrow terminates, is called latest finish time

d. All the above.

35. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

36. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

37. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

38. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

39. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

40. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

41. Which one is not drafting equipment ?

a. drafter

b. dust cloth

c. scale

d. None of the above

42. The unseen/inner edge of an object is represented by

a. solid line

b. dotted line

c. hatch line

d. none of the above

43. To draw very light line which pencil is used

a. B

b. HB

c. H

d. 2H

44. Hatching line is a line which makes an angle of 45° with

a. horizontal

b. vertical

c. main line of the section

d. all of the above

45. Present worth Annuity (PWA) is generally known as

a. Premium annuities

b. Income annuities

c. Future annuities

d. All of these

46. The wages of supervisors and material handlers are charged as :

a. Over head

b. direct labour cost

c. indirect labour cost

d. None of these

47. The financial analysis :

a. helps a share holder to compare the expected return on his investment in the firm against the expected return from other alternative investment.

b. helps a bank to know the financial position of the firm for granting a loan to the firm.

c. helps to judge the success of the firm's financial plans.

d. All of these.

48. In Nepal, which of the following act contains the provisions on health and safety of the workers?

a. contract act

b. industry act

c. labour act

d. all of the above

49. The Engineering Council members consist of the following members now more

a. 12

b. 14

c. 16

d. 18

50. The space required for living the worker is 15m with considering the maximum height of room above floor level.

a. 3m

b. 3.5m

c. 4m

d. 5 m