***Engineer***

1. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

2. If ΣH and ΣV are the algebraic sums of the forces resolved horizontally and vertically respectively, and ΣM is the algebraic sum of the moments of forces about any point, for the equilibrium of the body acted upon

a. ΣH = 0

b. ΣV = 0

c. ΣM = 0

d. all the above.

3. The ratio of tangential and normal components of a stress on an inclined plane through θ° to the direction of the force, is :

a. cos θ

b. tan θ

c. cos θ

d. sec θ

4. According to Unwin's formula, the diameter d of a rivet of plate of thickness t is :

a. d = 6.05 t

b. d = 1.5 t + 4

c. d = 5 t

d. d = t + 1.5

5. At either end of a plane frame, maximum number of possible transverse shear forces, are

a. one

b. two

c. three

d. four

6. If h is the difference in level between end points separated by l, then the slope correction is  . The second term may be neglected if the value of h in a 20 m distance is less than

a.  m

b. 1 m

c. 2 m

d. 3 m

7. If a linear traverse follows a sharp curve round a large lake where it is difficult to have long legs, the accuracy of the traverse may be improved by

a. taking short legs

b. making repeated observations of angular and linear measurements

c. making a subsidiary traverse to determine the length of a long leg

d. all the above.

8. For orientation of a plane table with three points A, B and C, Bessel's drill is

a. Align b through a and draw a ray towards c, align a through b and draw a ray towards c, finally align c through the point of intersection of the previously drawn rays

b. Align c through a and draw a ray towards b, align a through c and draw a ray towards b, finally align b through the point of intersection of the previously drawn rays

c. Align c through b and draw a ray towards a, align b through c and draw a ray towards a, finally align a, through the point of intersection of the previously, drawn rays

d. In the first two steps any two of the points may be used and a ray drawn towards the third point, which is sighted through the point of intersection of previously drawn rays in the final step.

9. The direction of steepest slope on a contour, is

a. along the contour

b. at an angle of 45° to the contour

c. at right angles to the contour

d. none of these.

10. The back staff reading on a B.M. of R.L. 500.000 m is 2.685 m. If foresight reading on a point is 1.345 m, the reduced level of the point, is

a. 501.345 m

b. 501.340 m

c. 504.030 m

d. 502.585 m.

11. Short offsets are measured with

a. an ordinary chain

b. an invar tape

c. a metallic tape

d. a steel tape.

12. The apparent error on reversal is

a. equal to the actual error

b. twice the actual error

c. thrice the actual error

d. none of these.

13. Dorry's testing machine is used for

a. crushing test of stone

b. hardness test of stone

c. impact test of stone

d. water absorption test.

14. Asbestos

a. is a natural fibrous mineral substance

b. is composed of hydrous silicates of calcium and magnesium (CaSiO3, 3MgSiO3)

c. contains iron oxide and alumina

d. all the above.

15. German silver is an alloy of

a. silver, gold and lead

b. copper, nickel and zinc

c. copper, brass and zinc

d. brass, silver and zinc.

16. A rock contains only one mineral. It is called

a. homogeneous

b. non-homogeneous

c. monomineralic

d. polymineralic.

17. Black marble is generally found in the district of

a. Jaipur

b. Jabalpur

c. Jaisalmer

d. Pune.

18. Pick up the correct statement from the following:

a. In basic Bessemer process, the steel heats the converter

b. In open-hearth process, the furnace heats the steel

c. In Siemens process, the impurities of pig iron are oxidised by the oxygen of the ore

d. all the above.

19. Proper batching ensures

a. durability

b. workability

c. strength

d. all the above.

20. For quality control of Portland cement, the test essentially done is

a. soundness

b. tensile strength

c. consistency

d. all the above.

21. Pick up the correct statement from the following:

a. Expansion joints are provided to accommodate thermal expansion

b. Construction joints are provided to control shrinkage cracks

c. Expansion joints need not be provided in foundation concrete

d. All the above.

22. To obtain a very high strength concrete, use very fine grained

a. Granite

b. Magnetite

c. Barite

d. Volcanic scoria.

23. The high strength of rapid hardening cement at early stage, is due to its

a. finer grinding

b. burning at high temperature

c. increased lime cement

d. higher content of tricalcium.

24. Sedimentation analysis is based on the assumption:

a. soil particles are spherical

b. particles settle independent of other particles

c. walls of the jar do not affect the settlement .

d. all the above.

25. You are given a sample of soil containing coarse grains to determine its water content, you will use

a. pycnometer

b. oven-drying method

c. calcium carbide method

d. alcohol method.

26. According to Coulomb's wedge theory, the active earth pressure slides the wedge

a. down and outwards on a slip surface

b. up and inwards on a slip surface

c. horizontal upward and parallel to base

d. horizontal inward and parallel to base.

27. The compressibility of clays, is caused due to:

a. expulsion of double layer water from in between the grains

b. sliping of particles to new positions of greater density

c. bending of particles as elastic sheets

d. all the above.

28. The vane shear test is used for the in-situ determination of the undrained strength of the intact fully saturated

a. sands

b. clays

c. gravels

d. highly organic soils.

29. The consolidation time for soils

a. decreases with increasing permeability

b. increases rapidly with increasing size of soil mass

c. is independent of the magnitude of the stress change.

d. All the above.

30. An event is indicated on the network by a number enclosed in

a. a square

b. a triangle

c. an ellipse

d. all the above.

31. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

32. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

33. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

34. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

35. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

36. For the construction of buildings, the subheads of the estimate are

a. Brickwork, Stone work, Roofing

b. Brickwork Flooring, Wood work, Steel work

c. Plastering or pointing, finishing, water supply and sanitary work

d. All the above.

37. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

38. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

39. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

40. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

41. The principal of oblique view drawing is to keep:

a. width axes at 90°

b. width and height axes at 90°

c. width, height and depth axes at 90°

d. all of the above

42. Working drawings:

a. must provide complete information to produce the object

b. need not provide complete information

c. should be supplemented by further details TOPS

d. should be supplemented by specifications

43. Topographic map is used to represent the

a. artificial detail

b. natural detail

c. construction detail

d. all of the above

44. The scale should never be used as a ........ for drawing straight lines.

a. set-square

b. working edge

c. straight edge

d. none of the above

45. Pick up the correct statement from the following:

a. The ability of a company to meet obligations which are likely to mature in short term, is called liquidity.

b. The liquidity ratio may be defined as a relationship of current liabilities and current assests and advances.

c. The liquidity ratios are used to indicate the financial position of the firm.

d. All of these

46. Which one of the following questions is relevant to the construction estimates :

a. Did the estimators precisely evaluate site conditions ?

b. Did the estimators use short cut methods which may be un realistic in their situation ?

c. How much money will the contractor's risk, loosing if he Were to submit bid on the raw estimate of cost.

d. All of these

47. Keeping in view, the feasibility order of magnitude, the preliminary, conceptual or budget estimates, are prepared by :

a. construction manager

b. owner himself/herself

c. construction manager

d. None of these

48. For the registiration of professional engineer amount fee required is:

a. NRS 5000/-

b. NRS 10000/-

c. NRS 15000/-

d. NRS 20000/-

49. What is the provision in Engineer Council Act and Regulation in Nepal for the punishment na person practices Engineering work without registering into the Nepal Engineering Council?

a. Rs 1000 find or imprisonment of 1 month or both

b. Rs 5000 fine

c. Rs10000 fine

d. Rs3000 fine or Imprisonment of 3 month of both

50. Which one is not concerned with the NEC

a. licensing (Registration) of Engineers

b. accreditation of certificates of academic qualifications

c. recognition of the academic institution

d. authority of opening engineering college