***Engineer***

1. Principal planes are subjected to

a. normal stresses only

b. tangential stresses only

c. normal stresses as well as tangential stresses

d. none of these.

2. A composite beam is composed of two equal strips one of brass and other of steel. If the temperature is raised

a. brass experiences compressive force

b. composite beam gets subjected to a couple

c. composite beam bends

d. All the above.

3. At any point of a beam, the section modulus may be obtained by dividing the moment of inertia of the section by

a. depth of the neutral axis

b. maximum tensile stress at the section

c. maximum compressive stress at the section

d. none of these.

4. The shear force on a simply supported beam is proportional to

a. sum of the forces

b. sum of the transverse forces

c. algebraic sum of the transverse forces of the section

d. curvature of the neutral axis.

5. If a constant section beam is subjected to a uniform bending moment throughout, its length bends to

a. a circular arc

b. a parabolic arc

c. a catenary

d. none of these.

6. Imaginary line passing through points having equal magnetic declination is termed as

a. isogon

b. agonic line

c. isoclinic line

d. none of these.

7. In an adjusted level, when the bubble is central, the axis of the bubble tube becomes parallel to

a. line of sight

b. line of collimation

c. axis of the telescope

d. None of these.

8. The boundary of water of a still lake, represents

a. level surface

b. horizontal surface

c. contour line

d. a concave surface.

9. The tangent to the liquid surface in a level tube, is parallel to the axis of the level tube at

a. every point of the bubble

b. either end of the bubble

c. the mid-point of the bubble

d. no where.

10. Locating the position of a plane table station with reference to three known points, is known as

a. intersection method

b. radiation method

c. resection method

d. three point problem.

11.  is a conventional sign of

a. mosque

b. temple

c. church

d. idgah.

12. The combined effect of curvature and refraction over a distance L kilometres is

a. 67.2 L2 mm

b. 76.3 L2 mm

c. 64.5 L2 mm

d. none of these.

13. Generally wooden moulds are made from

a. shisham wood

b. deodar wood

c. teak wood

d. hard wood.

14. Good quality stones must

a. be durable

b. be free from clay

c. resist action of acids

d. all the above.

15. Bituminous fells are used for

a. covering A.C. sheets

b. covering sloping roofs

c. D.P.C.

d. none to these.

16. For sanitary pipes and chemical stonewares,

a. salt glazing is used

b. lead glazing is used

c. opaque glazing is used

d. None of these.

17. The frog of a brick is normally made on its

a. top face

b. bottom face

c. longer face

d. shorter side.

18. Fibre glass

a. has a higher strength to weight ratio

b. is shock proof and fire retardent

c. does not decay

d. all the above.

19. The factor which affects workability, is

a. shape and size of the aggregates

b. grading and surface textures of the aggregates

c. air entraining agents

d. all the above.

20. Separation of coarse aggregates from mortar during transportation, is known

a. creeping

b. segregation

c. shrinkage

d. none of these.

21. Slump test of concrete is a measure ofits

a. consistency

b. compressive strength

c. tensile strength

d. impact value.

22. Addition of pozzolana to cement causes

a. loss of heat of hydration

b. reduction in bleeding

c. increase in curing time

d. all the above.

23. Water required per bag of cement, is

a. 14 kg

b. 21 kg

c. 28 kg

d. 35 kg

24. Geologic cycle for the formation of soil, is

a. Weathering → upheaval → transportation → deposition

b. Transportation → upheaval → weathering → deposition

c. Weathering → transportation → deposition → upheaval

d. None of these.

25. A compacted soil sample using 10% moisture content has a weight of 200 g and mass unit weight of 2.0 g/cm3. If the specific gravity of soil particles and water are 2.7 and 1.0, the degree of saturation of the soil is

a. 11.1%

b. 55.6%

c. 69.6%

d. none of these.

26. The ratio of the weight of water to the weight of solids in a given mass of soil, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

27. Sedimentation analysis is based on the assumption:

a. soil particles are spherical

b. particles settle independent of other particles

c. walls of the jar do not affect the settlement .

d. all the above.

28. For determing the specific gravity of soil solids, using a pycnometer of 500 cc., the following data is available : Weight of dry empty pycnometer = 125 g Weight of dried soil and pycnometer = 500 g Weight of dried soil and distilled = 850 g water filled in pycnometer up to top The specific gravity of soil soilds, is

a. 2.0

b. 2.25

c. 2.50

d. 2.75

29. The minimum water content at which the soil retains its liquid state and also possesses a small shearing strength against flowing, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

30. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

31. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

32. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

33. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

34. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

35. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

36. The expected out turn of half brick partition wall per mason per day is

a. 1.5 m3

b. 2.0 m3

c. 4.0 m2

d. 5.0 m2

37. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

38. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

39. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

40. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

41. Working edge of drawing board is kept ........... of the user.

a. left

b. right

c. ahead

d. all of the above

42. Horizontal lines are drawn

a. left to right

b. right to left

c. top to bottom

d. both (a) and (b) above

43. In engineering drawing, auxiliary views are used to show details of

a. incline surface

b. horizontal surface

c. vertical surface

d. none of the above

44. In orthographic projection, the projection's are always .......... to the plane of projection

a. perpendicular

b. parallel

c. 60°

d. 30°

45. Pick up the correct statement from the following:

a. The capital required to get a project started, is called first cost.

b. The costs associated with a new or existing project that remain unaffected by the changes in activity level over the normal range of operation of the project, are called fixed costs.

c. The group of costs that vary proportionately to the changes in the activity level of a new or existing project are called variable costs.

d. All of these

46. The annuity which refers to a debt payment for recovering the initial amount or capital in equal periodical payments, is known as;

a. Present Worth Annuity

b. Sinking fund annuity

c. Compound annuity

d. Capital recovery annuity

47. The sunk costs include :

a. a past expenditure

b. an unrecovered balance

c. an invested capital that cannot be retreived

d. All of these

48. For the registiration of professional engineer amount fee required is:

a. NRS 5000/-

b. NRS 10000/-

c. NRS 15000/-

d. NRS 20000/-

49. According to labour act considering health condition is

a. first aid after accident

b. safety on inst. and cloth

c. education& training

d. none of the above

50. Engineer shall not disclose, without...............confidential information concerning the business affairs or technical processes of any present or former client or employer or public body on which the serve:

a. editing

b. getting paid

c. remuneration

d. consent