***Engineer***

1. The force in BC of the truss shown in the given figure, is

a. 3.0t tension

b. t tension

c. t compression

d. None of these.

2. Pick up the correct statement from the following:

a. For a uniformly distributed load, B.M. curve is a parabola

b. For a load varying linearly, the shear force curve is a parabola

c. For a load varying linearly, the B.M. curve is a cubic parabola

d. All the above.

3. A cantilever of length 2 cm and depth 10 cm tapers in plan from a width 24 cm to zero at its free end. If the modulus of elasticity of the material is 0.2 x 106 N/mm2, the deflection of the free end, is

a. 3 mm

b. 4 mm

c. 5 mm

d. 6 mm

4. The moment diagram for a cantilever carrying linearly varying load from zero at its free end and to maximum at the fixed end will be a

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

5. Rankine-Golden formula accounts for direct as well as buckling stress and is applicable to

a. long columns

b. short columns

c. intermediate columns

d. all the above.

6. Probable systematic error in precise levelling as recommended by International Geodetic Association should not exceed (where k is in kilometers.)

a. ± 0.1 k mm

b. ± 0.2 k mm

c. ± 0.1 k

d. 0.2 k mm.

7. Contour lines of different elevations can unite to form one line, only in the case of

a. a vertical cliff

b. a saddle

c. a water shed line

d. a hill top.

8. Pick up the correct statement from the following :

a. spherical aberration may be minimised by replacing the single lens by a combination of the lenses.

b. in telescope objectives, a combination of convex lens and concave lens is used.

c. in eyepieces, two plano-convex lenses placed at a certain distance apart are used

d. all the above.

9. Greater accuracy in linear measurements, is obtained by

a. tacheometry

b. direct chaining

c. direct taping

d. all the above.

10. In levelling operation,

a. The first sight on any change point is a back sight

b. The second sight on any change point is a fore sight

c. The line commences with a fore sight and closes with a back sight

d. The line commences with a back sight and closes with a foresight.

11. Cross hairs in surveying telescopes, are fitted

a. in the objective glass

b. at the centre of the telescope

c. at the optical centre of the eye piece

d. in front of the eye piece.

12. Which one of the following mistakes/errors may be cumulative + or - :

a. bad ranging

b. bad straightening

c. erroneous length of chain

d. sag.

13. Blister steel

a. is full of fissures and cavities

b. can not be forged

c. can be easily welded

d. all the above.

14. For melting one tonne of cast iron

a. 700 m3 air is required

b. 20 kg limestone is required

c. one quintal coke is required

d. all the above.

15. Calcination of iron ores is done

a. to remove carbonic acid

b. by roasting in heaps

c. after dressing

d. all the above.

16. Stones used for the construction of retaining walls must be

a. soft

b. hard

c. light

d. heavy.

17. Pick up the correct statement from the following:

a. During rusting, first ferrous bicarbonates are formed

b. On further oxidation ferrous bicarbonates get converted to ferric bicarbonates

c. Ultimately hydrated ferric oxide is formed during rusting and carbon dioxide gets liberated

d. All the above.

18. Seasoning of timber is done for

a. increasing moisture content

b. decreasing moisture content

c. increasing strength of timber

d. none to these.

19. 'Ware house pack' of cement means

a. pressure exertion of the bags of upper layers

b. pressure compaction of the bags on lower layers

c. packing the ware house

d. none of these.

20. For given workability the grading requiring the least amount of water is one that gives

a. greatest surface area for the given cement and aggregates

b. least surface area for the given cement and aggregates

c. least weight for the given cement and aggregates

d. greatest weight for the given cement and aggregates

21. For preparing a test-specimen, it is necessary

a. to mix coarse aggregates

b. to mix water to the cement, fine aggregates and coarse aggregates

c. to oil inner surface of the mould and the base plate, before placing concrete

d. all the above.

22. The impurity of mixing water which affects the setting time and strength of concrete, is

a. sodium chlorides

b. sodium carbonates and bicarbonates

c. calcium chorides

d. calcium bicarbonates

23. Ordinary Portland cement is manufactured from

a. lime stone and clay

b. gypsum and lime

c. pozzolana

d. lime, pozzolana and clay.

24. The triaxial apparatus is usually used for

a. unconsolidated-undrained test

b. consolidated-undrained test

c. drained test

d. all the above tests.

25. Pick up the correct statement from the following:

a. A soil classification system is meant to provide an accepted and systematic method of describing the various types of soils eliminating personal factors

b. The first category of soil classification is based on grain size of the soil

c. The second category of soil classification is based on fine as well as coarse grains

d. All the above.

26. The minimum water content at which the soil just begins to crumble when rolled into threads 3 mm in diameter, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

27. The length/diameter ratio of cylindrical specimens used in triaxial test, is generally

a. 1.5

b. 2

c. 2.5

d. 3

28. 'Talus' is the soil transported by

a. wind

b. water

c. glacier

d. gravitational force.

29. The liquidity index is defined as a ratio expressed as percentage of

a. plastic limit minus the natural water content, to its plasticity index

b. natural water content minus its plastic limit to its plasticity index

c. natural water content plus its plastic limit to its plasticity index

d. liquid limit minus the natural water content to the plasticity index.

30. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

31. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

32. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

33. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

34. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

35. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

36. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

37. The expected out turn of brick work in cement mortar in foundation and plinth per mason per day, is

a. 1.00 m3

b. 1.25 m3

c. 1.50 m3

d. 1.75 m3

38. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

39. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

40. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

41. ......... is used for drawing curves which can't be drawn by a compass

a. bow compass

b. protractor

c. french curve

d. scale

42. Set squares are used for drawing ........ ....... .. and ........... lines.

a. vertical

b. inclined

c. parallel

d. all of the above

43. In the first angle method of orthographic projection

a. the object comes between the observer and the plane

b. the plane comes between the object and the observer

c. the observer comes between the object and the plane

d. none of the above

44. The best drawing paper used for water colour is :

a. hand made paper (hand made drawing sheet)

b. tracing drawing sheet

c. ordinary drawing sheet

d. ammonia paper

45. The sunk costs include :

a. a past expenditure

b. an unrecovered balance

c. an invested capital that cannot be retreived

d. All of these

46. Pick up the correct statement from the following:

a. The manifestation of the time value of money is termed as interest.

b. Interest on borrowing = present amount owed - original loan

c. The original investment (or loan) is referred to as principal.

d. All of these.

47. The estimator for definitive estimates must be able :

a. and an all-around construction expert.

b. to read the plans and specifications to determine accurate quantities of permanent materials and installed Equipment.

c. to express the job material requirements in dimensions suitable for costing and construction supplies.

48. Who is the Chairperson of House Rent Fixation Committee?

a. CDO

b. LDO

c. chairperson of DDC

d. section officer

49. Engineer shall not disclose, without...............confidential information concerning the business affairs or technical processes of any present or former client or employer or public body on which the serve:

a. editing

b. getting paid

c. remuneration

d. consent

50. What percent of interest is taken for the advance amount paid to the contractor by employer?

a. 1% of advance amount

b. 2% of advance amount

c. 3% of advance amount

d. none of the above