***Engineer***

1. For calculating the permissible stress  is the emprical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

2. The locus of reaction of a two hinged semi-circular arch, is

a. straight line

b. parabola

c. circle

d. hyperbola.

3. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

4. In a beam, the neutral plane

a. may be its centre

b. passes through the C.G. of the area of cross-section

c. does not change during deformation

d. none of these.

5. A beam of length L supported on two intermediate rollers carries a uniformly distributed load on its entire length. If sagging B.M. and hogging B.M. of the beam are equal, the length of each overhang, is

a. 0.207 L

b. 0.307 L

c. 0.407 L

d. 0.5 L.

6. In case of reduction of levels by the height of instrument method,

a. ∑ B.S. - ∑ F.S. = difference in R.L.S of the first station and last station

b. ∑ (R.L. + I + F.S.) - first R.L = ∑ (H.I. + No. of R.L.s.)

c. both (a) and (b) above

d. neither (a) nor (b).

7. Pick up the correct statement from the following :

a. in Galileo's telescope, the rays from the object get refracted at the objective and are intercepted by the eyepiece before a real image is formed

b. a line passing through the optical centre of the objective traversing through the eyepiece, is called line of sight

c. the line of sight which passes through the intersection of cross-lines marked on a diaphragm fixed in front of the eyepiece in a plane at right-angles to the axis, is called the line of collimation

d. all the above.

8. If the whole circle bearing of a line is 180°, its reduced bearing is

a. S 0° E

b. S 0° W

c. S

d. N.

9. The difference of level between a point below the plane of sight and one above, is the sum of two staff readings and an error would be produced equal to

a. the distance between the zero of gradient and the foot of the staff

b. twice the distance between the zero of graduation and the foot of the staff

c. thrice the distance between the zero of graduation and the foot of the staff

d. none of the above.

10. Locating the position of a plane table station with reference to three known points, is known as

a. intersection method

b. radiation method

c. resection method

d. three point problem.

11. A level when set up 25 m from peg A and 50 m from peg B reads 2.847 on a staff held on A and 3.462 on a staff held on B, keeping bubble at its centre while reading. If the reduced levels of A and B are 283.665 m and 284.295 m respectively, the collimation error per 100 m is

a. 0.015 m

b. 0.030 m

c. 0.045 m

d. 0.060 m

12. If d is the distance between equidistant odd ordinates, the Simpson's rule for the areas, is

a. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

b. [h1 + hn x 2(h2 + h4 + ... + hn - 1) + 4(h3 + h5 + ... + hn - 2)]

c. [h1 + hn + 2(h3 + h5 + ... + hn - 2) + 4(h2 + h4 + ... + hn - 1)]

d. none of these.

13. Good quality cement contains higher percentage of

a. Tricalcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite

14. The method of addition polymerization is used for obtaining:

a. polypropylene

b. polyvinylchloride

c. polystyrene

d. All of these.

15. Pick up the metal refractory from the following:

a. Molyblendum

b. Tungsten

c. Zicronium

d. All of these.

16. The rocks which are formed due to pouring of magma at the earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Igneous rocks

17. The lime which contains mainly calcium oxide and slacks with water, is

a. quick lime

b. hydraulic lime

c. poor lime

d. none of these.

18. The wedging is adopted for quarrying costly stratified rock such as

a. marble

b. limestone

c. sandstone

d. All the above.

19. Pick up the correct statement from the following:

a. Angular and rough aggregates reduce the workability of the concrete

b. Large size aggregates increase the workability due to lesser surface area

c. The slump of the concrete mix decreases due to an increase in temperature

d. All the above.

20. Setting time of cement increases by adding

a. gypsum

b. hydrogen peroxide

c. calcium chloride

d. sodium oxide

21. Water cement ratio is

a. weight of water to that of cement

b. weight of concrete to that of water

c. volume of concrete to that of water

d. both (a) and (b) of the above.

22. 1.S. Sieve Nos. 10 mm and 4.75 mm are geneally used for grading of

a. fine aggregates

b. neither (a) nor (b)

c. both (a) and (b)

d. none of these.

23. The compaction of concrete, improves

a. density

b. strength

c. durability

d. all the above.

24. The critical exist gradient of seepage water in soils, increases with

a. a decrease in specific gravity

b. a decrease in void ratio

c. both (a) and (c)

d. none of these.

25. The maximum possible value nf dry density is referred to as

a. dry density

b. zero air voids

c. saturation dry density

d. all the above.

26. The inventor of the term soil mechanics, was

a. Kray

b. Dr. Karl Terzaghi

c. Leygue

d. Fellenius.

27. The maximum value of effective stress in the past divided by the present value, is defined as over consolidation ratio (OCR). The O.C.R. of an over consolidated clay is

a. less than 1

b. 1

c. more than 1

d. None of these.

28. The clay soil mainly consists of

a. Montomorillonite

b. Illites

c. Vermiculite

d. All the above.

29. A plane inclined at an angle φ to the horizontal at which the soil is expected to stay in the absence of any lateral support, is known as

a. natural slope line

b. repose line

c. the φ line

d. all the above.

30. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

31. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

32. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

33. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

34. An event is indicated on the network by a number enclosed in

a. a square

b. a triangle

c. an ellipse

d. all the above.

35. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

36. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

37. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

38. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

39. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

40. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

41. A 100 m long line making 30° with the vertical, show its projection on horizontal plane

a. 100 m

b. 50 m

c. 86.6 m

d. 70.71 m

42. The space covered by a drawing in a paper after changing the scale from 1:100 to 1:50 will:

a. increase 400%

b. increase 200%

c. decrease 200

d. decrease 400%

43. Which one is not drafting equipment ?

a. drafter

b. dust cloth

c. scale

d. None of the above

44. Inking pen is used for drawing -- in ink

a. writing

b. curve

c. straight line

d. none of the above

45. The ratio of current assests to current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

46. Pick up the ratio which gives us sufficient information by which to judge the financial condition and performance of the firm, from the following:

a. Financial leverage ratio

b. Activity ratio

c. Portability

d. None of these

47. The project contractor relies on the cost of the estimate :

a. for submission of a competitive bid for a lumpsum contract

b. for a unit price contract

c. for preparation of a definitive estimate to help negotiate contract.

d. All of these

48. Workers are not allowed to work contiunously more than

a. 3hr

b. 5hr

c. 7hr

d. 4hr

49. NEC does not approach for

a. engineer

b. overseer to engineer

c. minimum degree B.E.

d. all of the above

50. The document announcing to prospective bidder that the owner is ready to receive bids is called

a. notice to bidders

b. instruction to bidders

c. bill of quantities

d. intension to acceptance