***Engineer***

1. A composite beam is composed of two equal strips one of brass and other of steel. If the temperature is raised

a. brass experiences compressive force

b. composite beam gets subjected to a couple

c. composite beam bends

d. All the above.

2. The assumption in the theory of bending of beams, is :

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

3. A compound truss may be formed by connecting two simple rigid frames, by

a. two bars

b. three bars

c. three parallel bars

d. three bars intersecting at a point.

4. At either end of a plane frame, maximum number of possible bending moments, are

a. two

b. three

c. four

d. zero.

5. Pick up the correct statement from the following :

a. The rate of change of bending moment is equal to rate of shear force

b. The rate of change of shear force is equal to rate of loading

c. neither (a) nor (b)

d. both (a) and (b).

6. The main plate of a transit is divided into 1080 equal divisions. 60 divisions of the vernier coincide exactly with 59 divisions of the main plate. The transit can read angles accurate upto

a. 10"

b. 15"

c. 20"

d. 30"

7. A dumpy level was set up at mid-point between pegs A and B, 80 m apart and the staff readings were 1.32 and 1.56. When the level was set up at a point 10 m from A on BA produced, the staff readings obtained at A and B were 1.11 and 1.39. The correct staff reading from this set up at S should be

a. 1.435

b. 1.345

c. 1.425

d. none of these.

8. Diurnal variation of magnetic declination is

a. greater at equator than nearer the poles

b. less at equator than nearer the poles

c. less in summer than in winter

d. same at all latitudes and during different months.

9. If Δ is the angle of deflection of a simple curve of radius R, the length of its long chord, is

a. R cos

b. 2R cos

c. R sin

d. 2R sin .

10. Surveys which are carried out to provide a national grid of control for preparation of accurate maps of large areas, are known

a. plane surveys

b. geodetic surveys

c. geographical surveys

d. topographical surveys.

11. The probable error of the adjusted bearing at the middle is

a.  rn

b.  rn

c.  rn

d.  rn.

12. In a theodolite

a. the telescope axis is perpendicular to transit axis

b. the axis of rotation is perpendicular to transit axis

c. the telescope axis, the transit axis and the rotation axis pass through the centre of theodolite

d. all the above.

13. The most fire resistant paints are :

a. enamel paints

b. aluminium paints

c. asbestos paints

d. cement paints.

14. Plywood is normally available

a. 1 mm thick

b. 2 mm thick

c. 2 to 3 mm thick

d. 3 mm to 4 mm thick.

15. The PVC doors and windows are preferred as they are

a. rot proof

b. termile proof

c. water proof

d. all of these.

16. The sequence of refractory materials according to increasing melting points is :

a. Dolomite, Magnesia, Bauxite, Chromite

b. Bauxite, Chromite, Dolomite, Magnesia

c. Magnesia, Bauxite, Dolomite, Chromite.

17. Plastic

a. consists of natural or synthetic binders

b. finished products are rigid and stable at normal temperature

c. is capable of flow when necessary heat and pressure are applied

d. All the above.

18. Soundness of cement is tested by

a. Vicat's apparatus

b. Le-chatelier apparatus

c. compressive strength testing apparatus

d. none of the these.

19. The shuttering of a hall measuring 4 m x 5 m, can be removed after

a. 7 days

b. 10 days

c. 14 days

d. 21 days

20. Pick up the correct statement from the following:

a. Gypsum (calcium sulphate) acts as an accelerator

b. Gypsum (calcium sulphate) acts as a retarder

c. Calcium chloride acts as an accelerator

d. Both (c) and (d).

21. Pick up the incorrect statement from the following:

a. Cement bags should preferably be piled on wooden planks

b. Cement bags should be placed such that bags of one layer does not touch the bags of the adjacent layer

c. Width and height of the pile should not exceed 3 m and 2.70 m respectively

d. None of these.

22. The grade of concrete not recommended by I.S. : 456, is

a. M 200

b. M 300

c. M 400

d. M 500

23. Horizontal construction joints in concrete walls are generally provided at

a. soffit level

b. window sill level

c. floor level

d. all the above.

24. Pick up the incorrect statement from the following:

a. Compaction has no effect on the structure of a soil

b. Permeability decreases with increase in the dry density of a compacted soil

c. A wet side compacted soil is more compressible than a dry side compacted soil

d. Dry side compaction soils swell more when given access to moisture

25. Pick up the correct statement from the following:

a. Kaolinite shows a very little sign of swelling on wetting

b. Kaolinite when wet, becomes moderately plastic because negative electro magnetic charges on platelets attrack water

c. Kaolinite is also called China clay

d. All the above.

26. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

27. Pick up the correct statement from the following:

a. When stress decreases, void, ratio decreases

b. When stress decreases, coefficient of permeability decreases

c. When stress decreases, coefficient of volume change decreases

d. When stress decreases void ratio, co-effi-cients of per-meability and volume change decrease.

28. For shear strength, triaxial shear test is suitable because

a. precise measurement of the pore pressure and volume change during the test is possible

b. stress distribution on the failure plane, is uniform

c. state of stress within the specimen during any stage of the test as well as at failure, is completely determined

d. all the above.

29. The vane shear test is used for the in-situ determination of the undrained strength of the intact fully saturated

a. sands

b. clays

c. gravels

d. highly organic soils.

30. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

31. Optimistic time, most likely time and pessimistic times for the activities of a network in the given figure are written above their arrows. If the contractual obligation time for the project is 75, the latest occurrence time for the event 2, is

a. 20

b. 25

c. 35

d. 15

32. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

33. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

34. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

35. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

36. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

37. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

38. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

39. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

40. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

41. A technical drawing of an object that shows the relationship or order of assembly of the various parts is known

a. orthographic projection

b. expolded view drawing

c. compacted view drawing

d. axonometric drawing

42. In case of aligned method of dimensioning, they can be readable form

a. bottoms and right side of drawing sheet

b. bottoms and left side of drawing sheet

c. bottoms and top side of drawing sheet

d. all of the above

43. Dimension lines and projection lines are drawn as:

a. continuous thick lines

b. continuous thin line with zig zags

c. continuous thin line

d. continuous thin and wary lines

44. The set of technical drawings used during the manufacturing phase of product including cvili construction, are known as

a. structural drawing

b. as built drawing

c. structural drawing

d. working drawing

45. In a cash flow series :

a. uniform gradient signifies that an income or disbursement changes by the same amount in each interest period.

b. Either an increase or a decrease in the amount of a cash flow is called the gradient.

c. The gradient in the cash flow may be positive or negative.

d. All of these

46. The product of CAF (S P) and PWF (S P) is:

a. 1/2

b. 1

c. 1/3

d. 1/4

47. Current assests less inventories divided by current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

48. The variation exceeding 15 % of the contract amount is approved by

a. gazetted 2nd class

b. gazetted 1st class

c. deaprtment head

d. none of the above

49. Prequilification is not required for any types of firm if the costing to buy an equipment is

a. less than 20 million

b. more than 20 million

c. any amount

d. all of the above

50. Code of conduct defines

a. what should be

b. what should not be

c. what had been

d. what will be