***Engineer***

1. The ratio of lateral strain to axial strain of a homogeneous material, is known

a. Yield ratio

b. Hooke's ratio

c. Poisson's ratio

d. Plastic ratio.

2. Total strain energy theory for the failure of a material at elastic limit, is known

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

3. A material may fail if

a. maximum strain exceeds

b. maximum shear stress exceeds

c. total strain energy exceeds

d. all the above.

4. The neutral axis of a beam cross-section must

a. pass through the centroid of the section

b. be equidistant from the top of bottom films

c. be an axis of symmetry of the section

d. none of these.

5. The cross sections of the beams of equal length are a circle and a square whose permissible bending stress are same under same maximum bending. The ratio of their flexural weights is,

a. 1.118

b. 1.338

c. 1.228

d. 1.108

6. The least count of a vernier scale is

a. sum of the smallest divisions of main and vernier scales

b. value of one division of the primary scale divided by total number of divisions of vernier scale

c. value of one division of vernier scale divided by total number of divisions of primary scale

d. none of these.

7. Tacheometric formula for horizontal distances using horizontal sights can also suitable be employed for inclined sights through θ by multiplying

a. the constants by cos2 θ

b. the constants by cos θ

c. the constants by sin θ

d. the multiplying constant by cos2 θ and additive constant by cos θ.

8. The bearings of the lines AB and BC are 146° 30' and 68° 30'. The included angle ABC is

a. 102°

b. 78°

c. 45°

d. none of these.

9. If the whole circle bearing of a line is 180°, its reduced bearing is

a. S 0° E

b. S 0° W

c. S

d. N.

10. Horizontal distances obtained tacheometerically are corrected for

a. slope correction

b. temperature correction

c. refraction and curvature correction

d. all the above.

11.  is the conventional sign of

a. temple

b. mosque

c. idgah

d. church

12. For preparation of a contour plan for a route survey

a. method of squares is used

b. method of trace contour is used

c. method of cross profile is used

d. indirect method of contouring is used.

13. The most important constituent of an oil paint, is

a. vehicle

b. pigment

c. base

d. all the above.

14. A bull nose brick is not used for

a. rounding off sharp corners

b. pillars

c. decoration purpose

d. arches.

15. The plastics made from cellulose resin

a. are as clear as glass

b. are tough and strong

c. possess excellent electrical properties

d. All the above.

16. A pug mill is used for

a. moulding brick earth

b. tempering brick earth

c. providing brick earth

d. all the above.

17. Bulking of sand is caused due to

a. surface moisture

b. air voids

c. viscosity

d. clay contents

18. Quick lime

a. reacts with carbon dioxide

b. may be used for white-washing

c. when mixed with water forms slaked lime

d. all the above.

19. Separation of coarse aggregates from mortar during transportation, is known

a. creeping

b. segregation

c. shrinkage

d. none of these.

20. The final operation of finishing floors, is known as

a. floating

b. trowelling

c. finishing

d. all the above.

21. The light weight aggregates are obtained from

a. sedimentary rocks

b. metamorphic rocks

c. igneous rocks

d. volcanic source.

22. Cement used for normal concrete construction, is obtained by burning a mixture of

a. argilaceous and calcareous materials

b. silicious and catcareous materials

c. silicious, argillaceous and calcareous materials

d. none of these.

23. Pick up the incorrect statement from the following:

a. A rich mix of concrete possesses higher strength than that a lean mix of desired workability with excessive equantity of water

b. The strength of concrete decreases as the water cement ratio increases

c. If the water cement ratio is less than 0.45, the concrete is not workable and causes honey-combed structure

d. Good compaction by mechanical vibrtions, increases the strength of concrete

24. Sedimentation analysis is based on the assumption:

a. soil particles are spherical

b. particles settle independent of other particles

c. walls of the jar do not affect the settlement .

d. all the above.

25. For testing a saturated clay for shear strength, the test recommended, is

a. direct shear test

b. triaxial compression test

c. unconfined compression test

d. all the above.

26. Buoyant unit weight equals the saturated density

a. multiplied by unit weight of water

b. divided by unit weight of water

c. plus unit weight of water

d. minus unit weight of water.

27. A fundamental equation of void ratio(e), specific gravity (G), water content (ω) and degree of saturation (Sr) is

a. e =

b. ω =

c. G =

d. Sr =

28. In a fow net

a. flow lines and equipotential lines cross each other at right angles

b. fields are rectangles whose length is twice the breadth

c. smaller the dimensions of the field, smaller will be the hydraulic gradient and velocity of flow through it

d. for homogeneous soil, the curves are smooth and circular.

29. Pick up the correct definition from the following:

a. The lateral pressure exerted by the soil when the retaining wall moves away from the back fill, is generally known as active earth pressure of the soil

b. The lateral pressure exerted by the soil when the retaining wail moves towards the soil, is generally known as 'Passive earth pressure of the soil'

c. The lateral pressure exerted by the soil when the retaining wall has no movement relative to the back fill, is known as 'earth pressure at rest of the soil'

d. All the above.

30. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

31. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

32. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

33. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

34. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

35. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

36. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

37. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

38. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

39. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

40. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

41. The projection in which the length, breadth and height of an object is shown in one view is known as:

a. pictorial projection

b. isometric projection

c. cavalier projection

d. cabinet projection

42. Presentation drawing are generally rendered due to

a. decoration

b. give real effects

c. attract the client

d. satisfaction

43. The area occupied by Ao (drawing sheet) is ......... times the area occupied by A3 (drawing sheet).

a. 2

b. 4

c. 8

d. 16

44. Circle and arcs of circles are drawn by means of a .......

a. lengthening bar

b. divider

c. compass

d. T-square

45. Earning per share is the most important ratio for

a. share holders

b. banks

c. company's management

d. All of these

46. The interest calculated on the basis of 365 days a year, is known as :

a. interest

b. ordinary simple interest

c. exact simple interest

d. None of these

47. Pick up the method used for project evaluation and selection in capital budgetting from the following:

a. Internal ratio of return

b. Net present worth

c. Profitability index

d. All the above

48. NEC act and its regulation........

a. 55 and 57

b. 54 and 55

c. 55 and 56

d. 57 and 58

49. Profession is defined as

a. systematic knowledge through specialized training

b. synonym to job or occupation

c. types of service to needy people

d. all of above

50. What is EPC contract ?

a. Engineering, Planning and costing

b. Evaluation, Planning and consulting

c. Engineering, procurement and Construction

d. none of above