***Engineer***

1. A compound bar consists of two bars of equal length. Steel bar cross-section is 3500 mm2 and that of brass bar is 3000 mm2. These are subjected to a compressive load 100,000 N. If Eb = 0.2 MN/mm2 and Eb = 0.1 MN/mm2, the stresses developed are:

a. σb = 10 N/mm2, σs = 20 N/mm2

b. σb = 8 N/mm2, σs = 16 N/mm2

c. σb = 6 N/mm2, σs = 12 N/mm2

d. σb = 5 N/mm2, σs = 10 N/mm2

2. A composite beam is composed of two equal strips one of brass and other of steel. If the temperature is raised

a. brass experiences compressive force

b. composite beam gets subjected to a couple

c. composite beam bends

d. All the above.

3. The ratio of the area of cross-section of a circular section to the area of its core, is

a. 4

b. 8

c. 12

d. 16

4. For a beam, if fundamental equations of statics are not sufficient to determine all the reactive forces at the supports, the structure is said to be

a. determinate

b. statically determinate

c. statically indeterminate

d. none of these.

5. The maximum twisting moment a shaft can resist, is the product of the permissible shear stress and

a. moment of inertia

b. polar moment of inertia

c. polar modulus

d. modulus of rigidly.

6. The accuracy of measurement in chain surveying, does not depend upon

a. length of the offset

b. scale of the plotting

c. importance of the features

d. general layout of the chain lines.

7. The bearing of AB is 190° and that of CB is 260° 30'. The included angle ABC, is

a. 80° 30'

b. 99° 30'

c. 70° 30'

d. none of these

8. Offsets are measured with an accuracy of 1 in 40. If the point on the paper from both sources of error (due to angular and measurement errors) is not to exceed 0.05 cm on a scale of 1 cm = 20 m, the maximum length of offset should be limited to

a. 14.14

b. 28.28 m

c. 200 m

d. none of these.

9. In case of a double line river, contours are

a. stopped at the banks of the river

b. stopped at the edge of the river

c. drawn across the water

d. drawn by parabolic curves having their vertex at the centre of the water.

10. If the whole circle bearing of a line is 270°, its reduced bearing is

a. N 90° W

b. S 90° W

c. W 90°

d. 90° W.

11. Probable systematic error in precise levelling as recommended by International Geodetic Association should not exceed (where k is in kilometers.)

a. ± 0.1 k mm

b. ± 0.2 k mm

c. ± 0.1 k

d. 0.2 k mm.

12. In setting up a plane table at any station

a. levelling is done first

b. centering is done first

c. both levelling and centering are done simultaneously

d. orientation is done first.

13. Soundness test of cement determines

a. quality of free lime

b. ultimate strength

c. durability

d. initial setting

14. Teak wood is suitable for

a. sports articles

b. furnitures

c. railway sleepers

d. all the above.

15. Oil varnish generally consists of

a. synthetic resin and spirit

b. oil, wax and resin

c. resin, oil and turpentine

d. spirit, oil and wax.

16. Pick up the correct statement from the following:

a. Impurities float on the molten iron as slag

b. The slag contains lime about 45%

c. The molten iron is made to run, in a long channel formed in sand called 'sow'

d. All the above.

17. Cement paints usually

a. contain 5% to 10% colour pigments

b. are prepared with white cement

c. contain 5% sodium chloride

d. all the above.

18. Snowcrete is one of the patent forms of

a. distempers

b. water proof cement paints

c. enamel paints

d. cellulose paints.

19. According to I.S. : 456, the number of grades of concrete mixes, is

a. 4

b. 5

c. 6

d. 7

20. Pick up the incorrect statement from the following:

a. In properly graded aggregates, bulk density is more

b. In single size aggregates, bulk density is least

c. In single size aggregates, bulk density is maximum

d. None of these.

21. If fineness modulus of sand is 2.5, it is graded as

a. fine sand

b. medium sand

c. coarse sand

d. very coarse sand.

22. ISI has specified full strength of concrete after

a. 14 days

b. 21 days

c. 28 days

d. none of these.

23. Pick up the incorrect statement from the following:

a. Admixtures accelerate hydration

b. Admixtures make concrete water proof

c. Admixtures make concrete acid proof

d. Admixtures give high strength

24. In a purely cohesive soil, the critical centre lies at the intersection of

a. perpendicular drawn at l/3rd slope from toe and the locus of the centre

b. perpendicular drawn at 2/3rd slope from toe and the locus of the centre

c. directional angles

d. none of these.

25. Chemical weathering of soil is caused due to

a. carbonation

b. hydration

c. leaching

d. all the above.

26. Stoke's law sates that the velocity at which a grain settles out of suspension, the other factors remaining constant, is dependent upon

a. weight of grain

b. size of grain

c. shape and size of grain

d. shape, size and weight of grain.

27. If the coefficients of volume change and compressibility of a soil sample are respectively 6.75 x 10-2 and 3 x 10-2, the void ratio of the soil sample, is

a. 1.15

b. 1.20

c. 1.25

d. 1.30

28. Fundamental relationship between dry density (γd), specific gravity (G), water content (ω) and percentage of air voids (na) is :

a. γd =

b. γd =

c. γd =

d. γd =

29. Stoke's law does not hold good if the size of particle is smaller than

a. 0.0002 mm

b. 0.002 mm

c. 0.02 mm

d. 0.2 mm

30. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

31. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

32. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

33. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

34. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

35. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

36. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

37. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

38. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

39. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

40. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

41. 9H pencil is ......than the 7H pencil

a. softer

b. harder

c. medium

d. none of the above

42. Which is not pictorial projection?

a. oblique projection

b. orthographic projection

c. trignometric

d. axonometric projection

43. The projection of any feature on the horizontal plane is called

a. plan

b. elevation

c. map

d. all of the above

44. The point of intersection of co-ordinate axis is called

a. zero-zero

b. origin

c. cross point

d. all of the above

45. Pick up the correct statement from the following:

a. Ratio analysis is the procedure of determining and interpreting numerical relationship of various items of the financial statement.

b. All financial ratios are obtained by relating two sets of information contained in a Single financial statement.

c. The relationship between two accounting figures expressed mathematically, is known as a financial ratio.

d. All of these

46. Present worth Annuity (PWA) is generally known as

a. Premium annuities

b. Income annuities

c. Future annuities

d. All of these

47. The CRF (ep) is also known as: [CRF(EP) - 8% - 7], where

a. 8% is the rate of interest per year

b. money is borrowed for n = 7 years

c. both (a) and (b)

d. Neither (a) nor (b)

48. Which of the following an engineer should not follow:

a. act as faithful agent for client

b. accept compensation from more than one party for same service without the consent of all

c. indicate client the adverse consequences if judgment is over ruled

d. undertake only these assignments for which he is qualified

49. Profession is defined as

a. systematic knowledge through specialized training

b. synonym to job or occupation

c. types of service to needy people

d. all of above

50. Time extension may need to be granted due to

a. causes by force majeure

b. causes by purchaser

c. both a) and b) of above

d. none of the above