***Engineer***

1. The general expression for the B.M. of a beam of length l is the beam carries

a. a uniformly distributed load w/unit length

b. a load varying linearly from zero at one end to w at the other end

c. an isolated load at mid span

d. none of these.

2. Maximum strain theory for the failure of a material at the elastic limit, is known as

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

3.  is the equation for Euler's crippling load if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

4. If the stress in each cross-section of a pillar is equal to its working stress, it is called

a. body of equal

b. body of equal section

c. body of equal strength

d. none of these.

5. If the rivets in adjacent rows are staggered and outermost row has only one rivet, the arrangement of the rivets, is called

a. chain riveting

b. zig-zag riveting

c. diamond riveting

d. none of these.

6. In horizontal angles, the error due to imperfect levelling of the plate bubble is

a. large when sights are nearly level

b. large for long sights

c. less for steeply inclined sights

d. large for steeply inclined sights.

7. The real image of an object formed by the objective, must lie

a. in the plane of cross hairs

b. at the centre of the telescope

c. at the optical centre of the eye-piece

d. anywhere inside the telescope.

8. Pick up the correct specification of Ramsden eyepiece from the following :

a. the curved surfaces of plano-convex lenses face each other

b. the two lenses are separated by a distance equal to 2/3 of the focal length of either lens.

c. the distance between the diaphragm and the front lens of the eyepiece is kept equal to 1/4 th of the focal length of a lens so that rays from a point on the diaphragm enter the eye as a parallel beam

d. all the above.

9. The distances AC and BC are measured from two fixed points A and B whose distance AB is known. The point C is plotted by intersection. This method is generally adopted in

a. chain surveying

b. traverse method of surveys

c. triangulation

d. none of these.

10. An angles of 45° with a chain line may be set out with

a. optical square

b. open cross staff

c. Fench cross staff

d. prismatic square.

11. If the long chord and tangent length of a circular curve of radius R are equal the angle of deflection, is

a. 60°

b. 90°

c. 120°

d. 150°.

12. Pantagraph is used for

a. measuring distances

b. measuring areas

c. enlarging or reducing plans

d. setting out right angles

13. The most fire resistant paints are :

a. enamel paints

b. aluminium paints

c. asbestos paints

d. cement paints.

14. The initial setting time of lime-pozzolana, is

a. 30 minutes

b. 60 minutes

c. 90 minutes

d. 120 minutes.

15. Priming consists of

a. one part of white lead, 8 parts of chalk and four parts of twice boiled linseed oil

b. 8 parts of white lead, one part of chalk and four parts of twice boiled linseed oil

c. one part of white lead, 8 parts of chalk and one part of linseed oil

d. none to these.

16. Stuco paints are suitable for

a. stone masonry

b. brick walls

c. both (a) and (b)

d. neither (a) nor (b).

17. For lime concrete,

a. flexural strength at 90 days is 0.2 N/mm2

b. compressive strength at 90 days is 1.5 N/mm2

c. compressive strength at 26 days is 1.2 N/mm2

d. all the above.

18. During pudding

a. carbon is converted into carbonic acid gas

b. silicon forms a slag

c. metal is heated by the burning of gases

d. all the above.

19. If fineness modulus of sand is 2.5, it is graded as

a. fine sand

b. medium sand

c. coarse sand

d. very coarse sand.

20. The type of aggregates not suitable for high strength concrete and for pavements subjected to tension, is

a. rounded aggregate

b. irregular aggregate

c. angular aggregate

d. flaky aggregate

21. The bulk density of aggregates does not depend upon :

a. specific gravity of aggregates

b. grading of aggregates

c. size and shape of the container

d. none of these.

22. The mixture of different ingredients of cement, is burnt at

a. 1200°C

b. 1400°C

c. 1600°C

d. 1800°C

23. An aggregate is said to be flaky, if its least dimension is less than

a. 3/4 mean dimension

b. 3/5 mean dimension

c. 5/8 mean dimension

d. none of these.

24. A saturated soil sample has water content of 40% and specific gravity of soil particles 2.7. The void ratio of the soil, is

a. 0.4

b. 0.52

c. 1.08

d. none of these.

25. Soils containing organic matters

a. are of spongy nature

b. swell with decrease of moisture

c. shrink with increase of moisture content

d. none of these.

26. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

27. The critical exist gradient of seepage water in soils, is

a. inversely proportional to the specific gravity

b. directly proportional to the specific gravity

c. inversely proportional to the voids ratio

d. none of these.

28. The water content in a soil sample when it continues to loose weight without loosing the volume, is called

a. Shrinkage limit

b. Plastic limit

c. liquid limit

d. semi-solid limit.

29. If the unit weight of sand particles is 2.696 g/cc. and porosity in loose state is 44%, the critical hydraulic gradient for quick sand condition, is

a. 0.92

b. 0.93

c. 0.94

d. 0.95

30. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

31. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

32. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

33. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

34. Pick up the correct statement from the following with regards to C.P.M. network analysis of projects

a. Earliest occurrence time of the event from which the activity arrow originates plus the duration of the activity, is called earliest finish time of the activity

b. The latest occurrence time of the node of which the activity arrow terminates minus the duration of the activity, is called latest start time

c. The latest occurrence time for the node at which the activity arrow terminates, is called latest finish time

d. All the above.

35. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

36. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

37. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

38. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

39. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

40. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

41. The set of technical drawings used during the manufacturing phase of product including cvili construction, are known as

a. structural drawing

b. as built drawing

c. structural drawing

d. working drawing

42. In the first angle method of orthographic projection

a. the object comes between the observer and the plane

b. the plane comes between the object and the observer

c. the observer comes between the object and the plane

d. none of the above

43. Sketching is usually done:

a. with French curves

b. free hand

c. with drafting machine

d. with protractor

44. In the first angle projection the object is assumed to be placed in

a. fourth quadrant

b. third quadrant

c. second quadrant

d. first quadrant

45. The financial analysis :

a. helps a share holder to compare the expected return on his investment in the firm against the expected return from other alternative investment.

b. helps a bank to know the financial position of the firm for granting a loan to the firm.

c. helps to judge the success of the firm's financial plans.

d. All of these.

46. Probabilistic estimating of a construction project includes:

a. Labour

b. Productivity

c. wage scale

d. All of these

47. Which method is adopted to develop an approximate or conceptual estimate for perimeter works for buildings from the following:

a. Cost per function method

b. Cost per square metre method

c. Cost per cubic metre method

d. Cost per linear metre method

48. For procurement of works up to 2 million, buy which of the following method is to be considered?

a. by sealed tender

b. by direct negotiation

c. by bidding

d. by direct labour or direct negotiation

49. Procurement of services means:

a. buying logistics

b. hiring consultant

c. renting house

d. none of the above

50. As per the public Procurement Regulation, 2064 of Nepal, what is the maximum age for a technical proposal for consulting services ?

a. 90%

b. 80%

c. 70%

d. 60%