***Engineer***

1. The equivalent length of a column of length L, having both the ends hinged, is

a. 2L

b. L

c.

d.

2. The normal and tangential components of stress on an inclined plane through θ° to the direction of the force, will be equal if θ is

a. 45°

b. 30°

c. 60°

d. 90°

3. Pick up the correct statement from the following:

a. The structural member subjected to compression and whose dimensions are small as compared to its length, is called a stmt

b. The vertical compression members, are generally known as columns or stanchions

c. Deflection in lateral direction of a long column, is generally known as buckling

d. All the above.

4. The ratio of the effective length of a column and minimum radius of gyration of its cross-sectional area, is known

a. buckling factor

b. slenderness ratio

c. crippling factor

d. none of these.

5. Hooke's law states that stress and strain are

a. directly proportional

b. inversely proportional

c. curvilinearly related

d. none of these.

6. True meridians are generally preferred to magnetic meridians because

a. these converge to a point

b. these change due to change in time

c. these remain constant.

d. None of these.

7. If θ is the slope of the ground and l is the measured distance, the correction is

a. 2l sin2 θ/2

b. 2l cos2 θ/2

c. 2l tan2 θ/2

d. 2l cot2 θ/2.

8. If h is the difference in level between end points separated by l, then the slope correction is  . The second term may be neglected if the value of h in a 20 m distance is less than

a.  m

b. 1 m

c. 2 m

d. 3 m

9. In levelling operation

a. if first reading is more than second, it represents a rise

b. if  first reading is less than second, it represents a fall

c. if  second reading is less than first, it represents a fall

d. both (b) and (c).

10. The sum of the interior angles of a geometrical figure laid on the surface of the earth differs from that of the corresponding plane figure only to the extent of one second for every

a. 100 sq. km of area

b. 150 sq. km of area

c. 200 sq. km of area

d. none of these.

11. Pick up the correct statement from the following :

a. it is difficult to eliminate an error completely at first trial

b. instability of the instrument makes it almost impossible to adjust it satisfactorily

c. adjustment screws must be left bearing firmly but should never be forced

d. all the above.

12. If whole circle bearing of a line is 120°, its reduced bearing is

a. S 20° E

b. S 60° E

c. N 120° E

d. N 60° E.

13. Jhumb bricks are

a. under burnt

b. over burnt

c. kutcha

d. none of these.

14. To give a brilliant finish, the type of varnish used, is

a. spirit varnish

b. turpentine varnish

c. oil varnish

d. none of these.

15. A rock contains only one mineral. It is called

a. homogeneous

b. non-homogeneous

c. monomineralic

d. polymineralic.

16. Bituminous fells are used for

a. covering A.C. sheets

b. covering sloping roofs

c. D.P.C.

d. none to these.

17. Based on its dry weight, a freshly felled tree may contain water

a. 25%

b. 50%

c. 75%

d. 100%.

18. Pick up the correct statement from the following:

a. Quick lime contains calcium oxide

b. Slaked lime may be obtained from quick lime

c. Slaked lime is obtained by adding water to quick lime

d. All the above.

19. Non-uniform compaction may cause the concrete

a. porous

b. non-homogeneous

c. reduced strength

d. all the above.

20. Construction joints are provided

a. where the member is supported by other member

b. at 18 m apart in huge structures

c. in concrete wall at sill level of windows

d. all the above.

21. For preparing ordinary concrete, the quantity of water used, is

a. 10% by weight of aggregates plus 10% of weight of cement

b. 5% by weight of aggregates plus 30% of weight of cement

c. 30% by weight of aggregates plus 10% of weight of cement

d. none of these.

22. Addition of pozzolana to cement

a. increases strength

b. increases heat of hydration

c. decreases curing time

d. none of these.

23. The ratio of various ingredients (cement, sand, aggregates) in concrete of grade M 200, is

a. 1 : 3 : 6

b. 1 :   : 3

c. 1 : 1 : 2

d. 4 : 2 : 1

24. Factor of safety against sliding of a slope, is the ratio of

a. actual cohesion to that required to maintain stability of slope

b. shear strength to shear stress along the surface

c. neither (a) nor (b)

d. both (a) and (b).

25. Failure of a slope occurs only when total shear force is

a. equal to total shearing strength

b. greater than total shearing strength

c. less than total shearing strength

d. none of these.

26. If the failure of a finite slope occurs through the toe, it is known as

a. slope failure

b. face failure

c. base failure

d. toe failure.

27. In a fow net

a. flow lines and equipotential lines cross each other at right angles

b. fields are rectangles whose length is twice the breadth

c. smaller the dimensions of the field, smaller will be the hydraulic gradient and velocity of flow through it

d. for homogeneous soil, the curves are smooth and circular.

28. Pick up the incorrect statement from the following:

a. Below the water table, the pore water may be static.

b. The hydrostatic pressure depends on the depth below the water level.

c. The attractive forces between the particles, caused due to negative pressure of water held above the water table is called soil suction.

d. None of these.

29. The shear strength in plastic undrained clay, is due to

a. inter-granular friction

b. internal friction

c. cohesion

d. none of these.

30. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

31. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

32. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

33. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

34. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

35. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

36. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

37. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

38. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

39. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

40. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

41. To draw very light line which pencil is used

a. B

b. HB

c. H

d. 2H

42. The view actually appears to the observer when viewed from a fixed position is called

a. isometric drawing

b. orthographic projection

c. perspective drawing

d. none of the above

43. When only one drawing is to be made then its position should be

a. left side of paper

b. right side of paper

c. upper side of paper

d. centre of paper

44. The angle between isometric axes is

a. 30°

b. 60°

c. 90°

d. 120°

45. Which one of the following definitions, is correct ?

a. The ratio of total debt to share holder's equity is called 'debt ratio'.

b. The ratio debt-to-total assests is called Debt-to-total assest ratio.

c. The ratio of earnings before interest and taxes for a particular reporting period to the amount of interest charges for the period, is called interest coverage ratio.

d. All of these

46. Pick up the correct statement from the following:

a. The gross profit percentage is the average profit margin obtained on goods sold.

b. The relationship of contribution to sales is known as contribution ratio

c. The difference between sales and variable cost of sales, is called contribution.

d. All of these

47. Pick up the correct statement from the following:

a. The capital required to get a project started, is called first cost.

b. The costs associated with a new or existing project that remain unaffected by the changes in activity level over the normal range of operation of the project, are called fixed costs.

c. The group of costs that vary proportionately to the changes in the activity level of a new or existing project are called variable costs.

d. All of these

48. Which one is correct with respect to values, morals and ethics?

a. values are the rules

b. morals are the personal character

c. ethics are professional sstandards

d. all of the above

49. Maximum working hour for a labour per week should be

a. 42hr

b. 45hr

c. 48hr

d. 53hr

50. For the registiration of professional engineer amount fee required is:

a. NRS 5000/-

b. NRS 10000/-

c. NRS 15000/-

d. NRS 20000/-