***Engineer***

1. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

2. The ratio of moments of inertia of a triangular section about its base and about a centroidal axis parallel to its base, is

a. 1.5

b. 2.0

c. 2.5

d. 3.0

3. The forces acting normally on the cross section of a bar shown in the given figure introduce

a. compressive stress

b. tensile stress

c. shear stress

d. none of these.

4. Pick up the correct statement from the following :

a. The rate of change of bending moment is equal to rate of shear force

b. The rate of change of shear force is equal to rate of loading

c. neither (a) nor (b)

d. both (a) and (b).

5. The maximum bending moment due to a moving load on a simply supported beam, occurs

a. at the supports

b. under the load

c. anywhere on the beam

d. none of these.

6. Pick up the correct statement from the following :

a. The horizontal angle between magnetic meridian and true meridian at a place is called magnetic declination or variance of the compass

b. the imaginary lines which pass through points at which the magnetic declinations are equal at a given time are called isogonic lines

c. the isogonic lines through places at which the declination is zero are termed agonic lines

d. all the above.

7. The constant vertical distance between two adjacent contours, is called

a. horizontal equivalent

b. vertical equivalent

c. contour interval

d. contour gradient.

8. Whole circle bearing of a line is preferred to a quadrantal bearing merely because

a. bearing is not completely specified by an angle

b. bearing is completely specified by an angle

c. Sign of the correction of magnetic declination is different in different quadrants

d. its trigonometrical values may be extracted from ordinary tables easily.

9. Determining the difference in elevation between two points on the surface of the earth, is known as

a. levelling

b. simple levelling

c. differential levelling

d. longitudinal levelling.

10. The limiting length of an offset does not depend upon

a. accuracy of the work

b. method of setting out perpendiculars

c. scale of plotting

d. indefinite features to be surveyed.

11. The total change in level along the line is equal to total back sights

a. minus total fore sights

b. the total rises minus total falls

c. the reduced level of last point minus reduced level of the first point

d. all the above.

12. In a precision traverse, included angles are measured by setting the vernier

a. to read zero exactly on back station

b. to read 5° exactly on back station

c. some where near zero and reading both verniers on back station

d. all the above.

13. Pick up the correct statement from the following:

a. Bull's trench kiln a trench excavated in ground

b. Hoffman's kiln is constructed overground

c. Tunnel Kiln is constructed as a tunnel

d. All the above.

14. Good quality stones must

a. be durable

b. be free from clay

c. resist action of acids

d. all the above.

15. Softer variety of steel may be obtained by

a. Cementation process

b. crucible process

c. Bessemer process

d. Open hearth process.

16. Bitumen paints offer

a. pleasing surface

b. hard surface

c. smooth surface

d. protective surface

e. rough surface.

17. Turpentine oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

18. The most valuable timber may be obtained from

a. chir

b. shisham

c. sal

d. teak.

19. The process of hardening the concrete by keeping its surface moist is known

a. wetting

b. curing

c. compacting

d. none of these.

20. Strength of concrete with passage of time

a. increases

b. decreases

c. fluctuates

d. remains constant.

21. Wp and Wf are the weights of a cylinder containing partially compacted and fully compacted concrete. If the compaction factor  is 0.95, the workability of concrete is

a. very low

b. low

c. high

d. none of these.

22. Curing a concrete for long period ensures better

a. strength

b. water resistance

c. water tightness and durability

d. all the above.

23. An excess of flaky particles in concrete aggregates

a. increases the quantity of water and sand

b. affects the durability of concrete

c. more than 15% are not desirable

d. all the above.

24. Pick up the correct statement from the following:

a. Silty clayloam contains highest percentage of silt

b. Stiff boulder clay offers maximum shear strength

c. Soft chalk carries least safe load

d. All the above.

25. Pick up the correct statement from the following:

a. At low value of water content most soils tend to be stiff.

b. At high water content, the dry density decreases with an increase of water content.

c. An airless soil will have maximum possible value of dry density for the given water content

d. All the above.

26. For shear strength, triaxial shear test is suitable because

a. precise measurement of the pore pressure and volume change during the test is possible

b. stress distribution on the failure plane, is uniform

c. state of stress within the specimen during any stage of the test as well as at failure, is completely determined

d. all the above.

27. Cohesionless soil is

a. sand

b. silt

c. clay

d. clay and silt.

28. For determing the specific gravity of soil solids, using a pycnometer of 500 cc., the following data is available : Weight of dry empty pycnometer = 125 g Weight of dried soil and pycnometer = 500 g Weight of dried soil and distilled = 850 g water filled in pycnometer up to top The specific gravity of soil soilds, is

a. 2.0

b. 2.25

c. 2.50

d. 2.75

29. The quantity of seepage of water through soils is proportional to

a. coefficient of permeability of soil

b. total head loss through the soil

c. neither (a) nor (b)

d. both (a) and (b).

30. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

31. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

32. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

33. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

34. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

35. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

36. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

37. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

38. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

39. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

40. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

41. The shape of scales are

a. flat and triangular

b. flat and rectangular

c. flat and square

d. all of the above

42. Inking pen is used for drawing -- in ink

a. writing

b. curve

c. straight line

d. none of the above

43. Select the incorrect statement about orthographic projections :

a. the reference planes are also called the principle planes

b. the object must the situated in the first quadrant

c. the planes of projection and assumed to be transparent

d. projections are obtained by drawing perpendicular s from the object to the planes

44. The edge of the board on which T-square is made to slide is called

a. working edge

b. drawing line

c. border line

d. deader line

45. Pick up the correct statement regarding financial statement analysis from the following.

a. Final analysis always involves the use of various financial statements i.e., balance sheet and income statement.

b. The balance sheet is the summary of assests, liabilities and owner's equity of business at a point in time.

c. The income statement is the summary of revenues and expenses of a firm over a particular period of time.

d. All the above

46. The estimator for definitive estimates must be able :

a. and an all-around construction expert.

b. to read the plans and specifications to determine accurate quantities of permanent materials and installed Equipment.

c. to express the job material requirements in dimensions suitable for costing and construction supplies.

47. The ratio obtained by dividing 'quick assests' by current liabilities is called

a. Turnover ratio

b. Acid test ratio

c. Solvency ratio

d. None of these.

48. Engineer shall not disclose, without...............confidential information concerning the business affairs or technical processes of any present or former client or employer or public body on which the serve:

a. editing

b. getting paid

c. remuneration

d. consent

49. As per the NEC Act, 2055 the name of engineers

a. general, professional, and foreigner

b. special, local and international wngineer

c. civil, electrical and mechanical engineer.

d. computer, electronics and agriculture engineer

50. The general experience required for nomination of chairperson of NEC is:

a. 10 yrs

b. 15 yrs

c. 20 yrs

d. 25 yrs