***Engineer***

1. A simply supported beam which carries a uniformly distributed load has two equal overhangs. To have maximum B.M. produced in the beam least possible, the ratio of the length of the overhang to the total length of the beam, is

a. 0.207

b. 0.307

c. 0.407

d. 0.508

2. The ratio of crippling loads of a column having both the ends fixed to the column having both the ends hinged, is

a. 1

b. 2

c. 3

d. 4

3. A body is said to be in equilibrium if

a. it moves horizontally

b. it moves vertically

c. it rotates about its C.G.

d. none of these.

4. A joint of a frame is subjected to three tensile force P, Q and R equally inclined to each other. If P is 10 tonnes, the other forces will be

a. R + 10 tonnes and Q = zero

b. Q + R = 10 tonnes

c. Q - R = zero

d. Q and R each is equal to 10 tonnes.

5. In a three hinged arch, the bending moment will be zero

a. at right hinge only

b. at left hinge only

c. at both right and left hinges

d. at all the three hinges.

6. In horizontal angles, the error due to imperfect levelling of the plate bubble is

a. large when sights are nearly level

b. large for long sights

c. less for steeply inclined sights

d. large for steeply inclined sights.

7. A dumpy level is set up with its eye-piece vertically over a peg A. The height from the top of peg A to the centre of the eye-piece is 1.540 m and the reading on peg B is 0.705 m. The level is then setup over B. The height of the eye-piece above peg B is 1.490 m and a reading on A is 2.195 m. The difference in level between A and B is

a. 3.030 m

b. 0.770 m

c. 0.785 m

d. 1.770 m.

8. Angles to a given pivot station observed from a number of traverse stations when plotted, the lines to the pivot station intersect at a common point

a. angular measurements are correct and not the linear measurements

b. linear measurements are correct and not the angular measurements

c. angular and linear measurements are correct and not the plotting of traverse

d. angular and linear measurements and also plotting of the traverse are correct.

9. If i is the stadia distance, f is the focal length and d is the distance between the objective and vertical axis of the techeometer, the multiplying constant, is

a. f/i

b. i/f

c. (f + d)

d. f/d.

10. Pick up the correct statement from the following :

a. the tangent screw enables to give small movement under conditions of smooth and positive control

b. standing on the tripod is the levelling head or trib arch

c. the levelling screws are used to tilt the instrument so that its rotation axis is truly vertical

d. all the above.

11. The sag of 50 m tape weighing 4 kg under 5 kg tension is roughly

a. 0.053 m

b. 0.063 m

c. 0.073 m

d. 0.083 m

12. Number of subdivisions per metre length of a levelling staff is

a. 100

b. 200

c. 500

d. 1000

13. Stuco paints are suitable for

a. stone masonry

b. brick walls

c. both (a) and (b)

d. neither (a) nor (b).

14. Second class bricks

a. are of dark brown colour

b. produce a metallic sound when struck

c. are well burnt

d. are under burnt.

15. Oil varnish generally consists of

a. synthetic resin and spirit

b. oil, wax and resin

c. resin, oil and turpentine

d. spirit, oil and wax.

16. Whitworth compressed steel is obtained when molten steel is subjected to a pressure of

a. 9 kg/mm2

b. 13 kg/mm2

c. 15 kg/mm2

d. 10 kg/mm2.

17. Good quality stones must

a. be durable

b. be free from clay

c. resist action of acids

d. all the above.

18. Pick up the correct statement from the following:

a. The articles prepared from clay which is burnt at low temperature and cooled down slowly, are called earthen-ware

b. The articles prepared from refractory clays which as mixed with stone and crushed pottery, are called stone ware

c. The articles prepared from fine earthen ware which is white, thin and semi-transparent, are called procelain

d. All the above.

19. Pick up the incorrect statement from the following:

a. Concrete for which preliminary tests are conducted, is called controlled concrete

b. Bulking of sand depends upon the fineness of grains

c. Concrete mix 1 : 6 : 12, is used for mass concrete in piers.

d. All the above.

20. The maximum thickness of concrete floor of a cement warehouse, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

21. Pick up the incorrect statement from the following:

a. Admixtures accelerate hydration

b. Admixtures make concrete water proof

c. Admixtures make concrete acid proof

d. Admixtures give high strength

22. The impurity of mixing water which affects the setting time and strength of concrete, is

a. sodium chlorides

b. sodium carbonates and bicarbonates

c. calcium chorides

d. calcium bicarbonates

23. Non-uniform compaction may cause the concrete

a. porous

b. non-homogeneous

c. reduced strength

d. all the above.

24. Under-reamed piles are generally

a. driven piles

b. bored piles

c. precast piles

d. all the above.

25. Pick up the correct statement from the following:

a. A soil having pH value more than 7 is an acidic soil

b. A soil having pH value less than 7 is an acidic soil

c. A soil having pH value more than 7 is an alkaline soil

d. A soil containing chemicals for the manufacture of portland cement is preferred.

26. When drainage is permitted under initially applied normal stress only and full primarily consolidation is allowed to take place, the test is known as

a. quick test

b. drained test

c. consolidated undrained test

d. none of these.

27. For shear strength, triaxial shear test is suitable because

a. precise measurement of the pore pressure and volume change during the test is possible

b. stress distribution on the failure plane, is uniform

c. state of stress within the specimen during any stage of the test as well as at failure, is completely determined

d. all the above.

28. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

29. The angle of internal friction, is least for

a. angular -grained dense sand

b. round-grained loose sand

c. round-grained loose sand

d. clays.

30. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

31. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

32. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

33. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

34. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

35. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

36. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

37. The expected out turn of brick work in cement mortar in foundation and plinth per mason per day, is

a. 1.00 m3

b. 1.25 m3

c. 1.50 m3

d. 1.75 m3

38. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

39. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

40. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

41. In full sectional view, the object is imagined to be cut off

a. one third

b. one half

c. one fourth

d. one fifth

42. 9H pencil is ......than the 7H pencil

a. softer

b. harder

c. medium

d. none of the above

43. Perspective drawing is done for

a. not for civil engineers

b. show details of market products

c. both (a) and (b) of above

d. none of the above

44. Ground plane is

a. horizontal plane

b. vertical plane

c. inclined plane

d. none of the above

45. The ratio of current assests to current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

46. Current assests less inventories divided by current liabilities is known as

a. Liquidity ratio

b. Current ratio

c. Acid-Test (or Quick) ratio

d. Debts ratio

47. If a is the base amount expenditure, b is the increase in the operation cost each year over a period of n years, the total cost of maintenance is :

a. a + (n + 1) b

b. a + (n - 1) b

c. a x (n - 1) b

d. a - (n - 1) b

48. As per the Public Procurement Regulation of Nepal, 2064, What is the maximum consulting services that can be procured directly by a public agency?

a. Rs. 200,000/-

b. Rs. 300,000/-

c. Rs. 500,000/-

d. none of the above

49. Any Non-Nepali Engineer desiring to work in Nepal may apply for registration under:

a. Category A

b. Category B

c. Category C

d. Category D

50. Which one is not concerned with NEC

a. acredition of academic certificate

b. authority of opening engineering college.

c. appointment of teacher

d. all of the above