***Engineer***

1. A composite beam is composed of two equal strips one of brass and other of steel. If the temperature is raised

a. brass experiences compressive force

b. composite beam gets subjected to a couple

c. composite beam bends

d. All the above.

2. The locus of the moment of inertia about inclined axes to the principal axis, is

a. straight line

b. parabola

c. circle

d. ellipse.

3. Keeping the depth d constant, the width of a cantilever of length l of uniform strength loaded with a uniformly distributed load w varies from zero at the free end and

a.  at the fixed end

b.  at the fixed end

c.  at the fixed end

d.  at the fixed end

4. Pick up the correct statement from the following :

a. The rate of change of bending moment is equal to rate of shear force

b. The rate of change of shear force is equal to rate of loading

c. neither (a) nor (b)

d. both (a) and (b).

5. The areas of cross-section of a square beam and a circular beam subjected to equal bending moments, are same.

a. square beam is more economical

b. both the beams are equally strong

c. both the beams are equally economical

d. none of these.

6. Probable accidental error in precise levelling as recommended by International Geodetic Association, should not exceed (where k is in kilometers.)

a. ± 0.5 k mm

b. ± 1 k mm

c. ± 2 k mm

d. ± 5 k mm.

7. Pick up the correct statement from the following :

a. Cross staff is used for setting out right angles

b. Gradiometer is used for setting out any required gradient

c. Line ranger is used for locating intermediate stations on a survey line

d. All the above.

8. The best method of interpolation of contours, is by

a. estimation

b. graphical means

c. computation

d. all of these.

9. The instrument which is used in plane tabling for obtaining horizontal and vertical distances directly without resorting to chaining, is known as

a. Plane alidade

b. telescopic alidade

c. clinometer

d. tacheometer.

10. If α is the angle between the polar ray and the tangent at the point of commencement of a lemniscate curve, the equation of the curve, is

a. l = ksin 2α

b. l = ksin 3α

c. l = ktan 2α

d. l = kcos 2α

11. The back staff reading on a B.M. of R.L. 500.000 m is 2.685 m. If foresight reading on a point is 1.345 m, the reduced level of the point, is

a. 501.345 m

b. 501.340 m

c. 504.030 m

d. 502.585 m.

12. Chain surveying is well adopted for

a. small areas in open ground

b. small areas with crowded details

c. large areas with simple details

d. large areas with difficult details.

13. Portland cement manufactured from pure white chalk and clay but free from iron-oxide, is known as

a. quick setting cement

b. rapid hardening cement

c. white cement

d. low heat Portland cement.

14. Good quality stones must

a. be durable

b. be free from clay

c. resist action of acids

d. all the above.

15. Cast steel is manufactured by

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

16. The commonly used raw material in the manufacture of cement, is

a. slate

b. sand stone

c. lime stone

d. basalt.

17. Stones used for ornamental work must be

a. soft

b. hard

c. light

d. heavy.

18. Pick up the correct statement from the following:

a. The bulking of sand is taken into account while volumetric proportioning of the aggregates

b. The dry sand and the sand completely flooded with water, have practically the same volume

c. The expansion and contraction joints are provided if concrete structures exceed 12 m in length

d. All the above.

19. Le-Chatelier's apparatus is used for testing

a. soundness of cement

b. hardness of cement

c. strength of cement

d. durability of cement.

20. The entrained air in concrete

a. increases workability

b. decreases workability

c. decreases resistance to weathering

d. increases strength

21. The high strength of rapid hardening cement at early stage, is due to its

a. finer grinding

b. burning at high temperature

c. increased lime cement

d. higher content of tricalcium.

22. The size of fine aggregates does not exceed

a. 3.00 mm

b. 3.75 mm

c. 4.75 mm

d. 5.75 mm

23. Horizontal construction joints in concrete walls are generally provided at

a. soffit level

b. window sill level

c. floor level

d. all the above.

24. 'Talus' is the soil transported by

a. wind

b. water

c. glacier

d. gravitational force.

25. The earth pressure of a soil at rest, is proportional to (where φ is the angle of internal friction of the soil)

a. tan (45° + φ)

b. tan2 (45° - φ)

c. tan2 (45° + φ)

d. none of these

26. Pick up the correct statement from the following:

a. At low value of water content most soils tend to be stiff.

b. At high water content, the dry density decreases with an increase of water content.

c. An airless soil will have maximum possible value of dry density for the given water content

d. All the above.

27. The shear strength of a soil

a. is proportional to the cohesion of the soil

b. is generally known as the strength of the soil

c. is proportional to the tangent of the angle of internal friction

d. all the above.

28. The zero atmospheric pressure is at

a. mean sea level

b. water table

c. phreatic surface

d. both (c) and (d) of the above.

29. Geologic cycle for the formation of soil, is

a. Weathering → upheaval → transportation → deposition

b. Transportation → upheaval → weathering → deposition

c. Weathering → transportation → deposition → upheaval

d. None of these.

30. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

31. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

32. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

33. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

34. Pick up the correct statement from the following with regards to C.P.M. network analysis of projects

a. Earliest occurrence time of the event from which the activity arrow originates plus the duration of the activity, is called earliest finish time of the activity

b. The latest occurrence time of the node of which the activity arrow terminates minus the duration of the activity, is called latest start time

c. The latest occurrence time for the node at which the activity arrow terminates, is called latest finish time

d. All the above.

35. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

36. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

37. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

38. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

39. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

40. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

41. Mechanical lines are types of

a. mechanic drawing

b. line of drawn with free hand

c. line drawn with drawing equipment

d. none of the above

42. Size of A4 size paper is

a. 148 x 210 mm

b. 210 x 297 mm

c. 297 x 210 mm

d. 320 X 460 mm

43. In orthographic projection which one have all three views same

a. cube

b. circle

c. sphere

d. both (a) and (c) of above

44. In isometric view a circle is seen as :

a. a parabola

b. a hyperbola

c. a cone

d. an ellipse

45. The financial analysis :

a. helps a share holder to compare the expected return on his investment in the firm against the expected return from other alternative investment.

b. helps a bank to know the financial position of the firm for granting a loan to the firm.

c. helps to judge the success of the firm's financial plans.

d. All of these.

46. Which one of the following definitions, is correct ?

a. The ratio of total debt to share holder's equity is called 'debt ratio'.

b. The ratio debt-to-total assests is called Debt-to-total assest ratio.

c. The ratio of earnings before interest and taxes for a particular reporting period to the amount of interest charges for the period, is called interest coverage ratio.

d. All of these

47. Pick up the correct statement from the following:

a. An annuity is a series of equal payments occurring at equal period of time.

b. Annuity is called an equal payment or uniform payment series.

c. An annuity may have periods of time of any length but should always be of equal length.

d. All the above.

48. Engineering shall act in such a manner as to uphold and enhance the honor, and dignity of the engineering profession and shall act with ..... for bribery, fraud, and corruption.

a. a tolerance

b. a penchant

c. zero tolerance

d. a predilection

49. Which one type of membership is not categorized by NEC

a. general engineer

b. foreign engineer

c. professional engineer

d. supreintendent engineer

50. For the fulfillment of his/her professional duties Engineer should:

a. Paramount the health of the public

b. paramount the safety of the public .

c. welfare of the public

d. all of the above