***Engineer***

1. A spring of mean radius 40 mm contains 8 action coils of steel (N = 80000 N/mm2), 4 mm in diameter. The clearance between the coils being 1 mm when unloaded, the minimum compressive load to remove the clearance, is

a. 25 N

b. 30 N

c. 35 N

d. 40 N

2. The horizontal thrust on the ends of a two hinged semicircular arch of radius R carrying

a. a uniformly distributed load ω per unit run over its right half span, is

b. a uniformly distributed load ω per unit run over its entire span is

c. a distributed load varying from zero at the left end to ω per unit horizontal run at the right end, is

d. all the above.

3. To determine the force in BD of the truss shown in the given figure a section is passed through BD, CD and CE, and the moments are taken about

a. A joint

b. B joint

c. C joint

d. D joint.

4. The ratio of the moment of inertia of a circular plate and that of a square plate for equal depth, is

a. equal to one

b. more than one

c. equal to 3π/16

d. none of these.

5. In a solid arch, shear force acts

a. along the axis of the arch

b. perpendicular to the axis of arch

c. tangentially to the arch

d. none of these.

6. Keeping the instrument height as 1.5 m, length of staff 4 m, the slope of the ground as 1 in 10, the sight on the down-slope, must be less than

a. 30 m

b. 25 m

c. 20 m

d. 15 m

7. Pick up the correct statement from the following :

a. Cross staff is used for setting out right angles

b. Gradiometer is used for setting out any required gradient

c. Line ranger is used for locating intermediate stations on a survey line

d. All the above.

8. The method of reversal

a. is usually directed to examine whether a certain part is truly parallel or perpendicular to another

b. makes the erroneous relationship between parts evident

c. both (a) and (b)

d. neither (a) nor (b).

9. Ranging in chain survey means

a. establishing an intermediate point on the line

b. determining the distance between end points

c. determining the offset distance

d. none of these

10. The bearings of two traverse legs AB and BC are N52° 45' E and N34° 30' E respectively. The deflection angle is

a. 18° 15' N

b. 18° 15' W

c. 18° 15' R

d. 18° 15' L

11. The reduced level of a floor is 99.995 m, the staff reading on the floor is 1.505 m. If the inverted staff reading against the roof is 1.795 m, the floor level below the slab, is

a. 3.290 m

b. 3.300 m

c. 3.275 m

d. 2.790 m

12. True meridian of different places

a. converge from the south pole to the north pole

b. converge from the north pole to the south pole

c. converge from the equator to the poles

d. run parallel to each other.

13. The minimum compressive strength of 1st class bricks should be

a. 90 kg/cm2

b. 100 kg/cm2

c. 120 kg/cm2

d. 130 kg/cm2.

14. The initial setting time of hydraulic lime, is

a. 60 minutes

b. 90 minutes

c. 120 minutes

d. 150 minutes.

15. Pick up the plutonic rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

16. Bitumen felt

a. is used as water proofing material

b. is used as damp proofing material

c. is made from bitumen and hessian fibres

d. all the above.

17. Pick up the correct statement regarding low heat cement from the following:

a. Its initial setting time is about one hour

b. Its final setting time is about 10 hours

c. Its mainly used for mass concrete work

d. All the above.

18. Pick up the rock which is not  a sedimentary rock from the following:

a. sand stone

b. gypsum

c. dolerite

d. lignite.

19. C.R.R.I. charts are used to obtain a relatioship between strength of concrete and

a. water cement ratio

b. workability

c. grading of aggregate

d. fineness modulus

20. High temperature

a. increases the strength of concrete

b. decreases the strength of concrete

c. has no effect on the strength of concrete

d. none of these.

21. The cement whose strength is a little lower than the ordinary cement during the first three months but attains afterwards the same strength, is known as

a. low-heat Portland cement

b. rapid hardening Portland cement

c. Portland blast slag cement

d. Portland pozzolana cement

22. A flaky aggregate is said to be elongated if its length is

a. twice the mean size

b. thrice the mean size

c. four times the mean size

d. five times the mean size.

23. Pick up the correct statement from the following:

a. Water cement paste hardens due to hydration

b. During hardening cement binds the aggregates together

c. Cement provides strength, durability and water tightness to the concrete

d. All the above.

24. A pile is being driven with a drop hammer weighing 1800 kg and having a free fall of 1.00 m. If the penetration with last blow is 5 mm, the load carrying capacity of the pile, -according to the Engineering News formula, is

a. 100 tonnes

b. 50 tonnes

c. 20 tonnes

d. 10 tonnes.

25. The method of the slices is applicable to

a. homogenous soils

b. stratified soils

c. saturated soils

d. non-uniform slopes

26. A partially saturated sample of soil has a unit weight of 2.0 g/cm3 and specific gravity of soil particles is 2.6. If the moisture content in the soil is 20%, the degree of saturation is

a. 20%

b. 77%

c. 92%

d. none of these.

27. Negative skin friction on piles

a. is caused due to relative settlement of the soil

b. is caused in soft clays

c. decreases the pile capacity

d. all of the above.

28. For determining the ultimate bearing capacity of soil, the recommended size of a square bearing plate to be used in load plate test should be 30 to 75 cm square with a minimum thickness of

a. 10 mm

b. 15 mm

c. 20 mm

d. 25 mm

29. The pressure that builds up in pore water due to load increment on the soil, is termed

a. excess pore pressure

b. excess hydrostatic pressure

c. hydrodynamic pressure

d. all the above.

30. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

31. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

32. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

33. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

34. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

35. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

36. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

37. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

38. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

39. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

40. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

41. The length of the rectangle representing the development of the lateral surface of a right circular cylinder is equal to :

a. circumference of the circular base

b. twice the circumference of circular base

c. diameter of the circular base

d. radius of the circular base

42. The art of representation of an object by systemic lines on a paper is known as

a. drawing

b. art

c. dimensioning

d. profession

43. The section obtained when the section plane is inclined to the axis of cone, but parallel to one of the generators of a cone is called ......... ......

a. an ellipse

b. a parabola

c. a hyperbola

d. a circle

44. Working edge of drawing board is kept ........... of the user.

a. left

b. right

c. ahead

d. all of the above

45. Pick up the correct statement from the following:

a. The assumptions that all cash flows occur at the end of the interest period, is known as the end of period convention.

b. A cash flow diagram is a graphical representation of cash flows drawn on a time scale.

c. The cash flow diagram represents the statement of the problem and also includes what is given and what is to be found.

d. All of the above.

46. The estimator for definitive estimates must be able :

a. and an all-around construction expert.

b. to read the plans and specifications to determine accurate quantities of permanent materials and installed Equipment.

c. to express the job material requirements in dimensions suitable for costing and construction supplies.

47. In a cash flow series :

a. uniform gradient signifies that an income or disbursement changes by the same amount in each interest period.

b. Either an increase or a decrease in the amount of a cash flow is called the gradient.

c. The gradient in the cash flow may be positive or negative.

d. All of these

48. What is the abbreviation used for German Technical Co-operation?

a. GTZ

b. GTC

c. GTCO

d. all of the above

49. The sequence of priority in making ethical decision is (from top to bottom)

a. society, profession organization, individual

b. profession, organization, individual, society.

c. individual, organization, profession, society

d. profession, individual, organization society

50. Maximum working hour for a labour per week should be

a. 42hr

b. 45hr

c. 48hr

d. 53hr